| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Some consideration for the new MAP-IEs for M2M group in 802.16.1b |  |
| Date Submitted | 2011-11-01 |  |
| Source(s) | Rui Huang, Honggang Li, Shantidev Mohanty Intel Corporation | E-mail: rui.huang@intel.com |
| Re: | IEEE 802.16-11/0028 for IEEE 802.16 Letter Ballot #34 on P802.16p/11-0033 |  |
| Abstract | In this contribution we will provide further considerations on the necessity of new MAP IE for M2M group in 802.16.1b. |  |
| Purpose | For review and adoption into P802.16p/11-0033. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Some consideration for the new MAP-IEs for M2M group in 802.16.1b

Rui Huang, Honggang Li, Shantidev Mohanty

Intel Corporation

# Introduction

One of MGID open issues in the latest Adhoc discussion is the necessity of the new MAP-IE for M2M group. However, there is not any consensus reached. Therefore, in this contribution we will provide some further considerations about this.

# MAP IE for M2M Group

In [1] the potential M2M group control signaling includes:

- Group control signaling for M2M groups resource allocation and power control

- DL multicast service for M2M groups

- Group paging for M2M groups

In the following subsections, we will summarize the potential solutions to support these signaling respectively.

## Group control signaling

Basically M2M group control signaling includes resource allocation and power control information for the M2M group specified by MGID. Since all scheduled M2M members could share part of control messages, the overall control signaling overhead will be reduced greatly [1].

Therefore, M2M group control signaling is so efficient that should be supported in 16p. Table 1 summarizes several solutions to support this mechanism.

Table 1. Solutions to support M2M group control signaling

| Solutions | Pros | Cons |
| --- | --- | --- |
| Assignment AMAP IE mask by MGID | compatible to 16m: no any additional AMAP IE needed | Need to define MGID with consist length with STID |
| Assignment AMAP IE mask by STID | compatible to 16m: no any additional AMAP IE needed | MGID could not map with STID uniquely. |
| MAC Message | MGID could be embedded in MAC Header Not any constraints for MGID | UE need complete decoding the traffic packets blindly even these not for him. |

## DL multicast services

Multicast service provides an efficient way to transmit some common DL data to a group of M2M devices also. MGID in this using scenario should be served as the multicast group ID in 16m and 16e.

Table 2 summarizes several solutions to support DL multicast services for M2M group.

Table 2. Solutions to support M2M group DL multicast

| Alternatives | Pros | Cons |
| --- | --- | --- |
| MBS AMAP IE with MGID | compatible to 16m | Need to define MGID with consist length with EMBS ID |
| MBS AMAP IE with STID | compatible to 16m | - MGID could not map with STID uniquely. - Multicast EMBS ID could valid in both Idle and Connect mode. But STID could not. |

## Group paging

For this kind of usage, the solution based the broadcasting paged group ID should be only optimized solution which has been adopted by 16p.

# Conclusion

Regarding to the minimize the stand impaction, we can conclude that the optimum solution to support the group control signaling for M2M group is reuse current AMAP-IE with consistent MGID length with STID.

# Text Proposal

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

Proposal #1 : Add the following sections in P802.16p/11-0033

6.2.1.3.1 M2M Group Identifier (MGID)

A 12-bit value that uniquely identifies an M2M group which one or more M2M devices belong to.

---------------------------End Text Proposal----------------------------------

# References

- IEEE C802.16p-11/0152

- P802.16p/11-0033