| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M network re-entry combined with bandwidth request in 16m |  |
| Date Submitted | 2011-10-28 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel Jaesun Cha ETRI Hyunjeong Kang Samsung Yi-Ting Lin Tsung-Yu Tsai Institute for Information Industry (III) Heejeong Cho LGE | Email: Honggang.li@intel.com |
| Re: | IEEE 802.16-11/0028 for IEEE 802.16 Letter Ballot #34 on P802.16p/11-0033 |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M network re-entry combined with bandwidth request in 16m

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

Jaesun Cha

ETRI

Hyunjeong Kang

Samsung

Yi-Ting Lin, Tsung-Yu Tsai

Institute for Information Industry (III)

Heejeong Cho

LGE

# Introduction

The network re-entry process is used for MS back to the connected mode from the idle mode, after network re-entry, the MS can request the transport channel for data transmission. For M2M application, small packet transmission is very usual and upon sending it out, the device will back to sleep state immediately to save power, so the normal process of network re-entry and bandwidth request is no longer an efficient way.

A network reentry procedure combined with bandwidth request is proposed to optimize the network reentry and bandwidth request process for M2M short packet transmission, by embedding the bandwidth request in the AAI-RNG-REQ message.

# Proposal

In the optimized procedure as shown in fig 1, the M2M device can carry the bandwidth request in AAI-RNG-REQ message and the BS can acknowledge if the bandwidth request is accepted in AAI-RNG-RSP message and if accepted, allocate the channel in UL AMAP message according to the request from the M2M device, which can save the two random access procedures (one for network reentry and one for bandwidth request) to one procedure. The saved random channel resource can be used for more contention-based channel access by M2M devices.

Fig 1, M2M network reentry with bandwidth request

# Proposed texts

Adopt the following remedies in IEEE 802.16p-11/0033.

: Added texts marked in blue font with underline and removed texts marked in red font with strikeout

----------------------------------------------  Text Start -----------------------------------------------

[Remedy 1: Add the following text after line 30 in page 37.]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

…

During network reentry, the M2M device may request UL BW grant without a contention-based bandwidth request by including Bandwidth Request Indicator Bandwidth Request Size in an AAI-RNG-REQ message. If an ABS receives the AAI-RNG-REQ message with Bandwidth Request Indicator set to 1 Bandwidth Request Size, the ABS may allocate an UL bandwidth based on the request Bandwidth Request Size if the ABS possesses the M2M device information, without a contention-based bandwidth request from the M2M device by setting the Unsolicited bandwidth grant indicator in an AAI-RNG-RSP message to the M2M device. If the Unsolicited bandwidth grant indicator is enabled, the ABS should allocate UL bandwidth within the BR grant time duration after sending the AAI-RNG-RSP message.

[Remedy 2: modify table 678 in page 6.]

Table 684—AAI-RNG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| Bandwidth Request Indicator. | 1 | 1: indicates BW grant is required for transmission of BR header after completion of network reentry. | Optional |
| Bandwidth Request Size | 19 | Amount of bandwidth requested in bytes | Optional |
| ... | ... | ... |  |

[Remedy 3: Modify table 685 in page 9.]

Table 685—AAI-RNG-RSP message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ... | ... | ... |  |
| Unsolicited bandwidth grant indicator. | 1 | 1: indicates that an unsolicited UL allocation will be provided without request from AMS during network entry or network re-entry.. 0b0: accept bandwidth request, and the AMS doesn’t need to do bandwidth request after network re-entry 0b1: reject bandwith request, and the AMS needs to do bandwidth request after network re-entry | Shall be included when AMS is attempting network entry Shall be included if AAI-RNG-RSP message is transmitted in response to AAI-RNG-REQ message that includes bandwidth request indicator during network reentry from idle mode.. |
| ... | ... | ... |  |

----------------------------------------------  Text End -----------------------------------------------