| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Partial M2M group network reentry |  |
| Date Submitted | 2011-10-28 |  |
| Source(s) | Honggang Li Rui Huang Shantidev Mohanty Intel Lei Zhou Samsung Jinsoo Choi Giwon Park LGE Yi-Ting Lin III Ming-Hung Tao ITRI Jaesun Cha ETRI | Email: Honggang.li@intel.com |
| Re: | IEEE 802.16-11/0028 for IEEE 802.16 Letter Ballot #34 on P802.16p/11-0033 |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Partial M2M group network reentry

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

Lei Zhou

Samsung

Jinsoo Choi, Giwon Park

LGE

Yi-Ting Lin

III

Ming-Hung Tao

ITRI

Jaesun Cha

ETRI

# Proposal

To support group-based network reentry of a fixed M2M device, the BS can page the group of M2M devices to do the network reentry. If the number of devices in one group is too large to page, the BS can page part of group members in one paging message as proposed in this contribution, by setting the parameter “group access probability” within the MGID loop in paging message.

Fig 1, message flow for partial M2M group network reentry

# Texts Proposal

Adopt the following remedies in IEEE 802.16p-11/0033.

: Added texts marked in blue font with underline and removed texts marked in red font with strikeout

----------------------------------------------  Text Start -----------------------------------------------

[Remedy 1: Modify Table706 in page 17 as follows]

6.2.3.23  AAI-PAG-ADV (paging advertisement) Message

Table 706—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For (j=0; j<Num_MGID; j++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00: Performing network reentry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: MGID re-assignment |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M is supported |
| If(Action code == 0b00){ |  |  |  |
| Initial ranging backoff start. | 4 | Indicate the initial backoff window size for M2M devices |  |
| M2M network access type | 2 | Indicate the network access scheme for M2M device 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ, This type is only applicable to fixed M2M device (i.e., Localized_Idle_Mode flag was set to 1 at the idle mode initiation). Except fixed M2M device, mobile M2M device shall perform the contention-based ranging. 0b01: dedicated ranging channel allocation, S-RCH 0b10: dedicated ranging channel allocation, NS-RCH 0b11: No dedicated ranging channel |  |
| If ( M2M network re-entry type==0b01 or 0b10){ |  |  |  |
| Group paging change count | 2 | The value is increased whenever the whole M2M group is paged in a new round, the value rolls over from 0 to 3 |  |
| Group access probability | 2 | 0b00:25% 0b01:50% 0b10:100% 0b11: Reserved |  |
| M2M ranging opportunity subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity dedicated for M2M devices. |  |
| Periodicy of the M2M ranging | 3 | Indicates the periodicy of the ranging dedicated for M2M devices. 0b000: transmission in every frame 0b001: transmission in the first frame in every superframe 0b010: transmission in the first frame in every even numbered superframe, i.e., mod(superframe number, 2) = 0 0b011: transmission in the first frame in every 4th superframe, i.e., mod(superframe number, 4) = 0 [0b100~0b111: reserved] |  |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

[Remedy 2: Modify subclause 6.2.18.7.2 as indicated (page 36, line 16):]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

…

If fixed M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and the M2M network access type is set to 0b01 or 0b10, the M2M device will decide if it will do the network reentry in the dedicated ranging channel by comparing a generated random numner~U(0,1) with the parameter “Group Access Probability”, if the generated random number is bigger than the “Group Access Probability”, the M2M device will do the network reentry. Group Paging Change Count is used for indicating if there is a new round of group paging for one M2M group. If it is a new round of group paging for one M2M group, all group member can do the network reentry if needed.

----------------------------------------------  Text End -----------------------------------------------