| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Resource allocation for contention free network reentry |  |
| Date Submitted | 2011-10-28 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel | Email: Honggang.li@intel.com |
| Re: | IEEE 802.16-11/0028 for IEEE 802.16 Letter Ballot #34 on P802.16p/11-0033 |  |
| Abstract |  |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Resource allocation for contention free network reentry

Hongang Li, Rui Huang, Shantidev Mohanty

Intel

# Proposal

In C80216p-11_0227, a new “Fixed M2M Ranging Assignment A-MAP IE” was proposed to allocate the channel for AAI-RNG-REQ message. To minimize the change to current standard, instead of the proposal in C80216p-11_0227, we propose reusing CDMA Allocation A-MAP IE for the channel allocation to AAI-RNG-REQ message for contention free network reentry.

Fig 1, contention-free network reentry

To identify the receiver, there is 16-bit CRC mask for each CDMA Allocation A-MAP IE, the CRC mask includes 16-bit RA-ID, which is made up of LSB 5-bit superframe index, 2-bit frame index, 6-bit preamble code index, and 2-bit opportunity index.

In the contention-free network reentry, the RA-ID can be generated by the ABS and specified in the paging message for the M2M device to detect the CDMA Allocation A-MAP IE.

To generate a RA-ID without the possibility of collision with other contention or contention-free attempts, the ABS can generate the RA-ID by selecting a future 5-bit superframe index which is not used by any contention-based attempt in the past and now, and a unique 6-bit preamble code index to a contention-free attempt, to avoid the collision with contention-free attempts.

Of course, the 2-bit frame index and 2-bit opportunity index can be also used to differentiate a unique contention-free attempt in some other way, combined with the superframe index and preamble code index.

# Texts Proposal

Adopt the following remedies in IEEE 802.16p-11/0033.

: Added texts marked in blue font with underline and removed texts marked in red font with strikeout

----------------------------------------------  Text Start -----------------------------------------------

[Remedy 1: Modify Table706 in page 17 as follows]

6.2.3.23  AAI-PAG-ADV (paging advertisement) Message

Table 706—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| For (j=0; j<Num_AMSs; j++) { |  |  |  |
| … |  |  |  |
| If(Action code == 0b0){ |  |  |  |
| Initial ranging backoff start | 4 | Indicate the initial backoff window size for M2M devices |  |
| } |  |  |  |
| M2M network access type | 2 | Indicate the network access scheme for M2M device 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ 0b01: dedicated ranging channel allocation in AAI-PAG-ADV 0b10: dedicated ranging channel allocation in broadcast assignment A-MAP IE 0b11: No dedicated ranging channel |  |
| If( M2M network re-entry type==0b00){ | - |  |  |
| Assignment A-MAP offset for AAI-RNG-REQ |  | This parameter indicates the offset that M2M device start to monitor the resource (i.e., Assignment A-MAP IE) for the AAI-RNG-REQ message. |  |
| RA-ID for contention free network reentry | 15 | TBD |  |
| } |  |  |  |
| … |  |  |  |
| For (j=0; j<Num_MGID; j++) { |  | Num_MGID indicates the number of MGIDs included in this paging message [0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| ... |  |  |  |
| If(Action code == 0b00){ |  |  |  |
| Initial ranging backoff start. | 4 | Indicate the initial backoff window size for M2M devices |  |
| M2M network access type | 2 | Indicate the network access scheme for M2M device 0b00: Resource allocation (i.e., Assignment A-MAP offset) for AAI-RNG-REQ, This type is only applicable to fixed M2M device (i.e., Localized_Idle_Mode flag was set to 1 at the idle mode initiation). Except fixed M2M device, mobile M2M device shall perform the contention-based ranging. 0b01: dedicated ranging channel allocation, S-RCH 0b10: dedicated ranging channel allocation, NS-RCH 0b11: No dedicated ranging channel |  |
| … |  |  |  |
| If( M2M network re-entry type==0b00){ | - |  |  |
| Assignment A-MAP start offset for AAI-RNG-REQ. |  | This parameter indicates the offset that M2M device start to monitor the resource (i.e., Assignment A-MAP IE) for the AAI-RNG-REQ message.. |  |
| RA-ID for contention free network reentry | 15 | TBD |  |
| Resource monitor timer. |  | Time duration that M2M device monitors the resource (i.e., Assignment A-MAP IE) for AAI-RNG-REQ message.. |  |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

[Remedy 2: Modify subclause 6.2.18.7.2 as indicated (page 35, line 62):]

6.2.18.7.2 Network re-entry from idle mode for M2M devices

…

When the M2M device receives the group paging message (i.e., AAI-PAG-ADV with MGID) and M2M network access type is set to 0b00 (i.e., Resource allocation for AAI-RNG-REQ), it starts to monitor the CDMA Allocation A-MAP IE Assignment A-MAP IE with a specified RA-ID to obtain the resource for AAI-RNG-REQ message at 'Assignment A-MAP start offset for AAI-RNG-REQ' during the 'Resource monitor timer'. If the M2M device does not decode the CDMA Allocation A-MAP IE Assignment A-MAP IE until the expiration of the 'Resource monitor timer', it performs contention-based ranging.

…

[Remedy 3: Modify subclause 6.2.18.7.2 as indicated (page 36, line 22):]

When the M2M device receives the individual paging message (i.e., AAI-PAG-ADV with FMDID) and M2M network access type is 0b00 (i.e., Resource allocation for AAI-RNG-REQ), it decodes the CDMA Allocation A-MAP IE Assignment A-MAP IE with a specified RA-ID to obtain the resource of AAI-RNG-REQ message at 'Assignment A-MAP offset for AAI-RNG-REQ'.

----------------------------------------------  Text End -----------------------------------------------