| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Abnormal Power Down Indication Procedure Using Abnormal Power Down Header |  |
| Date Submitted | 2011-11-8 |  |
| Source(s) | Tsung-Yu Tsai, Yi-Ting Lin Institute for Information Industry (III) | E-mail: tyt@iii.org.tw |
| Re: | Change Request on Pre-Draft IEEE P802.16p-11/0033 |  |
| Abstract | This contribution proposes method to signal Abnormal power down indication |  |
| Purpose | To be discussed and adopted by Machine-to-Machine(M2M) Task Group for IEEE P802.16p-11/0033 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Abnormal Power Down Indication Procedure Using Abnormal Power Down Header

Tsung-Yu Tsai, Yi-Ting Lin

Institute for Information Industry (III)

# Introduction

In 6.2.29.1 of the current draft [1], there are two possible way for an M2M device in connected state to signal the abnormal power down indication to the BS when it does not have available bandwidth.

- Using predefined BR index with 3 step BR procedure

- Perform 5-step BR procedure as defined in 6.3.6 to request bandwidth, and then sends AAI-RNG-REQ message with Ranging Purpose Indication set to 0b1110 to indicate the occurring of abnormal power down event upon the UL bandwidth is granted. This procedure is depicted in the left of Fig. 1.

In this contribution, we propose an efficient (with less steps) mechanism for abnormal power down indication as an alternative of the existing methods which performs 5-step BR procedure and sends AAI-RNG-REQ message. In the proposed mechanism, upon the M2M device receives the UL bandwidth allocation for standalone BR header (i.e. the third step of the Fig. 1), the M2M device sends an Abnormal Power Down Signaling header instead of sending BR header. The indication procedure can be finished upon the BS receives the Abnormal Power Down Signaling header, thus the total number of step needed is reduced to three. Moreover, the Abnormal Power Down Signaling header contains 18-bit MSB of Authorization Key(AK). It is for the confirmation purpose and avoids malicious users to send a fake Abnormal Power Down Signaling header and impact the M2M service. The right of Fig. 1 illustrates the entire procedure of the proposed mechanism.

Fig. 1 The Existing and Proposed Method

# Reference

[1] IEEE P802.16.1/D2, “IEEE Draft Standard for WirelessMAN-Advanced for Broadband Wireless Access Systems,” Oct. 2011.

[2] IEEE P802.16p-11/0033, “DRAFT Amendment to IEEE Standard for WirelessMAN-Advanced Air Interface for Broadband Wireless Access Systems – Enhancements to Support Machine-to-Machine Applications,” Oct. 2011.

# Proposed text with change marks

Remedy 1: Modify Page 42, lines 21-36 as follows

6.2.29.1 Abnormal Power Down Reporting in Connected State

If the MS is in connected state with uplink bandwidth already allocated and available, then it may use the available bandwidth to send an abnormal power down signaling header (as defined in 6.2.2.1.3.10) this or AAI-RNG-REQ message containing the Ranging Purpose Indication with value 0b1110.

If the MS is in connected state but does not have available UL bandwidth, then it may use the procedure defined in 6.3.6 to request bandwidth. Upon receiving bandwidth allocation for standalone BR header it may send the abnormal power down signaling header AAI-RNG-REQ message containing the Ranging Purpose Indication with value 0b1110.

If the MS is in connected state but does not have available UL bandwidth, then it may use the quick access procedure defined in 6.2.11 to report the abnormal power down event. The Predefined BR index may be used to indicate that an abnormal or involuntary power down has occurred.

Remedy 2: Modify Page 5, lines 62-63 as follows

| 01001 | M2M Abnormal Power Down Header |
| --- | --- |
| 0100001010-11111. | Reserved |

Remedy 3: Add the following after line 37 on Page 6

Add new subclause 6.2.2.1.3.10

6.2.2.1.3.10 M2M Abnormal Power Down header

When a M2M device detects an abnormal power down event, it sends M2M abnormal power down signaling header indicating that an abnormal or involuntary power down has occurred. M2M abnormal power down signaling header is defined in Table 670b.

Table 670b – M2M abnormal power down header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| M2M abnormal power down header() { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 |
| Type | 5 | MAC Signaling header type = 0b1000 |
| Length | 3 | Indicates the length of the signaling header in bytes |
| STID | 12 | STID of the M2M device that transmits emergency report signaling header |
| STID_Valid_Offset | 3 | STID_Valid_Offset of the M2M device that sends emergency report. If the assigned STID is not shared with other M2M devices, M2M device shall set this field to zero. |
| Emergency Type | 1 | 0b000: Power Outage 0b001: Reserved |
| Truncated AK | 18 | The 18 MSB of the Authorization Key (AK) maintained between the M2M device’s and the ABS. AK is derived according to 16.2.5.2.1.1.2. |
| } |  |  |