| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MGID Related Corrections |  |
| Date Submitted | 2011-11-10 |  |
| Source(s) | Erik Colban Huawei Anshuman Nigam | Voice: +1-858-775-2494 E-mail: ecolban@huawei.com erik.colban@drawmetry.com anshun@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #33, as announced in document IEEE 802.16-11/0027 |  |
| Abstract | This contribution proposes some clarifications and correction related to the use of M2M Groups and MGIDs |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MGID Corrections

Erik Colban

Huawei Technologies

Anshuman Nigam

Samsung

# Abstract

This contribution addresses text in P802.16p/D1 related to M2M groups and MGIDs. It proposes some additional specifications, clarifications and corrections related to this part of the draft.

# Introduction

The changes in this contribution reflect agreement from ad-hoc activities in M2M TG. The following was agreed to by the ad-hoc:

    - Create new space for M2MCID from the CID space

      - Applies only to 16p upgraded BSs

    - Use these M2MCID in Paging

    - Have non overlapping Zone concept

      - Zone ID is 15 bits

    - An M2M Device belongs to only one M2M Group Zone

    - Data via assignment MAP

    - Update M2MCID on Zone change

# Proposed Changes

## Change 1

Change text on page 4 as follows:

6.3.1 Addressing and connections

Insert the following texts at the end of subclause 6.3.1

One or more BS in an area of the network may be grouped into an M2M zone and identified by an M2M DEVICE GROUP ZONE ID. A BS may belong to at most one M2M group zone. A 15-bit M2M device group ID (MGID) uniquely identifies an M2M device group in the domain of the net-work entity that assigns MGID that one or more M2M devices belong to. The domain of the network entity is identified by M2M DEVICE GROUP ZONE ID. The BS may broadcast the M2M DEVICE GROUP ZONE ID of the zone to which it belongs may be broadcasted in the DCD message.

The M2M multicast connection ID (M2MCID) uniquely identifies a downlink multicast service flow shared by a group of M2M devices within an M2M zone. Implicitly, it is also used to identify the group of M2M devices that share the downlink multicast SF. An M2M device may share more than one downlink multicast SF each identified by an M2MCID. All M2MCIDs that are assigned to an M2M device belong to the same M2M group zone.

An The MGID M2MCID is assigned to a service flow of an M2M device by a network entity after initial network entry

through during the DSA procedure and released during the DSD procedure or an explicit network exit (e.g., power down location update). The assigned MGID M2MCID shall be retained by an M2M device even in idle mode unless the M2M device exits from the network or the network explicitly deletes the service flow associated with the MGID M2MCID. The MGID M2MCID can be re-assigned during nNormal Operation mode and idle mode. During nNormal Operation mode, the MGID M2MCID may be changed, and deleted by DSC, and DSD procedures respectively.

During the iIdle mMode, the MGID M2MCID may be changed by a location update procedure (i.e. BS-initiated location update) or during network reentry through the RNG-RSP message. When the BS updates the MGID through the BS-initiated location update, the BS can trigger the group location update as well as individual location update. When the BS changes the MGID of all M2M devices within the multicast group, tThe BS can trigger the group location update via a paging message. In Normal Operation, the BS can update the M2MCID for a M2M device group using the MAC Group Management Control Message (MGMC).

When  the  M2M  device  performs  the  timer  based  location update,  if  the  BS  needs  to  update  the  MGID M2MCID of  M2M device, the BS may send a RNG-RSP message with a new MGID M2MCID is sent by the BS in response to the RNG-REQ message.

A BS may use the MOB_PAG-ADV message to indicate the update of the MGID M2MCID and its new value to all the M2M devices in  a  group.  When  an  idle  mode  M2M  device  that  belongs  to  the  M2M  device  group  (indentified  by  its MGID M2MCID) receives a paging message directed to containing its an MGID M2MCID TLV identifying one of its service flows and the an Action Code TLV with value is set to 0b11, this M2M  device  shall  update  its  the MGID M2MCID based  on  the  value  indicated  by  MGID M2MCID Re-assignment  TLV  (see 11.17.5)

After receiving the updated MGID value, the M2M device shall send an acknowledgement (ACK) message to the BS. This ACK message is carried in the RNG-REQ message.

If the BS does not receive the an acknowledgement message from any of the some of the M2M devices belonging to that M2M device group which MGID has been updated, it assumes that those M2M devices missed the MGID update information. Iit may repeat the procedure in the next paging cycle of those M2M devices or it may send a RNG-RSP message containing the new MGID M2MCID to each of them. n the next paging cycle BS may ask those M2M devices to perform location update and

may send them a unicast message with the new MGID value (RNG-RSP).

The BS may use the M2M Group MAC Control (MGMC) message with the MGID M2MCID to send the same information to multiple M2M devices. The M2M device may respond to acknowledge this message with M2M

ACK MAC Control (MAMC) message.

## Change 1.3

On page 5, line 10, change 111 | MGMC | M2M Group MAC Control Message| TBD Broadcast

## Change 1.4

6.3.2.3.6 RNG-RSP (ranging response) message

Add the following text at the end of the subclause 6.3.2.3.6 as indicated

The following TLV parameter may be included in a RNG-RSP message to update MGID M2MCID in an M2M device.

MGID M2MCID Update

Bits 0-15: current MGID value

Bits 16-31: new MGID value

## Change 1.5

Change MGID to M2MCID globally

## Change 1.7

Add the following lines on page 9, line 54

The following TLV may be included when paging an M2M device:

M2M Paging Parameter (see 11.17.7)

The following TLV may be included when paging an M2M device:

Ranging backoff start (see 11.17.8)

## Change 3

On page 14, line 29, make the following changes:

6.3.34 Support of multicast operation for machine to machine application

A BS may provide a multicast service for group of M2M devices that share a downlink multicast service flow. The BS shall initiate the establishment of a service flow using the DSA procedures.  During the establishment of the SF, the SF is assigned an M2MCID, which uniquely identifies the SF. The M2M device shall retain these identifiers in Idle Mode (see 6.3.1). The BS shall provide the mapping between the SF and the M2MCID during the DSA signaling and may modify this mapping using the DSC procedures or by using M2MCID Update TLV during network re-entry.

Add new subclause 6.3.34.1

6.3.34.1 M2M multicast operation in idle mode

A BS may provide a multicast service for M2M devices in idle mode with or without requiring network

reentry of the M2M devices. Before a BS sends DL multicast data, the BS may shall transmit the paging message including the multicast traffic indication to the M2M devices during the paging listening intervals of the M2M devices. If an M2M device receives the paging message indicating multicast traffic reception without network reentry during its paging listening interval, the M2M device shall start receiving the DL multicast data

without the idle mode termination.

The Multicast transmission start time TLV may be included in the paging message in order to indicate when

the DL multicast data is sent by the BS. The value of Multicast transmission start time TLV shall be less

than the start time of the next paging listening interval of the M2M devices receiving the MOB_PAG-ADV

message. The M2M device may power down until the frame indicated by the Multicast transmission start

time TLV in the MOB_PAG-ADV message.

When the multicast data transmission ends, the BS shall notify the end of multicast data transmission to the

group  of  M2M  devices  by  sending  the  MOB_MTE-IND  message.  Upon  receiving  the  MOB_MTE-IND

message, the M2M devices may enter the paging unavailable interval as specified in 6.3.22.4.

In order to receive the M2M multicast data during idle mode M2M devices in idle mode shall use M2M

Multicast Traffic Reception timer if this timer is assigned during DSA procedure. If Multicast transmission

start  time  TLV  is  included  in  the  MOB_PAG-ADV  message  indicating  the  multicast  traffic  reception

(Action code = 0b10), M2M devices receiving the MOB_PAG-ADV message shall start the Multicast Traf-

fic  Reception  timer  at  the  frame  indicated  by  the  Multicast  transmission  start  time  TLV.  Otherwise,  the

M2M  device  shall  start  the  Multicast  Traffic  Reception  timer  when  the  M2M  device  receives  the

MOB_PAG-ADV. The M2M Multicast Traffic Reception timer shall be reset whenever the multicast data

are received. The M2M device shall stop the M2M Multicast Traffic Reception timer when the MOB_MTE-

IND message is received. If the M2M Multicast Traffic Reception timer expires, the M2M device shall enter

the paging unavailable interval as specified in 6.3.22.4.

## Change 3.5

Insert the following change on pge 22, line 50:

Make the following changes to Table 658:

| Transport, Secondary Man- agement, Tunnel or Manage- ment Tunnel, Multicast management CID | 2m+1–– n0xFE9F | For the secondary management connection, the same value is assigned to both the DL and UL connection. Tunnel CID is used for tunnel transport connections. Management Tun- nel CID is used for tunnel management connections. Multi- cast management CID is used for the downlink multicast management services. |
| --- | --- | --- |
| M2MCID | n+1 – 0xFE9F | M2M multicast connection identifiers |

## Change 3.6

Add the following text to page page23, line 64:

Insert the following row at the end of Table 678:

| M2M GROUP ZONE ID | ZZ | 2 | Bits 0-14: M2M group zone identifier Bit 15: Reserved | WirelessMAN OFDMA |
| --- | --- | --- | --- | --- |