| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Co-ordinated Group Paging in IEEE 802.16p |  |
| Date Submitted | 2011-11-06 |  |
| Source(s) | Honggang Li, Rui Huang Shantidev Mohanty Intel Corporation | Voice: +1-503-805-2567 E-mail: Shantidev.mohanty@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #33: Announcement |  |
| Abstract | This contribution proposes the methods to carry out co-ordinated group paging. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Co-ordinated Group Paging in IEEE 802.16p

Honggang Li, Rui Huang, Shantidev Mohanty

Intel Corporation

# Abstract

This contribution proposes methods to have co-ordination between group and individual paging in IEEE 802.16p systems.

# Background

When one device needs to receive both Individual traffic and Group traffic while in idle mode, it has to support the mechanism for carrying out both Individual and Group Paging operation.

This contribution proposes methods to carry out Individual and Group Paging in a co-ordinated manner. It may be noted that for paging operation, each device or a group of devices are assigned two parameters that govern the overall idle mode and paging operations. These parameters are Paging Cycle and Paging Offset.

# Proposed Changes

[Editor’s note] Add the following text after line 42 in page 12 of IEEE P802.16p/D1

-------------------------------------------Change Start-------------------------------------------------------

The device uses one PAGING CYCLE and PAGING OFFSET for both individual and group paging. In this case, the device wakes up once during the PAGING CYCLE to listen for Individual and Group Paging Events. In this case, all the devices in the same MGID group may share the same value of PAGING CYCLE and PAGING OFFSET. If the device is part of more than one MGID group, it uses the same PAGING CYCLE and PAGING OFFSET which the same as the PAGING CYCLE and PAGING OFFSET for its individual paging.

In another case, the device has different PAGING CYCLE and PAGING OFFSET for individual and group paging. Thus, it has Group Paging Listening Interval, GPLI, (based on PAGING CYCLE and PAGING OFFSET of group paging) and Individual Paging Listening Interval, IPLI,  (based on PAGING CYCLE and PAGING OFFSET of individual paging) .

The device does not wake up during all the GPLI. Instead, it may selective wake up during some GPLIs based on the indication it received about the group paging operation during IPLI. Thus, the network uses the MOB-PAG-ADV sent during the IPLI to provide indication about pending/upcoming Group Paging Event. When the device receives such indication, it wakes up during the next one or more following GPLI intervals.

-------------------------------------------Change End-------------------------------------------------------