| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD) (initial working document revised) |  |
| Date Submitted | 2011-01-13 |  |
| Source(s) | Jin Lee (M2M System Requirements Editor) Ron Murias (16p TG Chair) | Email: jin1.lee@lge.com E-mail: ron@murias.ca |
| Re: |  |  |
| Abstract | Initial IEEE 802.16p Machine to Machine (M2M) System Requirements Document |  |
| Purpose | Initial M2M System Requirements working document to enable capturing the requirements for the prospective IEEE 802.16p Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Machine to Machine (M2M) System Requirements Document

1	Overview	3

2	References	3

3	Definitions	4

4	Abbreviations and Acronyms	4

5	General Requirements	5

5.1	Compatibility with other 802.16 equipment	5

5.2	Complexity	5

5.3	M2M Services	5

5.4	802.16p M2M Reference System Architecture	5

6	Functional Requirements	6

6.1	Low Power Consumption	6

6.2	Large Numbers of Devices	6

6.3	Small Burst Transmissions	7

6.4	Security Support	8

7	Performance Requirements	8

# Overview

The 802.16p amendment shall be developed in accordance with the Machine to Machine (M2M) communication project authorization request (PAR) form and five criteria in the approved version of IEEE 802.16ppc-10/0003r7 [1]. The scope of the resulting project shall be as follows:

“This amendment specifies IEEE Std 802.16 medium access control (MAC) enhancements and minimal orthogonal frequency division multiple access (OFDMA) Physical Layer (PHY) modifications in licensed bands to support lower power consumption at the device, support by the Base Station of significantly larger numbers of devices, efficient support for small burst transmissions, and improved device authentication.”

This document represents the high-level system architecture and requirements for the 802.16p amendment. Both functional and performance requirements are included. All content included in any draft of the 802.16p amendment shall meet these requirements. The purpose of this amendment is to enable M2M applications and services in which the device communications require wide area wireless coverage in licensed bands, and are automated rather than human-initiated or human-controlled. The current IEEE 802.16 standard and the amendments under development do not address the unique features and requirements of M2M applications, such as very low power consumption, large number of devices, short burst transmissions, and device integrity.  The current scope of 802.16p covers the above mentioned requirements. It is recognized that these are not all-encompassing to the Machine-to-Machine applications space. However, in order to meet the significant market potential of M2M applications in a timely fashion, and to maintain competiveness of the IEEE 802.16 family of standards, it is important to accelerate the completion and evaluation of the 802.16p standard.

The M2M Technical Report [12] provides an overview of important M2M use cases that can benefit from wide area network connectivity. Key examples of M2M use cases include Smart Grid, Vehicular, and Healthcare. The requirements arising from these applications are covered in the next section for reference. In some sense, Smart Grid applications have the most mature requirements, given the momentum of industry players and governments world-wide to automate grid operations. We intend for 802.16p to, at minimum, provide new features that improve efficiency of communications for the Smart Grid.

# References

[1] IEEE 802.16ppc-10/0003r9: Machine to Machine (M2M) Communication PAR Form and Five Criteria.

[2] IEEE Std 802.16-2009: IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Fixed Broadband Wireless Access Systems, May 2009.

[3] IEEE P802.16m: IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems

[4] IEEE 802.16ppc-10/0002r7: Machine to Machine (M2M) Communication Study Report.

[5]	IEEE P802.16m System Requirements Document (SRD)

[6] IEEE 802.16gman-10/0038: (Draft) 802.16n System Requirements Document, 2010-09-16

[7] DRAFT-T31-127-R020-v01-M, “Recommendations and Requirements for WiMAX Machine to Machine (M2M), WiMAX Forum document, November 2010.

[8] 3GPP TS 22.368: Service requirements for Machine-Type Communications (MTC), Stage 1, Release 10, March 2010.

[9] NIST Priority Action Plan 2, “Wireless standards for Smart Grid,” sgs-2010-00003r000 Priority_Action_Plan2_r05, July, 2010.

[10] NIST framework and roadmap for Smart Grid interoperability standards, v1.0.

[11] Department of Energy, “Communications requirements for Smart Grid Technologies,” October, 2010.

[12] IEEE 802.16p-10/0005 Machine to Machine (M2M) Communications Technical Report

# Definitions

| Terms | Description |
| --- | --- |
| Machine-to-Machine (M2M) communication | This is information exchange between user devices through a Base Station, or between a device and a server in the core network through a Base Station that may be carried out without any human interaction. |
| M2M ASN | Access Service Network that supports M2M service |
| M2M device | 802.16p MS with M2M functionality |
| M2M subscriber | Consumer of M2M service |
| M2M Server | An entity to communicate with M2M devices. M2M server provides an interface which can be accessed by M2M subscriber. |
| M2M feature | Unique characteristic of M2M application that is supported by M2M ASN. One or more features may be needed to support an application |
| M2M group | A group of devices that share one or more features in common, and/or belong to same M2M subscriber |
| Device collaboration | A technique where a certain device helps the other device’s transmission |

# Abbreviations and Acronyms

| M2M | Machine to Machine |
| --- | --- |
| MS | Mobile Station |
| BS | Base Station |
| ASN | Access Service Network |
| CSN | Connectivity Service Network |
| HAN | Home Area Network |
| PHY | Physical layer |
| MAC | Medium Access Control |
| OFDMA | Orthogonal Frequency Division Multiple Access |
| QoS | Quality of Service |
| RRM | Radio Resource Management |
| SRD | System Requirements Document |
| NIST | National Institute of Standards and Technology |

# General Requirements

## Compatibility with other 802.16 equipment

IEEE 802.16p BS provides continuing support for MSs compliant with WirelessMAN-Advanced Air Interface or WirelessMAN-OFDMA R1 Reference System.

## Complexity

IEEE 802.16p provides IEEE Std 802.16 medium access control (MAC) enhancements and minimal OFDMA PHY modifications to provide functionalities for efficient Machine to Machine communication.

## M2M Services

IEEE 802.16p shall support services required for M2M applications (e.g. smart grid, applications on consumer devices, vehicular tracking and healthcare). Detailed M2M services are specified in IEEE 802.16ppc-10/0002r7 Machine to Machine (M2M) Communication Study Report.

## 802.16p M2M Reference System Architecture

Figure 1 captures the high level system architecture for IEEE 802.16p based M2M communications. The IEEE 802.16p M2M device is an IEEE 802.16 MS with M2M functionality. The M2M server is an entity that communicates to one or more IEEE 802.16p M2M devices. The M2M server also provides an interface which can be accessed by an M2M subscriber. The M2M subscriber is a user of M2M services (e.g. a utility company). The M2M Server may reside within or outside of the Connectivity Service Network (CSN) and can provide M2M specific services for one or more IEEE 802.16p M2M devices. The M2M application runs on the IEEE 802.16p M2M device and the M2M server.

The 802.16p M2M service system architecture supports following types of M2M communication:

a) Point-to-point communication between IEEE 802.16p M2M device and IEEE 802.16p M2M server

b) Point-to-multipoint communication between the IEEE 802.16p BS and IEEE 802.16p M2M devices

In the M2M service system architecture shown in Figure 1, an IEEE 802.16p M2M device can act as an aggregation point for non IEEE 802.16 M2M devices. The non IEEE 802.16 M2M devices use different radio interfaces such as IEEE 802.11, IEEE 802.15, Bluetooth, Power Line Communications, etc. This aggregation function is shown only to illustrate an applicable use case for an 802.16p M2M device, however air interface changes to 802.16p device to support aggregation function is out of scope of 802.16p project.

Figure 1 802.16p M2M service reference system architecture

# Functional Requirements

This section provides requirements related to M2M features that are included in the scope of 802.16p PAR[1].  It shall be possible to subscribe to different M2M features independently according to the application or network environment.

## Low Power Consumption

### The 802.16p system shall support mechanisms for low power consumption in M2M devices.

### The 802.16p system shall provide optimized operation for M2M devices with no/low mobility to conserve power.

### The 802.16p system shall support optimized operation for time controlled, time tolerant and in-frequent traffic to conserve power.

#### The 802.16p system shall support efficient radio resource management for time-controlled and time-tolerant traffic.

### The 802.16p system shall support longer inactive periods for increased power savings for M2M devices associated with certain M2M applications.

### The 802.16p system may support device collaboration without defining a new 16p peer-to-peer connection (i.e. all MS transmissions must be terminated at the BS).

## Large Numbers of Devices

The 802.16p system shall support very large numbers of devices.

### The 802.16p system shall be able to identify a large number of devices individually or by group.

### An 802.16p M2M device shall be able to join one or more M2M groups.

### The 802.16p system shall support efficient group management for large numbers of M2M devices.

### The 802.16p system shall support optimized signaling for resource allocation for large numbers of M2M devices individually or by group.

### The 802.16p system shall support efficient and reliable delivery of control and data to and from a large number of devices.

### The 802.16p system shall support efficient mechanisms for network access from large numbers of M2M devices.

#### The 802.16p system shall provide mechanisms to reduce access network congestion or overload from large numbers of M2M Devices.

### The 802.16p system shall have the ability to distinguish between “normal power down” (e.g. associated with a voluntary power outage event) and “abnormal power down” (associated with an involuntary power outage event).

### The 802.16p system shall provide mechanisms for allowing access with different levels of priority for M2M devices across different classes of M2M services.

### The 802.16p system shall support large numbers of M2M devices in connected state as well as in idle state. Signaling shall be optimized for transitions between the two states.

### The 802.16p system shall support various QoS levels for M2M services.

## Small Burst Transmissions

The 802.16p system shall support very small burst transmissions.

### The 802.16p system shall support efficient transmission of small burst sizes. The 802.16p system shall minimize protocol and signaling overhead to support small burst transmission (e.g. support short burst transmission in OFDMA networks without requiring full connection establishment).

## Security Support

The 802.16p system shall support integrity and authentication of M2M devices, as well as integrity and privacy of M2M application traffic which requires a secure connection.

### The 802.16p system shall support a device validity check between the device and the network.

### The 802.16p system shall enable a flexible security suite that can be adjusted per the security requirements of the M2M application.

### The 802.16p system shall support efficient security for small burst transmissions.

### The 802.16p system shall support efficient security for M2M applications with infrequent/low traffic.

### The 802.16p system shall support using the M2M device location information for different applications (e.g. security).

# Performance Requirements

[Editor’s Note: To be added as result of call for contributions]