| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications for E-MBS (P802.16.1/D2) |  |
| Date Submitted | 2011-11-08 |  |
| Source(s) | Seho Kim, Yeongmoon Son, Ji-Yun Seol, Taeyoung Kim Samsung Electronics Zheng Yan-Xiu ITRI | seho42.kim@samsung.com |
| Re: | “P802.16.1/D2”, in response to the IEEE802.16 Working Group Letter Ballot #32: Announcement, IEEE802.16-11/0026 |  |
| Abstract | This contribution provide clarification for E-MBS |  |
| Purpose | To discuss and adopt the proposed text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for E-MBS (P802.16.1/D2)

Seho Kim, Yeongmoon Son, Ji-Yun Seol, Taeyoung Kim

Samsung Electronics

- Problem statement

- AAI-E-MBS-CFG message include information necessary for E-MBS operation such as E-MBS ID and E-MBS MAP information. In case of E-MBS MAP information, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isizeoffset, and E-MBS MAP MIMO Mode need to be provided through AAI-E-MBS CFC message. However, there is no E-MBS MAP MIMO Mode in AAI-E-MBS CFC message.

Thus, E-MBS MAP MIMO Mode field should be removed in AAI-E-MBS CFC message and the MIMO encoding format of E-MBS MAP needs to be informed in 6.3.5.3.3 Enhanced-Multicast Broadcast Services MAP (E-MBS MAP) as SFBC since most of Assignment A-MAP IE is encoded by SFBC.

- There is no definition of E-MBS zone specific pilot patterns for 1 stream in current 16m standard. Actually, E-MBS zone specific pilot patterns for 1 stream would be the 1st pilot stream of E-MBS zone specific pilot patterns for 1 and 2 stream where 2nd pilot stream will be empty. In this case, both 1stream vertical encoded and SFBC encoded E-MBS bursts have same number of coded bits. Since SFBC encoding provides the diversity gain, SFBC encoded E-MBS burst will provide better PER than 1stream vertical encoded one. Thus, there is no need of 1stream vertical encoding in E-MBS burst allocation.

- Text changes in IEEE P802.16.1/D2

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1. Change subclause 6.2.3.61 on line 37 page 261 as follows:]

6.2.3.61 AAI-E-MBS-CFG message

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode, and E-MBS ID and FID mappings between serving E-MBS zone and neighboring E-MBS zone for the same content. AAI-E-MBS-CFG shall be advertised at the superframe whose number (Nsuperframe) from SFH meets the following condition:

[Remedy 2. Change subclause 6.3.5.3.3 on line 31 of page 598 as follows:]

6.3.5.3.3 Enhanced-Multicast Broadcast Services MAP (E-MBS MAP)

A 16-bit CRC is generated based on the contents of the E-MBS MAP. Following randomization, the resulting sequence of bits shall be encoded by the CTC described in 6.3.10.1. SFBC with precoding shall be used for transmitting the E-MBS MAP.

[Remedy 3. Change subclause 6.3.5.5.3 on page 673 as follows:]

6.3.5.5.3 E-MBS Control Information elements

The resource mapping for E-MBS allows are defined in E-MBS-DATA_IEs as defined in Table 211.

Table 211—E-MBS DATA information elements

| Syntax | Size (bits) | Description/Notes |
| --- | --- | --- |
| E-MBS-DATA_IE () { | — | — |
| No. of E-MBS IDs+FIDs | 8 | Total number of E-MBS streams in the IE |
| for (i = 0; i< No. of E-MBS IDs +FIDs, i++){ | — |  |
| E-MBS ID+FID | 16 | E-MBS ID + Flow ID of an E-MBS stream |
| ISizeOffset | 5 | Depends on supported modes, 32 modes assumed as baseline |
| MEF | 1 | E-MBS MIMO Encoding Format 0b0: SFBC 0b1: Vertical Encoding with 2 streams |
| if (MEF=0b1){ | — | — |
| Mt |  | Number of streams for transmission for Nt = 2, Nt = 4, and Nt = 8 (Mt ≤ Nt ) 0b0: 1 stream 0b1: 2 streams |
| } | — | — |
| E-MBS AAI subframe offset | variable | Includes the index of the AAI subframe where the E-MBS data burst ends and the first AAI subframe of the first frame of each superframe is reserved to transmit superframe header, where any E-MBS data burst is not transmitted. MSI == 0b00: 7 bits MSI == 0b01: 8 bits MSI == 0b10: 9 bits MSI == 0b11: 10 bits The AAI subframe offset index begins at the beginning of the MSI. |
| E-MBS Resource Indexing | 7 | Includes the location of the SLRU index where the E-MBS data burst ends. The first E-MBS stream begins in the SLRU immediately after E-MBS MAP in the E-MBS region The EMBS resource indexing field indexes SLRUs of a subframe in the E-MBS region as indicated in the Zone Allocation bitmap transmitted in the AAI-SCD message described in 6.2.3.31. |
| } | — | — |
| } | — | — |

-------------------------------  Text End  ---------------------------------------------------