| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixing ranging code conflicts in IEEE802.16Rev3/D2 |  |
| Date Submitted | 2011-11-07 |  |
| Source(s) | Yeongmoon Son, Samsung Electronics | ym1004.son@samsung.com |
| Re: | IEEE 802.16-11/0029, IEEE 802.16 Working Group Letter Ballot #35. |  |
| Abstract | This contribution provides suggested remedy to solve conflict of ranging codes between RS and Soft-combining. |  |
| Purpose | To be discussed and adopted by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fixing ranging code conflicts in IEEE802.16Rev3/D2

Yeongmoon Son

Samsung Electronics

- Problem statement

Text of IEEE802.16j-2009 and text of IEEE802.16-2009 are merged into Rev3/D2. As a result, there are some conflicts in raging code usage. We need to fix it.

# Proposed Text

Remedy #1:

[Modify table 674 on page 1456 line 15 as follows:]

| Table 674.—UCD PHY-specific channel encodings.—WirelessMAN-OFDMA (continued) |  |  |  |
| --- | --- | --- | --- |
| Name | Type (1 byte) | Length | Value |
| … |  |  |  |
| Start of ranging codes group | 155 | 1 | Indicates the starting number, S, of the group of codes used for this UL. If not specified, the default value shall be set to zero. All the ranging codes used on this UL shall be between S and ((S+O+N+M+L+P+Q+K+J+H+G+K+J+I+H+G+F) mod 256) where N is the number of initial ranging codes M is the number of periodic ranging codes L is the number of BR codes O is the number of HO ranging codes P is the number of RS initial ranging codes Q is the number of RS dedicated codes H is the number of HO ranging codes in HO Ranging Code set 1 J is the number of BR ranging codes in BR Ranging Code set 1 K is the number of Initial ranging codes in Initial Ranging Code set 1 G is the number of HO ranging codes in HO Ranging Code set 2 K is the number of Initial ranging codes in Initial Ranging Code set 1 J is the number of BR ranging codes in BR Ranging Code set 1 I is the number of HO ranging codes in HO Ranging Code set 1 H is the number of HO ranging codes in HO Ranging Code set 2 G is the number of RS initial ranging codes F is the number of RS dedicated codes The range of values is 0 ≤ S ≤ 255. |
| … |  |  |  |
| RS Initial Ranging Code | 219 | 1 | Number of RS initial ranging CDMA codes. Possible values are 0–255. |
| RS dedicated codes | 220 | 1 | Number of RS dedicated codes. Possible values are 0.–255. |
| Initial Ranging Code set 1 | 219 | 1 | Number of Initial ranging CDMA codes for soft combining. Possible values are 0–255. Default value is 0 |
| HO Ranging Code set 1 | 229220 | 1 | Number of HO ranging CDMA codes for soft combining. Possible values are 0–255. Default value is 0 |
| BR Ranging Code set 1 | 221 | 1 | Number of BR ranging CDMA codes for soft combining Possible values are 0–255. Default value is 0 |
| HO Ranging Code set 2 | 222 | 1 | Number of HO ranging CDMA codes for soft combining without fragmentation of RNG-REQ message. Possible values are 0–255. Default value is 0 |
| RS Initial Ranging Code | 223 | 1 | Number of RS initial ranging CDMA codes. Possible values are 0–255. Default value is 0 |
| RS dedicated codes | 224 | 1 | Number of RS dedicated codes. Possible values are 0.–255. Default value is 0 |

Remedy #2:

[Modify the section 8.4.7.3 on page 1426 line 8 as follows:]

8.4.7.3 Ranging codes

…..

The number of available codes is 256, numbered 0..255. Each BS uses a subgroup of these codes, where the

subgroup is defined by a number S, 0 ≤S ≤255. The group of codes shall be between S and

((S + O + N + M +L+K+J+I+H+G+F) mod 256).

- The first N codes produced are for initial ranging. Clock the PRBS generator 144 (S mod 256) times to 144 ((S + N) mod 256) – 1 times.

- The next M codes produced are for periodic ranging. Clock the PRBS generator 144 ((N + S) mod 256) times to 144 ((N + M + S) mod 256) – 1 times.

- The next L codes produced are for BRs. Clock the PRBS generator 144 ((N + M + S) mod 256) times to 144 ((N + M + L + S) mod 256) – 1 times.

- The next O codes produced are for HO ranging. Clock the PRBS generator 144 ((N + M + L + S) mod 256) times to 144 ((N + M + L + O + S) mod 256) – 1 times.

For  the MS and BS supporting OFDMA- initial ranging with soft combining (see 6.3.10.3.1.1), OFDMA HO ranging with soft combining (see 6.3.10.4.2 and 6.2.2.3.23.1), and Contention-based CDMA bandwidth requests using soft combining (see 6.3.6.5.1), the corresponding codes shall be generated by the following methods.

- The next K codes produced are for initial ranging code for soft combining. Clock the PRBS generator 144 ((N + M + L + O + S) mod 256) times to 144 ((K+ N + M + L + O + S) mod 256) – 1 times.

- The next J codes produced are for BR ranging code for soft combining. Clock the PRBS generator 144 ((K+ N + M + L + O + S) mod 256) times to 144 ((J+ K+ N + M + L + O + S) mod 256) – 1 times.

- The next HI codes produced are for HO ranging for soft combining. Clock the PRBS generator 144 ((J+ K+ N + M + L + O + S) mod 256) times to 144 ((HI + J+ K+ N + M + L + O + S) mod 256) – 1 times.

- The next GH codes produced are for HO ranging for soft combining without fragmentation of RNG-REQ message. Clock the PRBS generator 144 ((HI +J+ K+ N + M + L + O + S) mod 256) times to 144 ((GH+ HI + J+ K+ N + M + L + O + S) mod 256) – 1 times.

For the RS, the corresponding codes shall be generated by the following methods.

- The next G codes produced are for RS initial ranging CDMA codes. Clock the PRBS generator 144 ((H+ I + J+ K+ N + M + L + O + S) mod 256) times to 144 ((G + H + I + J+ K+ N + M + L + O + S) mod 256) – 1 times.

- The next F codes produced are for RS dedicated codes. Clock the PRBS generator 144 ((G + H + I + J+ K+ N + M + L + O + S) mod 256) times to 144 ((F + G + H + I + J+ K+ N + M + L + O + S) mod 256) – 1 times.