| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Merging of 802.1Q and 802.3 CSs |  |
| Date Submitted | 2007-11-02 |  |
| Source(s) | David Johnston Intel Corporation 2111 NE 25th Avenue Hillsboro, OR, USA | Voice: 503 629 0238 Fax: dj.johnston@intel.com david.johnston@ieee.org |
| Re: | 802.16 Revision |  |
| Abstract | Instructions to merge the 802.1Q and 802.3 CSs. |  |
| Purpose | To address the incomplete and architecturally incompatible 802.1Q subpart of the packet CS by merging its classification behavior with the 802.3 subpart of the packet CS |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Merging the 802.1Q Specific Subpart and the 802.3 Specific Subpart of the Packet CS

David Johnston

Intel Corporation

# Explanation

The 802.1Q specific subpart of the Packet CS is incompletely specified and architecturally flawed.

The specification lacks a proper definition of the CS SAP and its primitives when operating in the 802.1Q CS mode.

The 802.1Q specific subpart attempts to mandate that SDUs passing through it contain a VLAN header. However this is inconsistent with the 802.1D and 802.1Q specification which provide for the dynamic configuration of the tagging and untagging behavior of bridge ports and provide for a richer collection of single and multiple tags than the 802.1Q subpart considers.

It is not appropriate for the MAC to try to force the presence of absence of VLAN headers. That is the role of the bridge function defined in 802.1D and 802.1Q, which is outside the MAC. It is appropriate for such configuration to be permitted, but that is already handled by 802.1D and 802.1Q.

However the simple deletion of the 802.1Q specific subpart would also delete classifiers that are useful in classifying VLAN tag content.

The 802.3 specific subpart does have a well defined SAP. 802.16k by reference mandates that it be the same as the 802.3 MAC SAP.

Accordingly, the logical approach is to remove the 802.1Q subpart of the packet CS, but retain the classification rules and merge them into the 802.3 specific subpart, where they can be used in configurations where VLAN headers are known to exist.

There is another related problem with the classifier tables. There is no plain IP mode. All IP modes have some specificity towards the IP version or header compression. This proposal adds bits for “Packet, IP” to the classifier tables.

# Proposed Text Changes

The text below provides the changes required to remove the 802.1Q CS from the draft revised 802.16 standard and merge the 802.1Q classifiers into the 802.3 specific subpart of the packet CS along with a number of editorial and technical corrections to the associated text.

[Strike all of 5.2.5, 5.2.5.1 and 5.2.5.2]

[Remove the 802.1Q/VLAN block from Figure 23]

[Modify 11.7.7.1]

11.7.7.1 Classification/PHS options and SDU encapsulation support

This parameter indicates which classification/PHS options and SDU encapsulation the SS supports. By default, Packet, IPv4 and 802.3/Ethernet shall be supported, thus absence of this parameter in REG-REQ means that named options are supported by the SS. When the length field of the TLV is 2, it indicates that bits 16-31 should be considered to be equal to zero.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 7 | 2 or 4 | Bit 0: ATM Bit 1: Packet, IPv4 Bit 2: Packet, IPv6 Bit 3: Packet, 802.3/Ethernet Bit 4: Packet, 802.3 with 802.1Q VLAN Headers Bit 5: Packet, IPv4 over 802.3/Ethernet Bit 6: Packet, IPv6 over 802.3/Ethernet Bit 7: Packet, IPv4 over 802.3/Ethernet with 802.1Q VLAN Headers Bit 8: Packet, IPv6 over 802.3/Ethernet with 802.1Q VLAN Headers Bit 9: Packet, IEEE 802.3/Ethernet (with optional IEEE 802.1Q VLAN tags) and ROHC header compression Bit 10: Packet, IEEE 802.3/Ethernet (with optional IEEE 802.1Q VLAN tags) and ECRTP header compression Bit 11: Packet, IP (v4 or v6) with ROHC header compression Bit 12: Packet, IP (v4 or v6) with ECRTP header compression Bit 13: GPCS Bit 14: Packet, IP Bits 145–31: Reserved; shall be set to zero | REG-REQ, REG-RSP |

[Modify 11.13.19.1]

11.13.19.1 CS Specification Parameter

This parameter specifies the CS that the connection being set up shall use.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145.146].28 | 1 | Bit 0: GPCS (Generic Packet Convergence Sublayer) Bit 1: Packet, IPv4 Bit 2: Packet, IPv6 Bit 3: Packet, 802.3/Etherneta Bit 4: Packet, 802.1Q VLAN Reserved Bit 5: Packet, IPv4 over 802.3/Etherneta Bit 6: Packet, IPv6 over 802.3/Etherneta Bit 7: Packet, IPv4 over 802.1Q VLAN Reserved Bit 8: Packet, IPv6 over 802.1Q VLAN Reserved Bit 9: ATM Bit 10: Packet, IEEE 802.3/Etherneta and ROHC header compression Bit 11: Packet, IEEE 802.3/Etherneta and ECRPT header compression Bit 12: Packet, IPb with ROHC header compression Bit 13: Packet, IPb with ECRTP header compression Bit 14: Packet, IPb Bits 145–31: Reserved | DSA-REQ |

[Modify 11.13.19.2]

Each CS defines a set of parameters that are encoded within a subindex under the “cst” values listed below. In the cases of IP over IEEE 802.x3, the relevant IP and IEEE 802.x3 parameters shall be included in the DSx-REQ message.

| cst | CS |
| --- | --- |
| 99 | ATM |
| 100 | Packet, IPv4 |
| 101 | Packet, IPv6 |
| 102 | Packet, IEEE 802.3/Ethernet |
| 103 | Packet, IEEE 802.1Q VLAN Packet IP |
| 104 | Packet IPV4 over IEEE 802.3/Ethernet |
| 105 | Packet IPV6 over IEEE 802.3/Ethernet |
| 106 | Packet IPV4 over IEEE 802.1Q VLAN Reserved |
| 107 | Packet IPV6 over IEEE 802.1Q VLAN Reserved |
| 108 | Packet, IP with ROHC header compression |
| 109 | Packet, IP with ECRTP header compression |
| 110 | Packet, IP over IEEE 802.3/Ethernet with ROHC header compression |
| 111 | Packet, IP over IEEE 802.3/Ethernet with ECRTP header compression |

[Modify 12.1.1.6.1]

12.1.1.6.1 Packet CS parameters for DSA-REQ—BS-initiated

- Packet Classification Rule(s) (UL service flows only, default is no classification)

- Classifier Rule Priority (default to 0)

- Classifier Rule Index

- IP Type of Service/DSCP (only for IP CSs, default = don’t classify on this)

- Protocol (only for IP CSs, default = don’t classify on this)

- IP Masked Source Address (only for IP CSs, default = don’t classify on this)

- IP Destination Address (only for IP CSs, default = don’t classify on this)

- Protocol Source Port Range (only for IP CSs, default = don’t classify on this)

- Protocol Destination Port Range (only for IP CSs, default = don’t classify on this)

- Ethernet Destination MAC Address (only for Ethernet CSs, default = don’t classify on this)

- Ethernet Source MAC Address (only for Ethernet CSs, default = don’t classify on this)

- Ethertype/IEEE 802.2 SAP (only for Ethernet CSs, default = don’t classify on this)

- IEEE 802.1D User Priority (only for VLAN CSs Ethernet CSs on which VLAN headers carry the priority bits, default = don’t classify on this)

- IEEE 802.1Q VLAN_ID (only for VLAN Ethernet CSs, default = don’t classify on this)

- Associated PHSI (default is no PHS for this classifier match)

- Vendor-specific Classifier Parameters

- PHS Rule(s)

- PHSI

- PHSS

- PHSF

- PHSM (default is suppress all bytes of the suppression field)

- PHSV (default is verify)

- Vendor-specific PHS Parameters

[Modify 12.1.1.6.3]

12.1.1.6.3 Packet CS Parameters for DSC-REQ—BS Initiated

- Classifier Dynamic Service Change Action(s)

- Packet Classification Rule(s) (UL service flows only, 1 per Action)

- Classifier Rule Index

- Classifier Rule Priority (default to 0)

- IP Type of Service/DSCP (only for IP CSs, default = don’t classify on this)

- Protocol (only for IP CSs, default = don’t classify on this)

- IP Masked Source Address (only for IP CSs, default = don’t classify on this)

- IP Destination Address (only for IP CSs, default = don’t classify on this)

- Protocol Source Port Range (only for IP CSs, default = don’t classify on this)

- Protocol Destination Port Range (only for IP CSs, default = don’t classify on this)

- Ethernet Destination MAC Address (only for Ethernet CSs, default = don’t classify on this)

- Ethernet Source MAC Address (only for Ethernet CSs, default = don’t classify on this)

- Ethertype/IEEE 802.2 SAP (only for Ethernet CSs, default = don’t classify on this)

- IEEE 802.1D User Priority (only for VLAN CSsEthernet CSs on which VLAN headers carry the priority bits, default = don’t classify on this)

- IEEE 802.1Q VLAN_ID (only for VLAN Ethernet CSs, default = don’t classify on this)

- Associated PHSI (default is no PHS for this classifier match)

- Vendor-specific Classifier Parameters

- PHS Dynamic Service Change Action

- PHS Rule(s) (1 per Action)

- PHSI

- PHSS

- PHSF

- PHSM (default is suppress all bytes of the suppression field)

- PHSV (default is verify)

- Vendor-specific PHS Parameters