| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Computation of CMAC value not compliant with NIST special publication 800-38B |  |
| Date Submitted | 2007-10-29 |  |
| Source(s) | Erik Colban Navid Ehsan Yair Borlas Wee Peng Goh NextWave Broadband Inc.* | Voice: [Telephone Number (optional)]] E-mail: ecolban@nextwave.com nehsan@nextwave.com ybourlas@nextwave.com wgoh@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Working Group 802.16 Letter Ballot #26 as announced in IEEE 802.16-07/049 |  |
| Abstract | In defining the CMAC calculation, the 802.16 standard is not consistent with the NIST publication 800-38B for the CMAC specification. This deviation from the NIST specification has to be clearly stated in the standard. |  |
| Purpose | Adopt the proposed changes in 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Computation of CMAC value not compliant with NIST special publication 800-38B

Erik Colban and Navid Ehsan

NextWave Broadband Inc.

# Problem Description

In defining the CMAC calculation, the standard is not consistent with the NIST publication 800-38B for the CMAC specification. In the NIST publication it is specified that a CMAC of length Tlen is derived using the MSB of the result, whereas in the 802.16 standard, the 64-bit CMAC is derived from the LSB of the 128-bit CMAC. This deviation from the NIST specification has to be clearly stated in the standard.

# Specific Changes to the Standard

Page 570, line 42:  Add a note as follows:

The LSBs of the digest shall be truncated to yield a 64-bit length digest. Note: This is different from the recommendation in NIST special publication 800-38B where the MSB is used to derive the CMAC value.