| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MBS_DATA_Time_Diversity_IE Corrections |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Lei Wang, Sina Zahedi, Yair Bourlas, Wee Peng Goh NextWave Broadband Inc.* | Voice: E-mail: lwang@nextwave.com; ybourlas@nextwave.com; wgoh@nextwave.com; szahedi@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #26b as announced in IEEE 802.16-08/006 |  |
| Abstract | This document proposes fixes in MBS_DATA_Time_Diversity_IE. |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MBS_DATA_Time_Diversity_IE Corrections

Lei Wang, Sina Zahedi, Yair Bourlas, Wee Peng Goh,

NextWave Wireless

# Problem statement

The field “OFDMA symbol offset” in MBS_DATA_Time_Diversity IE indicates the starting offset of the MBS HARQ region, and the MBS HARQ region defined by the MBS_DATA_Time_Diversity_IE spans over the entire set of subchannels. If the MBS region contains an MBS MAP message, the OFDMA symbol offset of the starting location of the HARQ region need to point to a symbol after the last symbol occupying that MBS MAP message. Using this method, the area under the MBS map remains unused. Note that the MBS MAP allocation is specified as a block allocation by the MBS MAP IE in the DL MAP. Depending on the shape of MBS MAP allocation, such a waste could be serious as shown in the figure below.

# Proposed Corrections

Change the interpretation of MBS HARQ region defined by the parameter “OFDMA symbol offset” so that the full MBS region can be utilized. If subchannels remain unoccupied by the MBS MAP, allow MBS HARQ data sub-bursts in this region to start at the same symbol as the MBS MAP at the subchannel logically numbered after the largest subchannel occupied by the MBS MAP as shown in the figure below.

# Suggested Changes in Rev2/D3

In Rev2/D3, replace the contents from page 244 line 1 to page 245 line 3 by the following, where the new text is marked by blue and underlined and the deleted text is red with strikethrough:

Table 157 —MBS_DATA_Time_Diversity_IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| MBS_DATA_Time_Diversity_IE() { | — | — |
| MBS_MAP Type = 1 | 2 bits | — |
| MBS Burst Frame Offset | 2 bits | This indicates the burst located by this IE will be shown after MBS Burst Frame offset + 2 frames |
| OFDMA symbol offset | 8bits | This indicates starting position of the region of MBS Bbursts with respect to start of the next (MBS Burst Frame offset + 2)-th frame. |
| # of Data Sub-bursts | 4 bits | n = # Data Sub-bursts with the same frame offset |
| For(i=0; i<n; i++){ |  |  |
| Multicast CID | 12 bits | 12 LSBs of CID for multicast. |
| N_EP code | 4 bits | — |
| N_SCH code | 4 bits | — |
| AI_SN | 1 bit | — |
| SPID | 2 bits | — |
| ACID | 4 bits | — |
| Next MBS MAP change indication | 1 bit | This indicates whether the size of MBS MAP message of next MBS frame for these multicast CIDs included this IE will be different from the size of this MBS MAP message. |
| Next MBS frame offset | 8 bits | — |
| Next MBS OFDMA Symbol offset | 8 bits | — |
| If (Next MBS MAP change indication = 1) { | — | — |
| Next MBS No. OFDMA symbols | 6 bits | It is to indicate the size of MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| Next MBS No. OFDMA subchannels | 6 bits | It is to indicate the size of MBS_MAP message in Next MBS portion where the BS shall transmit the next MBS frame for multicast CIDs in this IE. |
| } | — | — |
| } | — | — |
| } | — | — |

OFDMA symbol offset

This indicates the starting position of the region for HARQ-coded MBS Bbursts allocated in the frame identified by with the same MBS_Burst_Frame_offset. There are two possible cases for the starting point of the MBS region:

1) If the MBS Frame Offset and OFDMA symbol offset point to the start of a region that contains an MBS_MAP message, then the MBS data sub-bursts specified in this MBS_DATA_Time_Diversity_IE are allocated in a frequency-first one-dimensional way starting at the same symbol as that MBS_MAP message and the subchannel logically numbered after the largest subchannel occupied by that MBS_MAP message.

2) Otherwise, The the MBS region defined by the MBS_DATA_Time_Diversity_IE begins from at the first subchannel of the OFDMA symbol identified by the MBS Frame Offset and OFDMA symbol offset, and in this region, MBS data sub-bursts, indicated specified by this MBS_DATA_Time_Diversity_IE at the same in this MBS_MAP message, are allocated in a frequency-first, one-dimensional way in the order of MBS DATA Time Diversity IE at a MBS_MAP message.