| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Byte and Bit ordering of TLV encodings in REP-RSP message |  |
| Date Submitted | 2007-11-05 |  |
| Source(s) | Yeongmoon Son Samsung Electronics #416, Maetan-3dong, Yeongtong-gu Suwon-city, South Korea | Voice: +82-31-279-5845 E-mail: ym1004.son@samsungmcom *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Working Group 802.16 Letter Ballot #26 as announced in IEEE 802.16-07/049 |  |
| Abstract | This contribution clarifies Byte and Bit ordering of TLV encodings in REP-RSP message |  |
| Purpose | Adopt the proposed change in IEEE802.16REV2/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Byte and Bit ordering of TLV encodings in REP-RSP message

Yeongmoon Son

Samsung Electronics

# 1. Introduction

There are a lot of ambiguities for Byte and Bit ordering of TLV encodings in REP-RSP as follows.

- 1 byte and 1 byte: which one occupies the most significant 8 bits?

- Second byte: Is it the most significant 8 bits or the least significant 8 bits?

- First MM bits and Next NN bits: Do Fist MM bits mean the most significant MM bits or the least significant MM bits? What about Next NN bits?

We need to explicitly clarify that ambiguity.

# 2. Proposed text change

[Modify the 1st table in the section 11.12 REP-RSP management message encodings on Page 1262 of IEEE802.16REV2/D1, as follows]

| REP-REQ Report type | Name | Type | Length | Value |
| --- | --- | --- | --- | --- |
| .. | .. | .. | .. | .. |
| Bit #1 = 1 | CINR report | 1.5 | 2 | 1 byteBit #15~Bit #8 : mean (see also 8.2.2, 8.3.9, 8.4.11) for details) 1 byteBit #7~Bit #0 : standard deviation |
| Bit #2 = 1 | RSSI report | 1.6 | 2 | 1 byteBit #15~Bit #8 : mean (see also 8.2.2, 8.3.9, 8.4.11) for details) 1 byteBit #7~Bit #0 : standard deviation |

[Modify the 2nd table in the section 11.12 REP-RSP management message encodings on Page 1262 of IEEE802.16REV2/D1, as follows]

| REP-REQ Channel Type request | Name | Type | Length | Value |
| --- | --- | --- | --- | --- |
| .. | .. | .. | .. | .. |
| Channel Type = 01 | Band AMC Report | 2.2 | 4 | First 12 bits for the band indicating bitmap and Next 25 bits for CINR reports (5 bits per each band) Bit #31 ~ Bit #20 : Band Indication Bitmap (Bit #31 for Band with index 11, Bit #30 for Band with 10 .. Bit#20 for Band with index 0) Bit #19 ~ Bit #0 : CINR reports for 4 selected Bands. (5 bits per each band. Band with lower index has lower significant 5 bits) |
| Channel Type = 01 | Enhanced Band AMC Report | 2.4 | 5 | First 12 bits for the band indicating bitmap and next 25 bits for CINR measurement (5 bits per each band) Bit #39 ~ Bit #28 : Band Indication Bitmap (Bit #39 for Band with index 11, Bit #38 for Band with index 10 .. Bit#28 for Band with index 0) Bit #27 ~ Bit #3 : CINR reports for 5 selected Bands. (5 bits per each band. Band with lower index has lower significant 5 bits) Bit #2 ~ Bit #0 : Reserved |
| Channel Type = 10 | Safety Channel Report | 2.3 | 5 | The first 20 bits for the reported bin indices and the next 20 bits for CINR reports (5 bits for each bin) Bit #39 ~ Bit #20 : Reported Bin Indication Bitmap (Bit #39 for Bin with index 19, Bit #38 for Bin with index 18 .. Bit#20 for Bin with index 0) Bit #19 ~ Bit #0 : CINR reports for 4 selected Bins. (5 bits per each bin. Bin with lower index has lower significant 5 bits) |
| .. | .. | .. | .. | .. |

[Modify the table in the section 11.12 REP-RSP management message encodings on Page 1263 of IEEE802.16REV2/D1, as follows]

| REP-REQ Preamble physical CINR request | Name | Type | Length | Value |
| --- | --- | --- | --- | --- |
| Bits 0–2 = 0b000 | PUSC zone with Use All SC = 0 | 2.6 | 1 or 2 | Bits 0–4: Mean of physical CINR estimate for PUSC zone with Use All SC = 0 and PRBS_ID indicated in Zone-Specific Physical CINR Request. Bit 5: Report type: 0: CINR estimated from pilot subcarriers. 1: CINR estimated from data subcarriers. Bits 6–7: Reserved; shall be set to zero. Bits 8–12: Standard deviation of CINR estimate for PUSC zone with Use All SC = 0 and PRBS_ID indicated in Zone-Specific CINR Request. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |
| Bits 0–2 = 0b001 | PUSC zone with Use All SC = 1 | 2.7 | 1 or 2 | Bits 0–4: Mean of physical CINR estimate for PUSC zone with Use All SC = 1 and PRBS_ID indicated in Zone-Specific Physi-cal CINR Request. CINR reported corresponds to a subset of major groups as specified in CINR Type Request. Bit 5: Report type: 0: CINR estimated from pilot subcarriers. 1: CINR estimated from data subcarriers. Bits 6–7: Reserved; shall be set to zero. Bits 8–12: Standard deviation of CINR estimate for PUSC zone with Use All SC = 1 and PRBS_ID indicated in Zone-Specific CINR Request. CINR reported corresponds to a subset of major groups as specified in CINR Type Request. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |
| Bits 0–2 = 0b010 | FUSC zone | 2.8 | 1 or 2 | Bits 0–4: Mean of physical CINR estimate for FUSC zone with PRBS_ID indicated in Zone-Specific Physical CINR Request. Bit 5: Report type: 0: CINR estimated from pilot subcarriers. 1: CINR estimated from data subcarriers. Bits 6–7: Reserved; shall be set to zero. Bits 8–12: Standard deviation of CINR estimate for FUSC zone with PRBS_ID indicated in Zone-Specific CINR Request. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |

[Modify the 1st table in the section 11.12 REP-RSP management message encodings on Page 1264 of IEEE802.16REV2/D1, as follows]

| REP-REQ Preamble physical CINR request | Name | Type | Length | Value |
| --- | --- | --- | --- | --- |
| Bits 0–2 = 0b011 | Optional FUSC zone | 2.9 | 1 or 2 | Bits 0–4: Mean of physical CINR estimate for Optional FUSC with PRBS_ID indicated in Zone-Specific Physical CINR Request. Bit 5: Report type: 0: CINR estimated from pilot subcarriers. 1: CINR estimated from data subcarriers. Bits 6–7: Reserved; shall be set to zero. Bits 8–12: Standard deviation of CINR estimate for Optional FUSC with PRBS_ID indicated in Zone-Specific CINR Request. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |
| .. | .. | .. | .. | .. |
| Bits 0–2 = 0b101 | AMC zone | 2.11 | 1 or 2 | Bits 0–4: Mean of physical CINR estimate for AMC AAS zone or AMC zone with dedicated pilots with PRBS_ID indicated in Zone-Specific Physical CINR Request. Bit 5: Report type: 0: CINR estimated from pilot subcarriers. 1: CINR estimated from data subcarriers. Bits 6–7: Reserved; shall be set to zero. Bits 8–12: Standard deviation of CINR estimate for AMC AAS zone or AMC zone with dedicated pilots. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |

[Modify the 2nd table in the section 11.12 REP-RSP management message encodings on Page 1264 of IEEE802.16REV2/D1, as follows]

| REP-REQ Preamble physical CINR request | Name | Type | Length | Value |
| --- | --- | --- | --- | --- |
| Bits #0-1 = 0b00 | The estimation of physical CINR measured from preamble for frequency reuse configuration = 1 | 2.12 | 1 or 2 | Bits 0–4: The mean of physical CINR estimation measured from preamble for frequency reuse configuration = 1. Bits 5–7: Reserved; shall be set to zero. Bits 8–12: The standard deviation of CINR estimation measured from preamble for frequency reuse configuration = 1. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |

[Modify the 1st table in the section 11.12 REP-RSP management message encodings on Page 1265 of IEEE802.16REV2/D1, as follows]

| REP-REQ Preamble physical CINR request | Name | Type | Length | Value |
| --- | --- | --- | --- | --- |
| Bits #0-1 = 0b01 | The estimation of physical CINR measured from preamble for frequency reuse configuration = 3 | 2.13 | 1 or 2 | Bits 0–4: The mean of physical CINR estimation measured from preamble for frequency reuse configuration = 3. Bits 5–7: Reserved; shall be set to zero. Bits 8–12: The standard deviation of CINR estimation measured from preamble for frequency reuse configuration = 3. Bits 13–15: Reserved; shall be set to zero. NOTE—The second byte Bit #15~Bit#8 shall only be sent if length = 2. |
| Bits #0-1 = 0b10 | The estimation of physical CINR measured from preamble for Band AMC zone. | 2.14 | 4 | The estimation of physical CINR measured from preamble for band AMC subchannel. First 12 bits for the band indicating bitmap and Next 20 bits for CINR reports (5 bits per each band). Bit #31 ~ Bit #20 : Band Indication Bitmap (Bit #31 for Band with index 11, Bit #30 for Band with 10 .. Bit#20 for Band with index 0) Bit #19 ~ Bit #0 : CINR reports for 4 selected Bands. (5 bits per each band. Band with lower index has lower significant 5 bits) |
| Bits #0-1 = 0b10 | The enhanced estimation of physical CINR measured from preamble for Band AMC zone. | 2.15 | 5 | The enhanced estimation of physical CINR measured from preamble for Band AMC subchannel. First 12 bits for the band indicating bitmap and Next 25 bits for CINR reports (b bits per each band) Bit #39 ~ Bit #28 : Band Indication Bitmap (Bit #39 for Band with index 11, Bit #38 for Band with index 10 .. Bit#28 for Band with index 0) Bit #27 ~ Bit #3 : CINR reports for 5 selected Bands. (5 bits per each band. Band with lower index has lower significant 5 bits) Bit #2 ~ Bit #0 : Reserved |