| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Generic L2 Services Transport Management PDU |  |
| Date Submitted | 2007-11-02 |  |
| Source(s) | David Johnston Intel Corporation 2111 NE 25th Avenue Hillsboro, OR, USA | Voice: 503 629 0238 Fax: dj.johnston@intel.com david.johnston@ieee.org |
| Re: | 802.16 Revision |  |
| Abstract | Instructions to delete the Secondary Management Connection |  |
| Purpose | To provide a generic extensible management transport PDU to support L2 network intitiated services such as location based services and OMA-DM network initiated provisioning |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Generic L2 Services Transport Management PDU

David Johnston

Intel Corporation

# Explanation

WiMAX has defined two L2 services that require network initiation. Carrying these services over L3 causes the network initiation to fail when they interact with a firewall that impedes L3 network initiated packets. The services currently defined by WiMAX that have this problem are a location based service and OMA-DM OTA bootstrap and notification.

The problem can be avoided if the service is provided at layer two. However there is no extensible carrier for this class of service at layer 2.

Specific instances of non-generic management pass through messages in 802.16 include the EAP_Transfer packet and the MOB-MIH-MSG packet that carries L2.5-esque protocols from M-SAP to M-SAP without further interpretation.

There is a security concern if we provide a generic pass through that it may be abused as a transport for L3 data. As such if any generic L2 pass through is defined, the services it may carry need to be strictly enumerated and other encodings (E.G. of a type byte indexing the service) must be rejected.

# Proposed Text Changes

The text below provides the changes required to add a generic L2 management transport PDU. It includes an enumerated protocol type field and language that dictates the modes under which the transport packets may be used and under which they must be discarded.

It allows for an external body (WiMAX) to own part of the state space of the protocol type field and retains all other parts of the state space reserved for IEEE use.

[Modify end of Table 38, page 84, lines 27-28]

| 69 | LBS-ADV | Location information broadcast for LBS | broadcast |
| --- | --- | --- | --- |
| 70 | WMX-Transfer | Generic L2 management transfer | Primary management, broadcast |
| 7071-255 | --- | Reserved | --- |

[Insert new subclause 6.3.2.3.65, page 276, line 60]

6.3.2.3.65 WiMAX L2 Management Transfer (WMX_Transfer) message

The WMX-Transfer message carries generic L2 service protocols.

| Syntax | Size | Notes |
| --- | --- | --- |
| WMX_Transfer() { |  |  |
| Management Message Type = 70 | 8 bits |  |
| Protocol Type | 8 bits |  |
| Length | 16 bits |  |
| Payload |  | Contents depend on the protocol type |
| TLV Encoded Information | Variable | To carry the HMAC/CMAC Tuple |
| } |  |  |

Protocol Type Field

The Protocol Type field indicates the protocol that is carried in the payload field. If the protocol type field is of an unknown type, the message shall be discarded. The type field shall be chosen from table xxx below.

Table XXX --- WMX-Transfer message protocol type field encodings.

| Protocol Type | Meaning | Notes |
| --- | --- | --- |
| 0 - 127 | Reserved |  |
| 128 - 255 | WiMAX L2 Protocols | Encodings in this range are enumerated in [x] WiMAX Stage 3 specification. |

Encodings in the range 0 -127 shall not be used if not directly enumerated in table XXX.

Encodings in the range 128 – 255 shall not be used if not directly enumerated in [x] WiMAX Stage 3 specification.

Length Field

The length shall encode an integer equal to the length of the payload in octets.

Payload field

The payload field shall contain the protocol payload determined by the protocol type field.

TLV Encoded Information

The TLV Encoded Information field shall contain the HMAC/CMAC tuple, except when the message is carried on the broadcast management connection, where it shall not contain the HMAC/CMAC tuple.

[Insert new function in 14.1.1, page 1593, line 24]

Function

ACM - Accounting Management
GEN – Generic L2 Services
HO – Handover
IMM - Idle Mode Management
LBS - Location Based Services
MBS - Multicast Broadcast Service
NEM - Network Entry Management
RRM - Radio Resource Management
SFM - Service Flow Management
SM - Security Management
SMC - Secondary Management Connection
SSM - Subscriber Station Management

[Insert new action type in table in 14.1.2.2, page 1596, line 29]

• MIH-IND,

• WMX-IND,

• Spare Capacity Report,

[Insert new subclause 14.2.12, page 1713, line 46]

14.2.12 Generic L2 Management Service Procedures

The Generic L2 Management Primitives provide transport of 802.16 and externally defined L2 management frames between the 802.16 entity and the NCMS. This enables extensible NCMS hosted L2 management services to be carried over the air without requiring an expanding number of supporting management frame types.

Figure XXX —GEN primitive Flow between 802.16 entity and NCMS

14.2.12.1 M-WMX-IND

Function:

This primitive used by the 802.16 entity to indicate on the M-SAP the reception of a WMX-Transfer message on the air interface and to convey the payload carried in the message to the NCMS. This primitive is used by the NCMS to request on the M-SAP that the 802.16 entity transmits a WMX-Transfer message containing payload carried in the primitive.

Semantics of the service primitive:

M-WMX-IND

(

Event_Type: WMX-IND,

Destination: NCMS, BS, MS,

Attribute_List:

WMX Protocol Type

WMX-Transfer frame,

)

WMX Protocol Type

Protocol type of the protocol carried in the WMX-Transfer Frame field

WMX-Transfer frame

Frame as determined by the protocol type field

When generated:

- 802.16 Entity to NCMS:

This primitive is generated by the 802.16 entity when the 802.16 entity receives a WMX_transfer message from a peer 802.16 entity.

- NCMS to 802.16 Entity:

This primitive is generated by the NCMS when the NCMS needs to convey a WMX transfer frame through the 802.16 entity to a peer 802.16 entity.

Effect of receipt:

- 802.16 Entity to NCMS

On receipt of this primitive from the M-SAP by the NCMS, the NCMS shall confirm that the protocol type field is enumerated as in table xxx (note to editor.. link to the protocol type table in 6.3.2.3.65) and convey the WMX transfer frame to the associated management entity. If the protocol type field is not understood or if the protocol type field is not one enumerated in table XXX, the frame shall be discarded.

- NCMS to 802.16 Entity

On receipt of this primitive from the M-SAP by the 802.16 entity, the 802.16 entity shall transmit a WMX transfer message containing the WMX transfer payload conveyed in the WMX transfer frame field of the primitive.