| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS_ADV TLVs |  |
| Date Submitted | 2007-11-14 |  |
| Source(s) | Lei Wang, Michael Wengler, Yair Bourlas, Wee Peng Goh, Erik Colban NextWave Broadband* | Voice: [Telephone Number (optional)]] E-mail: lwang@nextwave.com; ybourlas@nextwave.com; wgoh@nextwave.com; mwengler@nextwave.com ecolban@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Working Group 802.16 Letter Ballot #26 as announced in IEEE 802.16-07/049 |  |
| Abstract | This document proposes additional fields in LBS_ADV TLVs. |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

LBS_ADV TLVs

Lei Wang, Michael Wengler, Yair Bourlas, Wee Peng Goh, Erik Colban

NextWave Broadband Inc.

# Problem statement

The LBS_ADV message can provide more information about the BSs to enable MS positions to be derived with accuracy required to support Emergency Services regulations.

# Suggested Changes in Rev2/D1

In Rev2/D1, replace the contents from page 276 line 53 to page 276 line 59 by the following, where the new texts are marked by blue and underlined; the deleted texts are red with strikethrough:

6.3.2.3.64 Location Based Services (LBS-ADV) message

A BS may use the LBS-ADV message to broadcast the LBS information. The message may be broadcast periodically without solicitation. This message is sent from the BS to all MSs on a broadcast CID.

| Syntax | Size | Notes |
| --- | --- | --- |
| LBS-ADV_Message_Format( ) { | LBS-ADV |  |
| Management message type = 69 |  |  |
| Number_of_BS | 8 bits | Total number of serving BS and neighbor BSs |
| For (j = 0 ; j < Number_of_BS ; j++) { |  |  |
| Length | 8 bits | Length of message information within the iteration of Number_of_BS in bytes. |
| BSID | 24 bits | The least significant 24 bits of the Base Station ID parameter in the DL-MAP message of the Serving BS or Neighbor BS. |
| Preamble index/Subchannel Index | 8 bits | PHY-specific preamble for the BS |
| TLV encoded information | variable | TLV specific |
| } |  |  |
| } |  |  |

The LBS-ADV shall include the following TLV:

Base Station Information (see 11.21.1)

The Base Stations location, timing, and transmit information.

In Rev2/D1, replace the contents on pages 1306 line 1 through 1306 line 5 by the following, where the new texts are marked by blue and underlined; the deleted texts are red with strikethrough:

11.20 Device Coordinates

11.21 LBS-ADV message encodings

### 11.21.1 Base Station Information

This compound TLV is used for encoding information used to determine the position of a device BS.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Base Station Information | 45 | variable | Compound | LBS-ADV |

Either the Absolute Position TLV or the Relative Position TLV shall be present in the Base Station Information TLV. The first occurrence of this compound TLV in a message shall contain the Absolute Position TLV, and the position given by this TLV serves as a reference position of all instances of the Relative Position TLV in the same message.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Absolute Position | 45.1 | 15 | Bits # 0-5: longitude resolution 1-34 - number of valid bits in fixed-point value of longitude value 35 - LBS not supported Others - reserved Bits # 6-14: longitude integer Bits # 15-39: longitude fraction Bits # 40-45: latitude resolution 1-34 - number of valid bits in fixed-point value of latitude value 35 - LBS not supported Others - reserved Bits # 46-54: latitude integer Bits # 55-79: latitude fraction Bits # 80-83: altitude type 1 - meters 2 - floors Others - reserved Bits # 84-89: altitude resolution 1-30 - number of valid bits in fixed-point value of altitude value 31 - LBS not supported Others - reserved Bits # 90-111: altitude integer Bits # 112-119: altitude fraction |
| Relative Position | 45.2 | 6 | Bit #0-15: Signed distance East of reference position in units of meters. Bit #16-31: Signed distance North of reference position in units of meters Bit #32-47: Signed distance above reference position in units of meters |
| TX Power and Azimuth | 45.3 | 2 | Bit #0-7: EIRP of Transmitted preamble in steps of 0.5 dB, ranging from -44 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) Bit #8-15: Direction of TX Beam. 0xFF indicates omnidirectional transmit. For directional antennas, center of beam direction heading expressed in units of 5 degrees per LSB. For example, North: 0x00 or 0x48, antenna faces North, 0x12 antenna faces East, 0x24 antenna faces South, 0x36 antenna faces West. Values from 0x49 through 0xFE are not allowed |
| GPS Time | 45.4 | 6 | Bit #0-11: GPS second modulo 2048 Bit #12-39: GPS second fraction Bit#40-47: GPS Time Acc. For unsigned integer values 0x00-0x3F: Log2(Time error in pico sconds). For unknown accuracy in time error, 0xFF. Values 0x40-0xFE not allowed. |

The fields indicate the MS / BS location in latitude, longitude, and altitude fields within the Absolute Position TLV indicate the MS/BS location  in latitude, longitude, and altitude that are based on the LCI (Location Configuration Information) format as defined in RFC3825. Latitude and longitude are represented in 34 bits fixed-point 2s-complement number, consisting of 9 bits of integer and 25 bits of fraction. Altitude is represented in 30 bits fixed-point 2s-complement number with 22 bits of integer and 8 bits of fraction. Latitude and longitude should be normalized to within +/- 90 degrees and +/- 180 degrees, respectively. Each field also includes resolution bits that define the number of valid bits in the fixed-point value. Here are the definition of 2s-complement number.

- Positive numbers

  - LatitideLatitude - North

  - Longitude - East

  - Altitude - above ground

- Negative numbers

  - LatitideLatitude– South

  - Longitude - West

  - Altitude - below ground

The Relative Position TLV bits are used to indicate the positions of additional BSs as offsets from the Latitude, Longitude and Altitude values of the first Absolute Position TLV in the message.  Negative values denote positions West, South, and below the reference position. The scaling of these numbers allows positions of BS within 32 km from the first BS to be described.

The TX_Pwr field informs the receiving MS of how much preamble power is radiated from this BS antenna.  This information may be used by the mobile to aid in estimating the mobile’s range from the BS based on RSSI.

The TX_Azimuth field informs the receiving mobile of the direction from the BS antenna that this antenna radiates.  This information may be used by the mobile to aid in estimating the mobile’s location relative to the BS.

The GPS Time field informs the receiving mobile of the precise time at which BS frames are being transmitted.  This may be used by the mobile to calibrate the mobile’s own internal clock in reference to the GPS time standard.  This may be particularly valuable for determining GPS satellite signal search windows in mobiles equipped to detect GPS satellites.  GPS Second and Second fraction allow the mobile to use DL Frame arrival times as timing signals aligned with GPS time.  GPS Time Acc aids the mobile in estimating how much error with respect to GPS time the BS may have when using this calibration.