| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Including Frame Number in ARQ Feedback |  |
| Date Submitted | 2007-11-02 |  |
| Source(s) | Brett Schein (Beceem) | bschein@beceem.com |
|  | Anuj Puri (Beceem) | apuri@beceem.com |
|  | Manish Airy (Beceem) | mairy@beceem.com |
|  | Frank Zhou (Beceem) | fzhou@beceem.com |
| Re: | IEEE 802.16Rev2/D1 – Unsolicited Technical Proposal to solve a basic problem in ARQ Feedback |  |
| Abstract | ARQ Feedback is generated and transmitted over the air at an unpredictable time. The proposed solution adds a frame number to the ARQ Feedback to remove uncertainty and prevent unnecessary ARQ block retransmission. |  |
| Purpose | Adopt the proposed solution and incorporate in the P802.16Rev2 draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Including Frame Number in ARQ Feedback

Brett Schein, Anuj Puri, Manish Airy, Frank Zhou - Beceem

# Problem Statement

The standard does not specify exactly when ARQ Feedback is generated or transmitted over the air-interface.  So ARQ Feedback generation and transmission is implementation dependent.  A more fundamental problem is that scheduling delays are not deterministic.  Most drastically, random access delay in the UL can lead to large variation between the time ARQ Feedback is generated versus transmitted over the air.  Therefore, upon receiving ARQ Feedback, the ARQ transmitter does not know exactly which of the previous frames were taken into account when the ARQ Feedback was generated.  This leads to unnecessary retransmission of ARQ blocks or added delay waiting for ARQ_RETRY_TIMEOUT to expire, depending upon what each vendor decides to do about this.

The RECEIVER_ARQ_ACK_PROCESSING_TIME, an optional parameter, provides a minimum turnaround time.  Specifically, if RECEIVER_ARQ_ACK_PROCESSING_TIME = 3, then ARQ Feedback received on frame N cannot possibly incorporate transmissions occurring within the 3 previous frames.  But it does not require that the ARQ Feedback message incorporates every transmission occurring 4 or more frames ago.

Similarly, the ARQ_RECEIVER_DELAY cannot provide a deterministic turnaround time.  In fact, the standard does not even specify whether this is an estimated turnaround time, a minimum turnaround time, a maximum turnaround time, or something else.  But since the scheduling delay is variable, a single number cannot resolve the ambiguity.

To fix this problem, we propose adding frame number information to the ARQ Feedback itself.  This would indicate the latest frame number that was taken into account when the ARQ Feedback message was generated.  This eliminates all ambiguity in interpreting the ARQ Feedback, thus eliminating unnecessary retransmission or delay.

# Proposed text changes to the 802.16Rev2 draft

Make the following change (underlined) to the ARQ Feedback Payload format in Table 190 on page 282 of IEEE 802.16Rev2/D1:

| Syntax | Size | Notes |
| --- | --- | --- |
| ARQ_Feedback_Payload_Format() { |  |  |
| frame_num | 8 | 8 lsb’s of latest frame number taken into account when this message was generated. |
| do |  |  |
| ARQ_Feedback_IE(last) | variable | Insert as many as desired, until last==TRUE. See 6.3.4.2. |
| until (last) |  |  |
| } |  |  |