| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | OFDMA PHY (section 8.4), FDD/HFDD Specifications Augmentation |  |
| Date Submitted | 2007-11-14 |  |
| Source(s) | Dan Gal (Alcatel-Lucent ) | dgal@alcatel-lucent.com |
| Re: | IEEE 802.16Rev2/D1, Letter Ballot 26. Technical Comments |  |
| Abstract | The current draft Standard does not provide adequate specifications for ensuring FDD/HFDD operation of systems based on OFDMA-PHY. The contribution proposes augmentation to the frame structure. |  |
| Purpose | For discussion and adoption as the basis for subsequent text changes in P802.16Rev2 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

OFDMA PHY (section 8.4), FDD/HFDD Specifications Augmentation

Dan Gal, Alcatel-Lucent

# 1.	 Introduction

In the current draft Standard (802.16Rev2/D1), the OFDMA PHY (section 8.4) supports both TDD and FDD frequency duplexing modes. However, there is inadequate specification detail required to implement interoperable FDD systems consisting of a mix of HD (Half-Duplex) and FD (Full-Duplex) mobile stations. To augment the Standard, this proposal defines a detailed OFDMA FDD/HFDD frame structure for a likely deployment scenario consisting, primarily, of HD as well as FD MSs.

The contribution is designed to facilitate discussion and drive consensus. Subsequent text changes to the draft Standard would be generated.

## 2.	 Proposal

Insert the following text at the beginning of  section 8.4.4.2 PMP frame structure

“The frame structure of OFDMA FDD systems shall accommodate the simultaneous operation of both FD-FDD (full duplex) and H-FDD subscriber stations. FDD base stations shall be full duplex. The downlink and uplink frames shall be of the same length. Upon network entry, the SS/MS shall identify itself as either FD-FDD or H-FDD transceiver. The BS downlink and uplink allocation scheduling shall include allowance for H-FDD transitioning from Tx (transmit) mode to Rx (receive) mode and vice versa as well as to switch the frequency of operation (FDL or FUL).  As a general rule, H-FDD MS shall be in receive mode except when they are scheduled or allowed to transmit.

The BS uplink frame timing shall take into account the round trip delay (RTD) associated with the farthest MS location in the cell (6.66 μsec RTD for each 1 Km of MS to BS distance). The MS shall align its UL frame with that of the MS which is located at the edge of the cell.

The FDD frame structure shall maximize the utilization of the BS UL resources by properly coordinating the DL and UL allocations for H-FDD MSs. Figure 1 describes the frame structure of a system comprising of both H-FDD and FD-FDD MS. To this end, allocations for H-FDD MS shall be consistently be made in either one of two regions in the frame referred to as group 1 and group 2 regions.

Figure 1: FDD Frame Structure Optimized for Half-Duplex MSs

The frame is configured into four regions;

- Frame control region, consisting of the Preamble, FCH and MAP (including two sub-MAPS)

- HD group 1 region (shown in Fig.1 as A (Rx) and B (Tx)

- HD group 2 region (shown in Fig.1 as B (Rx) and A (Tx)

- UL control region, shared by both H-FDD and FD-FDD MSs.

Figure 2 is similar to figure 1 except that it also shows the one-way and round-trip-delay (RTD) offset of the MS-received DL frame and BS-received UL frame, respectively.

Figure 2: FDD Frame Structure Optimized for Half-Duplex MSs, RTD Offset Shown

The transition gaps (MS) and the transition gap allowances (BS) shown in figure 1-4 (MS-Tx) apply to all H-FDD MSs.  The transition gaps (MS) and the transition gap allowances (BS) shown in figure 1-3 (MS-Rx) apply only to those H-FDD MSs whose allocations border with the control region. All H-FDD MS that transition within the group region do so without the need for BS allowance.

The size (in symbols) and location within the frame of each region shall be configurable and defined in the MAP (as size symbol offsets from the frame beginning edge).

A change of an MS’s group region allocation should be done at frames in which the MS has no data to receive or transmit.

An FD-FDD MS shall be able to get simultaneous allocations in both group regions of the frame as well as in the DL symbols corresponding to the control region. “