| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Computation of CINR Standard Deviation for Band AMC Operation |  |
| Date Submitted | 2008-03-17 |  |
| Source(s) | Chandy Sankaran Joe Schumacher Motorola | Voice: +1 (847) 632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Revision 2 |  |
| Abstract | The existing algorithm for calculating the standard deviation statistic for CINR samples effectively renders the Band AMC mode of operation unusable. A modification is proposed that remedies this problem. |  |
| Purpose | Review and approve for 802.16 Revision 2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Computation of CINR Standard Deviation for Band AMC Operation

Chandy Sankaran, Joe Schumacher

Motorola

# Introduction

Two conditions need to be satisfied for a mobile to request a transition into band AMC mode (from PUSC mode):

- The average CINR of the whole bandwidth should be larger than the band AMC entry average CINR for at least band AMC allocation timer frames.

- The maximum of the standard deviation of the individual band’s CINR measurements should be lower than the band AMC allocation threshold (σMAX) for at least band AMC allocation timer frames.

The method for computing the standard deviation as outlined in IEEE P802.16 (e.g., 802.16e-2006, Rev2/D3) is performed using linear values of CINR moments and not decibel values of the CINR moments. This causes a problem when checking for condition (II) above.

# Problem Definition

In the current version of the standard, the standard deviation is to be computed in the following manner:

- Compute the 1st moment of CINR as:

- Compute the 2nd moment of CINR and the standard deviation as:

The problem with the above method is that  and are not computed as decibel values. This makes it difficult to use a single value of σMAX for a range of CINR values as shown below:

  - Let xn, k = (                + ε) be the mean CINR for frame n in band k. Assume x2[n−1,k]=μ2[n−1,k]. Then:

- = (α. xn, k + (1-α).                 )

- =                       = 0.43ε  (assume α = 0.75)

  - Let σMAX = 6 dB. Then allowed pairs of                   and xn, k are:

- = 0dB        xn, k < 10dB

- = 10dB      xn, k < 12.8dB

- = 15dB      xn, k < 15.5dB

- = 20dB      xn, k < 20.4dB

- = 30dB      xn, k < 30.04dB

Thus in the current standard, σMAX needs to be high before an MS with high CINR will request AMC:

  - High σMAX      low CINR users that cannot effectively use AMC are more likely to request AMC (AMC helps deserving low CINR users)

  - Given a mix of users with different speeds, it becomes difficult to identify users that can effectively use AMC and users that cannot effectively use AMC.

Figure 1: Allowed CINR variation for band AMC operation

To illustrate this behavior, Figure 1 shows the allowed CINR variation for an MS to be able to request band AMC transition. The red line is the CINR value and the bounding blue lines are the allowed CINR variation. The blue line is 3σMAX, linear of the red line. Assuming a Gaussian CINR distribution, this should capture more than 99.9% of the expected distribution of the CINR. It is clear from this figure that low CINR users have a larger allowed variation as compared to high CINR users.

Figure 2 illustrates how the proposed solution fixes the problem described earlier.

Figure 2. Illustration of proposed solution

# Proposed Resolution

The proposed solution is to compute  and as decibel values:

- Compute the 1st moment of CINR as:

  - where:

- Compute the 2nd moment of CINR and the standard deviation as:

  - and standard deviation as:

# Text Changes

In equations 160 and 161, indicate that the computations are performed using decibel values for the mean and standard deviation, i.e.,:

[Change equation 160 on page 1003 as indicated:]

μCINR,dB[k]=CINRdB[0]k=0(1−αavg)μCINR,dB[k+1]+αavgCINRdB[k]k>0

where

CINRlinear[k] is a linear measurement of CINR for the k-th measurement; and n is number of consecutive frames in which no measurement is made. In frames where no measurement is made, the MS shall report the latest averaged results., and

CINRdB[k]=10log10(CINRlinear[k]).

[Change equation 161 on page 1003 as indicated:]

χCINR,dB2[k]=(CINRdB[0])2k=0(1−αavg)χCINR,dB2[k+1]+αavg(CINRdB[k])2k>0