| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Diversity Zone for Handoff |  |
| Date Submitted | 2007-11-2 |  |
| Source(s) | Kamesh Medepalli (Beceem Communications) | kmedepalli@beceem.com |
|  | Djordje Tujkovic (Beceem Communications) | dtujkovic@beceem.com |
|  | Sriram Mudulodu (Beceem Communications) | smudulodu@beceem.com |
|  | Louay Jalloul (Beceem Communications) | ljalloul@beceem.com |
|  | Frank Zhou (Beceem) | fzhou@beceem.com |
| Re: | IEEE 802.16Rev2/D0b – Improving reliability and latency of the Handoff procedure. |  |
| Abstract | This contribution proposes a simple mode of Handoff operation that uses macro diversity to improve reliability of the Handoff process in the P802.16Rev2 standard. |  |
| Purpose | Adopt the proposed solution in the P802.16Rev2 draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Diversity Zone for Handoff

Beceem Communications

# Problem Statement

The current IEEE 802.16 standard specifies three different methods for HandOff (HO), each with its own advantages and disadvantages. The main objective of this contribution is to augment the HO procedures defined in the standard by introducing the concept of a diversity HO zone which combines the benefits of different HO forms. The current IEEE 802.16 standard specifies the following:

  - Hard Handover from one BS (serving BS) to another BS (target BS)

    - HO happens by the MS and BS exchanging MAC management messages

    - The MS essentially switches from one BS to another

    - This is the only form of Handoff that is part of the WiMAX profile

  - Fast Base Station Switching (FBSS)

    - Still a hard handover, i.e., it is a break-then-make form of handover

    - But the MS switches from one BS to another in a fast, proactive manner

    - Not part of the WiMAX profile

  - Macro Diversity Handover (MDHO)

    - Unlike the above two, it is a make-before-break handover

    - Multiple BS-s collectively transmit the same information to the MS

    - MS typically combines the received signals using separate FFT for each BS

    - Diversity selection is done on the Uplink

    - Not part of the WiMAX profile

Each of the three methods have their own share of advantages and disadvantages, which we list below:

Hard Handover from one BS to another BS (default method)

The main advantage of the HHO method is that it is the simplest of all the three, both from BS and ASN standpoint as well as MS standpoint. However, there are several deficiencies of this approach. The HO signaling includes multiple UL messages which need contention based BW allocation. As a result, HO latency can be high and the HO procedure may be unreliable. As a result, the BS and/or MS needs proper choice of HO triggers and frequency planning to achieve reasonable performance. It is also important to note that this technique does not leverage diversity combining (across BS-s). It is only diversity switching.

Fast Base Station Switching (FBSS)

The main advantage of FBSS is that it can potentially be faster than HHO as the CQICH channel can be used for HO signaling. However, this procedure requires accurate tuning of add/delete thresholds for Active set maintenance (i.e., the set of potential Anchor BS-s). Like HHO, this too does not leverage diversity combining (across BS-s). It is only diversity switching

Macro Diversity Handover (MDHO)

The main advantage of MDHO is that it is the only diversity combining HO technique described in the standard. As a result, MDHO can lead to improved reliability of the overall HO process as the interfering signal becomes desired signal (interfering BS/sector transmits the user information along with the current serving BS). This advantage, however, comes with the disadvantage that it results in a significant increase in complexity on the MS side. The allocations from different BS-s overlap in the same “logical” portion of the time-frequency rectangle while each BS continues to use its own DL permutation to send traffic to the MS. This means that MS needs to track channel from individual base stations, thereby increasing the complexity of the mobile station’s receiver. Moreover, the other disadvantage of MDHO is that the MS and BS constantly maintain an active set which is the set of BS-s that can potentially participate in the MDHO handover with the MS. This results in additional overhead and complexity to the system (FBSS also has this disadvantage).

In the next section we describe the method for improving the reliability and latency of the Handoff process using the diversity HO zone.

# Details of the Diversity HO Zone

In the diversity HO zone, all BS-s (sectors) involved in transmitting the signal to the BS use the same 	DL sub-carrier permutation. This results in increased CINR and reliability of messages/traffic to the MS. 	On the Uplink, selection diversity can be used by the BS-s to improve the quality. The figures below 	illustrates the diversity zone concept.

BS 1

BS 2

BS 3

MS1

S2

S2

S2

S1

S1

S1

S0

S0

S0

BS1

BS2

BS3

FCH

Maps

Data

for

MS1

FCH

Maps

Data

for

MS1

BS1, Sector 2

BS2, Sector 0

Different physical and/or logical sub-channels in all other zones

Identical physical AND logical sub-channels in Diversity HO zone

The group of BS-s participating in the diversity HO zone for an MS can be done in two ways. First, is to 	reuse the existing active set maintenance procedures described for MDHO (same applicable for FBSS). 	Second is to choose a pre-determined the pattern of BS-s that participate in the zone. For example, in the 	figure above, BS1 sector 2, BS2 sector 0 and BS3 sector 1 can form one group. The latter approach is 	clearly sub-optimal with respect to the set of sectors that are best suited for an MS. However, it incurs 	very little overhead, relatively.

# Proposed text changes to the 802.16Rev2 draft

The allocations in the diversity HO zone shall be made using the Diversity_HO_zone_IE() and the zone 	itself shall be described using the STC_DL_Zone_IE(). The Diversity_HO_zone_IE() shall be sent only 	by the Anchor BS and the format shall be as specified below:

| Syntax | Size | Notes |
| --- | --- | --- |
| Diversity_HO_Zone_IE() { |  |  |
| Extended-2 DIUC | 4 bits | Diversity_HO_Zone_IE()=0x0D |
| Length | 8 bits | -- |
| DIUC | 4 bits | -- |
| CID | 16 bits |  |
| OFDMA Symbol Offset | 8 bits | The offset of the OFDMA symbol in which the burst starts, measured in OFDMA symbols from beginning of the downlink frame in which the DL-MAP is transmitted. |
| Subchannel offset | 6 bits | The lowest index OFDMA subchannel used for carrying the burst, starting from subchannel 0. |
| Boosting | 3 bits | Refer to Table 273 |
| No. OFDMA Symbols | 6 bits | -- |
| No. Subchannels | 6 bits | -- |
| Repetition Coding Information | 2 bits | 0b00—No repetition coding 0b01—Repetition coding of 2 used 0b10—Repetition coding of 4 used 0b11—Repetition coding of 6 used |
| If !(byte boundary) { |  |  |
| Padding Nibble | Variable | Padding to reach byte boundary |
| } |  |  |
| } |  |  |