| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CQICH allocation request header correction |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Kiseon Ryu LG Electronics | Voice: +82-31-450-2067 E-mail: ksryu@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16 Working Group Letter Ballot #26 |  |
| Abstract | This document proposes fix of CQICH allocation request header. |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CQICH allocation request header correction

Kiseon Ryu,

LG Electronics

# Problem statement

In the current CQICH allocation request header, there is a problem for MS to use CQICH allocation request header.

Definition of feedback type in CQICH allocation request header refers to wrong number of table 474 which is UL Allocation Start IE. And there are two methods for BS to allocate CQICH to MS, which uses CQICH allocation IE and CQICH enhanced allocation IE. The current definition of Feedback type in CQICH request header can refer to only one IE between two UL-MAP IEs. So, It is needed to redefine the fields in CQICH allocation request header, for being compatible with both CQICH allocation IE and CQICH enhanced allocation IE.

# Proposed Correction

We propose to redefine the feedback type in CQICH allocation request header so that MS can request CQICH feedback type of CQICH allocation IE and CQICH enhanced allocation IE.

# Suggested Changes in Rev2/D1

[In Rev2/D1, on page 59, replace Figure 30 as follows]

Figure 30— CQICH allocation request

[On page 59, modify the table 12 as follows]

Table 12 — Description of the fields of CQICH allocation request header

| Name | Length (bit) | Description |
| --- | --- | --- |
| Type | 3 | The type of CQICH allocation request header is defined in Table 8 |
| Feedback Type | 3 | Set according to feedback type defined in Table 474. When FBSSI is set to 1, this field is neglected. |
| CQICH allocation type | 1 | 0 : CQICH allocation IE 1 : CQICH enhanced allocation IE |
| Feedback Type I | 2 | Feedback type for CQICH allocation IE 0b00 : PCINR Feedback 0b01 : ECINR Feedback 0b10 - 0b11: reserved |
| FBSSI | 1 | FBSS Indicator: Set when MS request CQICH during FBSS HO. |
| Preferred-Period(=p) | 3 | CQICH allocation period MS prefers. The value is defined in units of frame. When FBSSI is set to 1, the value contained in this field shall be neglected. |
| MIMO permutation feedback cycle | 2 | Set according to MIMO permutation feedback cycle defined in Table 470. When CQICH allocation type is set to 1, this field is neglected. |
| Feedback Type II | 3 | Feedback type for CQICH enhanced allocation IE. Set according to feedback type defined in Table 475. When CQICH allocation type is set to 0, this field is neglected. |
| Reserved | 12 7 | Shall be set to zero. |
| CID | 16 | MS basic connection identifier. |
| HCS | 8 | Header check sequence (same usage as HCS entry in Table 6). |