| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Idle Mode Retain Information |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Joe Schumacher Motorola* | Voice: +1-847-632-5978 E-mail: j.schumacher@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26, comment #36 on Rev 2/D1. |  |
| Abstract | Numerous inconsistent references are made to “Full service” in idle mode retention sections. |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Idle Mode Retain Information

Joe Schumacher

Motorola

# Introduction

A correction was made for Bit #6 in the "Idle Mode Retain Information" field in the Corrigendum 2 project regarding the retention of "MAC state machines, CS classifier information, etc.). However, the corresponding change was not made for Bit #7.

Reference is made to “Full Service” in the context of retaining state information in several areas of the document. This term needs to be clarified and used consistently throughout the standard.

# Discussion

Some aspects of the MS context may be stored in the network when the MS enters idle mode, depending on the negotiated values of the Idle Mode Retain Information TLV. Although some of the information retained by the BS may be implementation specific, certain elements should be specified to promote interoperability.

This contribution makes three general proposals:

- Replace the term “full service” with the term “non-idle service”

- Combine the definition of Bit #6 in the Idle Mode Retain Information TLV (see 6.3.2.3.26) to specify the retained information with the definition in 11.14 DREG-CMD/REQ message encodings. The definition in 6.3.2.3.26 was approved as part of the Corrigendum 2 project.

- Move the above definition to 11.14 DREG-CMD/REQ message encodings and reference this section wherever Idle Mode Retain Information is referenced.

# Resolution

6.3.2.3.26 DREG-CMD (de/reregister command) message

[change lines 34-53 on page 139 as indicated:]

Bit #6: Retain MS service and operation information associated with Full service non-idle service (see 11.14). This information does not include blocks currently in the ARQ window and associated timers. The MS service and operation information does include SFIDs and related description (QoS descriptors and CS classifier information) for all Service Flows that the MS has currently established as well as any SAs with their related keying information. The information retained by setting Bit #6 does not include those information associated with SBC-REQ/RSP messages, PKM-REQ/RSP messages, REG-REQ/RSP messages, Network Address, Time of Day, and TFTP messages unless otherwise specified by setting one or more Bits #0–#5.

Bit #7: Consider Paging Preference of each service flow in resource retention. Bit #7 is meaningful when Bit #2 and Bit #6 have a value of 1. If Bit #2, Bit #6 and Bit #7 is 1, MS service and operational information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see 11.14) are retained for service flows with positive Paging Preference. If Bit #2 and Bit #6 are 1 and Bit #7 is 0, MS service and operational information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see 11.14) are retained for all service flows.

6.3.2.3.42 DREG-REQ (SS deregistration request) message

[change lines 21-36 on page 168 as indicated:]

Bit #6: Retain MS service and operation information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see 11.14). The information retained by set-ting Bit #6 does not include those information associated with SBC-REQ/RSP messages, PKM-REQ/RSP messages, REG-REQ/RSP messages, Network Address, Time of Day, and TFTP messages unless otherwise specified by setting one or more Bits #0–#5.

Bit #7: Consider Paging Preference of each service flow in resource retention. Bit #7 is meaningful when Bit #2 and Bit #6 have a value of 1. If Bit #2, Bit #6 and Bit #7 is 1, MS service and operational information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see 11.14) are retained for service flows with positive Paging Preference. If Bit #2 and Bit #6 are 1 and Bit #7 is 0, MS service and operational information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see 11.14) are retained for all service flows.

11.14 DREG-CMD/REQ message encodings

[change “notes” column of table in 11.14, lines 23-55, “Idle Mode Retain” field as indicated:]

Bit 6: Retain MS service and operational information associated with full service (e.g., MAC state machines, CS classifier information). The information retained by setting bit 6 does not include the information associated with SBC-REQ/RSP messages, PKM-REQ/RSP messages, REG-REQ/RSP messages, network address, time of day, and TFTP messages unless otherwise specified by setting one or more bits 0–5. This information does not include blocks currently in the ARQ window or associated timers. The MS service and operation information does include SFIDs and related description (QoS descriptors and CS classifier information) for all Service Flows that the MS has currently established as well as any SAs with their related keying information. The information retained by setting Bit #6 does not include those information associated with SBC-REQ/RSP messages, PKM-REQ/RSP messages, REG-REQ/RSP messages, Network Address, Time of Day, and TFTP messages unless otherwise specified by setting one or more Bits #0–#5.

Bit 7: Consider Paging Preference of each Service Flow in resource retention. Bit #7 is meaningful when Bit #2 and Bit #6 have a value of 1. If Bit #2, Bit #6 and Bit #7 is 1, MS service and operational information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see above) are retained for Service Flows with positive Paging Preference. If Bit #2 and Bit #6 are 1 and Bit #7 is 0, MS service and operational information associated with Full service (MAC state machines, CS classifier information, etc.) non-idle service (see above) are retained for all Service Flows.

14.2.7.2.4 C-NEM-RSP (Action_Type = Deregistration)

[change lines 36-3,9 page 1680 as indicated:]

Bit #6: Omit TFTP management messages during re-entry processing

Bit #7: Full service and operational state transfer or sharing between serving BS and target BS (ARQ, timers, counters, MAC state machines, etc) non-idle service (see 11.14).