| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections for closed-loop MIMO codebooks in 802.16e |  |
| Date Submitted | 2008-01-07 |  |
| Source(s) | Ayelet Doron, Intel Corporation Qinghua Li, Intel Corporation | ayelet.doron@intel.com qinghua.li@intel.com |
| Re: | 802.16 Maintenance Task Group |  |
| Abstract | This contribution provides some typo corrections for 802.16e section 8.4.5.4.10.15 |  |
| Purpose | For discussion and approval by maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Corrections for closed-loop MIMO codebooks in 802.16e

Ayelet Doron, Qinghua Li, Intel Corporation

# 1. Purpose

In 802.16e section 8.4.5.4.10.15 "MIMO feedback for Tx beamforming", some mistakes were inserted. This contribution provides the needed corrections for generating the right CL-MIMO codebooks.

# 2. Details

The needed corrections are as follows:

- Page 786, line 11-12, Table 393—Operations to generate codebooks.  The row with “2, 6  |  H(V(2,1,6))  | — |  —” for Nt=2, L=6, i.e. 2 transmit antennas and 6-bit feedback codebook, should be deleted.  The 6-bit codebook is not defined in the standard and 3-bit codebook performance is close to the ideal in this case.

- Page 784, line 63-64, the last row on the page.  For the last term on the right hand side of the first equation, the index of v tilde should be changed from “i” to “1” as in the accepted contribution C802.16e-05/50r7.

- Page 784, line 63-64, the last row on the page.  For the second term on the right hand side of the first equation, the power of Q(u) should be changed from “i” to “i-1”.  Since the 2^L-th power of Q(u) is the identity matrix, the error makes the first and the last codewords equal.

- Page 784, line 63-64, the last row on the page.  For the second term on the right hand side of the third equation, the phase exponent of “e” should be changed from “\phi_1” to “\phi_i” as in the accepted contribution C802.16e-05/50r7.  Since the phase is to make the first entry of each codeword to be real, the phase should change with the codeword index.

After the correction of items 2, 3, and 4, the equations should look like (the changes are highlighted in yellow):

v1=1Nt1 ej⋅2πNt ... ej⋅2πNt⋅(Nt−1)T

vi=H(s)Qi−1(u)HH(s)v1	for i=2,....2L

vi=vie−j⋅Φi					for i=2,....2L

# 3. Text Changes

# In section 8.4.5.4.10.15 on page 786 of [1], Table 393—Operations to generate codebook, delete row as follows

| Nt, L / S | 2 | 3 | 4 |
| --- | --- | --- | --- |
| 2, 3 | HV2,1,3 |  |  |
| 3, 3 | HEV3,1,3 | HV3,1,3 |  |
| 4, 3 |  | HEV4,1,3 | HV4,1,3 |
| 2, 6 | HV2,1,6 |  |  |
| 3, 6 | HCV3,1,3,V2,1,3 | HCV3,1,3,HV2,1,3 |  |
| 4, 6 | HCV4,1,3,V3,1,3 | HEV4,1,6 | HV4,1,6 |

# In section 8.4.5.4.10.15 on page 784 of [1] the last row on the page should be changed to

vi=H(s)Qi−1(u)HH(s)v1	for i=2,....2L

vi=vie−j⋅Φi					for i=2,....2L

# Reference

[1] Part 16: Air Interface for Broadband Wireless Access Systems P802.16Rev2/D2 (December 2007)