| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections for closed-loop MIMO codebooks |  |
| Date Submitted | 2008-01-07 |  |
| Source(s) | Ayelet Doron, Intel Corporation Qinghua Li, Intel Corporation | ayelet.doron@intel.com qinghua.li@intel.com |
| Re: | IEEE802.16Rev2/D2 |  |
| Abstract | This contribution provides equation corrections for IEEE802.16Rev2/D2 section 8.4.5.4.10.15 |  |
| Purpose | For discussion and approval by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Corrections for closed-loop MIMO codebooks

Ayelet Doron, Qinghua Li, Intel Corporation

# 1. Purpose

When generating IEEE802.16e section 8.4.5.4.10.15 "MIMO feedback for Tx beamforming", some mistakes in the equations were not corrected at that time. This contribution provides the needed corrections for generating the right CL-MIMO codebooks.

# 2. Details

The needed corrections are as follows:

- Page 784, line 64, equation indices, vi and “un-bold s” are incorrect. Instead of:

vi=H(s) Qiu HHs⋅ vi,  for  i=2,…2L,     vi=vie−jφ1,   for i=2,…2L

should be:

vi=H(s) Qi−1u HHs v1,      for  i=2,…2L

vi=vie−jφi,                            for  i=2,…2L

- Page 785, line 8, wrong parenthesis. Instead of:

u=u1⋯uNt

should be:

u=u1⋯uNt

- Page 786, line 11-12, Table 393: The row starting with “2, 6  |  H(V(2,1,6))  | — |  —” for Nt=2, L=6, i.e. two transmit antennas and 6-bit feedback codebook, should be deleted, since the 6-bit codebook is not defined in the standard.

- Text Changes

[On page 784, line 64, replace equation]:

vi=H(s) Qiu HHs⋅ vi,  for  i=2,…2L,     vi=vie−jφ1,   for i=2,…2L

with:

vi=Hs Qi−1u HHs v1,      for  i=2,…2L

vi=vie−jφi,                            for  i=2,…2L

[On page 785, line 8, replace]

u=u1⋯uNt

with:

u=u1⋯uNt

# [On page 786, Table 393, delete 4th row as indicated]

| Nt, L / S | 2 | 3 | 4 |
| --- | --- | --- | --- |
| 2, 3 | HV2,1,3 |  |  |
| 3, 3 | HEV3,1,3 | HV3,1,3 |  |
| 4, 3 |  | HEV4,1,3 | HV4,1,3 |
| 2, 6 | HV2,1,6 |  |  |
| 3, 6 | HCV3,1,3,V2,1,3 | HCV3,1,3,HV2,1,3 |  |
| 4, 6 | HCV4,1,3,V3,1,3 | HEV4,1,6 | HV4,1,6 |