| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions for transparent transmit diversity |  |
| Date Submitted | 2008-03-20 |  |
| Source(s) | Yuval Lomnitz, Intel Corp Dov Andelman, Intel Corp Frank Zhou, Beceem Fred Vook, Jeff Zhuang, Motorola Ji-Yun Seol, Samsung Philippe Sehier, Dan Gal, Alcatel-Lucent Sylvain Labonte, Sequans Peretz Shkalim, Runcom Hang Jin, Herbert Ruck, Cisco Systems Weili Ren, Yuefeng Zhou, Nader Zein, Tomohiro Kikuma, NEC | yuval.lomnitz@intel.com dov.andelman@intel.com fzhou@beceem.com fred.vook@motorola.com jiyun.seol@samsung.com Philippe.Sehier@alcatel-lucent.fr sylvain@sequans.com peretzs@runcom.co.il hang_jin@navini.com weili.ren@nectech.co.uk |
| Re: | Letter Ballot 26b |  |
| Abstract | Currently CDD (cyclic delay diversity) is not defined in the 802.16 standard however widely used in practice. This contribution attempts to bridge this gap by formally defining transparent CDD and supplying necessary restrictions and definitions. |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Definitions for transparent transmit diversity

# Yuval Lomnitz, Dov Andelman, Intel

# Frank Zhou, Beceem

Fred Vook, Jeff Zhuang, Motorola

# Ji-Yun Seol, Samsung

# Philippe Sehier, Dan Gal, Alcatel-Lucent

# Sylvain Labonte, Sequans

# Peretz Shkalim, Runcom

# Hang Jin, Herbert Ruck, Cisco Systems

# Weili Ren, Yuefeng Zhou, Nader Zein, Tomohiro Kikuma, NEC

# Purpose

Currently CDD (cyclic delay diversity) is not defined in the 802.16 standard (OFDMA PHY) however widely used in practice in WiMAX system. This contribution attempts to bridge this gap by formally defining transparent CDD and supplying necessary restrictions. Although there is an effort to make these definitions in RPD in WiMAX forum we believe the correct place for them is the 802.16 standard since:

- These definitions are necessary for the interoperability of devices, not only in the scope of WiMAX.

- In order to refrain from contradictions between WiMAX and 802.16

This revision incorporates text from C802.16maint-08/070r2 to announce CDD parameters in the DCD and address issues raised in comment 2166.

# Discussion

## CDD enablement

Currently CDD (cyclic delay diversity) is not defined in the 802.16 standard. The reasons to use CDD in practice are many: first, this way a multi-antenna BS is able to split the transmit power between several antennas (and power amplifiers), and this way a BS with 2 antennas having power P which is able to produce power 2P during a MIMO zone, is able to produce power 2P rather than P during the first (non-MIMO) zone. The second reason is the diversity gain from using CDD. Other reasons are the benefits to link adaptation, power control and handoff from having a correct measure of signal strength from the preamble. So our first target is to enable CDD operation formally.

## CDD definitions and restrictions

Assuming CDD operation is enabled, some limitations need to be considered in order to enable design of receivers in an interoperable manner. CDD is considered a “transparent” technique to the receiver however it is not entirely transparent. First of all, CDD appears as additional delay spread. If the receiver is designed to support a given delay spread assuming known channel models, it is not guaranteed that its performance will not be compromised when additional unexpected delay appears. This depends on the value of the delay, and therefore it should be limited.

## Targets of this contribution

The targets of this contribution are:

- To define that CDD (and any cyclic/linear delay or filtering) is allowed

- To define the restrictions on CDD that enable MS and BS to design their receivers.

- To specify a CDD announcement TLV that enables performance improvement by specifying the CDD parameters to the MS.

# Text Changes

[add a new subclause]

8.4.8.7 Cyclic delay diversity (CDD)

The BS and MS may apply the cyclic delay diversity (CDD) technique in order to improve performance or split the power between multiple transmit antennas. The same signal (including data, pilots, preamble, midamble, etc) may be transmitted from several antennas simultaneously, with different cyclic delay applied to each signal in order to reduce the potential of nulling in the receiver's antenna. Cyclic delay means the samples in the useful symbol time Tb are shifted D samples forward, the last D samples are copied to the first D samples, and the CP is regenerated from the last samples of the rotated symbol. An equivalent description is to multiply all subcarriers by wk=exp−j2πDkN. Each transmit antenna uses a different delay value and potentially a different gain and phase.

In case of applying such techniques then in all references to "antenna 0" "antenna 1" and so forth, in the rest of section 8.4, shall refer to a logical antenna, representing the signal defined in 8.4.2.5, which may be transmitted from several physical antennas. An example is shown in figure YYY below.

Figure YYY – Example of single logical antenna split into two physical antennas with cyclic delays

For example, a 2 antenna BS using a 2 antenna STC zone, may use CDD in the first zone (with a single logical antenna) in order to effectively split the transmitted power between the two antennas and enjoy diversity gain.

The following requirements apply to CDD:

- The CDD parameters and arrangement of antennas shall not change within the zone. For this purpose the preamble is considered a part of the first PUSC zone.

- The CDD parameters (delays and powers) and arrangement of antennas shall be the same over zones which have broadcast pilots (dedicated pilots=0) and have the same number of antennas specified in the STC zone IE, over all frames. For this purpose the first PUSC zone is considered to have dedicated pilots=0 and number of antennas equals 1.

- All references to the power transmitted from an antenna (antenna #k) shall be interpreted as the total power transmitted from physical antennas associated with logical antenna #k

- Zone boosting limitations of 8.4.9.6 shall be kept with respect to the total power as defined above.

- The maximum delay in any physical antenna shall not exceed 1% of the useful symbol time Tb in any zone. The delay shall always be positive.

- If the power used by physical antennas associated with the same logical antenna is not equal, it is recommended that antennas transmitting higher power shall use smaller delay values.

[append the  following text to Table 544 at Page 1072, section 11.4.1]

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| CDD SISO Descriptor | 155 | variable | This TLV may be transmitted to specify CDD parameters to the MS. It applies to the first PUSC zone, as well as zones with STC=0b00 (1 logical antenna) and dedicated pilots=0 in STC DL zone IE. Each byte represents one CDD parameter: 5 LSBs for delay in samples (1 to 32) from physical antenna #0, 3 MSBs are reserved. If the Length of this CDD Descriptor is n bytes, then the number of Tx antenna is n+1. The power of each antenna is the same. | OFDMA |
| CDD STC Descriptor | 156 | variable | This TLV may be transmitted to specify CDD parameters to the MS. It applies to zones with STC=0b01 and 2/3 antennas select = 0b00 (STC using 2 antennas), i.e. 2 logical antennas, and dedicated pilots=0 in STC DL zone IE. Each byte represents one CDD parameter: 5 LSBs for delay in samples (1 to 32) from physical antenna #0, 3 MSBs are reserved. The same parameters apply to logical antenna 0 and 1. If the Length of this CDD Descriptor is n bytes, then the number of Tx antenna is 2(n+1). The power of each antenna is the same. | OFDMA |
| Second-tap CDD SISO Descriptor | 157 | Variable | This TLV may be transmitted to specify CDD parameters to the MS when two delay taps are used on each physical antenna. This TLV is used in conjunction with TLV 155 to specify the CDD parameters of the combined CDD scheme for SISO. TLV 155 is used to specify the first delay tap on each physical antenna. This TLV is then used to specify the second delay tap on each physical antenna. It applies to the first PUSC zone, as well as zones with STC=0b00 (1 logical antenna) and dedicated pilots=0 in STC DL zone IE. With M physical antennas, M+1 bytes are transmitted, where each of the first M bytes represents one CDD parameter: 6 LSBs for delay in samples (1 to 64) from the first lag of physical antenna #0, 2 MSBs indicate phase of the tap, where phase is restricted to {0, j, -1, -j}. On all physical antennas, the ratio of the power on first tap relative to the power on the second tap is indicated by the 4 LSBs (0 dB to 7 dB with 0.5 dB increment) of the M+1th byte, where the 4 MSBs are reserved. The sum of the power on the two taps shall equal one. | OFDMA |
| Second Tap CDD STC Descriptor | 158 | variable | This TLV may be transmitted to specify CDD parameters to the MS when two delay taps are used on each physical antenna. This TLV is used in conjunction with TLV 156 to specify the CDD parameters of the combined CDD scheme for STC zone. TLV 156 is used to specify the first delay tap on each physical antenna. This TLV is then used to specify the second delay tap on each physical antenna. It applies to zones with STC=0b01 and 2/3 antennas select = 0b00 (STC using 2 antennas), i.e. 2 logical antennas, and dedicated pilots=0 in STC DL zone IE. With M transmit antennas for each logical antenna, M+1 bytes are transmitted, where each of the first M byte represents one CDD parameter: 6 LSBs for delay in samples (1 to 64) from the first lag of physical antenna #0, 2 MSBs indicate phase, where phase is restricted to {0, j, -1, -j}. On all physical antennas, the ratio of the power on first tap relative to the power on the second tap is indicated by the 4 LSBs (0 dB to 7 dB with 0.5 dB increment) of the M+1th byte, where the 4 MSBs are reserved. The sum of the power on the two taps shall equal one. | OFDMA |