| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions for AMC permutation slot structure in MIMO zone |  |
| Date Submitted | 2008-01-10 |  |
| Source(s) | Yuval Lomnitz, Intel Corporation | Yuval.lomnitz@intel.com |
| Re: | IEEE802.16Rev2/D2 |  |
| Abstract | Completing definitions of AMC for MIMO zone |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Definitions for AMC permutation slot structure in MIMO zone

Yuval Lomnitz, Intel Corporation

# Purpose

In 802.16, three AMC slot structures are defined for SISO (1x6, 2x3, 3x2) but MIMO mapping (matrix A,B) is not defined for them. Rather, a different AMC slot (2x6) is defined for STC. This slot is not specified in the map (the zone switch IE only supports 1x6, 2x3, 3x2). We suggest completing the definition by supporting 2x6 directly in the map, and completing the definitions of the other slot sizes for MIMO.

- Text Changes

8.4.5.3.4 STC/DL Zone Switch IE format

[Change as follows in Table 327 (OFDMA STC DL Zone IE format)]

| AMC type | 2 | Indicates the AMC type in case permutation type = 0b11, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved 2x6 (only for STC using 2 antennas) Note that only 2x3 band AMC subchannel type (AMC Type = 0b01) is supported by MS |
| --- | --- | --- |

8.4.8.3.1.2.1 STC Mapping for optional AMC permutation

[Change the subclause as follows]

For the optional AMC permutation in STC zone, the data subchannels shall take an additional 2x6 (2 bins for 6 symbols) format may be used, in addition to the 1x6, 2x3 and 3x2 slot structures. The subcarrier permutation represented by Equation (88) in 8.4.6.3 shall not be applied for the optional AMC permutation within STC zones. The pilot pattern of 8.4.8.3.1.1 is used for all slot structures. The 2x6 slot contains 96 subcarriers and is mapped into 2 or more logical slots (according to the MIMO matrix) for the channel encoding operation (see 8.4.8.1.2.1.3)

For 2-antenna matrix A in 8.4.8.3.3, STC encoded data symbols shall be time mapped starting over the first 2 OFDMA symbols. The mapping starts at the lowest numbered subcarriers of lowest slot and continues in an ascending manner in subchannels first and then proceeds to the next two symbols in time. An illustration of the mapping rule for the antenna #0 is shown in Figure 276, assuming 2 Tx with Matrix A for a block of 2 slots a 2x6 AMC slot. The mapping of matrix A for the 3x2 slot is the same as the mapping for PUSC in Table 537. Matrix A is not supported for the 2x3 slot.

For 2-antenna vertically encoded matrix B in the optional AMC permutation, modulated data symbols shall

be sequentially mapped for two Tx antennas along the subcarriers of the first symbol in antenna-first order. The mapping

continues in an ascending manner in subchannels subcarriers first and then proceeds to the next symbol in time. An

illustration of the mapping rule for the antenna #0 in AMC2x6 is shown in Figure 277, assuming 2 Tx with vertically

encoded matrix B for a block of 2 slots. Figure 277 also shows the mapping rule for 2-antenna horizontally

encoded matrix B in the optional AMC permutation with AMC2x6, where each encoded stream is separately mapped to the corresponding antenna. The mapping of matrix B data for the 3x2 slot is the same as the mapping for PUSC in Table 537. The mapping of matrix B data for the 3x2 slot is the same as the first 3 symbols shown in Figure 277.

For a 3- or 4-antenna matrix A and matrix B in 8.4.8.3.4 and 8.4.8.3.5, STC encoded data symbols shall be

mapped at two adjacent subcarriers over two OFDMA symbols. When the subcarrier pair (over two

symbols) at frequency k+1 is allocated to pilots for antenna #0 or #1 and the pair at frequency k+2 is

allocated to pilots for antenna #2 or #3, then the pair at frequency k+3 shall be jointly encoded with the pair

at frequency k. This is illustrated in Figure 278, where blocks of 2 convolutional coded (CC) slots and

convolutional turbo coded (CTC) slots are separately shown. The mapping starts at the lowest numbered

subcarriers of the lowest slot and continues in an ascending manner in subchannels first and then proceeds to

the next two symbols in time. For a 3- and 4-antenna vertically/horizontally encoded matrix C in the optional AMC permutation, the same mapping rule for 2-antenna vertically/horizontally encoded matrix B shall be applied on the same frequency-time block with the 3- or 4-antenna pilot pattern.

[Change the titles of figures 276, 277 as follows]

Figure 276—Data mapping in the optional AMC 2x6 Zone with 2 Tx antenna and matrix A

Figure 277—Data mapping in the optional AMC 2x6 zone with 2 Tx antenna and matrix B

[Repeat the following change in all the tables indicated below]

| Subsection | Table |
| --- | --- |
| 8.4.5.3.8 MIMO DL Basic IE format | Table 331—MIMO DL Basic IE format |
| 8.4.5.3 DL-MAP IE format | Table 320—OFDMA DL-MAP IE format |
| 8.4.5.3.9 MIMO DL Enhanced IE format | Table 333—MIMO DL Enhanced IE format |
| 8.4.5.3.17 MIMO in Another BS IE | Table 340—MIMO in Another BS IE format |
| 8.4.5.3.18 Macro-MIMO DL Basic IE format | Table 341—Macro MIMO DL Basic IE() |

[The change:]

| If (Permutation = 0b11 and (AMC type is 2x3, 2x6 or 1x6)) { | — | — |
| --- | --- | --- |