| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions for AMC permutation slot structure in MIMO zone |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Yuval Lomnitz Dov Andelman Intel Corp. Dave Petchner Gokhan Korkmaz Arraycom | Voice: +972-3-9205773 E-mail: Yuval.lomnitz@intel.com dov.andelman@intel.com E-mail: dpechner@arraycom.com gkorkmaz@arraycomm.com |
| Re: | Letter Ballot 26a |  |
| Abstract | Completing definitions of AMC for MIMO zone |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Definitions for AMC permutation slot structure in MIMO zone

Yuval Lomnitz, Dov Andelman - Intel Corp. 
Dave Petchner, Gokhan Korkmaz - Arraycom

# Purpose

In 802.16, three AMC slot structures are defined for SISO (1x6, 2x3, 3x2) but MIMO mapping (matrix A,B) is not defined for them. Rather, a different AMC slot (2x6) is defined for STC. This slot is not specified in the map (the zone switch IE only supports 1x6, 2x3, 3x2). We suggest completing the definition by supporting 2x6 directly in the map, and completing the definitions of the other slot sizes for MIMO.

The following modifications are included in rev2 of this contribution:

- The text regarding minimum allocation in PUSC STC zone with dedicated pilots is extended to AMC, since this is required for the cases where the slot duration is smaller than one pilot period

- We correct two flaws in the pilot patterns of AMC in MIMO zone:

- The pilots for antenna 1 are concentrated on symbols 0,2,4 and for antenna 2 on symbols 1,3,5. This results in power fluctuation of 1.6dB between the symbols which degrades performance, especially when this scheme is used for the UL.

- The diagrams are confusing since they include also the “original” pilot locations for SISO on top of the MIMO configuration.

- We correct the pilot power for UL AMC MIMO: the pilot boosting for UL AMC is the same as for DL AMC, and likewise for UL AMC with MIMO it should be the same as for DL.

- We apply the corrections for the UL as well, since they are relevant to AMC with collaborative MIMO or single user MIMO in the UL. Note that minimum allocation rule is not applied for the UL, since the BS itself determines the allocation so can guarantee sufficient size.

# Discussion

## Pilot pattern

In the current draft the pilots for antenna 1 are concentrated on symbols 0,2,4 and for antenna 2 on symbols 1,3,5. This results in power fluctuation of 1.6dB between the symbols when considering the power per antenna, which degrades performance since it requires the transmitter to lower its power so that the peak power on these symbols will not be exceeded. This is an issue especially when this scheme is used for the UL (MS 0,1 transmit transmits the pattern of antennas 0,1 and therefore there are power fluctuations in the transmit power of each MS). In all other MIMO and SISO pilot patterns in 802.16 the issue of equal power per symbol per antenna has been considered and it seems to have been mistakenly neglected here.

We propose to exchange the roles of the antennas in the bottom AMC bin in order to equalize the power used from each antenna. In the downlink, the effect of power fluctuations may be smaller since the BS may not be power limited, and if the pilots are dedicated then the precoding of pilots may lead to averaging of this effect. In the UL, the effect is 0.8dB improvement in the transmit power (since for average power P, in the current scheme the transmit powers on alternating symbols are P+0.8dB and P-0.8dB, and therefore P  PMAX-0.8dB).

We show below that this change doesn’t affect the performance. The performance is shown using multiplication noise model for modeling the effect of channel estimation loss, similar to the models used for 802.16m evaluation methodology. The effective SNR is used to calculate an effective SE which is averaged over the 18x6 subchannel. The channel estimation filter is 2D-LMMSE with flat power delay profile of 5s. We present results for two cases of velocity (120Km/h and 10Km/h, both at 3.5Ghz carrier for worst case), and two cases of power normalization:

- Non-power limited: the mean power of the symbols is normalized to 1 (the noise is at 2=1/SNRNTX)

- Power limited: the mean power of the worst case symbol is normalized to 1 (other symbols have lower power and suffer SNR loss).

We show that we have no degradation in case (1), and enjoy the gain of the additional power in case (2). For each case we present the mean SE and the ratio of the SE-s of the two schemes.

### Pilot patterns

### Case 1A: V=120Km/h, non power limited

### Case 1B: V=10Km/h, non power limited

### Case 2A: V=120Km/h, power limited

### Case 2B: V=10Km/h, power limited

Text Changes

The text changes section is separated into the following subsections, which can be implemented seperately

- Text changes for slot structure support - DL

- Text changes for slot structure support – UL

- Text changes for allocation granularity

- Text changes for pilot positions

- Text changes for UL AMC MIMO Pilot Power

- Text changes for UL CSM

## Text changes for slot structure support - DL

8.4.5.3.4 STC/DL Zone Switch IE format

[Change as follows in Table 327 (OFDMA STC DL Zone IE format)]

| AMC type | 2 | Indicates the AMC type in case permutation type = 0b11, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved 2x6 (only for STC) Note that only 2x3 band AMC subchannel type (AMC Type = 0b01) is supported by MS |
| --- | --- | --- |

8.4.8.3.1.2.1 STC Mapping for optional AMC permutation

[Change the subclause as indicated]

For the optional AMC permutation in STC zone, the data subchannels shall take 2x6 (2 bins for 6 symbols) format may be used, in addition to the 1x6, 2x3 and 3x2 slot structures. The subcarrier permutation represented by Equation (88) in 8.4.6.3 shall not be applied for the optional AMC permutation within STC zones where STC field in STC DL Zone IE is not equal to 0b00. The pilot pattern of 8.4.8.3.1.1 is used for all slot structures. The 2x6 slot contains 96 subcarriers and is mapped into 2 or more consecutive logical slots (depending on the MIMO matrix) for channel encoding purposes (see 8.4.8.1.2.1.3)

For 2-antenna matrix A in 8.4.8.3.3, STC encoded data symbols shall be time mapped starting over the first 2 OFDMA symbols. The mapping starts at the lowest numbered subcarriers of lowest slot and continues in an ascending manner in subchannels first and then proceeds to the next two symbols in time. An illustration of the mapping rule for the antenna #0 is shown in Figure 276, assuming 2 Tx with Matrix A for a block of 2 slots a 2x6 AMC slot. The mapping of matrix A for the 3x2 slot is the same as the mapping for PUSC in Table 537. Matrix A is not supported for the 2x3 slot.

For 2-antenna vertically encoded matrix B in the optional AMC permutation, modulated data symbols shall

be sequentially mapped for two Tx antennas along the subcarriers of the first symbol in antenna-first order. The mapping

continues in an ascending manner in subchannels subcarriers first and then proceeds to the next symbol in time. An

illustration of the mapping rule for the antenna #0 in AMC2x6 is shown in Figure 277, assuming 2 Tx with vertically

encoded matrix B for a block of 2 slots. Figure 277 also shows the mapping rule for 2-antenna horizontally

encoded matrix B in the optional AMC permutation with AMC2x6, where each encoded stream is separately mapped to the corresponding antenna. The mapping of matrix B data for the 3x2 slot is the same as the mapping for PUSC in Table 537. The mapping of matrix B data for the 3x2 slot is the same as the first 3 symbols shown in Figure 277.

For a 3- or 4-antenna matrix A and matrix B in 8.4.8.3.4 and 8.4.8.3.5, STC encoded data symbols shall be

mapped at two adjacent subcarriers over two OFDMA symbols. When the subcarrier pair (over two

symbols) at frequency k+1 is allocated to pilots for antenna #0 or #1 and the pair at frequency k+2 is

allocated to pilots for antenna #2 or #3, then the pair at frequency k+3 shall be jointly encoded with the pair

at frequency k. This is illustrated in Figure 278, where blocks of 2 convolutional coded (CC) slots and

convolutional turbo coded (CTC) slots are separately shown. The mapping starts at the lowest numbered

subcarriers of the lowest slot and continues in an ascending manner in subchannels first and then proceeds to

the next two symbols in time. For a 3- and 4-antenna vertically/horizontally encoded matrix C in the optional AMC permutation, the same mapping rule for 2-antenna vertically/horizontally encoded matrix B shall be applied on the same frequency-time block with the 3- or 4-antenna pilot pattern.

[Change the titles of figures 276, 277 as follows]

Figure 276—Data mapping in the optional AMC 2x6 Zone with 2 Tx antenna and matrix A

Figure 277—Data mapping in the optional AMC 2x6 zone with 2 Tx antenna and matrix B

[Repeat the following change in all the tables indicated below]

| Subsection | Table |
| --- | --- |
| 8.4.5.3.8 MIMO DL Basic IE format | Table 331—MIMO DL Basic IE format |
| 8.4.5.3 DL-MAP IE format | Table 320—OFDMA DL-MAP IE format |
| 8.4.5.3.9 MIMO DL Enhanced IE format | Table 333—MIMO DL Enhanced IE format |
| 8.4.5.3.17 MIMO in Another BS IE | Table 340—MIMO in Another BS IE format |
| 8.4.5.3.18 Macro-MIMO DL Basic IE format | Table 341—Macro MIMO DL Basic IE() |

[The change:]

| If (Permutation = 0b11 and (AMC type is 2x3, 2x6 or 1x6)) { | — | — |
| --- | --- | --- |

## Text changes for slot structure support – UL

8.4.5.4.7 UL Zone Switch IE format

[Change in Table 371 as follows]

| AMC type | 2 | Indicates the AMC type in case permutation type = 0b11, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved 2x6 (only for collaborative MIMO and STC) |
| --- | --- | --- |

## Text changes for allocation granularity

11.8.3.7.5 OFDMA SS demodulator for MIMO support

[Change in the table as follows]

Bit #20: Allocation granularity in a DL PUSC/AMC STC zone with dedicated pilots

Bit #21: Concurrent allocation support in a DL PUSC/AMC STC zone with dedicated pilots

[Change the text below the table as follows]

Bits #20,#21 define allocation granularity support for PUSC/AMC STC zones with dedicated pilots. They do not apply to single antenna or single stream operation. The granularity is a function of the slot duration and the pilot period. For PUSC-STC with 2 antennas, the slot duration equals 2 symbols and the pilot period equals 4 symbols. For AMC NxM in STC zone the slot duration equals M symbols and the pilot period equals 6 symbols.

If bit #20 is set to 1, the allocations for an MS in a DL PUSC/AMC STC zone with dedicated pilots must meet the following constraints, applicable to both non-HARQ and HARQ sub-bursts:

1) The allocation must be a rectangle.

2) The smallest OFDMA symbol number of the allocation must be a multiple of the pilot period two slot-durations (4 symbols) relative to the smallest OFDMA symbol number of the zone.

3) The time duration of the allocation must be a multiple of the pilot period two slot-durations (4 symbols).

If bit #20 is set to 0, it indicates that the MS supports a granularity of one slot-duration (2 symbols for DL PUSC) for an allocation in any DL STC zone with dedicated pilots so long as the allocation in each subchannel in AMC and/or in each major group in PUSC is equal to or larger than the pilot period the minimum allocation duration of two slot-durations (4 symbols for DL PUSC).

If bit #21 is set to 1, no two allocations for the MS may occupy the same slot duration (2 symbols) in a DL PUSC/AMC STC zone with dedicated pilots. If bit #21 is set to 0, the MS can support multiple allocations in a given slot duration in any DL STC zone with dedicated pilots.

## Text changes for pilot positions

8.4.8.3.1.1 Allocation of pilot subcarriers

[change figure 273 to the following figure]

Figure 273—Pilot allocation for 2-antenna BS for the optional FUSC and the optional AMC zones

[change figure 275 to the following figure]

Figure 275—Pilot allocation for 4-antenna BS for the optional FUSC and the optional AMC zones

## Text changes for UL AMC MIMO Pilot Power

[Modify the paragraph on line 57, page 988 as follows]:

In UL PUSC STC and collaborative SM, the per pilot tone power is 3 dB (gain of 2) above the per data tone power for each transmit antenna.  In UL AMC STC and collaborative SM, the per pilot tone power is 5.5 dB (gain of 432) above the per data tone power for each transmit antenna.

## Text changes for UL CSM

8.4.8.4.1 Allocation of pilot subcarriers

[Change the second paragraph as follows]

For 2-antenna MS and the optional AMC, pilot allocation pattern shall be identical to that for the DL optional AMC with 2 antennas described in 8.4.8.3.1.1; all pilots in the even symbols marked as antenna 0 shall be allocated for antenna 0 or pattern A, while pilots in the odd symbols marked as antenna 1 shall be allocated for antenna #1 or pattern B. This is shown in Figure 273.

8.4.8.4.2 Allocation of data subchannels

[change the last paragraph as follows]

For the UL optional AMC permutation with matrix A and B, the data subchannels shall take 1x6 (1 bin for 6 symbols) format. The subcarrier permutation represented by Equation (88) 8.4.6.3 shall not be applied for the UL optional AMC permutation with matrix A and B. The data mapping rule is identical to that for the DL AMC permutation with 2 antennas. For the UL optional AMC permutation with collaborative spatial multiplexing, the subcarrier permutation represented by Equation (88) 8.4.6.3 shall not be applied and the data mapping shall be frequency-first over the 48 or 96 subcarriers of the slot for each of the MS.