| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC support for co-located coexistence with Wireless LAN and Wireless PAN (i.e. Bluetooth™) |  |
| Date Submitted | 2008-01-10 |  |
| Source(s) | Floyd Simpson Motorola, Inc. 8000 W. Sunrise Blvd Plantation, FL 33322 | Voice: +1 954-723-5269 Fax: +1 954-723-3391 Email: floyd.simpson@motorola.com |
| Re: | 802.16 Revision 2 |  |
| Abstract | Instructions to add MAC support for coexistence with Wireless LAN (WLAN) and Bluetooth (BT) to the 802.16 specification |  |
| Purpose | Co-located coexistence between 802.16 and WLAN/BT can utilize 802.16 power save class (PSC) to create non-interfering time periods of service operation opportunities for each radio. The 802.16 power saving class for co-located coexistence would be configured to be not deactivated during service operation. However, as the specification exists today, it is unknown or unspecified what the BS behavior (i.e. will the BS deactivate a PSC gratuitously?) will be when a power saving class is requested for co-located coexistence purposes by the MS. Furthermore, there are other ameliorating actions the BS could take to enhance the coexistence between WLAN/BT and 802.16 radios operating in the same MS. Therefore, explicit MAC support for co-located coexistence with WLAN and Bluetooth is proposed. |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

MAC Support for Co-located Coexistence with BT and WLAN

Floyd Simpson, Motorola, Inc.
Martin Lorenz, Intel
Yigal Bitran, Altair Semiconductor
Jérémy Gosteau, Sequans Communications

# Explanation

Co-located coexistence between 802.16 and WLAN/BT can utilize IEEE 802.16 power save class (PSC) to create non-interfering time periods of service operation opportunities for each radio operating in the same MS. The 802.16 power saving class for co-located coexistence would be configured to be not deactivated during service operation. However, as the specification exists today, it is unknown or unspecified what the BS behavior (i.e. will the BS deactivate a PSC gratuitously?) will be when a power saving class is requested for co-located coexistence purposes by the MS. Furthermore, there are other ameliorating actions the BS could take to enhance the coexistence between WLAN/BT and 802.16 radio co-located in the same MS. Therefore, explicit MAC support for co-located coexistence with WLAN and Bluetooth is proposed.

For the BS to offer support for coexistence of the co-located WLAN/BT and 802.16 radio of an attached MS, the BS must first know that the MS has this capability. The MS also must know that the BS has the capability to support its coexistence requirements so that it can know what to do when service operation requiring coexistence begins.  This is accomplished by adding new a new TLV in REG-REQ/RSP for this purpose.

When the MS needs to operate a service requiring coexistence support from the BS, the MS would setup/redefine a PSC to do so. To request BS co-located coexistence support from co-located coexistence capable BS, a new TLV is introduced in the MOB_SLP-REQ/RSP message for this purpose. Here the SLP-REQ from the MS would request that it’s PSC to not be explicitly deactivated (exception is during HO where it is already defined and implied in the 802.16 specification that PSC is deactivated during a HO) by the BS.

The last set of changes are specific behavioral changes required for BS supporting coexistence for 802.16 with WLAN/BT co-located in the MS.

# Proposed Rev2D2 Text Changes

The text below provides the proposed changes for 802.16 MAC to support coexistence between WLAN/BT and 802.16 MAC co-located in the MS. Insert text in ‘blue’ where indicated.

[Modify section 6.3.2.3.7 REG-REQ (registration request) message]

6.3.2.3.7 REG-REQ (registration request) message

The REG-REQ may contain the following TLVs:

IP Version (11.7.4)
SS Capabilities Encodings (11.7.8)
Vendor ID Encoding (11.1.5)
Vendor-specific information (11.1.6)
Convergence Sublayer Capabilities (11.7.7)
Co-located Coexistence Capability Supported (11.8.16) 
ARQ Parameters (11.7.1)
    ARQ and fragmentation parameters desired by the SS for establishing the secondary management connection. When the          TLV is not supplied, the SS is indicating its desire to not support ARQ on the connection. For purposes of the parameter         negotiation dialog, the parameters supplied in this message are equivalent to those supplied in the DSA-REQ message.

[Modify section 6.3.2.3.8 REG-RSP (registration response) message]

6.3.2.3.8 REG-RSP (registration response) message

The REG-RSP may contain the following TLVs :

SS Capabilities Encodings (11.7.8) 
   Response to the capabilities of the requester provided in the REG-REQ. Included in the response if the request included          capabilities information. The response indicates whether the capabilities may be used. If a capability is not recognized, the      response indicates that this capability shall not be used by the requester. Capabilities returned in the REG-RSP shall not          be set to require greater capability of the requester than is indicated in the REG-REQ.

IP Version (11.7.4)
Co-located Coexistence Capability Supported (11.8.16)
Vendor ID Encoding (of the responder; 11.1.5)
Vendor-specific information (11.1.6)

[Modify section 6.3.2.3.40 SLP-REQ (sleep request) message]

6.3.2.3.40 MOB_SLP-REQ (sleep request) message

The following TLV parameter may be included in MOB_SLP-REQ message transmitted when requesting an activation of power saving class. This TLV indicates the enabled action that MS performs upon reaching trigger condition in sleep mode.

Enabled-Action-Triggered
       Indicates possible action upon reaching trigger condition.

The following TLV parameter may be included in MOB_SLP-REQ message transmitted when requesting an activation of power saving class. This TLV indicates that the unavailability interval of the activated PSC is to be used for coexistence purposes in the MS and the BS is requested to use coexistence behavior for the PSC.

Co-located-Coexistence-Enabled
       Indicates PSC is also to support co-located coexistence.

The MOB_SLP-REQ shall include the following parameters encoded as TLV tuples: 
                              HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Modify section 6.3.2.3.41 SLP-RSP (sleep response) message]

6.3.2.3.41 MOB_SLP-RSP (sleep response) message

The following TLV parameter may be included in MOB_SLP-RSP message transmitted by the BS.

Enabled-Action-Triggered (11.1.8.1)
         This TLV indicates the enabled action that the MS performs upon reaching trigger condition in sleep mode.

Co-located-Coexistence-Enabled (11.1.8.4)
          This TLV indicates the BS uses co-located coexistence behavior for the PSC.

Next Periodic Ranging (11.1.8.3) 
          This value indicates the offset of frame in which MS shall be ready to perform a periodic ranging with respect to the                frame where MOB_SLP-RSP is transmitted.

[Add new section 6.3.21.8 Sleep mode supporting coexistence]

6.3.21.8 Sleep mode supporting coexistence

The MS may include the Co-located-Coexistence-Enabled TLV in MOB_SLP-REQ to define or define and activate a PSC for co-located coexistence support. The MS shall include the Co-located-Coexistence-Enabled TLV in the MOB_SLP-REQ if and only if at least one bit of this TLV value is set to 1 and shall not include it otherwise.

Only one active PSC supporting co-located coexistence should operate at any given time per MS. The active PSC supporting co-located coexistence shall be defined with Traffic_triggered_wakening_flag set to 0 and the TRF-IND_Required flag set to zero.

The BS shall not explicitly deactivate the active PSC supporting co-located coexistence. Furthermore, the BS shall honor, to all extent possible, the configuration for the PSC in the MS SLP-REQ. The expectation from the MS perspective is that the BS does not gratuitously reject or modify the configuration for the PSC in the MS SLP-REQ as multi-radio operation in the MS will suffer due to co-located radio interference. The MS shall deactivate the active PSC supporting co-located coexistence or switch to a PSC without co-located coexistence support required when co-located coexistence support is no longer needed.

When the PSC supporting co-located coexistence is for Wireless PAN support (bit 1 of Co-located-Coexistence-Enabled TLV is set to 1), the BS shall not provide any MS UL allocation in the first frame of the PSC Listen Window and shall provide any DL allocation as much as possible in the first frame of the PSC Listen Window. Furthermore, the BS should, to all extent possible, provide DL allocations such that DL allocations for MS with Co-located-Coexistence-Enabled active PSC come before MS that do not need co-located coexistence support.

When the PSC supporting co-located coexistence is for Wireless PAN/LAN support (bit 0 or bit 1 of Coexistence-Enabled TLV is set to 1), the BS shall not provide any MS DL/UL allocation in the Sleep Window of the PSC.

[Add new section 11.1.8.4 Coexistence-Enabled]

11.1.8.4 Co-located-Coexistence-Enabled

MS may include the Co-located-Coexistence-Enabled TLV in the MOB_SLP-REQ to indicate if coexistence behavior of the PSC is requested when the PSC is activated. If the BS supports the Co-located Coexistence Capability, the BS shall include this TLV in the MOB_SLP-RSP message transmitted in response to the MOB_SLP-REQ message containing this TLV.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 136 | 1 | Bit #0: Wireless LAN co-located coexistence Bit #1: Wireless PAN co-located coexistence Bit #2 – 7: reserved | SLP-REQ, SLP-RSP |

[Modify section 11.4.1 DCD channel encodings]

11.4.1 DCD channel encodings

[Add Co-located Coexistence Capability Supported TLV to Table 544]

| Name | Type (1 byte) | Length | Value | Phy Scope |
| --- | --- | --- | --- | --- |
| MBS zone identifier list | 61 | variable | This parameter shall include all MBS zone identifiers (i.e., n*MBS zone identifier) with which BS is associated. An MBS zone identifier is 1 byte long. bits #6 through #0 are the MBS Zone Identifier, bit #7 is set to 0 in each byte. | All |
| Co-located Coexistence Capability Supported | 62 | 1 | Bit #0: Wireless LAN co-located coexistence Bit #1: Wireless PAN co-located coexistence Bit #2 – 7: reserved | All |

[Add new section 11.8.16 Co-located Coexistence Capability Supported]

11.8.16 Co-located Coexistence Capability Supported

The Co-located Coexistence Capability Supported TLV indicates if co-located coexistence is supported. MSs and BSs that support co-located coexistence shall identify themselves by including this TLV.  For each bit, a value of 0 indicates “not supported” while 1 indicates “supported.”

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | 1 | Bit #0: Wireless LAN co-located coexistence Bit #1: Wireless PAN co-located coexistence Bit #2 – 7: reserved | REG-REQ, REG-RSP |