| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Persistent allocation method for reducing MAP overhead |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | June Moon Hoon Chang Kwanhee Roh Seho Kim Jaehee Cho Heewon Kang Changyoon Oh | Voice: +82-31-279-7529 E-mail: changyoon.oh@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26a |  |
| Abstract | The resource allocation scheme of 802.16e is extended to enable persistent allocations (also referred to as periodic allocations). |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Persistent allocation method for reducing MAP overhead

# 1. Introduction

When there are many small-sized bursts within a frame, e.g. VoIP traffic, MAP overhead can significantly be increased, limiting system capacity. Persistent allocation can reduce MAP overhead remarkably since allocation MAP message only appears once and retaining allocation afterward. This contribution proposes complete solution for supporting persistent allocation.

# 2. Proposed Solution

The periodicity of VoIP packet transmissions enables persistent resource allocation where the resource assignment signaling needs not to be sent for each packet transmission. In persistent allocation, MAP IEs including new resource assignments are sent to corresponding MSs in the beginning of periodic allocation, and for given persistent allocation period, resource assignments such as allocated resource size and MCS are assumed to be fixed and used by the corresponding MSs without explicit signaling.

However, resource for a VoIP MS needs to be deallocated when the MS goes into the silence state where the voice coder either transmits or suppresses lowest-rate voice packets, or should be reallocated when the MS’s MCS is changed to counteract channel variations.

Deallocation makes the allocated resource unused, and this can resultantly create fragmented unused resource, called resource hole if it is not replaced with other persistent allocation. It may be filled with another small-sized burst with non-persistent allocation, but when data traffic is multiplexed with VoIP traffic, the fragmented resource holes can degrades efficiency of the resource utilization as well as voice capacity. As a result, an efficient management mechanism is needed to cope with the resource holes problem. We propose efficient persistent resource allocation method in order to reduce the overhead of explicit signaling and improve the resource utilization.

In the proposed method, a new persistent allocation IE is designed to represent a group region and its resource allocation. It has its own Group ID, indicating a specified group as depicted in Fig. 1. The group consists of a set of users which have persistent resource within the Persistent IE. The users in same group shall have same allocation period (e.g. 4 frames for 20ms period VoIP). Assigning persistent subburst in a group region to an MS implicitly makes the MS belong to the group. Resources in a region are addressed by a pair of (slot offset, duration). The MSs in a group have a unique identification, called ‘order’, representing the order of the MSs’ resources in the group region.

Figure 1. Persistent resource allocation and group regions

When new persistent users enter the cell or the users in silence state move to talk state, new persistent allocation is needed. New persistent allocations in the group region are appended to the previously allocated slots as illustrated in Fig. 2 when Resource Shifting Indicator in MAP IE is set to ‘1’. The slot offset for the first new allocation is given in the MAP IE, and subsequent allocated resources are implicitly arranged in sequential order. For newly allocated persistent MSs, {CID, duration, MCS} fields are included. When Resource Shifting Indicator in MAP IE is set to ‘0’, new persistent allocation may not necessarily be appended to the previously allocated slots. The position of each allocation can be controlled by using Slot Offset IE.

Figure 2. Allocating new resource for persistent resource allocation

When the users in talk state move to the silence state, the resources assigned to them should be deallocated. These deallocated resources can be replaced with new persistent allocation or other non-persistent allocation provided that the size of deallocated resources are lager than the size of newly allocated resources. But this can not fully alleviate the resource hole problem. Thus a counter-measure is needed to alleviate this problem. If Resource Repacking Indicator in MAP IE is set to ‘1’, all the MSs within the same group shall modify their own order value and the location of resource allocation if their order value is higher than the order value specified in the deallocation message. For example, if MS2 with order value of 2 is deallocated then MSs with order value higher than 2(MS1, MS3 and MS5) shall decrease their own order by 1 and their start slot positions of persistent resources are also shifted by the amount of duration of deallocated resource as illustrated in Fig. 3. If Resource Repacking Indicator in MAP IE is set to ‘0’, the order value and the start slot position of the MSs in the same group remain unchanged.

Figure 3. Deallocating assigned resource in persistent resource allocation

If a resource is resized due to MCS change, voice codec rate change, or possibly MIMO mode change (SM/STC), the persistent allocations with order value higher than that of MCS changed allocation are moved forward or backward, depending on the change of the size if Resource Shifting Indicator is set to ‘1’. For example as shown in the Fig. 4, when the MCS is changed, a MCS change sub-IE is transmitted, and all MSs after the MCS changed MS use their resource at a new slot location, shifted by the length of the delta duration which is defined as the slot number difference between the previous assignment and the new assignment. When the MCS is changed to a higher MCS level, the delta duration is negative since the MS needs fewer slots to transmit fixed-sized VoIP packets and the following resources are shifted backward. On the other hand, when the MCS is changed to a lower MCS level, the delta duration becomes positive and the following resources are shifted forward. If Resource Shifting Indicator is set to ‘0’ the MCS change sub-IE shall not be used. In this case, instead of MCS change sub-IE, deallocation and persistent allocation shall be used.

Figure 4. Changing allocated resource size when the MCS level is adjusted.

If Resource Shifting scheme is used, deallocation or MCS change is accompanied by resource rearrangement which automatically fills the released resource holes in the group region. And newly allocated resources are appended after that. Therefore, unused resource holes cannot exist in this case.

By the way, persistent allocation may cause another problem which is called as MAP error propagation. Upon missing a MAP which contains resizing or deallocating persistent allocations, the MSs with persistent allocation may not be aware of any changes occurred in the missed MAP. To solve with this problem, which can occur in any other persistent allocation schemes, we introduce a change counter based approach. A persistent change counter (PCC) is increased for each occurrence of deallocation and MCS change event. Each group has its own PCC. For example, assume that allocation period of group-A is 4 and PCC value is 1 at the N-th frame. At the (N+4)-th frame, there are 2 deallocation events and 1 MCS change event. Then, PCC value of Group-A at the (N+4)-th frame should be 4 in this case. Whenever Deallocation or MCS change events for persistent resource occur, the BS sends PCC value in order to indicate possible change of persistent resource. When an MS with persistent allocation misses MAP message at the frame index corresponding to its own Group allocation period due to decoding failure, the MS shall transmit a specific CQICH codeword to the BS in order to indicate its MAP loss. The BS, in response to reception of the specific CQICH codeword from MSs, should send only the PCC value when there is no deallocation or MCS change event at the previous frame. MSs monitor the PCC value and it can figure out there are no changes of persistent resource at the previous frame. If deallocation and MCS change events occurred at the previous frame, the BS should send both PCC value and persistent resource change information so that the MS can recover its own persistent allocation using the resource change information. The persistent resource change information can be either explicit persistent allocation message or deallocation and/or MCS change sub-IEs transmitted in the previous frame. Note that the explicit persistent allocation message is targeted to only one MS but the deallocation or MCS change sub-IE can be used by multiple MSs. For example, if the number of MSs that reported MAP loss is small, BS can send explicit persistent allocation message to the MSs reported MAP loss to inform the newly changed allocation. Otherwise, BS can send MCS change and deallocation sub-IEs transmitted in the previous frame again that can be used by MSs reported MAP loss to solve the allocation mismatch problem. Note that the appearance order of deallocation sub-IE or MCS change sub-IE is directly related with change of PCC value. For example, if current PCC value is N and there are 1 deallocation sub-IE and 1 MCS change sub-IE is transmitted in current frame (e.g., deallocation comes earlier than MCS change), then the deallocation sub-IE describes the event corresponding to the PCC value of N, and the MCS change sub-IE describes the event corresponding to the PCC value of (N-1). Since the PCC value is increased for each occurrence of deallocation or MCS change event, each deallocation and MCS change event has its own PCC identification as is described above. Therefore, the MSs that missed MAP at the previous frame can recover its own persistent resource position by monitoring the PCC value and resource change information.

# 3. Proposed Text

[Add new section to 8.4.5.3.21.2]

8.4.5.3.21.2 Downlink Persistent Allocations

Downlink persistent allocations are used by the base station to make downlink time-frequency resource assignments which repeat periodically.  The logical time-frequency resource assigned by persistent allocation repeats at a periodic interval.  For downlink persistent allocations, the base station transmits the Persistent HARQ DL MAP IE.

The Persistent HARQ DL MAP IE defines one two-dimensional data region (a number of symbols by a number of subchannels). These allocations are further partitioned into subbursts by allocating a specified number of slots to each burst. Subbursts can be allocated either in persistent manner, or in non-persistent manner. The number of slots for each subburst is indicated by duration fields. The slots are allocated in a frequency-first order, starting from the slot with the smallest symbol number and smallest subchannel, and continuing to slots with increasing subchannel number. When DL Slot Offset IE appears in the Persistent HARQ DL MAP IE, subsequent subbursts allocation starts from the slot indicated by Slot offset field, and continuing to slots with increasing subchannel number. When the edge of the allocation is reached, the symbol number is increased by slot duration, as depicted in Figure xxx. Each subburst is separately encoded.

Figure xxx. Persistent HARQ DL Allocation

Table xxx defines detailed message format for Persistent HARQ DL MAP IE. Each Persistent HARQ DL MAP IE has its own Group ID, indicating a specified group. The Group consists of a set of users which have persistent resource within the Persistent HARQ DL MAP IE and have same allocation period. Assigning persistent subburst in a group region to an MS implicitly makes the MS belong to the group.

Several Sub-IEs are defined within the Persistent Subburst IE. DL Allocation sub-IE is used for subburst allocation, which can be either persistent, or non-persistent. Persistently allocated subbursts will repeatedly be allocated with a period indicated by Group allocation period until explicit deallocation message is sent. Deallocation sub-IE is used for deallocating persistently allocated surburst. DL MCS change sub-IE is used for changing allocation information (e.g. MCS level and slot length) for persistently allocated subburst. DL slot offset sub-IE is used for indicating slot start pointer for non-persistent subburst allocation. Subbursts allocated by DL allocation sub-IEs with non-persistent option appearing after DL slot offset sub-IE starts from the slot indicated by Slot offset field, and continuing to slots with increasing subchannel number.

Persistent subburst allocation starts from the slot right after previously allocated persistent resource when Resource Shifting Indicator is set to ‘1’. All the users within a group shall have persistent allocated resource, and also shall have his own order value, which is used as a user identifier for DL deallocation IE and DL MCS change IE. Order value is allotted only for persistent allocation, representing MS’s persistent resource allocation order. For persistent allocation when Resource Shifting is used, the DL Allocation sub-IE appears first in the Persistent HARQ DL MAP IE, and the MS shall obtain it’s order value by referring the Number of persistent allocations field in this IE. For the subsequent DL Allocation sub-IEs with persistent allocation, the MSs shall obtain their own order value by counting the number of preceding DL Allocation sub-IEs. When Resource Shifting is not used, order value is explicitly given by allocation sub-IE.

Deallocation and MCS change for persistent allocation shall accompany automatic resource repacking if the DL Resource Shifting Indicator field in Persistent HARQ DL MAP IE is set. If DL Resource Shifting Indicator field is set to ‘1’, MSs with persistent allocation in the same group shall react as follows. When DL deallocation sub-IE is sent, all the MSs with order value higher than specified order value within this sub-IE shall decrease their order value by one and their start slot position of persistent resource is also decreased by the amount of duration (DL deallocation sub-IE). When DL MCS change sub-IE is sent, all the MSs with order value higher than specified order value within this sub-IE shall adjust their start slot position by the amount of delta duration (DL MCS change sub-IE). If DL Resource Shifting Indicator field is set to ‘0’, the order value and the start slot position of MSs with persistent allocation in the same group shall not be changed

ACK channel start offset is specified for each group. MSs shall calculate their ACK channel indices for persistent allocation by the sum of ACK channel start offset and the order value of each MS. For non-persistent allocation specified within Persistent DL MAP IE, explicit ACK channel index is specified within DL allocation sub-IE.

HARQ retransmission packet for persistent allocation shall be assigned by non-persistent allocation

Table xxx - Persistent HARQ DL MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Persistent HARQ DL MAP IE() { |  |  |
| Extended-2 DIUC | 4 |  |
| Length | 8 |  |
| RCID type | 2 |  |
| Group ID | 4 | Group identifier for this Persistent IE |
| Number of persistent allocations | 5 | Used for obtaining 'order' information for persistent allocation |
| Duration type | 2 | Bit size of duration field: 10bits (0b00), 8bits (0b01) 6bits (0b10) |
| DL Persistent Control IE() | variable |  |
| While (data remains) { |  |  |
| Mode | 4 |  |
| If (mode == 0000) |  |  |
| Persistent DL HARQ Chase Sub-Burst IE () | variable |  |
| If (mode == 0011) |  |  |
| Persistent MIMO DL Chase HARQ Sub-Burst IE () | variable |  |
| } |  |  |
| } |  |  |

Number of persistent allocations

Indicates the number of persistent allocations in this group. This can be used to obtain 'order' value for persistent allocation

Duration type

Specifies bit size of duration field used in the Persistent HARQ DL MAP IE

Table xxx – DL Persistent control IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL Persistent control IE() { |  |  |
| Persistent control indicator | 4 |  |
| if (LSB#0 of Persistent control indicator == 1) { |  | Group Region indicator |
| OFDMA symbol offset | 8 |  |
| Subchannel offset | 7 |  |
| Number of OFDMA symbols | 7 |  |
| Number of subchannels | 7 |  |
| Rectangular burst indication | 1 |  |
| DL Resource Shifting Indicator | 1 | 0: Resource Shifting is not used 1: Resource Shifting is used |
| Reserved | 1 |  |
| } |  |  |
| if (LSB#1 of Persistent control indicator == 1) { |  | Group Period indicator |
| Group allocation period | 4 |  |
| } |  |  |
| if (LSB#2 of Persistent control indicator == 1 && DL Resource Shifting Indicator == 1) { |  | ACK channel start offset indicator for persistent allocation |
| ACK CH Start Offset | 8 |  |
| } |  |  |
| if (LSB#3 of Persistent control indicator == 1 && DL Resource Shifting Indicator == 1) { |  | PCC indicator |
| Persistent Change Counter | 8 |  |
| } |  |  |
| } |  |  |

DL Resource Shifting Indicator

The Resource Shifting Indicator enables resource shifting on a per Persistent HARQ DL MAP IE basis by utilizing the deallocation information of other MSs. If the indicator is set to 1, all MSs other than those MSs whose resource is being deallocated, or whose resource is changed by MCS change, track the location and the number of the deallocated slots or number of MCS changed slots and decide whether, and how, they should shift the resources allocated to them. If the location of the deallocated slots or MCS changed slots preceeds the resource currently allocated to an MS in the HARQ region, then the MS shall shift its position of allocated slots forward such that the amount of shift is equal to the number of deallocated slots in the HARQ region. If on the other hand, the location of the deallocated slots or MCS changed slots is after the currently allocated slots, then the MS does not change the current position of allocation.

Group allocation period

The group allocation period shall be set to the period of the persistent allocation, in units of frames. If the allocation period is set to n, the time-frequency resource assignment repeats every n frames.  For example, as illustrated below, if n=4, the time-frequency resource assignment is valid in frames N, N+4, N+8, etc.

ACK channel start offset

It is specified for each group. MSs shall calculate their ACK channel indices for persistent allocation by the sum of ACK channel start offset and the order value of each MS.

Persistent Change Counter (PCC)

For each group, PCC is increased for each occurrence of new MCS change or deallocation event. For example, assume that allocation period of group-A is 4 and PCC value is 1 at the N-th frame. At the (N+4)-th frame, there are 2 deallocation events and 1 MCS change event. PCC value of Group-A at the (N+4)-th frame should be 4 in this case. When an MS with persistent allocation fails to receive MAP message at the frame index corresponding to its own Group allocation period, the MS shall transmit a specific CQICH codeword to the BS in order to inform its MAP loss. The BS shall send PCC value when there is any deallocation or MCS change event at the current frame, or when there are any reception of the specific CQICH codeword from the MS. Note that each occurrence of deallocation sub-IE and MCS change sub-IE is directly related with the change of PCC value. This means that the first appearance of deallocation or MCS change sub-IE within the persistent HARQ DL MAP IE describes the most recent event related to the PCC value of N if the PCC value of current frame is N. Persistent Change Counter shall not be transmitted if DL Resource Repacking Indicator is set to ‘0’.

Table xxx –Persistent DL HARQ Chase Sub-burst IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Persistent DL HARQ Chase Sub-Burst IE () { |  |  |
| N sub-IE | 4 | Number of Sub-IEs |
| For (j=0; j<N_sub-IE; j++) { | 3 |  |
| Sub-IE type | 3 |  |
| Reserved | 1 |  |
| If (Sub-IE type==0) |  |  |
| DL Deallocation sub-IE() | Variable | Persistent deallocation |
| If (Sub-IE type==1) |  |  |
| DL MCS Change sub-IE() | 16 | MCS change for persistent resource |
| If (Sub-IE type==2) |  |  |
| DL Allocation sub-IE() | Variable | Non-persistent or persistent allocation |
| If (Sub-IE type==3) |  |  |
| DL Slot Offset sub-IE() | 12 | Indicates the start pointer in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the time-frequency resource. When this IE appears before DL_allocation_IE, it indicates allocation start position. |
| } |  |  |
| } |  |  |

Table xxx – Persistent MIMO DL Chase HARQ Sub-Burst IE ()

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Persistent MIMO DL Chase HARQ Sub-Burst IE () { |  |  |
| N sub-IE | 4 | Number of Sub-IEs |
| For (j=0; j<N_sub-IE; j++) { | 3 |  |
| Sub-IE type | 3 |  |
| Reserved | 1 |  |
| If (Sub-IE type==0) |  |  |
| DL Deallocation sub-IE() | variable | Persistent deallocation |
| If (Sub-IE type==1) |  |  |
| MIMO DL MCS Change sub-IE() | variable | MCS change for MIMO persistent resource |
| If (Sub-IE type==2) |  |  |
| MIMO DL Allocation sub-IE() | variable | Non-persistent or persistent MIMO allocation |
| If (Sub-IE type==3) |  |  |
| DL Slot Offset sub-IE() | 12 | Indicates the start pointer in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the time-frequency resource. When this IE appear before DL_allocation_IE, it indicates allocation start position. |
| } |  |  |
| } |  |  |

Table xxx – DL Deallocation sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL deallocation sub-IE() { |  |  |
| Order | 5 |  |
| If (DL Resource Shifting Indicator == 1) { |  |  |
| Duration | Variable | 10/8/6 bits |
| } |  |  |
| Padding | Variable | Padding to nibble; shall be set to 0 |
| } |  |  |

Table xxx – DL MCS change sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL MCS Change sub-IE() { |  |  |
| Order | 5 |  |
| DIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| Delta duration | 5 | Difference in slots between previously occupied subburst size and current subburst size |
| } |  |  |

Table xxx - DL Allocation sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL Allocation sub-IE() { |  |  |
| RCID | Variable |  |
| Allocation flag | 1 | 0: Non-persistent allocation 1: Persistent allocation |
| Order Indication |  |  |
| If (Order Indication == 1) { |  |  |
| Order | 5 |  |
| } |  |  |
| Duration | Variable | 10 bit in case of Duration type=00 8 bit in case of Duration type=01 6 bit in case of Duration type=10 |
| DIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| if (Allocation flag==1) { |  |  |
| Number of ACID | 4 | Number of HARQ channels associated with this persistent assignment. It enables ACID cycling |
| } |  |  |
| ACID | 4 |  |
| AI-SN | 1 |  |
| ACK disable | 1 |  |
| If (ACK_disable==0 and (Allocation_flag==0 or DL Resource Shifting Indicator==0) { |  |  |
| ACK channel | 8 | ACK channel index |
| } |  |  |
| Dedicated DL control Indicator | 2 |  |
| If LSB#0 of dedicated DL control indicator ==1){ |  |  |
| Duration (d) | 4 |  |
| If (Duration !=0b0000 ){ |  |  |
| Allocation index | 6 |  |
| Period (p) | 3 |  |
| Frame offset | 3 |  |
| } |  |  |
| } |  |  |
| If (LSB #1 of dedicated DL control indicator ==1){ |  |  |
| Dedicated DL control IE | Variable |  |
| } |  |  |
| } |  |  |

Allocation flag

Indicates whether this subburst allocation is persistent or not

ACID

The ACID field shall be set to the beginning ACID as described below.

Number of ACID

The Number of ACID (N_ACID) shall be set to the number of HARQ channel identifiers for persistent allocation.  The Number of ACID and ACID fields are used together to establish an implicit cycling of ACID as follows.  If N_ACID is greater than 1, and if the allocation period is greater than 0, the ACID corresponding to the first occurrence of the persistent allocation corresponds to the ACID field, and the ACID corresponding to the second occurrence of the persistent allocation corresponds to the ACID field + 1, etc.  The ACID corresponding to the N_ACIDth occurrence of the persistent allocation corresponds to the ACID field + N_ACID – 1, and the ACID corresponding to the (N_ACID + 1)th occurrence of the persistent allocation corresponds to the ACID field.  This process is repeated for additional occurrences of the persistent allocation.  As illustrated below, if the allocation period equals 4, if N_ACID = 4, and if ACID = 2, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc.

ACK channel index

The ACK channel shall be set to the number of the ACK channel within the HARQ ACK Region.  The mobile station shall use the indicated ACK channel for transmitting acknowledgment information for each packet received using the time-frequency resource referred to by non-persistent allocation.

AI_SN

The AI_SN shall be set to the initial AI_SN for each ACID in the sequence of ACID values.  The AI_SN shall toggle from 1 to 0 or 0 to 1 for each occurrence of a particular ACID.  For example, if the allocation period equals 4, if N_ACID = 4, if ACID = 2, and if AI_SN = 0, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc, and the AI_SN follows the pattern 0, 0, 0, 0, 1, 1, 1, 1, etc.

Table xxx – DL slot offset sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL slot offset sub-IE { |  |  |
| Slot offset | 10 |  |
| Reserved | 2 |  |
| } |  |  |

Slot offset

Indicates the start point in OFDMA slots, with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the time-frequency resource. When this IE appears before DL allocation sub-IE with non-persistent option, it indicates allocation start position. This does not affect persistent allocation

Table xxx – MIMO DL MCS change sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO DL MCS Change sub-IE() { |  |  |
| Order | 5 |  |
| Dedicated MIMO DL Control Indicator | 1 |  |
| if (Dedicated MIMO DL Control Indicator==1) { |  |  |
| Dedicated MIMO DL Control IE() | Variable |  |
| } |  |  |
| Delta_duration | 5 | Difference in slots between previously occupied subburst size and current subburst size |
| For (i=0; i<N_layer; i++) { |  |  |
| DIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| } |  |  |
| } |  |  |

Table xxx - MIMO DL Allocation sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO DL Allocation sub-IE() { |  |  |
| MU_indicator | 1 | Indicates whether this DL burst is intended for multiple SS |
| Dedicated MIMO DL Control Indicator | 1 |  |
| ACK disable | 1 |  |
| Order Indication | 1 |  |
| If (Order Indication == 1) { |  |  |
| Order | 5 |  |
| } |  |  |
| if (MU_indicator==0) { |  |  |
| RCID | Variable |  |
| } |  |  |
| if (Dedicated MIMO DL Control Indicator==1) { |  |  |
| Dedicated MIMO DL Control IE() | Variable |  |
| } |  |  |
| Allocation flag | 1 | 0: Non-persistent 1: Persistent alloc |
| Duration | Variable | 10/8/6 bits |
| For (i=0; i<N_layer; i++) { |  |  |
| if (MU_indicator==1) { |  |  |
| RCID | Variable |  |
| } |  |  |
| DIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| if (ACK_diable==0) { |  |  |
| if (Allocation flag==1) { |  |  |
| Number of ACID | 4 | Number of HARQ channels associated with this persistent assignment. It enables ACID cycling |
| } |  |  |
| ACID | 4 |  |
| AI-SN | 1 |  |
| If (Allocation_flag==0 or DL Resource Shifting Indicator == 0) { |  |  |
| ACK channel | 8 | ACK channel index |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to nibble; shall be set to 0 |
| } |  |  |

[Add new section to 8.4.5.4.24.3]

8.4.5.4.24.3 Uplink Persistent Allocations

Uplink persistent allocations are used to make uplink time-frequency resource assignments which repeats periodically. The logical time-frequency resource assigned by persistent allocation repeats at a periodic interval.  For uplink persistent allocations, the base station transmits the Persistent HARQ UL MAP IE.

The Persistent HARQ UL MAP IE defines data region by the starting symbol and subchannel. Subbursts defined within the Persistent HARQ UL MAP IE can be allocated either in persistent manner, or in non-persistent manner. The number of slots for each subburst is indicated by duration fields. The slots are allocated in a time-first order. When UL Slot Offset IE appears in the Persistent HARQ UL MAP IE, subsequent subbursts starts from the slot indicated by Slot offset field, and continuing to slots with increasing symbol number. When the edge of the allocation is reached, the subchannel number is increased. Each subburst is separately encoded.

Table yyy - Persistent HARQ UL MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Persistent HARQ UL MAP IE() { |  |  |
| Extended-2 UIUC | 4 |  |
| Length | 8 |  |
| RCID type | 2 |  |
| Group ID | 4 |  |
| Number of persistent allocations | 5 |  |
| Duration type | 2 | Bit size of duration field: 10bits (0b00), 8bits (0b01) 6bits (0b10) |
| UL Persistent Control IE() | variable |  |
| While (data remains) { |  |  |
| Mode | 3 |  |
| Reserved | 1 |  |
| If (mode == 000) |  |  |
| Persistent UL HARQ Chase Sub-Burst IE () | variable |  |
| If (mode == 011) |  |  |
| Persistent MIMO UL Chase HARQ Sub-Burst IE () | variable |  |
| } |  |  |
| } |  |  |

Number of persistent allocations

Indicates the number of persistent allocations in this group. This can be used to obtain 'order' value for persistent allocation

Duration type

Specifies bit size of duration field used in the Persistent HARQ UL MAP IE

Table yyy – UL Persistent Control IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL Persistent Control IE () { |  |  |
| Persistent control indicator | 3 |  |
| Reserved | 1 |  |
| if (LSB#0 of Persistent control indicator == 1) { |  | Group Region indicator |
| OFDMA symbol offset | 8 |  |
| Subchannel offset | 7 |  |
| UL Resource Shifting Indicator | 1 | 0: resource repacking is not used 1: resource repacking is used |
| } |  |  |
| if (LSB#1 of Persistent control indicator == 1 ) { |  | Group Period indicator |
| Group allocation period | 4 |  |
| } |  |  |
| if (LSB#2 of Persistent control indicator == 1 && UL Resource Shifting Indicator == 1) { |  | PCC indicator |
| Persistent Change Counter | 8 |  |
| } |  |  |

UL Resource Shifting Indicator

The Resource Shifting Indicator enables resource shifting on a per Persistent HARQ UL MAP IE basis by utilizing the deallocation information of other MSs. If the indicator is set to 1, all MSs other than those MSs whose resource is being deallocated, or whose resource is changed by MCS change, track the location and the number of the deallocated slots or number of MCS changed slots and decide whether, and how, they should shift the resources allocated to them. If the location of the deallocated slots or MCS changed slots preceeds the resource currently allocated to an MS in the HARQ region, then the MS shall shift its position of allocated slots forward such that the amount of shift is equal to the number of deallocated slots in the HARQ region. If on the other hand, the location of the deallocated slots or MCS changed slots is after the currently allocated slots, then the MS does not change the current position of allocation.

Group allocation period

The group allocation period shall be set to the period of the persistent allocation, in units of frames. If the allocation period is set to n, the time-frequency resource assignment repeats every n frames.  For example, if n=4, the time-frequency resource assignment is valid in frames N, N+4, N+8, etc.

Persistent Change Counter (PCC)

For each group, PCC is increased for each occurrence of new MCS change or deallocation event. For example, assume that allocation period of group-A is 4 and PCC value is 1 at the N-th frame. At the (N+4)-th frame, there are 2 deallocation events and 1 MCS change event. PCC value of Group-A at the (N+4)-th frame should be 4 in this case. When an MS with persistent allocation fails to receive MAP message at the frame index corresponding to its own Group allocation period, the MS shall transmit a specific CQICH codeword to the BS in order to inform its MAP loss. The BS shall send PCC value when there is any deallocation or MCS change event at the current frame, or when there are any reception of the specific CQICH codeword from the MS. Note that each occurrence of deallocation sub-IE and MCS change sub-IE is directly related with the change of PCC value. This means that the first appearance of deallocation or MCS change sub-IE within the Persistent HARQ UL MAP IE describes the most recent event related to the PCC value of N if the PCC value of current frame is N. Persistent Change Counter shall not be transmitted if UL Resource Repacking Indicator is set to ‘0’.

Table yyy –Persistent UL HARQ Chase Sub-burst IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Persistent UL HARQ Chase Sub-Burst IE () { |  |  |
| N sub-IE | 4 | Number of Sub-IEs |
| For (j=0; j<N_sub-IE; j++) { | 3 |  |
| Sub-IE type | 3 |  |
| Reserved | 1 |  |
| If (Sub-IE type==0) |  |  |
| UL Deallocation sub-IE() | variable | Persistent deallocation |
| If (Sub-IE type==1) |  |  |
| UL MCS Change sub-IE() | 16 | MCS change for persistent resource |
| If (Sub-IE type==2) |  |  |
| UL Allocation sub-IE() | variable | Non-persistent or persistent allocation |
| If (Sub-IE type==3) |  |  |
| UL Slot Offset sub-IE() | 12 |  |
| } |  |  |
| } |  |  |

Table yyy – Persistent MIMO UL Chase HARQ Sub-Burst IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Persistent MIMO UL Chase HARQ Sub-Burst IE () { |  |  |
| N sub-IE | 4 | Number of Sub-IEs |
| For (j=0; j<N_sub-IE; j++) { | 3 |  |
| Sub-IE type | 3 |  |
| Reserved | 1 |  |
| If (Sub-IE type==0) |  |  |
| UL Deallocation IE() | Variable | Persistent deallocation |
| If (Sub-IE type==1) |  |  |
| MIMO UL MCS Change IE() | 16 | MCS change for MIMO persistent resource |
| If (Sub-IE type==2) |  |  |
| MIMO UL Chase Allocation IE() | Variable | Non-persistent or persistent MIMO allocation |
| If (Sub-IE type==3) |  |  |
| UL Slot Offset IE() | 16 |  |
| } |  |  |
| } |  |  |

Table yyy – UL Deallocation sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL deallocation sub-IE() { |  |  |
| Order | 5 |  |
| If (UL Resource Shifting Indicator == 1) { |  |  |
| Duration | Variable | 10/8/6 bits |
| } |  |  |
| Padding | Variable | Padding to nibble; shall be set to 0 |
| } |  |  |

Table yyy – UL MCS change Sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL MCS Change sub-IE() { |  |  |
| Order | 5 |  |
| UIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| Delta duration | 5 | Difference in slots between previously occupied subburst size and current subburst size |
| } |  |  |

Table yyy - UL Allocation sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL Allocation sub-IE() { |  |  |
| RCID | Variable |  |
| Allocation flag | 1 | 0: Non-persistent allocation 1: Persistent allocation |
| Order Indication | 1 |  |
| If (Order Indication == 1) { |  |  |
| Order | 5 |  |
| } |  |  |
| Dedicated UL control Indicator | 1 |  |
| If (Dedicated UL control indicator ==1){ |  |  |
| Dedicated UL control IE | Variable |  |
| } |  |  |
| UIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| Duration | Variable | 10 bit in case of Duration type=00 8 bit in case of Duration type=01 6 bit in case of Duration type=10 |
| if (Allocation flag==1) { |  |  |
| Number of ACID | 4 | Number of HARQ channels associated with this persistent assignment. It enables ACID cycling |
| } |  |  |
| ACID | 4 |  |
| AI-SN | 1 |  |
| ACK disable | 1 |  |
| } |  |  |

Allocation flag

Indicates whether this subburst allocation is persistent or not

ACID

The ACID field shall be set to the beginning ACID as described below.

Number of ACID

The Number of ACID (N_ACID) shall be set to the number of HARQ channel identifiers for persistent allocation.  The Number of ACID and ACID fields are used together to establish an implicit cycling of ACID as follows.  If N_ACID is greater than 1, and if the allocation period is greater than 0, the ACID corresponding to the first occurrence of the persistent allocation corresponds to the ACID field, and the ACID corresponding to the second occurrence of the persistent allocation corresponds to the ACID field + 1, etc.  The ACID corresponding to the N_ACIDth occurrence of the persistent allocation corresponds to the ACID field + N_ACID – 1, and the ACID corresponding to the (N_ACID + 1)th occurrence of the persistent allocation corresponds to the ACID field.  This process is repeated for additional occurrences of the persistent allocation.  If the allocation period equals 4, if N_ACID = 4, and if ACID = 2, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc.

AI_SN

The AI_SN shall be set to the initial AI_SN for each ACID in the sequence of ACID values.  The AI_SN shall toggle from 1 to 0 or 0 to 1 for each occurrence of a particular ACID.  For example, if the allocation period equals 4, if N_ACID = 4, if ACID = 2, and if AI_SN = 0, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc, and the AI_SN follows the pattern 0, 0, 0, 0, 1, 1, 1, 1, etc.

Table yyy – UL Slot Offset sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL slot offset sub-IE { |  |  |
| Slot offset | 10 |  |
| Reserved | 2 |  |
| } |  |  |

Slot offset

Indicates the start pointer. When this IE appears before UL allocation sub-IE with non-persistent option, it indicates allocation start position. This does not affect persistent allocation

Table yyy – MIMO UL MCS Change sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO UL MCS Change sub-IE() { |  |  |
| Order | 5 |  |
| Dedicated MIMO UL Control Indicator | 1 |  |
| if (Dedicated MIMO UL Control Indicator==1) { |  |  |
| Dedicated MIMO UL Control IE() | Variable |  |
| } |  |  |
| Delta_duration | 5 |  |
| For (i=0; i<N_layer; i++) { |  | N_layer=2 when MU indicator is set to 1 |
| UIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| } |  |  |
| } |  |  |

Table yyy - MIMO UL Allocation sub-IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO UL Allocation sub-IE () { |  |  |
| MU_indicator | 1 | Indicates whether this UL burst is intended for multiple SS |
| Dedicated MIMO UL Control Indicator | 1 |  |
| ACK disable | 1 |  |
| Order Indication | 1 |  |
| If (Order Indication == 1) { |  |  |
| Order | 5 |  |
| } |  |  |
| if (MU_indicator==0) { |  |  |
| RCID | Variable |  |
| if (Dedicated MIMO UL Control Indicator==1) { |  |  |
| Dedicated MIMO UL Control IE() | Variable |  |
| } |  |  |
| } else |  |  |
| Matrix |  |  |
| } |  |  |
| Allocation flag | 1 | 0: Non-persistent 1: Persistent alloc |
| Duration | Variable | 10/8/6 bits |
| For (i=0; i<N_layer; i++) { |  | N_layer=2 when MU indicator is set to 1 |
| if (MU_indicator==1) { |  |  |
| RCID | Variable |  |
| } |  |  |
| UIUC | 4 |  |
| Repetition Coding Indication | 2 |  |
| if (ACK_diable==0) { |  |  |
| if (Allocation flag==1) { |  |  |
| Number of ACID | 4 | Number of HARQ channels associated with this persistent assignment. It enables ACID cycling |
| } |  |  |
| ACID | 4 |  |
| AI-SN | 1 |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to nibble; Shall be set to 0 |
| } |  |  |