| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Compressed MAC PDU Overhead |  |
| Date Submitted | 2008-01-18 |  |
| Source(s) | Xiao Xu and Hua Xu Motorola Inc. | Voice: 847-632-2176 E-mail: Hua.Xu@motorola.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #26a |  |
| Abstract | This document contains definitions and implementations of the compressed MAC header with purposes to reduce MAC overhead and improve system capacity. |  |
| Purpose | Review and adapt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Compressed MAC PDU Overhead

Xiao Xu and Hua Xu

Motorola Inc.

# Introduction

Generic MAC header (GMH) defined in IEEE 802.16e is 6 bytes.  Compare with typical 44 bytes VoIP MAC PDU, it uses over 13.6% of the resource.

Most of the fields specified in the GMH and other subheaders can be compressed for applications with small payload like VoIP.  In this proposal we introduce compressed MAC header, compressed fragmentation subheader, and compressed packing subheader.  With these new compressed MAC headers, we are able to reduce the MAC PDU overhead by 50% and therefore improve overall system performance.

# Proposed Text

Add a new section in section 6.3.2.1.1 and change the current section to 6.3.2.1.1.1 on Page 51 line 56.

# 6.3.2.1.1.2 Compressed MAC header

The compressed MAC header shall be used for applications such as VOIP, where the payload size is small. Flows using the compressed MAC header will not be ARQ-enabled due to their delay sensitivity. They will use HARQ only. The compressed header is illustrated in Figure 24.

Figure 24 – Compressed MAC header format

The fields of the compressed MAC header are defined in Table 7.  The coding of these fields is such that the first byte of a MAC header shall never have the value of 0xFX, where “X” means “do not care.” This prevents false detection on the stuff byte used in the transmission convergence sublayer (TCS).

Table 7—Compressed MAC header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| HT | 1 | Header type. Shall be set to zero. |
| CID Index | 4 | Index to connection identifier. The index, ranging from 0 – 15, increments based on the numerical order of the 16-bit CID with the first or smallest CID having the index of 0. Up to 16 connections allowed per MS. |
| EKS | 2 | Encryption key sequence. The index of the traffic encryption key (TEK) and initialization vector (IV) used to encrypt the payload. This field is only meaningful if encryption is enabled. Please refer to section 11.13.28 for how to enable encryption during service flow negotiation. |
| ESF | 1 | Extended Subheader field. If ESF = 0, the extended subheader is absent. If ESF = 1, the extended subheader is present and will follow the generic MAC header immediately. (See 6.3.2.2.7.) The ESF is applicable both in the DL and in the UL. |
| HCS | 6 | Header check sequence. A 6-bit field used to detect errors in the header. The transmitter shall calculate the HCS value for the first 18 bits of the cell header, and insert the result into the HCS field (the last 6 bit of the MAC header). It shall be the remainder of the division (Modulo 2) by the generator polynomial g(D) = D6 + D + 1 of the polynomial D6 multiplied by the content of the header excluding the HCS field. |
| LEN | 7 | Length. The length in bytes of the MAC PDU including the MAC header and the CRC if present. |
| Type | 3 | This field indicates the subheaders and special payload types present in the message payload. |

The definition of the Type field is indicated in Table 8.

Table 8—Type encodings

| Type bit | Value |
| --- | --- |
| #2 | Fragmentation subheader (FSH) 1 = present, 0 = absent |
| #1 | Packing subheader (PSH) 1 = present, 0 = absent |
| #0 least significant bit (LSB) | DL: Fast-feedback allocation subheader (FFSH) UL: Grant management subheader (GMSH) 1 = present, 0 = absent |

The capability of the BS and MS to use compressed MAC header is negotiated between BS and MS during SBC-REQ/RSP process. Please refer to section 6.4.3.3.23. If the capability to handle compressed MAC header is confirmed and a service flow has PDU sizes smaller than 127 bytes (e.g., VOIP), BS or MS shall indicate using it in the compressed MAC header TLV in the service flow encodings. (Section 11.13.9)

Add a new section in section 6.3.2.2.1 and change the current section to 6.3.2.2.1.1 on Page 69 line 51.

6.3.2.2.1.2 Compressed Fragmentation subheader (FSH)

If associated with the compressed MAC header, the format of the FSH is illustrated in Table 20.

Table 20— Compressed FSH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Fragmentation Subheader() { | -- | -- |
| FC | 2 | Indicates the fragmentation state of the payload: 00 = No fragmentation 01 = Last fragment 10 = First fragment 11 = Continuing (middle) fragment |
| if (ARQ-enabled Connection) | -- | -- |
| BSN | 11 | Sequence number of first block in the current SDU fragment. |
| else { | -- | -- |
| if (Type bit Extended Type) | -- | See Table 8 |
| FSN | 11 | Sequence number of the current SDU fragment. The FSN value increments by one (modulo 2048) for each fragment, including unfragmented SDUs. |
| Else | -- | -- |
| FSN | 36 | Sequence number of the current SDU fragment. The FSN value increments by one (modulo 864) for each fragment, including unfragmented SDUs. |
| } | -- | -- |
| Reserved | 3 | Shall be set to zero. |
| } | -- | -- |

Add a new section in section 6.3.2.2.3 and change the current section to 6.3.2.2.3.1 on Page 72 line 49.

6.3.2.2.3.2 Compressed Packing subheader (PSH)

When packing (see 6.4.4.4) is used, the MAC may pack multiple SDUs into a single MAC PDU. When packing variable-length MAC SDUs, the MAC precedes each one with a PSH. The PSH is defined in Table 23. If associated with a compressed MAC header, the PSH is defined in Table 25.

Table 23— Compressed PSH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Packing Subheader() { | -- | -- |
| FC | 2 | Indicates the fragmentation state of the payload: 00 = No fragmentation 01 = Last fragment 10 = First fragment 11 = Continuing (middle) fragment |
| if (ARQ-enabled Connection) | -- | -- |
| BSN | 11 | Sequence number of first block in the current SDU fragment. |
| else { | -- | -- |
| if (Type bit Extended Type) | -- | See Table 8 |
| FSN | 11 | Sequence number of the current SDU fragment. The FSN value increments by one (modulo 2048) for each fragment, including unfragmented SDUs. |
| Else | -- | -- |
| FSN | 36 | Sequence number of the current SDU fragment. The FSN value increments by one (modulo 864) for each fragment, including unfragmented SDUs. |
| } | -- | -- |
| Length | 118 | Length of the SDU fragment in bytes including the PSH. |
| } | -- | -- |

Modify section 11.8.2, page 1100 line 26.

11.8.2 Capabilities for Construction and Transmission of MAC PDUs

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 4 | 1 | Bit #0: Ability to receive requests piggybacked with data Bit #1: Ability to use 3-bit FSN values used when forming MAC PDUs on non-ARQ connections Bits #2–7: Reserved; shall be set to zero Add: Bit #2: Ability to use compressed MAC header Add: Bits #3–7: Reserved; shall be set to zero | SBC-REQ (see 6.3.2.3.23) SBC-RSP (see 6.3.2.3.24) |

Modify Table 570, page 1154 line 26.

Table 570—Service flow encodings

| Type | Parameter |
| --- | --- |
| 10 | Reserved |
| 10 | Compressed MAC header. |

Modify section 11.13.9 on page 1160 line 20.

11.13.9 Reserved

11.13.9 Compressed MAC header

The parameter specifies if the compressed MAC header (see section 6.3.2.1.1.2) shall be used in order to reduce MAC overhead. This service flow parameter is only meaningful if both BS and SS are capable of handling the compressed MAC header, as established during the SBC-REQ/RSP process. (See section 11.8.2)

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].10 | 1 | 1 = compressed MAC header is used 0 = compressed MAC header is not used and generic MAC header is used. | DSx-REQ DSx-RSP DSx-ACK REG-RSP |