| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Retry counter for message ranging |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Jaejeong(Brian) Shim, Kyeong-Tae Do Samsung Electronics* | Voice: E-mail: brian.shim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26a |  |
| Abstract | This contribution proposed a load balancing scheme during initial network entry, HO, or re-entry from idle mode. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Retry counter for message ranging

Samsung Electronics

# Problem description

# In the current 802.16 standard, the only retransmission counter for ranging is 'Contention Ranging Retries (Min: 16)'. But in OFDMA, we have two kinds of ranging mechanisms - CDMA code-based ranging and message-based ranging.

# All the retransmission counters for message transaction such as 'Registration Request Retries' have minimum values of 3 because the message TX has already UL allocation. The message transmission is not a contention based TX.

# If we use the 'Contention Ranging Retries (Min: 16)' for message ranging, the MS will retry up to 16 times if it detect an error during RNG-REQ/RSP transaction though it has UL allocation successfully. Generally, CDMA code is more robust than management message.

# The problem can happen during initial network entry and HO. Especially in HO case, the MS may suffer very long service interruption.

# Proposed Changes

# Adopt a new retransmission counter for message ranging. Make the original counter, 'Contention Ranging Retries (Min: 16)' be used in CDMA code-based ranging only.

Change #1:

Adopt the following changes in Figure 99:

Figure 99—CDMA initial ranging—SS (part 4)

Change #2: Add an item in Table 525 - Parameters and constants:

Add the following item into Table 525:

Table 525— Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| SS | Ranging Request Retries | Number of retries on ranging requests by RNG-REQ messages (OFDMA only) | 3 | — | — |

# References

[IEEE802.16-Rev2/D2]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D2 (December 2007). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.