| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Lost DL/UL-MAP Intervals |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Jaejeong(Brian) Shim, Samsung Electronics* | Voice: E-mail: brian.shim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26a |  |
| Abstract | This contribution proposed a load balancing scheme during initial network entry, HO, or re-entry from idle mode. |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Lost DL/UL-MAP Intervals

Samsung Electronics

# Problem description

# Current maximum values of Lost DL-MAP interval and Lost UL-MAP interval are both 600ms.

# The MS can initiate HO re-enrty by transmitting RNG-REQ with Serving BSID and CMAC before the Lost DL-MAP interval/Lost UL-MAP interval expires. In this case, BS can reuse MS context information to perform network re-entry with HO process optimization.

# However, if an MS suffers DL or UL-MAP lost for 600ms, it will restart its MAC state according to current 802.16 standards. Thus the BS cannot reuse the MS context information because the MS will start a full network entry after MAC restart.

# The value 600ms is reasonable during initial network entry phase. But it is too small after the initial network entry phase.

# The maximum value of System resource retain timer is 65535*100ms and that of Serving BS ID AGING-TIMER is 655 sec.

# Proposed Changes

Take a larger value (655 sec) for both Lost DL-MAP interval and Lost UL-MAP interval after MS’ completion of initial network entry.

Change #1: Adopt the following changes in Table 525 - Parameters and constants:

Adopt the following changes in Table 525:

Table 525— Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| SS | Lost DL-MAP Interval | Time since last received DL-MAP message before DL synchronization is considered lost _ | — | — | 600 ms (during initial network entry), 655 s (after initial network entry) |
| SS | Lost UL-MAP Interval | Time since last received UL-MAP message before UL synchronization is considered lost | — | — | 600 ms (during initial network entry), 655 s (after initial network entry) |

# References

[IEEE802.16-Rev2/D2]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D2 (December 2007). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.