| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced Retransmission Scheme for Persistent Allocations |  |
| Date Submitted | 2008-01-10 |  |
| Source(s) | Vladimir Yanover Alvarion Nadav Lavi Alvarion | E-mail: Vladimir.yanover@alvarion.com Nadav.lavi@alvarion.com |
| Re: | Letter Ballot 26a |  |
| Abstract | The resource allocation scheme of 802.16e is extended to enable support of efficient retransmissions in persistent allocations. |  |
| Purpose | Accept the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Enhanced Retransmission Scheme for Persistent Allocations

Nadav Lavi, Vladimir Yanover

Alvarion

# Introduction

Due to the fact that there is an element in the MAP per transmission/allocation, the MAP consumes large part of the DL sub-frame. To reduce this enormous overhead and to support large number of VoIP users, persistent MAP scheme was introduced. With this new scheme the MAP overhead is reduced substantially. However, when persistent allocations are used with HARQ, the major MAP overhead appears to be in the retransmissions MAP elements. To enable further reduction of MAP overhead we need to address HARQ retransmissions overhead.

This idea can be used in combination with virtually any format of persistent allocation MAP IE, particularly with the format proposed in [1].

# Proposed Solution

The persistent allocation scheme enables the transmission of a single IE in the MAP indicating multiple allocations/transmissions. This IE indicates a group of sub-bursts in the DL or bursts in the UL (and hence MSs) that are retransmitted or requested to be retransmitted. Utilizing this indication, and with the addition of saving the allocations parameters by the MSs (minor information storing) we can reduce the size of the retransmission IE.

Here is a summary of the scheme:

- In case a MS is given a persistent allocation it is expected to store the following parameters:

  - Its own allocation parameters, i.e., DL/UL sub-burst/burst location, MCS, and repetition.

  - The duration fields of the previous sub-bursts in the persistent allocation

  - The frame number the persistent allocation was sent

  - The persistent IE number in the DL or UL MAP

- Then in case specific DL or UL burst needs retransmission the retransmission IE will be included into the MAP with the following information:

  - Frame number and IE number where the persistent allocation was sent

  - Indication on which sub-bursts should be retransmitted

# Proposed Text Modification

Modify in the Table 346—HARQ DL MAP IE format:

1. The mode field of the existing HARQ DL MAP IE as follows:

| Mode | 4 | Indicates the mode of this HARQ region: 0b0000: Chase HARQ 0b0001: Incremental redundancy HARQ for CTC 0b0010: Incremental redundancy HARQ for Convolutional Code 0b0011: MIMO Chase HARQ 0b0100: MIMO IR HARQ 0b0101: MIMO IR HARQ for Convolutional Code 0b0110: MIMO STC HARQ 0b0111: Persistent Chase HARQ 0b1000: Group ReTx Chase HARQ 0b1001-0b1111: Reserved |
| --- | --- | --- |

2. Add the following “else if”  group to the existing HARQ DL MAP IE after the “else if” group of supposed Persistent IE

| } else if (Mode == 0b1000) { | − | − |
| --- | --- | --- |
| Group_ReTx_DL_HARQ_Chase_Sub-Burst_IE() | variable | − |

3. Add new section to 8.4.5.3.21

8.4.5.3.21.3 Downlink Group HARQ Retransmission

In case persistent allocation scheme is used, the Group Retransmission IE enables the BS to indicate which sub-bursts should be retransmitted without the need to include the PHY information which is already indicated in the Persistent Allocation MAP IE.

Every MS stores parameters of its allocation in the persistent IE, the physical transmission parameters (modulation, coding, repetition, boosting, and duration), and the duration of the sub-bursts appearing in the persistent IE before its sub-burst.

It is assumed that the ACID used in the retransmission is identical to that of the latest persistent allocation of the connection not including the current frame. Figure A illustrates the usage of ACIDs and the timeline for retransmission opportunities.

Figure A – Group Retransmission operation, ACIDs usage, and timeline.

In case the BS needs to perform further retransmissions after a new transmission occurs at the connection, i.e., after allocation period time, it shall use the regular non-persistent IEs, e.g, DL_HARQ_Chase_Sub-Burst_IE.

In case the BS needs the retransmission to use a different repetition and/or boosting it also shall use the regular non-persistent IEs.

For downlink group retransmission, the base station transmits the HARQ DL MAP IE, with the mode field set to the value 0b1000

Table XXX – DL Chase Group Retransmission IE

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| DL Chase Sub-Burst IE For Group ReTx{ | --- | --- |
| N sub burst | 4 | Equal to the number of 1 bits in the group bitmap field |
| N ACK channel | 4 | --- |
| Frame number | 10 | Reference to the frame number where the original persistent allocation was transmitted |
| Persistent IE number | 4 | Reference to the persistent IE in the DL map which defined the allocation for the original transmission |
| Group bitmap | variable | Indicating the MSs that are required to do retransmission (byte aligned) |
| } |  |  |

Frame number

The frame number when the persistent DL allocation IE was transmitted. This parameter together with Persistent IE number identifies the DL persistent allocation IE

Persistent IE number

This numbering refers only to persistent IEs within the DL MAP. This parameter together with Frame numberidentifies the DL persistent allocation IE

Figure B – Retransmission reference to the persistent IE.

Group bitmap

The bitmap provides an indication regarding the sub-bursts that are retransmitted. The bits in the bitmap correspond to consequent sub-bursts in the persistent allocation, in the order of bytes in the bitmap, starting from the MSB in each byte.

To interpret the Group Retransmission IE, the MS calculates its sub-burst position within the burst by summarizing durations of those of previous sub-bursts in the DL Persistent Allocation IE which have ‘1’ at the corresponding locations of the bitmap. Physical parameters used for the transmission are identical to those specified in the DL Persistent Allocation IE

4. Modify Table 368 as follows.

Table 368 - Extended-2 UIUC code assignment for UIUC = 11

| Extended-2 Type (hexadecimal) | Usage |
| --- | --- |
| 00 | CQICH Enhanced Allocation IE |
| 01 | HO Anchor Active UL-MAP IE |
| 02 | HO Active Anchor UL-MAP IE |
| 03 | Anchor BS Switch IE |
| 04 | UL Sounding Command IE |
| 05 | Reserved |
| 06 | MIMO UL Enhanced IE |
| 07 | HARQ UL MAP IE |
| 08 | HARQ ACKCH Region Allocation IE |
| 09 | MIMO UL Basic IE |
| 0A | Mini-subchannel allocation IE |
| 0B | Persistent HARQ UL MAP IE |
| 0C | Group HARQ Retransmission UL MAP IE |
| 0C …0D | Reserved |
| 0E | AAS SDMA UL IE |
| 0F | Feedback Polling IE |

5. Add a new section to 8.4.5.4.24

8.4.5.4.24.4 Uplink Group HARQ Retransmission

In case persistent allocation scheme is used, the UL Group Retransmission IE enables the BS to indicate which sub-bursts should be retransmitted without the need to include the PHY information which is already indicated in the Persistent Allocation MAP IE.

Every MS stores its location in the persistent IE, the physical transmission parameters (modulation, coding, repetition, boosting, and duration), and the duration of the sub-bursts appearing in the persistent IE before its sub-burst.

It is assumed that the ACID used in the retransmission is identical to that of the latest persistent allocation of the connection. Figure C illustrates the usage of ACIDs and the timeline for retransmission opportunities.

Figure C – Retransmission operation, ACIDs usage, and timeline.

In case the BS needs to perform further retransmissions after a new transmission occurs, i.e., after allocation period time, it shall use the regular non-persistent IEs, e.g, HARQ_UL-MAP_IE.

In addition, if the BS requires the retransmission to use a different repetition and/or boosting it also must use the regular non-persistent IEs.

Table YYY – UL Chase for group retransmission IE

| Syntax | Length (bits) | Comments |
| --- | --- | --- |
| UL Chase IE For Group ReTx{ | --- | --- |
| Extended2- UIUC | 4 | 0x0C |
| Length | 8 | Length of the IE |
| Frame number | 10 | Reference to the frame number where the original persistent allocation was transmitted |
| Persistent IE number | 4 | Reference to the persistent IE in the DL map which defined the allocation for the original transmission |
| Slot offset | 10 | Indicates the start of this Persistent allocation in OFDMA slots, with respect to the start of the UL subframe |
| Group bitmap | variable | Indicating the MSs that are required to do retransmission (byte aligned) |
| } |  |  |

Frame number

The frame number when the persistent UL allocation IE was transmitted. This parameter together with Persistent IE number identifies the DL persistent allocation IE

Persistent IE number

This numbering refers only to persistent IEs within the UL MAP. This parameter together with Frame number identifies the UL persistent allocation IE

To interpret the Group Retransmission IE, the MS calculates its burst position within the UL subframe by summarizing durations of those of previous sub-bursts in the UL Persistent Allocation IE which have ‘1’ at the corresponding locations of the bitmap. Physical parameters used for the transmission are identical to those specified in the UL Persistent Allocation IE

Figure D – Retransmission reference to the persistent IE.

Group bitmap

The bitmap provides an indication regarding the bursts that are retransmitted. The bits in the bitmap correspond to consequent sub-bursts in the persistent allocation, in the order of bytes in the bitmap, starting from the MSB in each byte.

To interpret the Group Retransmission IE, the MS calculates its sub-burst position within the UL subframe by summarizing durations of those of previous bursts in the UL Persistent Allocation IE which have ‘1’ at the corresponding locations of the bitmap. Physical parameters used for the transmission are identical to those specified in the UL Persistent Allocation IE

# References

- Persistent Allocation, IEEE contribution for Rev2/D2.