| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes in QoS parameters for UGS |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Daniel Cohn Alvarion | E-mail: daniel.cohn@alvarion.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #26a |  |
| Abstract | This contribution points out at inconsistencies in UGS QoS parameters and proposes a solution |  |
| Purpose | Propose changes to QoS parameter set for UGS scheduling type |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Changes in QoS parameters for UGS ST

Daniel Cohn

Alvarion

# Scope

The scope of this document is to point out at inconsistencies in IEEE 802.16Rev2/D2 definition of QoS parameters for the unsolicited grant service (UGS) type of data delivery services (ToDDS) and scheduling type (ST) and to propose minor changes for correcting this problem.

# Problem Statement

Table 194 in section 6.3.20.1.1 of IEEE 802.16Rev2/D2 defines QoS parameters for the UGS ToDDS. Section 6.3.5.2.1 in the first paragraph defines QoS parameters for the UGS ST.

The table below compares the QoS parameter sets defined by these two references.

| Section 6.3.20.1.1 definitions for UGS ToDDS | Section 6.3.5.2.1 definitions for UGS ST |
| --- | --- |
| Tolerated Jitter | Tolerated Jitter |
| SDU size (if fixed length SDU service flow) |  |
| Minimum reserved traffic rate | Minimum reserved traffic rate |
| Maximum latency | Maximum latency |
| Request/Transmit policy | Request/Transmit policy |
| Unsolicited grant interval | Unsolicited grant interval |
|  | Maximum sustained traffic rate |
|  | Uplink Grant Scheduling Type |

For consistency reasons, it is expected that QoS parameter sets for the UGS ToDDS and the UGS ST will be identical, with the exception of the Uplink Grant Scheduling Type parameter that is only relevant for the scheduling type. This is further supported by the fact that, according to table 193 in section 6.3.20.1, uplink UGS ToDDS service flows should be supported by UGS ST.

The following discrepancies therefore exist:

- Maximum sustained traffic rate (MSTR) is defined by 6.3.5.2.1 as a mandatory UGS ST QoS parameter but it is not part of UGS ToDDS parameter set according to 6.3.20.1.1

- For fixed-length SDU service flows, SDU size is defined by 6.3.20.1.1 as a UGS ToDDS parameter but not it is not part of ST ToDDS parameter set according to 6.3.5.2.1

# Proposed Solution

For the discrepancy indicated in a) above, we propose to eliminate MSTR as a QoS parameter for the UGS ST. Minimum reserved traffic rate (MRTR), together with unsolicited grant interval, is sufficient to determine the allocation size for UGS service flows.

For the discrepancy indicated in b) above, we propose to add SDU size as a QoS parameter for UGS ST for fixed-length-SDU service flows.

# Required Changes in Standard

## Changes in section 6.3.5.2.1

Change first paragraph as follows:

The UGS is designed to support real-time uplink service flows that transport fixed-size data packets on a

periodic basis, such as T1/E1 and Voice over IP without silence suppression. The service offers fixed-size

grants on a real-time periodic basis, which eliminate the overhead and latency of SS requests and assure that

grants are available to meet the flow’s real-time needs. The BS shall provide Data Grant Burst IEs to the SS

at periodic intervals based upon the Maximum Sustained Minimum Reserved Traffic Rate of the service flow. The size of these grants shall be sufficient to hold the fixed-length data associated with the service flow (with associated generic MAC header and GMSH) but may be larger at the discretion of the BS scheduler. In order for this service to work correctly, the Request/Transmission Policy (see 11.13.12) setting shall be such that the SS is prohibited from using any contention request opportunities for this connection. The mandatory QoS

parameters are Maximum Sustained Minimum Reserved Traffic Rate (11.13.6), Maximum Latency (11.13.14), Tolerated Jitter (11.13.13), Uplink Grant Scheduling Type (11.13.11), SDU size (for fixed length SDU service flows) (11.13.16), and Request/Transmission Policy (11.13.12) and Unsolicited Grant Interval (11.13.20). If present, the Minimum Reserved Traffic Rate parameter (11.13.8) shall have the same value as the Maximum Sustained Traffic Rate parameter.

Change third paragraph as follows:

The BS shall not allocate more bandwidth than the Maximum Sustained Minimum Reserved Traffic Rate parameter of the active QoS parameter set, excluding the case when the SI bit of the Grant Management field is set. In this case, the BS may grant up to 1% additional bandwidth for clock rate mismatch compensation.

Also referred to as “scheduling service”