| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover and User Matrices objects for handover performance characterization |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Joey Chou Intel Shawn Wallace Sprint Fran Burns Sprint | E-mail: joey.chou@intel.com E-mail: Shawn.Wallace@sprint.com E-mail: francis.burns@sprint.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #26a, on P802.16Rev2/D2, as announced in IEEE 802.16-07/058 |  |
| Abstract | This document proposes handover and user matrices objects that are needed to characterize handover performance. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Handover and User Matrices objects for handover performance characterization

Joey Chou, Shawn Wallace, Fran Burns

Intel, Sprint

# Introduction

This document proposes the following counter objects that are important to characterize the 802.16 WMAN handover performances.

- wmanIf2BsSsHandoverStatisticsTable

- wmanIf2BsSsUserMatricsTable

- wmanIf2BsSsCidMatricsTable

- wmanIf2BsPerformanceCountersTrap

# Proposed Text

# 2.1	MIB Module Description

13.1.3.4 wmanIf2BsPm

[Change Figure 324 as the following:]

Figure 324 – wmanIf2BsPm Structure

[Add subclauses 13.1.3.4.7 as the following:]

# 13.1.3.4.7 wmanIf2BsSsHandoverMatricsTable

# wmanIf2BsSsHandoverMatricsTable contains statistical information that can be used to characterize MS' performance during the handover.

# 13.1.3.4.8 wmanIf2BsSsUserMatricsTable

wmanIf2BsSsUserMatricsTable contains counter objects to track user matrices.

# 13.1.3.4.9 wmanIf2BsSsCidMatricsTable

wmanIf2BsSsCidMatricsTable tracks the number of basic and primary CIDs, and the average and maximum number of user CIDs..

# 2.2	ASN.1 Notations – wmanIf2BsSsHandoverMatricsTable

It proposes the following new code be added to the wmanIf2Mib.

--

-- Mobile Station handover matrics Table

--

wmanIf2BsSsHandoverMatricsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsHandoverMatricsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains matrics information that can be

used to characterize MS' performance during the handover.

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsPm 7 }

wmanIf2BsSsHandoverMatricsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsHandoverMatricsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsSsHandoverMatricsTable  1 }

WmanIf2BsSsHandoverMatricsEntry ::= SEQUENCE {

wmanIf2BsSsHandoverAttempt              Counter64,

wmanIf2BsSsHandoverSuccess              Counter64,

wmanIf2BsSsHandoverSuccessRate          INTEGER,

wmanIf2BsSsHandoverCancel               Counter64,

wmanIf2BsSsHandoverReject               Counter64,

wmanIf2BsSsHandoverCancelRate           INTEGER,

wmanIf2BsSsHandoverRejectRate           INTEGER,

wmanIf2BsSsAvgHandoverTime              INTEGER,

wmanIf2BsSsUnexpectedHandover           Counter64}

wmanIf2BsSsHandoverAttempt OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of MS initiated handover

attempts. The serving BS increments the counter by one each

time the BS receives MOB_MSHO-REQ from MS."

::= { wmanIf2BsSsHandoverMatricsEntry 1 }

wmanIf2BsSsHandoverSuccess OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of successful MS initiated

handover. The serving BS increments the counter by one each

time the BS sends MOB_BSHO-RSP to MS."

::= { wmanIf2BsSsHandoverMatricsEntry 2 }

wmanIf2BsSsHandoverSuccessRate OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsSsHandoverSuccessRate % =

wmanIf2BsSsHandoverSuccess / wmanIf2BsSsHandoverAttempt

* 100"

::= { wmanIf2BsSsHandoverMatricsEntry 3 }

wmanIf2BsSsHandoverCancel OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of MS initiated handover

cancellation. The BS serving increments the counter by one

each time the BS receives MOB_HO-IND with

HO_IND_type = 'HO cancel'"

::= { wmanIf2BsSsHandoverMatricsEntry 4 }

wmanIf2BsSsHandoverReject OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of MS initiated handover

rejected by MS. The serving BS increments the counter by

one each time the BS receives MOB_HO-IND with

HO_IND_type = 'HO reject'"

::= { wmanIf2BsSsHandoverMatricsEntry 5 }

wmanIf2BsSsHandoverCancelRate OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsSsHandoverCancelRate % =

wmanIf2BsSsHandoverCancel  / wmanIf2BsSsHandoverAttempt

* 100"

::= { wmanIf2BsSsHandoverMatricsEntry 6 }

wmanIf2BsSsHandoverRejectRate OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsSsHandoverRejectRate % =

wmanIf2BsSsHandoverReject  / wmanIf2BsSsHandoverAttempt

* 100"

::= { wmanIf2BsSsHandoverMatricsEntry 7 }

wmanIf2BsSsAvgHandoverTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average handover time Avg(N)

measured at target BS. The BS measures the number of

handovers occured withing each interval set by the

wmanIf2BsDataSampleInterval object. The formula of the

measurement is shown below:

T = Handover time is the amount of time between:

1. Target BS receives the backbone handover indication

message from the serving BS for a give MS

2. The given MS completes the ranging successfully by

receiving RNG-RSP from the target BS

N = the number of handover in a reporting interval

Avg(N) = the average amount of time after N handovers

Avg(0) = T, the 1st handover

Avg(N) = (Avg(N-1)*(N-1) + T) / N"

::= { wmanIf2BsSsHandoverMatricsEntry 8 }

wmanIf2BsSsUnexpectedHandover OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of unexpected MS initiated

handover attempts. The target BS increments the counter by

one each time the target BS receives RNG-REQ from a MS

initiated handover without backbone handover indication

message from the serving BS."

::= { wmanIf2BsSsHandoverMatricsEntry 9 }

# 2.3	ASN.1 Notations – wmanIf2BsSsUserMatricsTable

It proposes the user matrices objects.

--

-- Mobile Station user matrics Table

--

wmanIf2BsSsUserMatricsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsUserMatricsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains counter objects to track user matrics.

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsPm 8 }

wmanIf2BsSsUserMatricsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsUserMatricsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsSsUserMatricsTable  1 }

WmanIf2BsSsUserMatricsEntry ::= SEQUENCE {

wmanIf2BsSsActiveUsers                  Counter64,

wmanIf2BsSsPeakNormalModeUsers          Counter64,

wmanIf2BsSsPeakSleepModeUsers           Counter64,

wmanIf2BsSsPeakIdleModeUsers            Counter64,

wmanIf2BsSsAvgNormalModeUsers           INTEGER}

wmanIf2BsSsActiveUsers OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object captures the number of active users over a

reporting period by counting the number of SS's that have

active CIDs."

::= { wmanIf2BsSsUserMatricsEntry 1 }

wmanIf2BsSsPeakNormalModeUsers OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of peak normal mode users.

The serving BS increments the counter by one each time the

BS receives REG-REQ from a MS and returns REG-RSP."

::= { wmanIf2BsSsUserMatricsEntry 2 }

wmanIf2BsSsPeakSleepModeUsers OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of peak sleep mode users.

The serving BS increments the counter by one each time the

BS successfully puts a MS into sleep mod by doing the

following.

1. BS receives MOB_SLP-REQ from a MS and returns

MOB_SLP-RSP.

2. BS send MOB_SLP-RSP to a MS autonomously."

::= { wmanIf2BsSsUserMatricsEntry 3 }

wmanIf2BsSsPeakIdleModeUsers OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of peak idle mode users.

The serving BS increments the counter by one each time the

BS successfully puts a MS into the idle mode by doing the

following.

1. MS sends DREG-REQ with a De-Registration_Request_Code =

0x01; BS returns DREG-CMD message to the MS.

2. BS sends a DREG-CMD with an Action Code = 0x05 in

unsolicited manner; MS returns the DREG-REQ message with

De-Registration_Request_Code = 0x02"

::= { wmanIf2BsSsUserMatricsEntry 4 }

wmanIf2BsSsAvgNormalModeUsers OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average normal mode users Avg(N)

measured at BS. The BS measures the number of normal mode

users recorded within each interval set by the

wmanIf2BsDataSampleInterval object. The formula of the

measurement is shown below:

T = the number of normal mode users recorded in each sample

interval

N = the number of samples in a reporting interval

Avg(N) = the average of normal mode users

Avg(0) = T, the 1st sample

Avg(N) = (Avg(N-1)*(N-1) + T) / N"

::= { wmanIf2BsSsUserMatricsEntry 5 }

# 2.4	ASN.1 Notations – wmanIf2BsSsCidMatricsTable

It proposes the CID matrices objects.

--

-- Mobile Station connection ID matrics Table

--

wmanIf2BsSsCidMatricsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsCidMatricsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table tracks the number of basic and primary CIDs, and

the average and maximum number of user CIDs.

For example:

T = the number of user CIDs measured in a sample interval

N = the number of samples in a reporting interval

Avg(N) = the average anumber of user CIDs

Avg(0) = T, the 1st sample

Avg(N) = (Avg(N-1)*(N-1) + T) / N

Max(N) = the maximum number of user CIDs

Max(0) = T, the 1st sample

Max(N) = T if T > Max(N-1)

= Max(N-1) if T <= Max(N-1)

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsPm 9 }

wmanIf2BsSsCidMatricsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsCidMatricsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsSsCidMatricsTable  1 }

WmanIf2BsSsCidMatricsEntry ::= SEQUENCE {

wmanIf2BsSsBasicAndPrimaryCids          Counter64,

wmanIf2BsSsMaximumUserCids              Counter64,

wmanIf2BsSsAvgUserCids                  Counter64}

wmanIf2BsSsBasicAndPrimaryCids OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object captures the number of basic and primary CIDs

in a reporting period."

::= { wmanIf2BsSsCidMatricsEntry 1 }

wmanIf2BsSsMaximumUserCids OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the maximum anumber of user CIDs Avg(N)

, measured in a reporting interval"

::= { wmanIf2BsSsCidMatricsEntry 2 }

wmanIf2BsSsAvgUserCids OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average number of user CIDs Avg(N)

, measured in a reporting interval"

::= { wmanIf2BsSsCidMatricsEntry 3 }

# 2.5	ASN.1 Notations – wmanIf2BsPerformanceCountersTrap

It proposes the handover and user matrices object to be added to wmanIf2BsPerformanceCountersTrap.

wmanIf2BsPerformanceCountersTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsAuthenAttempt,

wmanIf2BsSsAuthenSuccess,

wmanIf2BsSsAuthenSuccessRate,

wmanIf2BsSsRangingAttempt,

wmanIf2BsSsRangingSuccess,

wmanIf2BsSsRangingSuccessRate,

wmanIf2BsAvgDlUserThroughput,

wmanIf2BsAvgUlUserThroughput,

wmanIf2BsAvgDlMacThroughput,

wmanIf2BsAvgUlMacThroughput,

wmanIf2BsAvgDlPhyThroughput,

wmanIf2BsAvgUlPhyThroughput,

wmanIf2BsPeakDlUserThroughput,

wmanIf2BsPeakUlUserThroughput,

wmanIf2BsPeakDlMacThroughput,

wmanIf2BsPeakUlMacThroughput,

wmanIf2BsPeakDlPhyThroughput,

wmanIf2BsPeakUlPhyThroughput,

wmanIf2BsAvgDlCellEdgeThroughput,

wmanIf2BsAvgUlCellEdgeThroughput,

wmanIf2BsDataRateMeasurements,

wmanIf2BsAvgNetworkEntryTime,

wmanIf2BsMaxNetworkEntryTime,

wmanIf2BsAvgNetworkReEntryTime,

wmanIf2BsMaxNetworkReEntryTime,

wmanIf2BsNumOfNetworkEntries,

wmanIf2BsNumOfNetworkReEntries,

wmanIf2BsDlPacketsSend,

wmanIf2BsDlPacketsErrored,

wmanIf2BsDlPacketErrorRate,

wmanIf2BsEventNotificationTime,

wmanIf2BsSsHandoverAttempt,

wmanIf2BsSsHandoverSuccess,

wmanIf2BsSsHandoverSuccessRate,

wmanIf2BsSsHandoverCancel,

wmanIf2BsSsHandoverReject,

wmanIf2BsSsHandoverCancelRate,

wmanIf2BsSsHandoverRejectRate,

wmanIf2BsSsAvgHandoverTime,

wmanIf2BsSsUnexpectedHandover,

wmanIf2BsSsActiveUsers,

wmanIf2BsSsPeakNormalModeUsers,

wmanIf2BsSsPeakSleepModeUsers,

wmanIf2BsSsPeakIdleModeUsers,

wmanIf2BsSsBasicAndPrimaryCids,

wmanIf2BsSsMaximumUserCids,

wmanIf2BsSsAvgUserCids}

STATUS      current

DESCRIPTION

"This event should be automatically generated every

wmanIf2BsCountersReportInterval interval to report the

performance counters for a given BS sector identified by

ifIndex. The performance counters will be reset after this

event being sent to EMS."

::= { wmanIf2BsTrapPrefix 6 }