| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications of Unsolicited MOB_SCN-RSP message |  |
| Date Submitted | 2008-01-13 |  |
| Source(s) | Lily Zhu, Peretz Feder, Dan Gal Alcatel-Lucent 67 Whippany Rd. Whippany, NJ 07981, USA | Voice: 973-386-6976 E-mail: lilyz@alcatel-lucent.com pfeder@alcatel-lucent.com dgal@alcatel-lucent.com |
| Re: | P802.16Rev2/D1 ballot comments |  |
| Abstract | Proposals to add clarification to handling of Unsolicited MOB_SCN-RSP message |  |
| Purpose | To clarify the MS behavior upon receiving unsolicited MOB_SCN-RSP message |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications of ‘Unsolicited MOB_SCN-RSP message’

Lily Zhu, Dan Gal, Peretz Feder

Alcatel-Lucent

# Problem Description

The current text of the standard, describing the usage of unsolicited MOB_SCN-RSP message, is ambiguous and may lead to different interpretations. In fact, views gathered from different equipment designers (from the same and across vendors) prior to the submission of this contribution indicate a multiplicity of interpretations to the current text. This contribution presents the issues which require clarification by way of changes in the Standard.

Section 6.3.22.1.2 of P802.16 Rev2/D2 (and earlier versions) defines an unsolicited MOB_SCN-RSP message (without assignment of scanning interval and a scan duration of zero, that can be used by the BS to trigger MS’s reporting of scanning rsulet see citation below) albeit,, without actually triggering a scanning action.  This is specifically intended for use by MS(s) that perform autonomous scanning of same-carrier neighbor BSs.

Line 7 pager 429: “For unsolicited MOB_SCN-RSP message transmitted without assignment of scanning interval, a value of  zero for Scan duration is used to trigger MS to report scanning result, without explicitly assigning scanning intervals to the MS. In this case, the MS shall only update Report Mode, Report Period and Report Metric based on the information received in MOB_SCN-RSP message.”

However, the above statement is not clear because the standard defined three distinct report modes in MOB_SCN-RSP message for scan report (see Report Mode in section 6.3.2.3.49):

- 0b00: no report

- 0b01: periodic report

- 0b10: event-triggered report

- 0b11: Reserved

MOB_SCN-REP message defined in §6.3.2.3.50 only states that MS shall report scan results when the “report mode” in MOB_SCN-RSP is set to either “0b01” or “0b10”.  
Issue 1: The MS’s behavior upon “report mode = 0b00” is not specified.

In addition, there is one particular scenario which is not addressed by the standard:

- When the MS performs autonomous scanning, the "periodic report” mode and the “event triggered report” mode provide a mechanism for the MS to report the results of a prior scanning it has triggered automatically, either

  - Periodically or

  - Based on some solicited or unsolicited trigger

Issue 2: This however, does NOT provide a method for the BS to solicit a one-time scan result (of all neighbor BSs and serving BS) on BS’s demand.

Issue 3: When periodic report is enabled for autonomous scanning, the method to stop the periodic reporting is not clearly defined in the standard.

The above three issues are separate, but the following change proposal could solve them and provide an efficient way to solicit a scan report on demand, which is required when autonomous scanning is used.

# Proposed Text Changes

The text below provides the changes required to clarify the behavior of “unsolicited MOB_SCN-RSP”.

When the “scan duration” field is set to the value of “0” in an unsolicited MOB_SCN-RSP message, the message should be used to configure MS’s scan reporting in autonomous scanning mode as follows:  
Note: Proposed new text below is shaded and in red

[Add new text in subclause 6.3.2.3.46, page 208, after line 34]

“When the “report mode” = 0b00 (no report), the MS shall send a single MOB_SCN-REP to the BS immediately after it receives the MOB_SCN-RSP message.  The MS shall stop any coordinated scanning or reporting activity after the immediate MOB_SCN-REP.  The autonomous scanning shall still continue as always.”

[Add new text in 6.3.2.3.46, page 211,  after lines 6]

“In the MOB_SCN-REP message, responding to the MOB_SCN-RSP message (scan duration = 0 and report mode = 0b00), the report mode can be set to either 0 or 1.”

[Add new text in subclause 6.3.22.1.2, page 429, after the first paragraph at line 13]

“During a race condition scenario, when the BS sends an unsolicited MOB_SCN-RSP independent of an MS initiated MOB_SCN-REQ, the BS shall respond to the MOB_SCN-REQ message with another MOB_SCN-RSP.  The last MOB_SCN-RSP from the BS shall overwrite its original unsolicited message.

When multiple MOB_SCN-RSP (solicited or unsolicited) messages are received by the MS (over time), the MS shall always respond to the most recently received unsolicited MON_SCN-RSP message.  For example: The MOB_SCN-RSP message with scan duration 0 and reporting mode “0b00” can also be used to stop a periodic scanning/reporting since the MS has to stop any scanning/reporting after it acts on this message.”