| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications of Unsolicited MOB_SCN-RSP message |  |
| Date Submitted | 2008-01-24 |  |
| Source(s) | Lily Zhu, Peretz Feder, Dan Gal Alcatel-Lucent 67 Whippany Rd. Whippany, NJ 07981, USA | Voice: 973-386-6976 E-mail: lilyz@alcatel-lucent.com pfeder@alcatel-lucent.com dgal@alcatel-lucent.com |
| Re: | P802.16Rev2/D2 ballot comments |  |
| Abstract | Proposals to add clarification to handling of Unsolicited MOB_SCN-RSP message |  |
| Purpose | To clarify the MS behavior upon receiving Unsolicited MOB_SCN-RSP message |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications of ‘Unsolicited MOB_SCN-RSP message’

Lily Zhu, Dan Gal, Peretz Feder

Alcatel-Lucent

# 1. Problem Description

The current draft standard (Rev2/D2) does not specify how the BS should retrieve a one-time scan results from the MS.

#### 1.1 Proposed solution

 Use the BS’s unsolicited MOB_SCN-RSP message defined in Section 6.3.22.1.2

The current Standard text:  page 429, line 7:

“For unsolicited MOB_SCN-RSP message transmitted without assignment of scanning interval, a value of  zero for Scan duration is used to trigger MS to report scanning result, without explicitly assigning scanning intervals to the MS. In this case, the MS shall only update Report Mode, Report Period and Report Metric based on the information received in MOB_SCN-RSP message.”

The standard defines the following report modes (in section 6.3.2.3.49).

- 0b00: no report

- 0b01: periodic report

- 0b10: event-triggered report

- 0b11: reserved

 We propose to designate 0b11 as the “One-time scan report”

#### 1.2 	Proposed text changes to Rev2/D2

- On page 204, table 145, (snapshot shown below), make the following changes:

 For the “Report mode” parameter, change bit-pair 0b11 code from “reserved” to “One-time report”.

0b00: no report

0b01: periodic report

0b10: event-triggered report

0b11: One-time scan report

(2)  [Modify a sentence in subclause 6.3.2.3.46, on page 208, line 6]

“For a periodic report (i.e., Report Mode is 0b01) and for One-time Scan report (Report Mode is 0b11), the MS should report the scanning results to its serving BS at the time indicated in the MOB_SCN-RSP message except when it is in the scanning interval. The MS shall include all available scanning results for the requested BSs specified in the said MOB_SCN-RSP message.”

(3)  [add the following new text on page 429, line 12]

“When the Report Mode is 0b11, the MS should report its actual scanning results for neighbors that were scanned due to a previous MOB_SCN-RSP as well as due to previous autonomous scanning.”

(4) [Insert new text in subclause 6.3.22.1.2, page 429, line 13]

“In the event of a race condition where after the BS sends an unsolicited MOB_SCN-RSP the MS sends a MOB_SCN-REQ before it receives the said BS’s MOB_SCN-RSP, the BS shall respond to such MOB_SCN-REQ with another unsolicited MOB_SCN-RSP message (in the interleaving interval of the previous MOB_SCN-RSP message) that shall overwrite the previous one.

When multiple MOB_SCN-RSP (solicited or unsolicited) messages are received by the MS (over time) and if the MS responds to the message, then the MS shall always respond to the most recent message. “