| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup and rationalizing of the capabilities management in ASN MIBs. |  |
| Date Submitted | 2008-01-16 |  |
| Source(s) | Krzysztof Dudzinski Airspan | Voice: E-mail: kdudzins@airspan.com |
| Re: | P802.16Rev2/D2 (LB26a) |  |
| Abstract | MIBs defined in the 802.16i drafts contain huge number of capabilities objects spread over large number of tables. This contribution proposes a way of rationalizing of the encoding of the important capabilities. It simplifies the model and improves the management protocol performance. |  |
| Purpose | Approve the contribution and apply the changes to subclause 13.2.3 of the standard document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Cleanup and rationalizing of the capabilities management in ASN MIBs.

Krzysztof Dudzinski

Airspan Communication Ltd.

# Overview

The MIBs define multiple sets of capabilities management tables. They are spread over multiple MIB modules (WMAN-IF2-BS-MIB, WMAN-IF2M-BS-MIB) and defined in the different tables based on their content e.g. OFDMA capabilities in OfdmPhy branch of the tree. However the capabilities are not of the SS running OFDMA or MS. The capabilities are to tell us if SS is capable of OFDMA or capable of mobility etc. BS doesn't have to grant a given capability. As such it is better to group all the capabilities in one simple set of 4 tables rather than in 3 or 4 sets of 4 tables each. This makes the MIB much simpler too.

Many of the capabilities are encoded as BITs. Again in the MIB there is a separate set of 4 objects (in 4 tables) for each such capability and additionally the textual convention is defined for each one of them. This is extremely inefficient and leads to heavy overhead in management protocols forced to get all objects independently to retrieve the complete capabilities of the SS. It is much better to group all binary encoded capabilities in a single set of 4 objects, each defining the full set of binary options of capabilities. The gain in protocol performance may lead to 1 pdu to get complete capabilities map for all SSs on the BS instead of multiple tens of pdus. The simplification of the MIB is substantial.

The proposed remedy defines 3 new textual conventions WmanIf2SsCapOptionsGeneral,

WmanIf2SsCapOptionsOfdm,

WmanIf2SsCapOptionsOfdma.

And a set of 12 objects defining complete capabilities of SS.

They all group large number of capabilities and replace about 140 objects and 8 tables!

# Editorial Instructions.

# General capabilities

Tables used for general capabilities management are:

wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapabilitiesConfigTable

## 1.1 Binary encoded objects (BITS)

Implement the content of the following BITS based textual conventions as a part of a single combined textual convention WmanIf2SsCapOptionsGeneral .

Pay attention to notes in brackets.

Delete all the objects using the textual conventions listed.

The list of textual conventions to delete:

- WmanIf2ArqSupportType

- WmanIf2IpVersionType

- WmanIf2MacCsBitMap

- WmanIf2PhsSupportType

- WmanIf2BwAllocSupport

- WmanIf2PduConstruction

- WmanIf2PackingSupport (ref. 11.7.8.11 in 16e ->11.7.8.7 in revision)

- WmanIf2ExtRtpsSupport (ref. 11.7.8.12 in 16e ->11.7.8.8 in revision)

- WmanIf2IpAllocMethod

- WmanIf2ArqAckType

- WmanIf2MacHeaderSupp (bits 0-7, 11-16 moved to options bitmap; bits 8-10 moved to new type WmanIf2MacHeaderSuppSdnSnPeriod , bits 17,18 moved to WmanIf2ExtCapability)

- WmanIf2PkmVersion

- WmanIf2AuthPolicy

- WmanIf2MacMode

- WmanIf2ExtCapability (bits 1,2 added, were in WmanIf2MacHeaderSupp and all bits moved to options)

Create new capability objects defined using newly created combined textual convention WmanIf2SsCapOptionsGeneral.

The list of new capability objects:

wmanIf2BsSsReqCapOptionsGeneral in wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapOptionsGeneral in wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsCapOptionsGeneral in wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapCfgOptionsGeneral in wmanIf2BsCapabilitiesConfigTable

## 1.2 Numerical objects (Integer, Unsigned integer)

Keep in use the following numerical capabilities textual conventions.

Apply the correction changes defined in brackets where appropriate.

- WmanIf2NumOfCid

- WmanIf2MaxDsxFlowType

- WmanIf2MaxMcpGroupCid

- WmanIf2MaxPkmFlowType

- WmanIf2MaxNumOfSaType

- WmanIf2MaxClassifiers

- WmanIf2SsTransitionGap

- xxxCapMaxNumBurstToMs (create textual convention for these objects)

- WmanIf2MaxMacLevel for both uplink and downlink objects

- xxxCapNumOfProvisionedSf (create textual convention for these objects)

- xxxCapPnWindowSize

## 1.3 Obsolete objects

Delete the following textual conventions and/or related objects as they are removed from the standard as a result of the amendments 802.16e, Cor2 and Rev2.

- WmanIf2MacCrcSupport (OK, already deleted in D6)

- WmanIf2AuthPolicyType (OK, already deleted in D6)

# 2. OFDM capabilities

Tables used for OFDM capabilities management are:

wmanIf2BsSsOfdmReqCapabilitiesTable,

wmanIf2BsSsOfdmRspCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesConfigTable

## 2.1 Binary encoded objects (BITS)

Implement the content of the following BITS based textual conventions as a part of a single combined textual convention WmanIf2SsCapOptionsOfdm.

Pay attention to notes in brackets.

Delete all the objects using the textual conventions listed.

The list of textual conventions to delete:

- WmanIf2OfdmFftSizes (OK)

- WmanIf2OfdmSsDeModType (OK)

- WmanIf2OfdmSsModType (OK)

- WmanIf2OfdmTcSublayer (OK)

- WmanIf2OfdmPrivMap (OK)

- WmanIf2OfdmUlPower (OK)

Create new capability objects defined using newly created combined textual convention WmanIf2SsCapOptionsOfdm. The new objects are created in the general capabilities tables to allow subsequent deletion of OFDM capabilities tables.

The list of new capability objects to create:

wmanIf2BsSsReqCapOptionsOfdm in wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapOptionsOfdm in wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsCapOptionsOfdm in wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapCfgOptionsOfdm in wmanIf2BsCapabilitiesConfigTable

## 2.2 Numerical objects (Integer, Unsigned integer)

Keep in use the following numerical capabilities textual conventions and/or objects.

Apply the correction changes defined in brackets where appropriate.

Move the objects listed or objects using listed textual conventions to corresponding general capabilities tables to allow subsequent deletion of OFDM capabilities tables.

- xxxCapLoopPwrControlSw (add and use textual convention WmanIf2CapMinNumFramesForPwrCtrlSwitch)

## 2.3 Obsolete objects

Delete the following textual conventions and/or related objects as they are removed from the standard as a result of the amendments 802.16e, Cor2 and Rev2.

- WmanIf2OfdmFocusedCt

## 2.4 OFDM capabilities tables

Finally delete the empty OFDM capabilities tables:

wmanIf2BsSsOfdmReqCapabilitiesTable,

wmanIf2BsSsOfdmRspCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesConfigTable

# 3. OFDMA capabilities

Tables used for OFDMA capabilities management are:

wmanIf2BsSsOfdmaReqCapabilitiesTable,

wmanIf2BsSsOfdmaRspCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesConfigTable

## 3.1 Binary encoded objects (BITS)

Implement the content of the following BITS based textual conventions as a part of a single combined textual convention WmanIf2SsCapOptionsOfdma .

Pay attention to notes in brackets.

Delete all the objects using the textual conventions listed.

The list of textual conventions to delete:

- WmanIf2OfdmaFftSizes (fft256 removed in Cor2 and bit made reserved)

- WmanIf2OfdmaMsDeModType

- WmanIf2OfdmaMsModType

- WmanIf2OfdmaPermutation

- WmanIf2OfdmaMimoCap (corrected bits as per Rev2)

- WmanIf2OfdmaPrivMap

- WmanIf2OfdmaAasCap

- WmanIf2OfdmaCinrCap

- WmanIf2OfdmaUlPower

- WmanIf2OfdmaMapCap

- WmanIf2OfdmaUlCntlCh

- WmanIf2OfdmaMsCistCap

- WmanIf2OfdmaMaxHarq

- WmanIf2OfdmaModMimo

- WmanIf2MultiBurst

- WmanIf2IncrHarqBuf

- WmanIf2ChaseHarqBuf

Create new capability objects defined using newly created combined textual convention WmanIf2SsCapOptionsOfdma. The new objects are created in the general capabilities tables to allow subsequent deletion of OFDMA capabilities tables.

The list of new capability objects to create:

wmanIf2BsSsReqCapOptionsOfdma in wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapOptionsOfdma in wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsCapOptionsOfdma in wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapCfgOptionsOfdma in wmanIf2BsCapabilitiesConfigTable

## 3.2 Numerical objects (Integer, Unsigned integer)

Keep in use the following numerical capabilities textual conventions and/or objects.

Apply the correction changes defined in brackets where appropriate.

Move the objects listed or objects using listed textual conventions to corresponding general capabilities tables to allow subsequent deletion of OFDMA capabilities tables.

- wmanIf2BsSsOfdmaReqCapNoUlHarqChannel (created textual convention WmanIf2OfdmaNoHarqChannelCapType and renamed from wmanIf2BsSsOfdmaReqCapNoDlHarqChannel)

- wmanIf2BsSsOfdmaReqCapNoDlHarqChannel (new object as per 11.8.3.7.2)

- WmanIf2SdmaPilotCap

## 3.3 Obsolete objects

Delete the following textual conventions and/or related objects as they are removed from the standard as a result of the amendments 802.16e, Cor2 and Rev2 or for other reason as stated in the notes in brackets.

- WmanIf2OfdmaDemMimo (duplicates WmanIf2OfdmaMimoCap)

- WmanIf2OfdmaUlMimo (deleted in Cor2)

## 3.4 OFDMA capabilities tables cleanup

Finally delete the empty OFDMA capabilities tables:

wmanIf2BsSsOfdmaReqCapabilitiesTable,

wmanIf2BsSsOfdmaRspCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesConfigTable

# ASN1 code with complete solution for WMAN-IF2-BS-MIB

# --

# -- Textual conventions

# --

# WmanIf2CidType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Type of CID."

# SYNTAX INTEGER (0..65535)

# WmanIf2MaxTxPowerType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "This type is used to define maximum available power for

# BPSK, QPSK, 16-QAM and 64-QAM constellations. The maximum

# power parameters are reported in dBm and quantized in 0.5

# dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

# (encoded 0xFF). Values outside this range shall be

# assigned the closest extreme. SSs that do not support

# QAM64 shall report the value of 0x00 in the maximum QAM64

# power field."

# REFERENCE

# "Subclause 11.8.3.2 in IEEE Std 802.16-2004"

# SYNTAX INTEGER (0..255)

# WmanIf2MacVersion ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Version number of IEEE 802.16."

# SYNTAX INTEGER

# {

# ieee802Dot16Of2001(1),

# ieee802Dot16cOf2002(2),

# ieee802Dot16aOf2003(3),

# ieee802Dot16Of2004(4),

# ieee802Dot16e(5),

# tbd(6)

# }

# WmanIf2NumOfCid ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The object of this type shows the number of CIDs that

# SS can support."

# REFERENCE

# "Subclause 11.7.6 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER (1..65535)

# WmanIf2MaxDsxFlowType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The object of this type specifies the maximum number of

# concurrent DSA, DSC, or DSD transactions that may be

# outstanding."

# REFERENCE

# "Subclause 11.7.8.2 in IEEE Std 802.16-2004"

# SYNTAX INTEGER (0..255)

# WmanIf2MaxMcaFlowType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The object of this type specifies the maximum number of

# concurrent MCA transactions that may be outstanding."

# REFERENCE

# "Subclause 11.7.8.4 in IEEE Std 802.16-2004"

# SYNTAX INTEGER (0..255)

# WmanIf2MaxMcpGroupCid ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The object of this type indicates the maximum number of

# simultaneous Multicast Polling Groups the SS is

# capable of belonging to."

# REFERENCE

# "Subclause 11.7.8.5 in IEEE Std 802.16-2004"

# SYNTAX INTEGER (0..255)

# WmanIf2MaxPkmFlowType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The object of this type specifies the maximum number of

# concurrent PKM transactions that may be outstanding."

# REFERENCE

# "Subclause 11.8.4.5 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER (0..255)

# WmanIf2MaxNumOfSaType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "This field specifies maximum number of supported

# security association of the SS."

# REFERENCE

# "Subclause 11.8.4.6 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER (0..255)

# WmanIf2MaxClassifiers ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The object of this type indicates the maximum number of

# admitted Classifiers that the SS is allowed to have."

# REFERENCE

# "Subclause 11.7.7.2 in IEEE Std 802.16-2004"

# SYNTAX INTEGER (0..65535)

# WmanIf2SsTransitionGap ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "This field indicates the transition speed SSTTG and SSRTG

# for TDD and H-FDD SSs. Allowed values are:

# OFDM mode: TDD and H-FDD 0..100

# Other modes: TDD: 0..50; H-FDD: 0..100"

# REFERENCE

# "Subclause 11.8.3.1 in IEEE Std 802.16-2004"

# SYNTAX INTEGER (0..100)

# WmanIf2MaxMacLevel ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "maximum amount of MAC level data including MAC headers and

# HARQ retransmission bursts the MS is capable of processing

# in the DL/UL part of a single MAC frame."

# REFERENCE

# "Subclause 11.7.8.10 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER (0..65535)

# WmanIf2ChannelNumber ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Physical channel number"

# SYNTAX INTEGER (0..199)

# WmanIf2BsIdType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Defines the encoding of BSID. The BSID is a 6 byte number

# and follows the encoding rules of MacAddress textual

# convention, i.e. as if it were transmitted

# least-significant bit first. The value should be displayed

# with 2 parts clearly separated by a colon e.g:

# 001DFF:00003A. The most significant part is representing

# the Operator ID. "

# SYNTAX OCTET STRING (SIZE (6))

# WmanIf2OfdmFftSize ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "FFT size for OFDM PHY

# 0b000: 2048

# 0b011: 256"

# REFERENCE

# "Subclause 11.8.3.6.1, IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# fft2048(0),

# fft256(3)

# }

# WmanIf2OfdmFecCodeType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "FEC code type and modulation type"

# REFERENCE

# "Table 356 and Table 362 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# bpskCc1Over2(0),

# qpskRsCcCc1Over2(1),

# qpskRsCcCc3Over4(2),

# sixteenQamRsCcCc1Over2(3),

# sixteenQamRsCcCc3Over4(4),

# sixtyFourQamRsCcCc2Over3(5),

# sixtyFourQamRsCcCc3Over4(6),

# qpskBtc1Over2(7),

# qpskBtc3Over4(8),

# sixteenQamBtc3Over4(9),

# sixteenQamBtc4Over5(10),

# sixtyFourQamBtc2Over3(11),

# sixtyFourQamBtc5Over6(12),

# qpskCtc1Over2(13),

# qpskCtc2Over3(14),

# qpskCtc3Over4(15),

# sixteenQamCtc1Over2(16),

# sixteenQamCtc3Over4(17),

# sixtyFourQamCtc2Over3(18),

# sixtyFourQamCtc3Over4(19)

# }

# WmanIf2HarqAckDelay ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "HARQ ACK delay for UL and DL bursts

# 1 = one frame offset

# 2 = two frames offset

# 3 = three frames offset"

# REFERENCE

# "Table 353 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# oneframeoffset(1),

# twoframesoffset(2),

# threeframesoffset(3)

# }

# WmanIf2AasBeamSel ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Boolean to indicate whether unsolicited AAS Beam Select

# messages (see 6.3.2.3.41 in IEEE 802.16e-2005) should be

# sent by the MS.

# 0: MS should not send AAS Beam Select Messages

# 1: MS may send AAS Beam Select Messages"

# REFERENCE

# "Table 353 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# notAllowed(0),

# allowed(1)

# }

# WmanIf2OfdmaFftSize ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "FFT size for OFDMA PHY

# 0b000 = 2048

# 0b001 = 1024

# 0b010 = 512

# 0b100 = 128"

# REFERENCE

# "Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# fft2048(0),

# fft1024(1),

# fft512(2),

# reserved(3),

# fft128(4)

# }

# WmanIfPermutationType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Permutation type for broadcast region in HARQ zone"

# REFERENCE

# "Table 353 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# pusc(1),

# fusc(2),

# optionalFusc(3),

# amc(4)

# }

# WmanIf2HoSupportType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The types of handover supported."

# REFERENCE

# "Table 358 in IEEE Std 802.16e-2005"

# SYNTAX BITS

# {

# handover(0),

# mdHandover(1),

# fbssHandover(2)

# }

# WmanIf2OfdmaUcdFecCode ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "UCD FEC code type and modulation type"

# REFERENCE

# "Table 357 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# qpskCc1Over2(0),

# qpskCc3Over4(1),

# sixteenQamCc1Over2(2),

# sixteenQamCc3Over4(3),

# sixtyFourQamCc1Over2(4),

# sixtyFourQamCc2Over3(5),

# sixtyFourQamCc3Over4(6),

# qpskBtc1Over2(7),

# qpskBtc3Over4(8),

# sixteenQamBtc3Over5(9),

# sixteenQamBtc4Over5(10),

# sixtyFourQamBtc5Over8(11),

# sixtyFourQamBtc4Over5(12),

# qpskCtc1Over2(13),

# reserved(14),

# qpskCtc3Over4(15),

# sixteenQamCtc1Over2(16),

# sixteenQamCtc3Over4(17),

# sixtyFourQamCtc1Over2(18),

# sixtyFourQamCtc2Over3(19),

# sixtyFourQamCtc3Over4(20),

# sixtyFourQamCtc5Over6(21),

# qpskZtCc1Over2(22),

# qpskZtCc3Over4(23),

# sixteenQamZtCc1Over2(24),

# sixteenQamZtCc3Over4(25),

# sixtyFourQamZtCc1Over2(26),

# sixtyFourQamZtCc2Over3(27),

# sixtyFourQamZtCc3Over4(28),

# qpskLdpc1over2(29),

# qpskLdpc2over3A(30),

# qpskLdpc3over4A(31),

# sixteenQamLdpc1over2(32),

# sixteenQamLdpc2over3A(33),

# sixteenQamLdpc3over4A(34),

# sixtyFourQamLdpc1over2(35),

# sixtyFourQamLdpc2over3A(36),

# sixtyFourQamLdpc3over4A(37),

# qpskLdpc2over3B(38),

# qpskLdpc3over4B(39),

# sixteenQamLdpc2over3B(40),

# sixteenQamLdpc3over4B(41),

# sixtyFourQamLdpc2over3B(42),

# sixtyFourQamLdpc3over4B(43),

# qpskCcOptIntv1over2(44),

# qpskCcOptIntv3over4(45),

# sixteenQamCcOptIntv1over2(46),

# sixteenQamCcOptIntv3over4(47),

# sixtyFourQamCcOptIntv2over3(48),

# sixtyFourQamCcOptIntv3over4(49),

# qpskLdpc5over6(50),

# sixteenQamLdpc5over6(51),

# sixtyFourQamLdpc5over6(52)

# }

# WmanIf2OfdmaDcdFecCode ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "DCD FEC code type and modulation type"

# REFERENCE

# "Table 363 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# qpskCc1Over2(0),

# qpskCc3Over4(1),

# sixteenQamCc1Over2(2),

# sixteenQamCc3Over4(3),

# sixtyFourQamCc1Over2(4),

# sixtyFourQamCc2Over3(5),

# sixtyFourQamCc3Over4(6),

# qpskBtc1Over2(7),

# qpskBtc3Over4Or2Over3(8),

# sixteenQamBtc3Over5(9),

# sixteenQamBtc4Over5(10),

# sixtyFourQamBtc2Over3Or5Over8(11),

# sixtyFourQamBtc5Over6Or4Over5(12),

# qpskCtc1Over2(13),

# reserved14(14),

# qpskCtc3Over4(15),

# sixteenQamCtc1Over2(16),

# sixteenQamCtc3Over4(17),

# sixtyFourQamCtc1Over2(18),

# sixtyFourQamCtc2Over3(19),

# sixtyFourQamCtc3Over4(20),

# sixtyFourQamCtc5Over6(21),

# qpskZtCc1Over2(22),

# qpskZtCc3Over4(23),

# sixteenQamZtCc1Over2(24),

# sixteenQamZtCc3Over4(25),

# sixtyFourQamZtCc1Over2(26),

# sixtyFourQamZtCc2Over3(27),

# sixtyFourQamZtCc3Over4(28),

# qpskLdpc1over2(29),

# qpskLdpc2over3A(30),

# qpskLdpc3over4A(31),

# sixteenQamLdpc1over2(32),

# sixteenQamLdpc2over3A(33),

# sixteenQamLdpc3over4A(34),

# sixtyFourQamLdpc1over2(35),

# sixtyFourQamLdpc2over3A(36),

# sixtyFourQamLdpc3over4A(37),

# qpskLdpc2over3B(38),

# qpskLdpc3over4B(39),

# sixteenQamLdpc2over3B(40),

# sixteenQamLdpc3over4B(41),

# sixtyFourQamLdpc2over3B(42),

# sixtyFourQamLdpc3over4B(43),

# qpskCcOptIntv1over2(44),

# qpskCcOptIntv3over4(45),

# sixteenQamCcOptIntv1over2(46),

# sixteenQamCcOptIntv3over4(47),

# sixtyFourQamCcOptIntv2over3(48),

# sixtyFourQamCcOptIntv3over4(49)

# }

# WmanIf2SdmaPilotCap ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "This field indicates SDMA pilot pattern support for AMC

# zone."

# REFERENCE

# "Subclause 11.8.3.7.17 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# noSupport(0),

# sdmaPilotAandB(1),

# allSdmaPilotPatterns(2)

# }

# WmanIf2GlobalSrvClass ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Global Service Class Name contains 8 information fields

# that map to predefined QoS attributes as shown in

# subclause 6.3.14.4.1.

# bit#0:     Uplink/Downlink indicator

# 0 - uplink

# 1 - downlink

# bit#1-6:   Maximum sustained traffic rate in bps that is

# defined in Table 124b

# bit#7:     0 - no traffic indication

# 1 - traffic indication

# bit#8-13:  Maximum traffic burst defines the maximum burst

# size that must be accommodated for the service.

# bit#14-19: Minimum reserved traffic rate parameter

# specifies the minimum rate, in bits per second,

# reserved for this service flow.

# bit#20-25: Maximum latency specifies the maximum interval

# between the reception of a packet at CS of BS

# or SS and the arrival of the packet to the peer

# device.

# bit#26:    SDU indicator specifies whether the SDUs on the

# service flow are fixed-length or variable-length.

# 0 - variable length

# 1 - fixed length

# bit#27:    Paging indicator of an MS preference for the

# reception of paging advisory messages during

# idle mode. When set, it indicates that the BS

# may present paging advisory messages or other

# indicative messages to the MS when data SDUs

# bound for the MS are present while the MS is in

# Idle Mode.

# 0 - no paging generation

# 1 - paging generation"

# REFERENCE

# "Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

# SYNTAX BITS

# {

# ulDlIndicator(0),

# maxSustainedRate0(1),

# maxSustainedRate1(2),

# maxSustainedRate2(3),

# maxSustainedRate3(4),

# maxSustainedRate4(5),

# maxSustainedRate5(6),

# trafficIndication(7),

# maxTrafficBurst0(8),

# maxTrafficBurst1(9),

# maxTrafficBurst2(10),

# maxTrafficBurst3(11),

# maxTrafficBurst4(12),

# maxTrafficBurst5(13),

# minReservedRate0(14),

# minReservedRate1(15),

# minReservedRate2(16),

# minReservedRate3(17),

# minReservedRate4(18),

# minReservedRate5(19),

# maxLatency0(20),

# maxLatency1(21),

# maxLatency2(22),

# maxLatency3(23),

# maxLatency4(24),

# maxLatency5(25),

# sduIndicator(26),

# pagingGeneration(27),

# reserved0(28),

# reserved1(29),

# reserved2(30),

# reserved3(31)

# }

# WmanIf2LinkDirection ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The direction of a connection."

# SYNTAX INTEGER

# {

# downstream(1),

# upstream(2)

# }

# WmanIf2DataEncryptAlgId ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Data encryption algorithm identifiers."

# REFERENCE

# "Table 375 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# none(0),

# des56BitCbcMode(1),

# aes128BitCcmMode(2),

# aes128BitCbcMode(3),

# aes128BitCtrMode(128)

# }

# WmanIf2DataAuthAlgId ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Data authentication algorithm identifiers."

# REFERENCE

# "Table 376 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# noDataAuthentication(0),

# aes128BitCcmMode(1)

# }

# WmanIf2TekEncryptAlgId ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "TEK encryption algorithm identifiers."

# REFERENCE

# "Table 377 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# tripleDes128BitKey(1),

# rsa1024BitKey(2),

# aes128BitKeyEcbMode(3),

# aes128BitKeyWrap(4)

# }

# WmanIf2PkmErrorCode ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "This error code provides further information about an

# Authorization Reject, Key Reject, Authorization Invalid,

# or TEK Invalid.

# 0 - no failure

# 1 - unauthorized SS

# 2 - unauthorized SAID

# 3 - unsolicited

# 4 - invalid key sequence

# 5 - key request authentication failure

# The following are error code for permanent authorization

# failure that indicates any reattempts at authorization

# would continue to result in Authorization Rejects.

# 6 - the BS does not have the CA certificate belonging

# to the issuer of an SS certificate

# 7 - SS certificate has an invalid signature

# 8 - ASN.1 parsing failure during verification of SS

# certificate

# 9 - SS certificate is on the 'hot list'

# 10 - inconsistencies between certificate data and data

# in accompanying PKM attributes

# 11 - SS and BS have incompatible security capabilities"

# REFERENCE

# "Subclause 11.9.10 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# noFailure(0),

# unauthorizedSs(1),

# unauthorizedSaid(2),

# unsolicited(3),

# invalidKeySequence(4),

# keyReqAuthFailure(5),

# umknownManufactur(6),

# invalidSignature(7),

# asn1ParsingFailure(8),

# ssCaOnHotList(9),

# dataInconsistency(10),

# ssBsIncompatibleSc(11)

# }

# WmanIf2CertificateStat ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The reason why a SS's certificate is deemed valid

# or invalid:

# 0 - return unknown if the SS is running PKM mode

# 1 - means the certificate is valid because it chains

# to a valid certificate

# 2 - means the certificate is valid because it has been

# provisioned to be trusted

# 3 - means the certificate is invalid because it has been

# provisioned to be untrusted.

# 4 - means the certificate is invalid because it chains

# to an untrusted certificate.

# 5 - refer to errors in parsing, validity periods, etc,

# of SS certificate

# 6 - refer to errors in parsing, validity periods, etc,

# of CA certificate"

# REFERENCE

# "Subclause 7.2.5.1 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# unknown(0),

# validSsChained(1),

# validSsTrusted(2),

# invalidSsUntrusted(3),

# invalidCAUntrusted(4),

# invalidSsOther(5),

# invalidCAOther(6)

# }

# WmanIf2SaType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The type of Security Association (SA)."

# REFERENCE

# "Table 379 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# primarySa(0),

# staticSa(1),

# dynamicSa(2)

# }

# WmanIf2SaServiceType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "This attribute indicates service types of the corresponding

# SA type. This attribute shall be defined only when the SA

# type is Static SA or Dynamic SA. The GTEK shall be used to

# encrypt connection for group multicast service. The MTK

# shall be used to encrypt connection for MBS service."

# REFERENCE

# "Subclause 11.9.35 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# unicastService(0),

# groupMulticastService(1),

# mbsService(2)

# }

# WmanIf2SfSchedulingType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The scheduling service provided by a SC for an upstream

# service flow. If the parameter is omitted from an upstream

# QOS Parameter Set, this object takes the value of

# bestEffort (2). This parameter must be reported as

# undefined (1) for downstream QOS Parameter Sets."

# SYNTAX INTEGER

# {

# undefined(1),

# bestEffort(2),

# nonRealTimePollingService(3),

# realTimePollingService(4),

# extRealTimePollingService(5),

# unsolicitedGrantService(6)

# }

# WmanIf2PhsRuleVerify ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The value of this field indicates to the sending entity

# whether or not the packet header contents are to be

# verified prior to performing suppression. If PHSV is

# enabled, the sender shall compare the bytes in the packet

# header with the bytes in the PHSF that are to be

# suppressed as indicated by the PHSM."

# REFERENCE

# "Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# phsVerifyEnable(0),

# phsVerifyDisable(1)

# }

# WmanIf2ClassifierBitMap ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "A bit of of this object is set to 1 if the parameter

# indicated by the comment was present in the classifier

# encoding, and 0 otherwise.

# Note: that BITS are encoded most significant bit first,

# so that if e.g. bits 6 and 7 are set, this object is

# encoded as the octet string '030000'H."

# REFERENCE

# "Subclause 11.13.19.3.4 in IEEE Std 802.16e-2005"

# SYNTAX BITS

# {

# priority(0),

# ipTos(1),

# ipProtocol(2),

# ipMaskedSrcAddr(3),

# ipMaskedDestAddr(4),

# srcPort(5),

# destPort(6),

# destMacAddr(7),

# srcMacAddr(8),

# ethernetProtocol(9),

# userPriority(10),

# vlanId(11),

# ipv6FlowLabel(12),

# actionRule(13)

# }

# WmanIf2SfState ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Defines the state of a service flow."

# SYNTAX INTEGER

# {

# authorized(1),

# admitted(2),

# active(3)

# }

# WmanIf2ServClassName ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Defines the type of service class name."

# SYNTAX OCTET STRING (SIZE (2..128))

# WmanIf2CsSpecification ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Defines the types of convergence sublayer."

# REFERENCE

# "Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER

# {

# reserved(0),

# packetIpV4(1),

# packetIpV6(2),

# packet802dot3Ethernet(3),

# packet802dot1QVlan(4),

# packetIpV4Over802dot3(5),

# packetIpV6Over802dot3(6),

# packetIpV4Over802dot1Q(7),

# packetIpV6Over802dot1Q(8),

# atm(9),

# packet802dot3EthernetRohcHc(10),

# packet802dot3EthernetEcrtpHc(11),

# packetIp2RohcHc(12),

# packetIp2EcrtpHc(13)

# }

# WmanIf2TekState ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "TEK State."

# REFERENCE

# "Subclause 7.2.5.1 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# start(1),

# opWait(2),

# opReauthWait(3),

# operational(4),

# rekeyWait(5),

# rekeyReauthWait(6)

# }

# WmanIf2OfdmaFecCodeType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "FEC code type and modulation type"

# REFERENCE

# "Table 356 and Table 362 in IEEE Std 802.16-2004"

# SYNTAX INTEGER

# {

# qpskCc1Over2(0),

# qpskCc3Over4(1),

# sixteenQamCc1Over2(2),

# sixteenQamCc3Over4(3),

# sixtyFourQamCc2Over3(4),

# sixtyFourQamCc3Over4(5),

# qpskBtc1Over2(6),

# qpskBtc2Over3(7),

# sixteenQamBtc3Over5(8),

# sixteenQamBtc4Over5(9),

# sixtyFourQamBtc5Over8(10),

# sixtyFourQamBtc4Over5(11),

# qpskCtc1Over2(12),

# qpskCtc2Over3(13),

# qpskCtc3Over4(14),

# sixteenQamCtc1Over2(15),

# sixteenQamCtc3Over4(16),

# sixtyFourQamCtc2Over3(17),

# sixtyFourQamCtc3Over4(18),

# sixtyFourQamCtc5Over6(19),

# qpskZtCc1Over2(20),

# qpskZtCc3Over4(21),

# sixteenQamZtCc1Over2(22),

# sixteenQamZtCc3Over4(23),

# sixtyFourQamZtCc2Over3(24),

# sixtyFourQamZtCc3Over4(25)

# }

# WmanIf2Ipv6FlowLabel ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The value of this field specifies the matching values for

# the IPv6 Flow label field. As the flow label field has a

# length of 20 bits, the first 4 bits of the most significant

# byte shall be set to 0x0 and disregarded."

# SYNTAX OCTET STRING (SIZE (3))

# WmanIf2ActionRule ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "The value of this field specifies an action associated with

# the classifier rule

# Bit #0: 0 = none.

# 1 = Discard packet"

# REFERENCE

# "Subclause 11.13.19.3.4.17 in IEEE Std 802.16e-2005"

# SYNTAX BITS { discardPacket(0) }

# WmanIf2BsSsCapType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "xxxReq: Indicates that SS reports its capabilities in

# RNG-REQ, SBC-REQ, or REG-REQ messages.

# xxxRsp: Indicates that BS confirms SS capabilities in

# RNG-RSP, SBC-RSP, or REG-RSP messages."

# SYNTAX INTEGER

# {

# xxxReq(1),

# xxxRsp(2)

# }

# WmanIf2BsCapType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "inheritedBsCap: Indicates BS' basic HW and SW capabilities.

# configedBsCap: Indicates BS' capabilities that has been

# configured."

# SYNTAX INTEGER

# {

# inheritedBsCap(1),

# configedBsCap(2)

# }

# WmanIf2CapMaxNumBurstToMsType ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Maximum number of bursts transmitted concurrently to the MS,

# including all bursts without CID or with CIDs matching the MS's CIDs."

# REFERENCE

# "Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

# SYNTAX Integer32 (1..16)

# WmanIf2CapMaxNumOfProvisionedSf ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "When a BS is to transmit multiple DSA transactions for

# provisioned service flows, this object indicates how many

# DSA transactions with provisioned service flows will be

# transmitted."

# REFERENCE

# "Subclause 11.7.19 in IEEE Std 802.16e-2005"

# SYNTAX Integer32 (0..255)

# WmanIf2MacHeaderSuppSdnSnPeriod ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Indicates SDU_SN extended subheader transmission period (p)

# for connection with ARQ disabled = once every 2^p MAC PDUs.

# Correspondsto bits 8-10 of the MAC header and extend subheader support."

# REFERENCE

# "Subclause 11.7.25 in IEEE Std 802.16e-2005"

# SYNTAX INTEGER (0..7)

# WmanIf2CapMinNumFramesForPwrCtrlSwitch ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Defines the minimum number of frames that SS takes to switch

# from the open loop power control scheme to the closed loop

# power control scheme or vice versa."

# REFERENCE

# "Subclause 11.8.3.6.7 in IEEE Std 802.16-2008"

# SYNTAX Integer32 (0..255)

# WmanIf2CapOfdmaNoHarqChannel ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Specifies the number of H-ARQchannels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# REFERENCE

# "Subclause 11.8.3.7.2 (demodulator) and 11.8.3.7.3 (modulator) in IEEE 802.16-2008."

# SYNTAX Integer32 (0..15)

# WmanIf2SsCapOptionsGeneral ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Description."

# REFERENCE

# "IEEE Std 802.16-2008 subclauses:

# bit 0:       11.7.8.1 (ARQ support)

# bit 1-2:     11.7.4 (IP version)

# bit 3-16:    11.7.7.1 (CS support)

# bit 17-18:   11.7.7.2 (PHS support)

# bit 19:      11.8.1 (BW allocation - duplex support)

# bit 20-21:   11.8.2 (PDU construction and transmission)

# bit 22:      11.7.8.11 (Packing)

# bit 23:      11.7.8.12 (Extended rtPS)

# bit 24-27:   11.7.11 (SMC IP address allocation method)

# bit 28-31:   11.7.23 (ARQ ACK type)

# bit 32-45:   11.7.25 (Mac header support)

# bit 46-47:   11.8.4.1 (Pkm version)

# bit 48-53:   11.8.4.2 (Authorization policy support)

# bit 54-58:   11.8.4.3 (Message Authentication Code mode)

# bit 59-61:   11.8.6 (Extended subheader support)

# "

# SYNTAX BITS

# {

# arqSupport(0),

# ipv4Support(1),

# ipv6Support(2),

# csSupportAtm(3),

# csSupportIpv4(4),

# csSupportIpv6(5),

# csSupport802Dot3(6),

# csSupport802Dot1Q(7),

# csSupportIpv4Over802Dot3(8),

# csSupportIpv6Over802Dot3(9),

# csSupportIpv4Over802Dot1Q(10),

# csSupportIpv6Over802Dot1Q(11),

# csSupport802Dot3RohcHc(12),

# csSupport802Dot3EcrtpHc(13),

# csSupportIpv4Over6RohcHc(14),

# csSupportIpv4Over6EcrtpHc(15),

# csSupportGpcs(16),

# phsSupportAtm(17),

# phsSupportPacket(18),

# bwAllocSupportFullDuplex(19),

# pduConstructionpiggybackedRequests(20),

# pduConstructionFsn3Bits(21),

# packingSupported(22),

# extendedRtpsSupported(23),

# smcIpAddressAllocDhcp(24),

# smcIpAddressAllocMobileIpv4(25),

# smcIpAddressAllocMobileDhcpv6(26),

# smcIpAddressAllocMobileIpv6Autoconfig(27),

# arqSelectiveAck(28),

# arqCumulativeAck(29),

# arqCumWithSelAckEntry(30),

# arqCumWithBlockSeqAck(31),

# headerSupportBwReqUlTxPowerReport(32),

# headerSupportBwReqCinrReport(33),

# headerSupportCqichAllocationReq(34),

# headerSupportPhyChannelReport(35),

# headerSupportBwReqUlSleepCntl(36),

# headerSupportSnReport(37),

# headerSupportFeedbackReport(38),

# headerSupportSduSn(39),

# headerSupportDlSleepControl(40),

# headerSupportFeedbackRequest(41),

# headerSupportMimcModeFeedback(42),

# headerSupportUlTxPowerReport(43),

# headerSupportMiniFeedback(44),

# headerSupportSnRequest(45),

# pkmVersionSupport1(46),

# pkmVersionSupport2(47),

# authPolicySupportRsaInitialEntry(48),

# authPolicySupportEapInitialEntry(49),

# authPolicySupportAuthEapInitialEntry(50),

# authPolicySupportRsaReentry(51),

# authPolicySupportEapReentry(52),

# authPolicySupportAuthEapReentry(53),

# macModeHmac(54),

# macModeCmac(55),

# macModeHmac64(56),

# macModeHmac80(57),

# macModeHmac96(58),

# extSubheader(59),

# headerSupportShortPduSn(60),

# headerSupportLongPduSn(61)

# }

# WmanIf2SsCapOptionsOfdm ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Description."

# REFERENCE

# "IEEE Std 802.16-2008 subclauses:

# bits 0-1:     11.8.3.6.1 (OFDM FFT sizes)

# 2-7:     reserved

# bits 8-13:    11.8.3.6.2 (OFDM SS demodulator)

# 14-15:   reserved

# bits 16-21:   11.8.3.6.3 (OFDM SS modulator)

# 22-23:   reserved

# bits 24:      11.8.3.6.5 (OFDM TC sublayer)

# 25-31:   reserved

# bits 32-33:   11.8.3.6.6 (OFDM private map)

# 34-39:   reserved

# bits 40-41:   11.8.3.6.7 (OFDM SS uplink power control)

# 42-47:   reserved"

# SYNTAX BITS

# {

# ofdmFftSize256(0),

# ofdmFftSize2048(1),

# reserved2(2),

# reserved3(3),

# reserved4(4),

# reserved5(5),

# reserved6(6),

# reserved7(7),

# ofdmSsDemodulatorQam64(8),

# ofdmSsDemodulatorBtc(9),

# ofdmSsDemodulatorCtc(10),

# ofdmSsDemodulatorStc(11),

# ofdmSsDemodulatorAas(12),

# ofdmSsDemodulatorSuchan(13),

# reserved14(14),

# reserved15(15),

# ofdmSsModulatorQam64(16),

# ofdmSsModulatorBtc(17),

# ofdmSsModulatorCtc(18),

# ofdmSsModulatorSubchan(19),

# ofdmSsModulatorFocusedCtBwReq(20),

# ofdmSsModulatorUlCyclicDelay(21),

# reserved22(22),

# reserved23(23),

# ofdmTcSublayerSupport(24),

# reserved25(25),

# reserved26(26),

# reserved27(27),

# reserved28(28),

# reserved29(29),

# reserved30(30),

# reserved31(31),

# ofdmRegularMap(32),

# ofdmCompressedMap(33),

# reserved34(34),

# reserved35(35),

# reserved36(36),

# reserved37(37),

# reserved38(38),

# reserved39(39),

# ofdmUlOpenLoopPwrCtrl(40),

# ofdmUlAasPreamblePwrCtrl(41),

# reserved42(42),

# reserved43(43),

# reserved44(44),

# reserved45(45),

# reserved46(46),

# reserved47(47),

# reserved48(48)

# }

# WmanIf2SsCapOptionsOfdma ::= TEXTUAL-CONVENTION

# STATUS current

# DESCRIPTION

# "Description."

# REFERENCE

# "IEEE Std 802.16-2008 subclauses:

# bits 0:          reserved

# 1-4:        11.8.3.7.1 (OFDMA FFT sizes)

# 5-7:        reserved

# bits 8-14,16-18: 11.8.3.7.2 (OFDMA SS demodulator)

# 15,19-23:   reserved

# bits 24-31:      11.8.3.7.3 (OFDMA SS modulator)

# bits 32-39:      11.8.3.7.4 (OFDMA permutation)

# bits 40-61:      11.8.3.7.5 (OFDMA SS demodulator for MIMO)

# 62-63:      reserved

# bits 64-71:      11.8.3.7.7 (OFDMA private map)

# bits 72-76:      11.8.3.7.8 (OFDMA AAS)

# 77-79:      reserved

# bits 80-87:      11.8.3.7.9 (OFDMA CINR measurements)

# bits 88-89:      11.8.3.7.11 (OFDMA power control)

# 90-95:      reserved

# bits 96-100:     11.8.3.7.12 (OFDMA map)

# 101-103:    reserved

# bits 104-106,108-111: 11.8.3.7.13 (OFDM uplink control)

# 107:        reserved

# bits 112-123:    11.8.3.7.14 (OFDMA CSIT capability)

# 124-127:    reserved

# bits 128-135:    11.8.3.7.15 (OFDMA max bursts in HARQ)

# bits 136-144:    11.8.3.7.16 (OFDMA modulator for MIMO)

# 145-151:    reserved

# bits 152-153:    11.8.3.7.18 (OFDMA multicast bursts)

# 154-159:    reserved

# bits 160-172:    11.8.3.7.19.1 (OFDMA HARQ incremental buffer)

# 173-175:    reserved

# bits 176-190:    11.8.3.7.19.1 (OFDMA HARQ chase)

# 191:        reserved

# bits 192-198:    11.8.3.7.20 (OFDMA parameter set)

# 199:        reserved"

# SYNTAX BITS

# {

# reserved0(0),

# ofdmaFftSize2048(1),

# ofdmaFftSize128(2),

# ofdmaFftSize512(3),

# ofdmaFftSize1024(4),

# reserved5(5),

# reserved6(6),

# reserved7(7),

# ofdmaDemodulatorQam64(8),

# ofdmaDemodulatorBtc(9),

# ofdmaDemodulatorCtc(10),

# ofdmaDemodulatorStc(11),

# ofdmaDemodulatorCcWithInterleaver(12),

# ofdmaDemodulatorHarqChase(13),

# ofdmaDemodulatorHarqCtclr(14),

# reserved15(15),

# ofdmaDemodulatorHarqCclr(16),

# ofdmaDemodulatorLdpc(17),

# ofdmaDemodulatorDedicatedPilots(18),

# reserved19(19),

# reserved20(20),

# reserved21(21),

# reserved22(22),

# reserved23(23),

# ofdmaModulatorQam64(24),

# ofdmaModulatorBtc(25),

# ofdmaModulatorCtc(26),

# ofdmaModulatorStc(27),

# ofdmaModulatorHarqChase(28),

# ofdmaModulatorHarqCtcIr(29),

# ofdmaModulatorHarqCcIr(30),

# ofdmaModulatorHarqLdpc(31),

# odfmaPermutationOptionalPusc(32),

# odfmaPermutationOptionalFusc(33),

# odfmaPermutationAmc1x6(34),

# odfmaPermutationAmc2x3(35),

# odfmaPermutationAmc3x2(36),

# odfmaPermutationAmcWithHarqMap(37),

# odfmaPermutationTusc1(38),

# odfmaPermutationTusc2(39),

# ofdmaDemodMimo2AntStcMatrixA(40),

# ofdmaDemodMimo2AntStcMatrixBVCoding(41),

# ofdmaDemodMimo2AntStcMatrixBHCoding(42),

# ofdmaDemodMimo4AntStcMatrixA(43),

# ofdmaDemodMimo4AntStcMatrixBVCoding(44),

# ofdmaDemodMimo4AntStcMatrixBHCoding(45),

# ofdmaDemodMimo4AntStcMatrixCVCoding(46),

# ofdmaDemodMimo4AntStcMatrixCHCoding(47),

# ofdmaDemodMimo3AntStcMatrixA(48),

# ofdmaDemodMimo3AntStcMatrixB(49),

# ofdmaDemodMimo3AntStcMatrixCVCoding(50),

# ofdmaDemodMimo3AntStcMatrixCHCoding(51),

# ofdmaDemodMimoCalcPrecodeWight(52),

# ofdmaDemodMimoAdaptiveRateCtrl(53),

# ofdmaDemodMimoCalcChanMatrix(54),

# ofdmaDemodMimoAntGroup(55),

# ofdmaDemodMimoAntSelect(56),

# ofdmaDemodMimoCodebookPrecode(57),

# ofdmaDemodMimoLongTermPrecode(58),

# ofdmaDemodMimoMidamble(59),

# ofdmaDemodAllocGranDlPuscStc(60),

# ofdmaDemodConcurentAllocDlPuscStc(61),

# reserved62(62),

# reserved63(63),

# ofdmaPrivateMapHarqMap(64),

# ofdmaPrivateMap(65),

# ofdmaPrivateMapReduced(66),

# ofdmaPrivateMapChainEnable(67),

# ofdmaPrivateMapDlFrameOffset(68),

# ofdmaPrivateMapUlFrameOffset(69),

# ofdmaPrivateMapChainConcurency0(70),

# ofdmaPrivateMapChainConcurency1(71),

# ofdmaAasZone(72),

# ofdmaAasDiverityMapScan(73),

# ofdmaAasFeedbackRsp(74),

# ofdmaAasDlPreamble(75),

# ofdmaAasUlPreamble(76),

# reserved77(77),

# reserved78(78),

# reserved79(79),

# ofdmaCinrPhysicalPreamble(80),

# ofdmaCinrPhysicalPilotSubc(81),

# ofdmaCinrPhysicalDataSubc(82),

# ofdmaCinrEffectivePreamble(83),

# ofdmaCinrEffectivePilotSubc(84),

# ofdmaCinrEffectiveDataSubc(85),

# ofdmaCinr2CqiChannel(86),

# ofdmaFreqSelectivityReport(87),

# ofdmaPwrCtrlOpenLoop(88),

# ofdmaPwrCtrlAasPreamble(89),

# reserved90(90),

# reserved91(91),

# reserved92(92),

# reserved93(93),

# reserved94(94),

# reserved95(95),

# ofdmaMapHarq(96),

# ofdmaMapExtendedHarqIe(97),

# ofdmaMapSubMapForFirstZone(98),

# ofdmaMapSubMapForOtherZones(99),

# ofdmaMapDlRegionDefinition(100),

# reserved101(101),

# reserved102(102),

# reserved103(103),

# ofdmaUlCtrlt3BitMimoFastFeedback(104),

# ofdmaUlCtrltEnhancedFastFeedback(105),

# ofdmaUlCtrltUlAck(106),

# reserved107(107),

# ofdmaUlCtrltUepFastFeedback(108),

# ofdmaUlCtrltFastDlMeasurementFeedback(109),

# ofdmaUlCtrltPriSecFastFeedback(110),

# ofdmaDiucCqiFastFeedback(111),

# ofdmaCsitTypeA(112),

# ofdmaCsitTypeB(113),

# ofdmaPowerAssignment(114),

# ofdmaSoundingRspTime0(115),

# ofdmaSoundingRspTime1(116),

# ofdmaSoundingRspTime2(117),

# ofdmaMaxSimuSoundInst0(118),

# ofdmaMaxSimuSoundInst1(119),

# ofdmaMaxSimuSoundInst2(120),

# ofdmaMaxSimuSoundInst3(121),

# ofdmaNoP9Or18ForCsitTypeA(122),

# ofdmaCsitNotSupported(123),

# reserved124(124),

# reserved125(125),

# reserved126(126),

# reserved127(127),

# ofdmaMaxHarqBurstsUl0(128),

# ofdmaMaxHarqBurstsUl1(129),

# ofdmaMaxHarqBurstsUl2(130),

# ofdmaMaxHarqBurstsUlNonHarqIncluded(131),

# ofdmaMaxHarqBurstsDl0(132),

# ofdmaMaxHarqBurstsDl1(133),

# ofdmaMaxHarqBurstsDl2(134),

# ofdmaMaxHarqBurstsDl3(135),

# ofdmaMimoMod2AntStcMatrixA(136),

# ofdmaMimoMod2AntStcMatrixBVCoding(137),

# ofdmaMimoMod2AntStcMatrixBHCoding(138),

# ofdmaMimoModBeamforming(139),

# ofdmaMimoModAdaptiveRateControl(140),

# ofdmaMimoModSingleAnt(141),

# ofdmaMimoModCollaborativeSm1Ant(142),

# ofdmaMimoModCollaborativeSm2Ants(143),

# ofdmaMimoModDisableUlSubchRotation(144),

# reserved145(145),

# reserved146(146),

# reserved147(147),

# reserved148(148),

# reserved149(149),

# reserved150(150),

# reserved151(151),

# ofdmaMcastBurstsDlMultiFecTypes(152),

# ofdmaMcastBurstsUlMultiFecTypes(153),

# reserved154(154),

# reserved155(155),

# reserved156(156),

# reserved157(157),

# reserved158(158),

# reserved159(159),

# ofdmaHarqIncrBufDlNep0(160),

# ofdmaHarqIncrBufDlNep1(161),

# ofdmaHarqIncrBufDlNep2(162),

# ofdmaHarqIncrBufDlNep3(163),

# ofdmaHarqIncrBufDlAggFlag(164),

# reserved165(165),

# reserved166(166),

# reserved167(167),

# ofdmaHarqIncrBufUlNep0(168),

# ofdmaHarqIncrBufUlNep1(169),

# ofdmaHarqIncrBufUlNep2(170),

# ofdmaHarqIncrBufUlNep3(171),

# ofdmaHarqIncrBufUlAggFlag(172),

# reserved173(173),

# reserved174(174),

# reserved175(175),

# ofdmaHarqChaseBufDlComb0(176),

# ofdmaHarqChaseBufDlComb1(177),

# ofdmaHarqChaseBufDlComb2(178),

# ofdmaHarqChaseBufDlComb3(179),

# ofdmaHarqChaseBufDlComb4(180),

# ofdmaHarqChaseBufDlComb5(181),

# ofdmaHarqChaseBufDlAggFlag(182),

# reserved183(183),

# ofdmaHarqChaseBufUlComb0(184),

# ofdmaHarqChaseBufUlComb1(185),

# ofdmaHarqChaseBufUlComb2(186),

# ofdmaHarqChaseBufUlComb3(187),

# ofdmaHarqChaseBufUlComb4(188),

# ofdmaHarqChaseBufUlComb5(189),

# ofdmaHarqChaseBufUlAggFlag(190),

# reserved191(191),

# odfmaParamSetPhyA(192),

# odfmaParamSetPhyB(193),

# odfmaParamSetHarq0(194),

# odfmaParamSetHarq1(195),

# odfmaParamSetHarq2(196),

# odfmaParamSetMacA(197),

# odfmaParamSetMacB(198),

# reserved199(199)

# }

# --

# -- Node definitions

# --

# -- 1.0

# std OBJECT IDENTIFIER ::= { iso 0 }

# -- 1.0.8802

# iso8802 OBJECT IDENTIFIER ::= { std 8802 }

# -- 1.0.8802.16

# wman OBJECT IDENTIFIER ::= { iso8802 16 }

# -- 1.0.8802.16.999.1

# wmanIf2MibObjects OBJECT IDENTIFIER ::= { wmanIf2BsMibD6Cap 1 }

# -- 1.0.8802.16.999.1.2

# wmanIf2BsCm OBJECT IDENTIFIER ::= { wmanIf2MibObjects 2 }

# -- 1.0.8802.16.999.1.2.1

# wmanIf2BsRegisteredSsTable OBJECT-TYPE

# SYNTAX SEQUENCE OF WmanIf2BsRegisteredSsEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table contains the basic capability information

# of SSs that have been negotiated and agreed between

# BS and SS via REG-REQ and REG-RSP messages. An entry

# in this table indicates the SS has entered and registered

# into the BS."

# REFERENCE

# "Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsCm 1 }

# -- 1.0.8802.16.999.1.2.1.1

# wmanIf2BsRegisteredSsEntry OBJECT-TYPE

# SYNTAX WmanIf2BsRegisteredSsEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table provides one row for each SS that has been

# registered in the BS. The primary index is the ifIndex

# with an ifType of ieee80216WMAN, indicating the BS sector

# with which the SS is associated. wmanIf2BsSsMacAddress

# identifies the SS being registered."

# INDEX { ifIndex, wmanIf2BsSsMacAddress }

# ::= { wmanIf2BsRegisteredSsTable 1 }

# WmanIf2BsRegisteredSsEntry ::=

# SEQUENCE {

# wmanIf2BsSsMacVersion

# WmanIf2MacVersion,

# wmanIf2BsSsMaxTxPower64Qam

# WmanIf2MaxTxPowerType,

# wmanIf2BsSsMaxTxPower16Qam

# WmanIf2MaxTxPowerType,

# wmanIf2BsSsMaxTxPowerQpsk

# WmanIf2MaxTxPowerType,

# wmanIf2BsSsMaxTxPowerBpsk

# WmanIf2MaxTxPowerType,

# wmanIf2BsSsAasBroadcastPermission

# INTEGER,

# wmanIf2BsSsVendorIdEncoding

# OCTET STRING,

# wmanIf2BsSs2ndMgmtArqBlockSize

# INTEGER,

# wmanIf2BsSs2ndMgmtArqRxPurgeTimeout

# INTEGER,

# wmanIf2BsSs2ndMgmtArqDeliverInOrder

# TruthValue,

# wmanIf2BsSs2ndMgmtArqSyncLossTimeout

# INTEGER,

# wmanIf2BsSs2ndMgmtArqBlockLifetime

# INTEGER,

# wmanIf2BsSs2ndMgmtArqUpLinkRxDelay

# INTEGER,

# wmanIf2BsSs2ndMgmtArqDnLinkRxDelay

# INTEGER,

# wmanIf2BsSs2ndMgmtArqUpLinkTxDelay

# INTEGER,

# wmanIf2BsSs2ndMgmtArqDnLinkTxDelay

# INTEGER,

# wmanIf2BsSs2ndMgmtArqWindowSize

# INTEGER,

# wmanIf2BsSs2ndMgmtArqEnable

# TruthValue,

# wmanIf2BsSsIpManagementMode

# INTEGER,

# wmanIf2BsSsManagementSupport

# INTEGER,

# wmanIf2BsSsSecondaryCid

# WmanIf2CidType,

# wmanIf2BsSsPrimaryCid

# WmanIf2CidType,

# wmanIf2BsSsBasicCid

# WmanIf2CidType,

# wmanIf2BsSsMacAddress

# MacAddress

# }

# -- 1.0.8802.16.999.1.2.1.1.24

# wmanIf2BsSsMacVersion OBJECT-TYPE

# SYNTAX WmanIf2MacVersion

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This parameter specifies the version of 802.16 to which the

# message originator conforms."

# REFERENCE

# "Subclause 11.1.3 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 24 }

# -- 1.0.8802.16.999.1.2.1.1.23

# wmanIf2BsSsMaxTxPower64Qam OBJECT-TYPE

# SYNTAX WmanIf2MaxTxPowerType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The maximum available power for 64-QAM constellations.

# The maximum power parameters are reported in dBm and

# quantized in 0.5 dBm steps ranging from -64 dBm (encoded

# 0x00) to 63.5 dBm (encoded 0xFF). Values outside this

# range shall be assigned the closest extreme. SSs that do

# not support QAM64 shall report the value of 0x00. This

# parameter is only applicable to systems supporting the SCa,

# OFDM or OFDMA PHY."

# REFERENCE

# "Subclause 11.8.3.2 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 23 }

# -- 1.0.8802.16.999.1.2.1.1.22

# wmanIf2BsSsMaxTxPower16Qam OBJECT-TYPE

# SYNTAX WmanIf2MaxTxPowerType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The maximum available power for 16-QAM constellations.

# The maximum power parameters are reported in dBm and

# quantized in 0.5 dBm steps ranging from -64 dBm (encoded

# 0x00) to 63.5 dBm (encoded 0xFF). Values outside this

# range shall be assigned the closest extreme. This parameter

# is only applicable to systems supporting the SCa, OFDM or

# OFDMA PHY."

# REFERENCE

# "Subclause 11.8.3.2 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 22 }

# -- 1.0.8802.16.999.1.2.1.1.21

# wmanIf2BsSsMaxTxPowerQpsk OBJECT-TYPE

# SYNTAX WmanIf2MaxTxPowerType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The maximum available power for QPSK. The maximum power

# parameters are reported in dBm and quantized in 0.5 dBm

# steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

# (encoded 0xFF). Values outside this range shall be assigned

# to closest extreme. This parameter is only applicable to

# systems supporting the SCa, OFDM or OFDMA PHY."

# REFERENCE

# "Subclause 11.8.3.2 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 21 }

# -- 1.0.8802.16.999.1.2.1.1.20

# wmanIf2BsSsMaxTxPowerBpsk OBJECT-TYPE

# SYNTAX WmanIf2MaxTxPowerType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The maximum available power for BPSK. The maximum power

# parameters are reported in dBm and quantized in 0.5 dBm

# steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

# (encoded 0xFF). Values outside this range shall be assigned

# the closest extreme. This parameter is only applicable to

# systems supporting the SCa, OFDM or OFDMA PHY."

# REFERENCE

# "Subclause 11.8.3.2 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 20 }

# -- 1.0.8802.16.999.1.2.1.1.19

# wmanIf2BsSsAasBroadcastPermission OBJECT-TYPE

# SYNTAX INTEGER

# {

# contBasedBwReqPermitted(0),

# contBasedBwReqNotPermitted(1)

# }

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This parameter specifies if SS can issue contention-based

# bandwidth request or not."

# REFERENCE

# "Subclause 11.6 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 19 }

# -- 1.0.8802.16.999.1.2.1.1.18

# wmanIf2BsSsVendorIdEncoding OBJECT-TYPE

# SYNTAX OCTET STRING (SIZE (3))

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The value field contains the vendor identification

# specified by the 3 byte vendor-specific organizationally

# unique identifier of the SS or BS MAC address.A vendor ID

# used in a REG-REQ shall be the Vendor ID of the SS sending

# the request. A vendor ID used in a REG-RSP shall be the

# Vendor ID of the BS sending the response."

# REFERENCE

# "Subclause 11.1.5 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 18 }

# -- 1.0.8802.16.999.1.2.1.1.17

# wmanIf2BsSs2ndMgmtArqBlockSize OBJECT-TYPE

# SYNTAX INTEGER (1..2040)

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This parameter specifies the size of a ARQ block. This

# parameter shall be established by negotiation during the

# connection setup. The requester includes its desired

# setting in the REQ message. The receiver of the REQ

# message shall take the smaller of the value it prefers and

# value in the REQ message. The minimum value is included in

# the RSP message."

# REFERENCE

# "Subclause 11.13.18.8 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 17 }

# -- 1.0.8802.16.999.1.2.1.1.16

# wmanIf2BsSs2ndMgmtArqRxPurgeTimeout OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "10 us"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Indicates the time interval the ARQ window is advanced

# after a fragment is received. A value of 0 means Infinite."

# REFERENCE

# "Subclause 11.13.18.7 in IEEE Std 802.16-2004"

# DEFVAL { 0 }

# ::= { wmanIf2BsRegisteredSsEntry 16 }

# -- 1.0.8802.16.999.1.2.1.1.15

# wmanIf2BsSs2ndMgmtArqDeliverInOrder OBJECT-TYPE

# SYNTAX TruthValue

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Indicates whether or not data is to be delivered

# by the receiving MAC to its client application

# in the order in which data was handed off to the

# originating MAC."

# REFERENCE

# "Subclause 11.13.18.6 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 15 }

# -- 1.0.8802.16.999.1.2.1.1.14

# wmanIf2BsSs2ndMgmtArqSyncLossTimeout OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "10 us"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The maximum interval before declaring a loss

# of synchronization of the sender and receiver

# state machines. A value of 0 means Infinite."

# REFERENCE

# "Subclause 11.13.18.5 in IEEE Std 802.16-2004"

# DEFVAL { 0 }

# ::= { wmanIf2BsRegisteredSsEntry 14 }

# -- 1.0.8802.16.999.1.2.1.1.13

# wmanIf2BsSs2ndMgmtArqBlockLifetime OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "10 us"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The maximum time interval an ARQ fragment will be

# managed by the transmitter ARQ machine, once

# initial transmission of the fragment has occured.

# If transmission or retransmission of the fragment

# is not acknowledged by the receiver before the

# time limit is reached, the fragment is discarded.

# A value of 0 means Infinite."

# REFERENCE

# "Subclause 11.13.18.4 in IEEE Std 802.16-2004"

# DEFVAL { 0 }

# ::= { wmanIf2BsRegisteredSsEntry 13 }

# -- 1.0.8802.16.999.1.2.1.1.12

# wmanIf2BsSs2ndMgmtArqUpLinkRxDelay OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The object defines the ARQ receiver delay for

# uplink transmission."

# REFERENCE

# "Subclause 11.13.18.3 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 12 }

# -- 1.0.8802.16.999.1.2.1.1.11

# wmanIf2BsSs2ndMgmtArqDnLinkRxDelay OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The object defines the ARQ receiver delay for

# downlink transmission."

# REFERENCE

# "Subclause 11.13.18.3 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 11 }

# -- 1.0.8802.16.999.1.2.1.1.10

# wmanIf2BsSs2ndMgmtArqUpLinkTxDelay OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The object defines the ARQ transmitter delay for

# uplink transmission."

# REFERENCE

# "Subclause 11.13.18.3 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 10 }

# -- 1.0.8802.16.999.1.2.1.1.9

# wmanIf2BsSs2ndMgmtArqDnLinkTxDelay OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The object defines the ARQ transmitter delay for

# downlink transmission."

# REFERENCE

# "Subclause 11.13.18.3 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 9 }

# -- 1.0.8802.16.999.1.2.1.1.8

# wmanIf2BsSs2ndMgmtArqWindowSize OBJECT-TYPE

# SYNTAX INTEGER (1..1024)

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Indicates the maximum number of unacknowledged

# fragments at any time for 2nd management connection."

# REFERENCE

# "Subclause 11.13.18.2 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 8 }

# -- 1.0.8802.16.999.1.2.1.1.7

# wmanIf2BsSs2ndMgmtArqEnable OBJECT-TYPE

# SYNTAX TruthValue

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "True(1) ARQ enabling is requested for the 2nd

# management channel."

# REFERENCE

# "Subclause 11.13.18.1 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 7 }

# -- 1.0.8802.16.999.1.2.1.1.6

# wmanIf2BsSsIpManagementMode OBJECT-TYPE

# SYNTAX INTEGER

# {

# unmanaged(0),

# ipManaged(1)

# }

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The IP management mode parameter dictates whether

# the provider intends to manage the SS on an ongoing

# basis via IP-based mechanisms."

# REFERENCE

# "Subclause 11.7.3 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 6 }

# -- 1.0.8802.16.999.1.2.1.1.5

# wmanIf2BsSsManagementSupport OBJECT-TYPE

# SYNTAX INTEGER

# {

# unmanagedSs(0),

# managedSs(1)

# }

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object indicates whether or not the SS is managed."

# REFERENCE

# "Subclause 11.7.2 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 5 }

# -- 1.0.8802.16.999.1.2.1.1.4

# wmanIf2BsSsSecondaryCid OBJECT-TYPE

# SYNTAX WmanIf2CidType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The value of this object indicates the secondary

# management CID present in the REG-RSP message. The value

# should be null if the 2nd management connection is not

# available."

# REFERENCE

# "Subclause 6.4.2.3.8 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 4 }

# -- 1.0.8802.16.999.1.2.1.1.3

# wmanIf2BsSsPrimaryCid OBJECT-TYPE

# SYNTAX WmanIf2CidType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The value of this object indicates the primary CID of the

# SS received from the RNG-RSP message."

# REFERENCE

# "Subclause 6.3.2.3.6 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 3 }

# -- 1.0.8802.16.999.1.2.1.1.2

# wmanIf2BsSsBasicCid OBJECT-TYPE

# SYNTAX WmanIf2CidType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "The value of this object indicates the SS's basic CID

# that was sent in the RNG-RSP message."

# REFERENCE

# "Subclause 6.3.2.3.6 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 2 }

# -- 1.0.8802.16.999.1.2.1.1.1

# wmanIf2BsSsMacAddress OBJECT-TYPE

# SYNTAX MacAddress

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "The MAC address of SS is received from the RNG-REQ

# message. When SS registers, this MAC address is entered

# into the table, and used as the identifier to the SS."

# REFERENCE

# "Subclause 6.3.2.3.5 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsRegisteredSsEntry 1 }

# -- 1.0.8802.16.999.1.2.3

# wmanIf2BsSsReqCapabilitiesTable OBJECT-TYPE

# SYNTAX SEQUENCE OF WmanIf2BsSsReqCapabilitiesEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table contains the basic capability information of SSs

# that have been reported by SSs to BS using RNG-REQ, SBC-REQ

# and REG-REQ messages. Entries in this table should be

# created when an SS registers with a BS."

# ::= { wmanIf2BsCm 3 }

# -- 1.0.8802.16.999.1.2.3.1

# wmanIf2BsSsReqCapabilitiesEntry OBJECT-TYPE

# SYNTAX WmanIf2BsSsReqCapabilitiesEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table provides one row for each SS that has been

# registered in the BS. This table augments the table

# wmanIf2BsRegisteredSsTable."

# AUGMENTS { wmanIf2BsRegisteredSsEntry }

# ::= { wmanIf2BsSsReqCapabilitiesTable 1 }

# WmanIf2BsSsReqCapabilitiesEntry ::=

# SEQUENCE {

# wmanIf2BsSsReqCapUplinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsSsReqCapDsxFlowControl

# WmanIf2MaxDsxFlowType,

# wmanIf2BsSsReqCapMcaFlowControl

# WmanIf2MaxMcaFlowType,

# wmanIf2BsSsReqCapMcpGroupCidSupport

# WmanIf2MaxMcpGroupCid,

# wmanIf2BsSsReqCapPkmFlowControl

# WmanIf2MaxPkmFlowType,

# wmanIf2BsSsReqCapMaxNumOfSupportedSA

# WmanIf2MaxNumOfSaType,

# wmanIf2BsSsReqCapMaxNumOfClassifier

# WmanIf2MaxClassifiers,

# wmanIf2BsSsReqCapTtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsSsReqCapRtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsSsReqCapDownlinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsSsReqCapMaxNumBurstToMs

# WmanIf2CapMaxNumBurstToMsType,

# wmanIf2BsSsReqCapMaxMacLevelDlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsSsReqCapMaxMacLevelUlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsSsReqCapPnWindowSize

# INTEGER,

# wmanIf2BsSsReqCapMacHeaderSdnSnPeriod

# WmanIf2MacHeaderSuppSdnSnPeriod,

# wmanIf2BsSsOfdmReqCapLoopPwrControlSw

# Unsigned32,

# wmanIf2BsSsOfdmaReqCapSdmaPilot

# WmanIf2SdmaPilotCap,

# wmanIf2BsSsOfdmaReqCapNoUlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsSsOfdmaReqCapNoDlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsSsReqCapOptionsGeneral

# WmanIf2SsCapOptionsGeneral,

# wmanIf2BsSsReqCapOptionsOfdm

# WmanIf2SsCapOptionsOfdm,

# wmanIf2BsSsReqCapOptionsOfdma

# WmanIf2SsCapOptionsOfdma

# }

# -- 1.0.8802.16.999.1.2.3.1.1

# wmanIf2BsSsReqCapUplinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object shows the number of Uplink transport CIDs the SS

# can support."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsReqCapabilitiesEntry 1 }

# -- 1.0.8802.16.999.1.2.3.1.3

# wmanIf2BsSsReqCapDsxFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxDsxFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent

# DSA, DSC, or DSD transactions that SS is capable of having

# outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 3 }

# -- 1.0.8802.16.999.1.2.3.1.4

# wmanIf2BsSsReqCapMcaFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxMcaFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent MCA

# transactions that SS is capable of having outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 4 }

# -- 1.0.8802.16.999.1.2.3.1.5

# wmanIf2BsSsReqCapMcpGroupCidSupport OBJECT-TYPE

# SYNTAX WmanIf2MaxMcpGroupCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object indicates the maximum number of

# simultaneous Multicast Polling Groups the SS is

# capable of belonging to."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 5 }

# -- 1.0.8802.16.999.1.2.3.1.6

# wmanIf2BsSsReqCapPkmFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxPkmFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent PKM

# transactions that SS is capable of having outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 6 }

# -- 1.0.8802.16.999.1.2.3.1.7

# wmanIf2BsSsReqCapMaxNumOfSupportedSA OBJECT-TYPE

# SYNTAX WmanIf2MaxNumOfSaType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the maximum number of supported

# security associations of the SS."

# DEFVAL { 1 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 7 }

# -- 1.0.8802.16.999.1.2.3.1.10

# wmanIf2BsSsReqCapMaxNumOfClassifier OBJECT-TYPE

# SYNTAX WmanIf2MaxClassifiers

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object indicates the maximum number of admitted

# Classifiers that the SS can support."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 10 }

# -- 1.0.8802.16.999.1.2.3.1.14

# wmanIf2BsSsReqCapTtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the SS's transition speed SSTTG

# for TDD and H-FDD SSs. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 14 }

# -- 1.0.8802.16.999.1.2.3.1.15

# wmanIf2BsSsReqCapRtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the SS's transition speed SSRTG

# for TDD and H-FDD SSs. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 15 }

# -- 1.0.8802.16.999.1.2.3.1.16

# wmanIf2BsSsReqCapDownlinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object shows the number of Downlink transport CIDs the

# SS can support."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsReqCapabilitiesEntry 16 }

# -- 1.0.8802.16.999.1.2.3.1.19

# wmanIf2BsSsReqCapMaxNumBurstToMs OBJECT-TYPE

# SYNTAX WmanIf2CapMaxNumBurstToMsType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum number of bursts transmitted concurrently to the MS

# , including all bursts without CID or with CIDs matching

# the MS's CIDs."

# REFERENCE

# "Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsReqCapabilitiesEntry 19 }

# -- 1.0.8802.16.999.1.2.3.1.23

# wmanIf2BsSsReqCapMaxMacLevelDlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per DL frame."

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 23 }

# -- 1.0.8802.16.999.1.2.3.1.24

# wmanIf2BsSsReqCapMaxMacLevelUlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per UL frame."

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsSsReqCapabilitiesEntry 24 }

# -- 1.0.8802.16.999.1.2.3.1.28

# wmanIf2BsSsReqCapPnWindowSize OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Specifies the size capability of the receiver PN window for

# SAs and management connections. The receiver shall track

# PNs within this window to prevent replay attacks (see

# 7.5.1.2.4)."

# REFERENCE

# "Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsReqCapabilitiesEntry 28 }

# -- 1.0.8802.16.999.1.2.3.1.29

# wmanIf2BsSsReqCapMacHeaderSdnSnPeriod OBJECT-TYPE

# SYNTAX WmanIf2MacHeaderSuppSdnSnPeriod

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Indicates whether or not the MS and BS support various

# types of MAC header and extended subheaders."

# REFERENCE

# "Subclause 11.7.25 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsReqCapabilitiesEntry 29 }

# -- 1.0.8802.16.999.1.2.3.1.30

# wmanIf2BsSsOfdmReqCapLoopPwrControlSw OBJECT-TYPE

# SYNTAX Unsigned32

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates he minimum number of frames that

# SS takes to switch from the open loop power control

# scheme to the closed loop power control scheme or

# vice versa."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 30 }

# -- 1.0.8802.16.999.1.2.3.1.31

# wmanIf2BsSsOfdmaReqCapSdmaPilot OBJECT-TYPE

# SYNTAX WmanIf2SdmaPilotCap

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the SDMA pilot pattern support

# for AMC zone."

# REFERENCE

# "Subclause 11.8.3.7.17 in IEEE 802.16e"

# ::= { wmanIf2BsSsReqCapabilitiesEntry 31 }

# -- 1.0.8802.16.999.1.2.3.1.32

# wmanIf2BsSsOfdmaReqCapNoUlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of uplink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 32 }

# -- 1.0.8802.16.999.1.2.3.1.33

# wmanIf2BsSsOfdmaReqCapNoDlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of downlink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 33 }

# -- 1.0.8802.16.999.1.2.3.1.34

# wmanIf2BsSsReqCapOptionsGeneral OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsGeneral

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded general capability options as reported by SS."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 34 }

# -- 1.0.8802.16.999.1.2.3.1.35

# wmanIf2BsSsReqCapOptionsOfdm OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdm

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDM specific capability options as reported by SS."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 35 }

# -- 1.0.8802.16.999.1.2.3.1.36

# wmanIf2BsSsReqCapOptionsOfdma OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdma

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDMA specific capability options as reported by SS."

# ::= { wmanIf2BsSsReqCapabilitiesEntry 36 }

# -- 1.0.8802.16.999.1.2.4

# wmanIf2BsSsRspCapabilitiesTable OBJECT-TYPE

# SYNTAX SEQUENCE OF WmanIf2BsSsRspCapabilitiesEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table contains the basic capability information of SSs

# that have been negotiated and agreed between BS and SS via

# RNG-REQ/RSP, SBC-REQ/RSP and REG-REQ/RSP messages.

# This table augments the wmanIf2BsRegisteredSsTable."

# REFERENCE

# "Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

# ::= { wmanIf2BsCm 4 }

# -- 1.0.8802.16.999.1.2.4.1

# wmanIf2BsSsRspCapabilitiesEntry OBJECT-TYPE

# SYNTAX WmanIf2BsSsRspCapabilitiesEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table provides one row for each SS that has been

# registered in the BS. This table augments the

# wmanIf2BsRegisteredSsTable. "

# AUGMENTS { wmanIf2BsRegisteredSsEntry }

# ::= { wmanIf2BsSsRspCapabilitiesTable 1 }

# WmanIf2BsSsRspCapabilitiesEntry ::=

# SEQUENCE {

# wmanIf2BsSsRspCapUplinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsSsRspCapDsxFlowControl

# WmanIf2MaxDsxFlowType,

# wmanIf2BsSsRspCapMcaFlowControl

# WmanIf2MaxMcaFlowType,

# wmanIf2BsSsRspCapMcpGroupCidSupport

# WmanIf2MaxMcpGroupCid,

# wmanIf2BsSsRspCapPkmFlowControl

# WmanIf2MaxPkmFlowType,

# wmanIf2BsSsRspCapMaxNumOfSupportedSA

# WmanIf2MaxNumOfSaType,

# wmanIf2BsSsRspCapMaxNumOfClassifier

# WmanIf2MaxClassifiers,

# wmanIf2BsSsRspCapTtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsSsRspCapRtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsSsRspCapDownlinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsSsRspCapMaxNumBurstToMs

# WmanIf2CapMaxNumBurstToMsType,

# wmanIf2BsSsRspCapMaxMacLevelDlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsSsRspCapMaxMacLevelUlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsSsRspCapNumOfProvisionedSf

# WmanIf2CapMaxNumOfProvisionedSf,

# wmanIf2BsSsRspCapPnWindowSize

# INTEGER,

# wmanIf2BsSsRspCapMacHeaderSdnSnPeriod

# WmanIf2MacHeaderSuppSdnSnPeriod,

# wmanIf2BsSsOfdmRspCapLoopPwrControlSw

# Unsigned32,

# wmanIf2BsSsOfdmaRspCapSdmaPilot

# WmanIf2SdmaPilotCap,

# wmanIf2BsSsOfdmaRspCapNoUlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsSsOfdmaRspCapNoDlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsSsRspCapOptionsGeneral

# WmanIf2SsCapOptionsGeneral,

# wmanIf2BsSsRspCapOptionsOfdm

# WmanIf2SsCapOptionsOfdm,

# wmanIf2BsSsRspCapOptionsOfdma

# WmanIf2SsCapOptionsOfdma

# }

# -- 1.0.8802.16.999.1.2.4.1.1

# wmanIf2BsSsRspCapUplinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated number of Uplink transport CIDs the SS can

# support."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 1 }

# -- 1.0.8802.16.999.1.2.4.1.3

# wmanIf2BsSsRspCapDsxFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxDsxFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated maximum number of concurrent DSA, DSC, or DSD

# transactions that may be outstanding."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 3 }

# -- 1.0.8802.16.999.1.2.4.1.4

# wmanIf2BsSsRspCapMcaFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxMcaFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated maximum number of concurrent

# MCA transactions that may be outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 4 }

# -- 1.0.8802.16.999.1.2.4.1.5

# wmanIf2BsSsRspCapMcpGroupCidSupport OBJECT-TYPE

# SYNTAX WmanIf2MaxMcpGroupCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated maximum number of simultaneous Multicast

# Polling Groups the SS is capable of belonging to."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 5 }

# -- 1.0.8802.16.999.1.2.4.1.6

# wmanIf2BsSsRspCapPkmFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxPkmFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated maximum number of concurrent PKM

# transactions that may be outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 6 }

# -- 1.0.8802.16.999.1.2.4.1.7

# wmanIf2BsSsRspCapMaxNumOfSupportedSA OBJECT-TYPE

# SYNTAX WmanIf2MaxNumOfSaType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated maximum number of supported security

# association of the SS."

# DEFVAL { 1 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 7 }

# -- 1.0.8802.16.999.1.2.4.1.10

# wmanIf2BsSsRspCapMaxNumOfClassifier OBJECT-TYPE

# SYNTAX WmanIf2MaxClassifiers

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Negotiated maximum number of admitted Classifiers

# that the SS is allowed to have."

# DEFVAL { 0 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 10 }

# -- 1.0.8802.16.999.1.2.4.1.14

# wmanIf2BsSsRspCapTtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the negotiated transition speed

# SSTTG for TDD and H-FDD SSs. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 14 }

# -- 1.0.8802.16.999.1.2.4.1.15

# wmanIf2BsSsRspCapRtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the negotiated transition speed

# SSRTG for TDD and H-FDD SSs. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 15 }

# -- 1.0.8802.16.999.1.2.4.1.17

# wmanIf2BsSsRspCapDownlinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object shows the number of Downlink transport CIDs the

# SS can support."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 17 }

# -- 1.0.8802.16.999.1.2.4.1.19

# wmanIf2BsSsRspCapMaxNumBurstToMs OBJECT-TYPE

# SYNTAX WmanIf2CapMaxNumBurstToMsType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum number of bursts transmitted concurrently to the MS

# , including all bursts without CID or with CIDs matching

# the MS CIDs."

# REFERENCE

# "Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 19 }

# -- 1.0.8802.16.999.1.2.4.1.23

# wmanIf2BsSsRspCapMaxMacLevelDlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per DL frame. A value of 0 indicates such

# limitation does not exist, except the limitation of

# the physical medium"

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 23 }

# -- 1.0.8802.16.999.1.2.4.1.24

# wmanIf2BsSsRspCapMaxMacLevelUlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per UL frame. A value of 0 indicates such

# limitation does not exist, except the limitation of

# the physical medium"

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsSsRspCapabilitiesEntry 24 }

# -- 1.0.8802.16.999.1.2.4.1.25

# wmanIf2BsSsRspCapNumOfProvisionedSf OBJECT-TYPE

# SYNTAX WmanIf2CapMaxNumOfProvisionedSf

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "When a BS is to transmit multiple DSA transactions for

# provisioned service flows, this object indicates how many

# DSA transactions with provisioned service flows will be

# transmitted."

# REFERENCE

# "Subclause 11.7.19 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 25 }

# -- 1.0.8802.16.999.1.2.4.1.29

# wmanIf2BsSsRspCapPnWindowSize OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Specifies the size capability of the receiver PN window for

# SAs and management connections. The receiver shall track

# PNs within this window to prevent replay attacks (see

# 7.5.1.2.4)."

# REFERENCE

# "Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 29 }

# -- 1.0.8802.16.999.1.2.4.1.30

# wmanIf2BsSsRspCapMacHeaderSdnSnPeriod OBJECT-TYPE

# SYNTAX WmanIf2MacHeaderSuppSdnSnPeriod

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Indicates whether or not the MS and BS support various

# types of MAC header and extended subheaders."

# REFERENCE

# "Subclause 11.7.25 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 30 }

# -- 1.0.8802.16.999.1.2.4.1.31

# wmanIf2BsSsOfdmRspCapLoopPwrControlSw OBJECT-TYPE

# SYNTAX Unsigned32

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates he minimum number of frames that

# SS takes to switch from the open loop power control

# scheme to the closed loop power control scheme or

# vice versa."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 31 }

# -- 1.0.8802.16.999.1.2.4.1.32

# wmanIf2BsSsOfdmaRspCapSdmaPilot OBJECT-TYPE

# SYNTAX WmanIf2SdmaPilotCap

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the SDMA pilot pattern support

# for AMC zone."

# REFERENCE

# "Subclause 11.8.3.7.17 in IEEE 802.16e"

# ::= { wmanIf2BsSsRspCapabilitiesEntry 32 }

# -- 1.0.8802.16.999.1.2.4.1.33

# wmanIf2BsSsOfdmaRspCapNoUlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of uplink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 33 }

# -- 1.0.8802.16.999.1.2.4.1.34

# wmanIf2BsSsOfdmaRspCapNoDlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of downlink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 34 }

# -- 1.0.8802.16.999.1.2.4.1.35

# wmanIf2BsSsRspCapOptionsGeneral OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsGeneral

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded general capability options as granted by BS for SS."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 35 }

# -- 1.0.8802.16.999.1.2.4.1.36

# wmanIf2BsSsRspCapOptionsOfdm OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdm

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDM specific capability options as granted by BS for SS."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 36 }

# -- 1.0.8802.16.999.1.2.4.1.37

# wmanIf2BsSsRspCapOptionsOfdma OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdma

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDMA specific capability options as granted by BS for SS."

# ::= { wmanIf2BsSsRspCapabilitiesEntry 37 }

# -- 1.0.8802.16.999.1.2.5

# wmanIf2BsBasicCapabilitiesTable OBJECT-TYPE

# SYNTAX SEQUENCE OF WmanIf2BsBasicCapabilitiesEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table contains the basic capabilities of the BS as

# implemented in BS hardware and software. These capabilities

# along with the configuration for them

# (wmanIf2BsCapabilitiesConfigTable) are used for negotiation

# of basic capabilities with SS using RNG-RSP, SBC-RSP and

# REG-RSP messages. The negotiated capabilities are obtained

# by interSubclause of SS raw reported capabilities, BS raw

# capabilities and BS configured capabilities. The objects in

# the table have read-only access. The table is maintained

# by BS."

# ::= { wmanIf2BsCm 5 }

# -- 1.0.8802.16.999.1.2.5.1

# wmanIf2BsBasicCapabilitiesEntry OBJECT-TYPE

# SYNTAX WmanIf2BsBasicCapabilitiesEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table provides one row for each BS sector."

# INDEX { ifIndex }

# ::= { wmanIf2BsBasicCapabilitiesTable 1 }

# WmanIf2BsBasicCapabilitiesEntry ::=

# SEQUENCE {

# wmanIf2BsCapUplinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsCapDsxFlowControl

# WmanIf2MaxDsxFlowType,

# wmanIf2BsCapMcaFlowControl

# WmanIf2MaxMcaFlowType,

# wmanIf2BsCapMcpGroupCidSupport

# WmanIf2MaxMcpGroupCid,

# wmanIf2BsCapPkmFlowControl

# WmanIf2MaxPkmFlowType,

# wmanIf2BsCapMaxNumOfSupportedSA

# WmanIf2MaxNumOfSaType,

# wmanIf2BsCapMaxNumOfClassifier

# WmanIf2MaxClassifiers,

# wmanIf2BsCapTtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsCapRtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsCapDownlinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsCapMaxNumBurstToMs

# WmanIf2CapMaxNumBurstToMsType,

# wmanIf2BsCapMaxMacLevelDlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsCapMaxMacLevelUlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsCapNumOfProvisionedSf

# WmanIf2CapMaxNumOfProvisionedSf,

# wmanIf2BsCapPnWindowSize

# INTEGER,

# wmanIf2BsCapMacHeaderSdnSnPeriod

# WmanIf2MacHeaderSuppSdnSnPeriod,

# wmanIf2BsSsOfdmCapLoopPwrControlSw

# Unsigned32,

# wmanIf2BsSsOfdmaCapSdmaPilot

# WmanIf2SdmaPilotCap,

# wmanIf2BsOfdmaCapNoUlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsOfdmaCapNoDlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsCapOptionsGeneral

# WmanIf2SsCapOptionsGeneral,

# wmanIf2BsCapOptionsOfdm

# WmanIf2SsCapOptionsOfdm,

# wmanIf2BsCapOptionsOfdma

# WmanIf2SsCapOptionsOfdma

# }

# -- 1.0.8802.16.999.1.2.5.1.1

# wmanIf2BsCapUplinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object shows the number of Uplink transport CIDs the

# BS can support per SS."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsBasicCapabilitiesEntry 1 }

# -- 1.0.8802.16.999.1.2.5.1.3

# wmanIf2BsCapDsxFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxDsxFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent

# DSA, DSC, or DSD transactions that BS allows each SS to

# have outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 3 }

# -- 1.0.8802.16.999.1.2.5.1.4

# wmanIf2BsCapMcaFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxMcaFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent

# MCA transactions that BS allows each SS to have."

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 4 }

# -- 1.0.8802.16.999.1.2.5.1.5

# wmanIf2BsCapMcpGroupCidSupport OBJECT-TYPE

# SYNTAX WmanIf2MaxMcpGroupCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object indicates the maximum number of simultaneous

# Multicast Polling Groups the BS allows each SS to belong

# to."

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 5 }

# -- 1.0.8802.16.999.1.2.5.1.6

# wmanIf2BsCapPkmFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxPkmFlowType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent

# PKM transactions that BS allows each SS to have."

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 6 }

# -- 1.0.8802.16.999.1.2.5.1.7

# wmanIf2BsCapMaxNumOfSupportedSA OBJECT-TYPE

# SYNTAX WmanIf2MaxNumOfSaType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies maximum number of supported security

# associations per SS that the BS allows."

# DEFVAL { 1 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 7 }

# -- 1.0.8802.16.999.1.2.5.1.10

# wmanIf2BsCapMaxNumOfClassifier OBJECT-TYPE

# SYNTAX WmanIf2MaxClassifiers

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object indicates the maximum number of admitted

# Classifiers per SS that the BS allows."

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 10 }

# -- 1.0.8802.16.999.1.2.5.1.14

# wmanIf2BsCapTtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the transition speed SSTTG for TDD

# and H-FDD SSs allowed by the BS. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsBasicCapabilitiesEntry 14 }

# -- 1.0.8802.16.999.1.2.5.1.15

# wmanIf2BsCapRtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the transition speed SSRTG for TDD

# and H-FDD SSs allowed by the BS. The usage is defined

# by WmanIf2SsTransitionGap."

# ::= { wmanIf2BsBasicCapabilitiesEntry 15 }

# -- 1.0.8802.16.999.1.2.5.1.16

# wmanIf2BsCapDownlinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This object shows the number of Downlink transport CIDs the

# SS can support."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsBasicCapabilitiesEntry 16 }

# -- 1.0.8802.16.999.1.2.5.1.19

# wmanIf2BsCapMaxNumBurstToMs OBJECT-TYPE

# SYNTAX WmanIf2CapMaxNumBurstToMsType

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum number of bursts transmitted concurrently to the MS

# , including all bursts without CID or with CIDs matching

# the MS CIDs."

# REFERENCE

# "Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsBasicCapabilitiesEntry 19 }

# -- 1.0.8802.16.999.1.2.5.1.23

# wmanIf2BsCapMaxMacLevelDlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per DL frame. A value of 0 indicates such

# limitation does not exist, except the limitation of

# the physical medium"

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 23 }

# -- 1.0.8802.16.999.1.2.5.1.24

# wmanIf2BsCapMaxMacLevelUlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per UL frame. A value of 0 indicates such

# limitation does not exist, except the limitation of

# the physical medium"

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsBasicCapabilitiesEntry 24 }

# -- 1.0.8802.16.999.1.2.5.1.25

# wmanIf2BsCapNumOfProvisionedSf OBJECT-TYPE

# SYNTAX WmanIf2CapMaxNumOfProvisionedSf

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "When a BS is to transmit multiple DSA transactions for

# provisioned service flows, this object indicates how many

# DSA transactions with provisioned service flows will be

# transmitted."

# REFERENCE

# "Subclause 11.7.19 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsBasicCapabilitiesEntry 25 }

# -- 1.0.8802.16.999.1.2.5.1.29

# wmanIf2BsCapPnWindowSize OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Specifies the size capability of the receiver PN window for

# SAs and management connections. The receiver shall track

# PNs within this window to prevent replay attacks (see

# 7.5.1.2.4)."

# REFERENCE

# "Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsBasicCapabilitiesEntry 29 }

# -- 1.0.8802.16.999.1.2.5.1.30

# wmanIf2BsCapMacHeaderSdnSnPeriod OBJECT-TYPE

# SYNTAX WmanIf2MacHeaderSuppSdnSnPeriod

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Indicates whether or not the MS and BS support various

# types of MAC header and extended subheaders."

# REFERENCE

# "Subclause 11.7.25 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsBasicCapabilitiesEntry 30 }

# -- 1.0.8802.16.999.1.2.5.1.31

# wmanIf2BsSsOfdmCapLoopPwrControlSw OBJECT-TYPE

# SYNTAX Unsigned32

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates he minimum number of frames that

# SS takes to switch from the open loop power control

# scheme to the closed loop power control scheme or

# vice versa."

# ::= { wmanIf2BsBasicCapabilitiesEntry 31 }

# -- 1.0.8802.16.999.1.2.5.1.32

# wmanIf2BsSsOfdmaCapSdmaPilot OBJECT-TYPE

# SYNTAX WmanIf2SdmaPilotCap

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field indicates the SDMA pilot pattern support

# for AMC zone."

# REFERENCE

# "Subclause 11.8.3.7.17 in IEEE 802.16e"

# ::= { wmanIf2BsBasicCapabilitiesEntry 32 }

# -- 1.0.8802.16.999.1.2.5.1.33

# wmanIf2BsOfdmaCapNoUlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of uplink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsBasicCapabilitiesEntry 33 }

# -- 1.0.8802.16.999.1.2.5.1.34

# wmanIf2BsOfdmaCapNoDlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of downlink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsBasicCapabilitiesEntry 34 }

# -- 1.0.8802.16.999.1.2.5.1.35

# wmanIf2BsCapOptionsGeneral OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsGeneral

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded general BS capability options."

# ::= { wmanIf2BsBasicCapabilitiesEntry 35 }

# -- 1.0.8802.16.999.1.2.5.1.36

# wmanIf2BsCapOptionsOfdm OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdm

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDM specific BS capability options."

# ::= { wmanIf2BsBasicCapabilitiesEntry 36 }

# -- 1.0.8802.16.999.1.2.5.1.37

# wmanIf2BsCapOptionsOfdma OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdma

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDMA specific BS capability options."

# ::= { wmanIf2BsBasicCapabilitiesEntry 37 }

# -- 1.0.8802.16.999.1.2.6

# wmanIf2BsCapabilitiesConfigTable OBJECT-TYPE

# SYNTAX SEQUENCE OF WmanIf2BsCapabilitiesConfigEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table contains the configuration for basic

# capabilities of BS. The table is intended to be used to

# restrict the Capabilities implemented by BS, for example in

# order to comply with local regulatory requirements. The BS

# should use the configuration along with the implemented

# Capabilities (wmanIf2BsBasicCapabilitiesTable) for

# negotiation of basic capabilities with SS using RNG-RSP,

# SBC-RSP and REG-RSP messages. The negotiated capabilities

# are obtained by interSubclause of SS reported capabilities,

# BS raw capabilities and BS configured capabilities. The

# objects in the table have read-write access. The rows are

# created by BS as a copy of wmanIf2BsBasicCapabilitiesTable

# and can be modified by NMS."

# ::= { wmanIf2BsCm 6 }

# -- 1.0.8802.16.999.1.2.6.1

# wmanIf2BsCapabilitiesConfigEntry OBJECT-TYPE

# SYNTAX WmanIf2BsCapabilitiesConfigEntry

# MAX-ACCESS not-accessible

# STATUS current

# DESCRIPTION

# "This table provides one row for each BS sector."

# INDEX { ifIndex }

# ::= { wmanIf2BsCapabilitiesConfigTable 1 }

# WmanIf2BsCapabilitiesConfigEntry ::=

# SEQUENCE {

# wmanIf2BsCapCfgUplinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsCapCfgDsxFlowControl

# WmanIf2MaxDsxFlowType,

# wmanIf2BsCapCfgMcaFlowControl

# WmanIf2MaxMcaFlowType,

# wmanIf2BsCapCfgMcpGroupCidSupport

# WmanIf2MaxMcpGroupCid,

# wmanIf2BsCapCfgPkmFlowControl

# WmanIf2MaxPkmFlowType,

# wmanIf2BsCapCfgMaxNumOfSupportedSA

# WmanIf2MaxNumOfSaType,

# wmanIf2BsCapCfgMaxNumOfClassifier

# WmanIf2MaxClassifiers,

# wmanIf2BsCapCfgTtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsCapCfgRtgTransitionGap

# WmanIf2SsTransitionGap,

# wmanIf2BsCapCfgDownlinkCidSupport

# WmanIf2NumOfCid,

# wmanIf2BsCapCfgMaxNumBurstToMs

# INTEGER,

# wmanIf2BsCapCfgMaxMacLevelDlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsCapCfgMaxMacLevelUlFrame

# WmanIf2MaxMacLevel,

# wmanIf2BsCapCfgNumOfProvisionedSf

# Unsigned32,

# wmanIf2BsCapCfgPnWindowSize

# INTEGER,

# wmanIf2BsCapCfgMacHeaderSdnSnPeriod

# WmanIf2MacHeaderSuppSdnSnPeriod,

# wmanIf2BsSsOfdmCapCfgLoopPwrControlSw

# Unsigned32,

# wmanIf2BsSsOfdmaCapCfgSdmaPilot

# WmanIf2SdmaPilotCap,

# wmanIf2BsOfdmaCapCfgNoUlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsOfdmaCapCfgNoDlHarqChannel

# WmanIf2CapOfdmaNoHarqChannel,

# wmanIf2BsCapCfgOptionsGeneral

# WmanIf2SsCapOptionsGeneral,

# wmanIf2BsCapCfgOptionsOfdm

# WmanIf2SsCapOptionsOfdm,

# wmanIf2BsCapCfgOptionsOfdma

# WmanIf2SsCapOptionsOfdma

# }

# -- 1.0.8802.16.999.1.2.6.1.1

# wmanIf2BsCapCfgUplinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object shows the configured number of Uplink transport

# CIDs the BS can support per SS."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsCapabilitiesConfigEntry 1 }

# -- 1.0.8802.16.999.1.2.6.1.3

# wmanIf2BsCapCfgDsxFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxDsxFlowType

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object specifies the configured maximum number of

# concurrent DSA, DSC, or DSD transactions that BS allows

# each SS to have outstanding."

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 3 }

# -- 1.0.8802.16.999.1.2.6.1.4

# wmanIf2BsCapCfgMcaFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxMcaFlowType

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent

# MCA transactions that BS is configured to allow each SS to

# have."

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 4 }

# -- 1.0.8802.16.999.1.2.6.1.5

# wmanIf2BsCapCfgMcpGroupCidSupport OBJECT-TYPE

# SYNTAX WmanIf2MaxMcpGroupCid

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object indicates the maximum number of simultaneous

# Multicast Polling Groups the BS is configured to allow

# each SS to belong to."

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 5 }

# -- 1.0.8802.16.999.1.2.6.1.6

# wmanIf2BsCapCfgPkmFlowControl OBJECT-TYPE

# SYNTAX WmanIf2MaxPkmFlowType

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object specifies the maximum number of concurrent

# PKM transactions that BS is configured to allow each SS

# to have."

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 6 }

# -- 1.0.8802.16.999.1.2.6.1.7

# wmanIf2BsCapCfgMaxNumOfSupportedSA OBJECT-TYPE

# SYNTAX WmanIf2MaxNumOfSaType

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This field specifies configured maximum number of supported

# security association per SS."

# DEFVAL { 1 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 7 }

# -- 1.0.8802.16.999.1.2.6.1.10

# wmanIf2BsCapCfgMaxNumOfClassifier OBJECT-TYPE

# SYNTAX WmanIf2MaxClassifiers

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object indicates the configured maximum number of

# admitted Classifiers per SS that the BS can support."

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 10 }

# -- 1.0.8802.16.999.1.2.6.1.14

# wmanIf2BsCapCfgTtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This field indicates the configured transition speed

# SSTTG for TDD and H-FDD SSs. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsCapabilitiesConfigEntry 14 }

# -- 1.0.8802.16.999.1.2.6.1.15

# wmanIf2BsCapCfgRtgTransitionGap OBJECT-TYPE

# SYNTAX WmanIf2SsTransitionGap

# UNITS "microsecond"

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This field indicates the configured transition speed

# SSRTG for TDD and H-FDD SSs. The usage is defined by

# WmanIf2SsTransitionGap."

# ::= { wmanIf2BsCapabilitiesConfigEntry 15 }

# -- 1.0.8802.16.999.1.2.6.1.16

# wmanIf2BsCapCfgDownlinkCidSupport OBJECT-TYPE

# SYNTAX WmanIf2NumOfCid

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This object shows the number of Downlink transport CIDs

# the SS can support."

# REFERENCE

# "Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsCapabilitiesConfigEntry 16 }

# -- 1.0.8802.16.999.1.2.6.1.19

# wmanIf2BsCapCfgMaxNumBurstToMs OBJECT-TYPE

# SYNTAX INTEGER (1..16)

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "Maximum number of bursts transmitted concurrently to the MS

# , including all bursts without CID or with CIDs matching

# the MS CIDs."

# REFERENCE

# "Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsCapabilitiesConfigEntry 19 }

# -- 1.0.8802.16.999.1.2.6.1.23

# wmanIf2BsCapCfgMaxMacLevelDlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per DL frame. A value of 0 indicates such

# limitation does not exist, except the limitation of

# the physical medium"

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 23 }

# -- 1.0.8802.16.999.1.2.6.1.24

# wmanIf2BsCapCfgMaxMacLevelUlFrame OBJECT-TYPE

# SYNTAX WmanIf2MaxMacLevel

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "Maximum amount of MAC level data the MS is capable of

# processing per UL frame. A value of 0 indicates such

# limitation does not exist, except the limitation of

# the physical medium"

# REFERENCE

# "Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

# DEFVAL { 0 }

# ::= { wmanIf2BsCapabilitiesConfigEntry 24 }

# -- 1.0.8802.16.999.1.2.6.1.25

# wmanIf2BsCapCfgNumOfProvisionedSf OBJECT-TYPE

# SYNTAX Unsigned32 (0..255)

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "When a BS is to transmit multiple DSA transactions for

# provisioned service flows, this object indicates how many

# DSA transactions with provisioned service flows will be

# transmitted."

# REFERENCE

# "Subclause 11.7.19 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsCapabilitiesConfigEntry 25 }

# -- 1.0.8802.16.999.1.2.6.1.29

# wmanIf2BsCapCfgPnWindowSize OBJECT-TYPE

# SYNTAX INTEGER (0..65535)

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Specifies the size capability of the receiver PN window for

# SAs and management connections. The receiver shall track

# PNs within this window to prevent replay attacks (see

# 7.5.1.2.4)."

# REFERENCE

# "Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsCapabilitiesConfigEntry 29 }

# -- 1.0.8802.16.999.1.2.6.1.30

# wmanIf2BsCapCfgMacHeaderSdnSnPeriod OBJECT-TYPE

# SYNTAX WmanIf2MacHeaderSuppSdnSnPeriod

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "Indicates whether or not the MS and BS support various

# types of MAC header and extended subheaders."

# REFERENCE

# "Subclause 11.7.25 in IEEE Std 802.16e-2005"

# ::= { wmanIf2BsCapabilitiesConfigEntry 30 }

# -- 1.0.8802.16.999.1.2.6.1.31

# wmanIf2BsSsOfdmCapCfgLoopPwrControlSw OBJECT-TYPE

# SYNTAX Unsigned32

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This field indicates he minimum number of frames that

# SS takes to switch from the open loop power control

# scheme to the closed loop power control scheme or

# vice versa."

# ::= { wmanIf2BsCapabilitiesConfigEntry 31 }

# -- 1.0.8802.16.999.1.2.6.1.32

# wmanIf2BsSsOfdmaCapCfgSdmaPilot OBJECT-TYPE

# SYNTAX WmanIf2SdmaPilotCap

# MAX-ACCESS read-write

# STATUS current

# DESCRIPTION

# "This field indicates the SDMA pilot pattern support

# for AMC zone."

# REFERENCE

# "Subclause 11.8.3.7.17 in IEEE 802.16e"

# ::= { wmanIf2BsCapabilitiesConfigEntry 32 }

# -- 1.0.8802.16.999.1.2.6.1.33

# wmanIf2BsOfdmaCapCfgNoUlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of uplink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsCapabilitiesConfigEntry 33 }

# -- 1.0.8802.16.999.1.2.6.1.34

# wmanIf2BsOfdmaCapCfgNoDlHarqChannel OBJECT-TYPE

# SYNTAX WmanIf2CapOfdmaNoHarqChannel

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "This field specifies the number of downlink H-ARQ

# channels (n) the SS supports, where n = 1..16.

# The value of this object should be 0..15."

# ::= { wmanIf2BsCapabilitiesConfigEntry 34 }

# -- 1.0.8802.16.999.1.2.6.1.35

# wmanIf2BsCapCfgOptionsGeneral OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsGeneral

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded general capability options cofigured on BS.

# These object typically restricts availability of BS supported features."

# ::= { wmanIf2BsCapabilitiesConfigEntry 35 }

# -- 1.0.8802.16.999.1.2.6.1.36

# wmanIf2BsCapCfgOptionsOfdm OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdm

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDM specific capability options cofigured on BS.

# These object typically restricts availability of BS supported OFDM specific features."

# ::= { wmanIf2BsCapabilitiesConfigEntry 36 }

# -- 1.0.8802.16.999.1.2.6.1.37

# wmanIf2BsCapCfgOptionsOfdma OBJECT-TYPE

# SYNTAX WmanIf2SsCapOptionsOfdma

# MAX-ACCESS read-only

# STATUS current

# DESCRIPTION

# "Combined BITS encoded OFDMA specific capability options cofigured on BS.

# These object typically restricts availability of BS supported OFDMA specific features."

# ::= { wmanIf2BsCapabilitiesConfigEntry 37 }