| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup and rationalizing of the capabilities management in ASN MIBs. |  |
| Date Submitted | 2008-01-23 |  |
| Source(s) | Krzysztof Dudzinski Airspan Joey Chou Intel | Voice: E-mail: kdudzins@airspan.com E-mail: joey.chou@intel.com |
| Re: | P802.16Rev2/D2 (LB26a) |  |
| Abstract | MIBs defined in the 802.16i drafts contain huge number of capabilities objects spread over large number of tables. This contribution proposes a way of rationalizing of the encoding of the important capabilities. It simplifies the model and improves the management protocol performance. |  |
| Purpose | Approve the contribution and apply the changes to subclause 13.2.3 of the standard document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Cleanup and rationalizing of the capabilities management in ASN MIBs.

Krzysztof Dudzinski

Airspan Communication Ltd.

# Overview

The MIBs define multiple sets of capabilities management tables. They are spread over multiple MIB modules (WMAN-IF2-BS-MIB, WMAN-IF2M-BS-MIB) and defined in the different tables based on their content e.g. OFDMA capabilities in OfdmPhy branch of the tree. However the capabilities are not of the SS running OFDMA or MS. The capabilities are to tell us if SS is capable of OFDMA or capable of mobility etc. BS doesn't have to grant a given capability. As such it is better to group all the capabilities in one simple set of 4 tables rather than in 3 or 4 sets of 4 tables each. This makes the MIB much simpler too.

Many of the capabilities are encoded as BITs. Again in the MIB there is a separate set of 4 objects (in 4 tables) for each such capability and additionally the textual convention is defined for each one of them. This is extremely inefficient and leads to heavy overhead in management protocols forced to get all objects independently to retrieve the complete capabilities of the SS. It is much better to group all binary encoded capabilities in a single set of 4 objects, each defining the full set of binary options of capabilities. The gain in protocol performance may lead to 1 pdu to get complete capabilities map for all SSs on the BS instead of multiple tens of pdus. The simplification of the MIB is substantial.

The proposed remedy defines 3 new textual conventions WmanIf2SsCapOptionsGeneral,

WmanIf2SsCapOptionsOfdm,

WmanIf2SsCapOptionsOfdma.

And a set of 12 objects defining complete capabilities of SS.

They all group large number of capabilities and replace about 140 objects and 8 tables!

# Editorial Instructions.

# General capabilities

Tables used for general capabilities management are:

wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapabilitiesConfigTable

## 1.1 Binary encoded objects (BITS)

Implement the content of the following BITS based textual conventions as a part of a single combined textual convention WmanIf2SsCapOptionsGeneral .

Pay attention to notes in brackets.

Delete all the objects using the textual conventions listed. Then delete the textual conventions.

The list of textual conventions to delete:

- WmanIf2ArqSupportType

- WmanIf2IpVersionType

- WmanIf2MacCsBitMap

- WmanIf2PhsSupportType

- WmanIf2BwAllocSupport

- WmanIf2PduConstruction

- WmanIf2PackingSupport (ref. 11.7.8.11 in 16e ->11.7.8.7 in revision)

- WmanIf2ExtRtpsSupport (ref. 11.7.8.12 in 16e ->11.7.8.8 in revision)

- WmanIf2IpAllocMethod

- WmanIf2ArqAckType

- WmanIf2MacHeaderSupp

- WmanIf2PkmVersion

- WmanIf2AuthPolicy

- WmanIf2MacMode

- WmanIf2ExtCapability (bits 1,2 added, were in WmanIf2MacHeaderSupp and all bits moved to options)

- WmanIf2MacHeaderSuppSdnSnPeriod

Create new capability objects defined using newly created combined textual convention WmanIf2SsCapOptionsGeneral.

The list of new capability objects:

wmanIf2BsSsReqCapOptionsGeneral in wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapOptionsGeneral in wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsCapOptionsGeneral in wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapCfgOptionsGeneral in wmanIf2BsCapabilitiesConfigTable

## 1.2 Numerical objects (Integer, Unsigned integer)

Keep in use the following numerical capabilities textual conventions.

Apply the correction changes defined in brackets where appropriate.

- WmanIf2NumOfCid

- WmanIf2MaxDsxFlowType

- WmanIf2MaxMcpGroupCid

- WmanIf2MaxPkmFlowType

- WmanIf2MaxNumOfSaType

- WmanIf2MaxClassifiers

- WmanIf2SsTransitionGap

- xxxCapMaxNumBurstToMs (create textual convention for these objects)

- WmanIf2MaxMacLevel for both uplink and downlink objects

- xxxCapNumOfProvisionedSf (create textual convention for these objects)

- xxxCapPnWindowSize

## 1.3 Obsolete objects

Delete the following textual conventions and/or related objects as they are removed from the standard as a result of the amendments 802.16e, Cor2 and Rev2.

- WmanIf2MacCrcSupport (OK, already deleted in D6)

- WmanIf2AuthPolicyType (OK, already deleted in D6)

# 2. OFDM capabilities

Tables used for OFDM capabilities management are:

wmanIf2BsSsOfdmReqCapabilitiesTable,

wmanIf2BsSsOfdmRspCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesConfigTable

## 2.1 Binary encoded objects (BITS)

Implement the content of the following BITS based textual conventions as a part of a single combined textual convention WmanIf2SsCapOptionsOfdm.

Pay attention to notes in brackets.

Delete all the objects using the textual conventions listed. Then delete the textual conventions.

The list of textual conventions to delete:

- WmanIf2OfdmFftSizes (OK)

- WmanIf2OfdmSsDeModType (OK)

- WmanIf2OfdmSsModType (OK)

- WmanIf2OfdmTcSublayer (OK)

- WmanIf2OfdmPrivMap (OK)

- WmanIf2OfdmUlPower (OK)

Create new capability objects defined using newly created combined textual convention WmanIf2SsCapOptionsOfdm. The new objects are created in the general capabilities tables to allow subsequent deletion of OFDM capabilities tables.

The list of new capability objects to create:

wmanIf2BsSsReqCapOptionsOfdm in wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapOptionsOfdm in wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsCapOptionsOfdm in wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapCfgOptionsOfdm in wmanIf2BsCapabilitiesConfigTable

## 2.2 Numerical objects (Integer, Unsigned integer)

Keep in use the following numerical capabilities textual conventions and/or objects.

Apply the correction changes defined in brackets where appropriate.

Move the objects listed or objects using listed textual conventions to corresponding general capabilities tables to allow subsequent deletion of OFDM capabilities tables.

- xxxCapLoopPwrControlSw (add and use textual convention WmanIf2CapMinNumFramesForPwrCtrlSwitch)

## 2.3 Obsolete objects

Delete the following textual conventions and/or related objects as they are removed from the standard as a result of the amendments 802.16e, Cor2 and Rev2.

- WmanIf2OfdmFocusedCt

## 2.4 OFDM capabilities tables

Finally delete the empty OFDM capabilities tables:

wmanIf2BsSsOfdmReqCapabilitiesTable,

wmanIf2BsSsOfdmRspCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesTable,

wmanIf2BsOfdmCapabilitiesConfigTable

# 3. OFDMA capabilities

Tables used for OFDMA capabilities management are:

wmanIf2BsSsOfdmaReqCapabilitiesTable,

wmanIf2BsSsOfdmaRspCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesConfigTable

## 3.1 Binary encoded objects (BITS)

Implement the content of the following BITS based textual conventions as a part of a single combined textual convention WmanIf2SsCapOptionsOfdma .

Pay attention to notes in brackets.

Delete all the objects using the textual conventions listed. Then delete the textual conventions.

The list of textual conventions to delete:

- WmanIf2OfdmaFftSizes (fft256 removed in Cor2 and bit made reserved)

- WmanIf2OfdmaMsDeModType

- WmanIf2OfdmaMsModType

- WmanIf2OfdmaPermutation

- WmanIf2OfdmaMimoCap (corrected bits as per Rev2)

- WmanIf2OfdmaPrivMap

- WmanIf2OfdmaAasCap

- WmanIf2OfdmaCinrCap

- WmanIf2OfdmaUlPower

- WmanIf2OfdmaMapCap

- WmanIf2OfdmaUlCntlCh

- WmanIf2OfdmaMsCistCap

- WmanIf2OfdmaMaxHarq

- WmanIf2OfdmaModMimo

- WmanIf2MultiBurst

- WmanIf2IncrHarqBuf

- WmanIf2ChaseHarqBuf

Create new capability objects defined using newly created combined textual convention WmanIf2SsCapOptionsOfdma. The new objects are created in the general capabilities tables to allow subsequent deletion of OFDMA capabilities tables.

The list of new capability objects to create:

wmanIf2BsSsReqCapOptionsOfdma in wmanIf2BsSsReqCapabilitiesTable,

wmanIf2BsSsRspCapOptionsOfdma in wmanIf2BsSsRspCapabilitiesTable,

wmanIf2BsCapOptionsOfdma in wmanIf2BsBasicCapabilitiesTable,

wmanIf2BsCapCfgOptionsOfdma in wmanIf2BsCapabilitiesConfigTable

## 3.2 Numerical objects (Integer, Unsigned integer)

Keep in use the following numerical capabilities textual conventions and/or objects.

Apply the correction changes defined in brackets where appropriate.

Move the objects listed or objects using listed textual conventions to corresponding general capabilities tables to allow subsequent deletion of OFDMA capabilities tables.

- wmanIf2BsSsOfdmaReqCapNoUlHarqChannel (created textual convention WmanIf2OfdmaNoHarqChannelCapType and renamed from wmanIf2BsSsOfdmaReqCapNoDlHarqChannel)

- wmanIf2BsSsOfdmaReqCapNoDlHarqChannel (new object as per 11.8.3.7.2)

- WmanIf2SdmaPilotCap

## 3.3 Obsolete objects

Delete the following textual conventions and/or related objects as they are removed from the standard as a result of the amendments 802.16e, Cor2 and Rev2 or for other reason as stated in the notes in brackets.

- WmanIf2OfdmaDemMimo (duplicates WmanIf2OfdmaMimoCap)

- WmanIf2OfdmaUlMimo (deleted in Cor2)

## 3.4 OFDMA capabilities tables cleanup

Finally delete the empty OFDMA capabilities tables:

wmanIf2BsSsOfdmaReqCapabilitiesTable,

wmanIf2BsSsOfdmaRspCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesTable,

wmanIf2BsOfdmaCapabilitiesConfigTable

# ASN1 code with complete solution for WMAN-IF2-BS-MIB

--

-- Textual conventions

--

WmanIf2CidType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Type of CID."

SYNTAX INTEGER (0..65535)

WmanIf2MaxTxPowerType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This type is used to define maximum available power for

BPSK, QPSK, 16-QAM and 64-QAM constellations. The maximum

power parameters are reported in dBm and quantized in 0.5

dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

(encoded 0xFF). Values outside this range shall be

assigned the closest extreme. SSs that do not support

QAM64 shall report the value of 0x00 in the maximum QAM64

power field."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

SYNTAX INTEGER (0..255)

WmanIf2MacVersion ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Version number of IEEE 802.16."

SYNTAX INTEGER

{

ieee802Dot16Of2001(1),

ieee802Dot16cOf2002(2),

ieee802Dot16aOf2003(3),

ieee802Dot16Of2004(4),

ieee802Dot16e(5),

tbd(6)

}

WmanIf2NumOfCid ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The object of this type shows the number of CIDs that

SS can support."

REFERENCE

"Subclause 11.7.6 in IEEE Std 802.16e-2005"

SYNTAX INTEGER (1..65535)

WmanIf2MaxDsxFlowType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The object of this type specifies the maximum number of

concurrent DSA, DSC, or DSD transactions that may be

outstanding."

REFERENCE

"Subclause 11.7.8.2 in IEEE Std 802.16-2004"

SYNTAX INTEGER (0..255)

WmanIf2MaxMcaFlowType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The object of this type specifies the maximum number of

concurrent MCA transactions that may be outstanding."

REFERENCE

"Subclause 11.7.8.4 in IEEE Std 802.16-2004"

SYNTAX INTEGER (0..255)

WmanIf2MaxMcpGroupCid ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The object of this type indicates the maximum number of

simultaneous Multicast Polling Groups the SS is

capable of belonging to."

REFERENCE

"Subclause 11.7.8.5 in IEEE Std 802.16-2004"

SYNTAX INTEGER (0..255)

WmanIf2MaxPkmFlowType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The object of this type specifies the maximum number of

concurrent PKM transactions that may be outstanding."

REFERENCE

"Subclause 11.8.4.5 in IEEE Std 802.16e-2005"

SYNTAX INTEGER (0..255)

WmanIf2MaxNumOfSaType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This field specifies maximum number of supported

security association of the SS."

REFERENCE

"Subclause 11.8.4.6 in IEEE Std 802.16e-2005"

SYNTAX INTEGER (0..255)

WmanIf2MaxClassifiers ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The object of this type indicates the maximum number of

admitted Classifiers that the SS is allowed to have."

REFERENCE

"Subclause 11.7.7.2 in IEEE Std 802.16-2004"

SYNTAX INTEGER (0..65535)

WmanIf2SsTransitionGap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This field indicates the transition speed SSTTG and SSRTG

for TDD and H-FDD SSs. Allowed values are:

OFDM mode: TDD and H-FDD 0..100

Other modes: TDD: 0..50; H-FDD: 0..100"

REFERENCE

"Subclause 11.8.3.1 in IEEE Std 802.16-2004"

SYNTAX INTEGER (0..100)

WmanIf2MaxMacLevel ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"maximum amount of MAC level data including MAC headers and

HARQ retransmission bursts the MS is capable of processing

in the DL/UL part of a single MAC frame."

REFERENCE

"Subclause 11.7.8.10 in IEEE Std 802.16e-2005"

SYNTAX INTEGER (0..65535)

WmanIf2ChannelNumber ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Physical channel number"

SYNTAX INTEGER (0..199)

WmanIf2BsIdType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Defines the encoding of BSID. The BSID is a 6 byte number

and follows the encoding rules of MacAddress textual

convention, i.e. as if it were transmitted

least-significant bit first. The value should be displayed

with 2 parts clearly separated by a colon e.g:

001DFF:00003A. The most significant part is representing

the Operator ID. "

SYNTAX OCTET STRING (SIZE (6))

WmanIf2OfdmFftSize ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"FFT size for OFDM PHY

0b000: 2048

0b011: 256"

REFERENCE

"Subclause 11.8.3.6.1, IEEE Std 802.16-2004"

SYNTAX INTEGER

{

fft2048(0),

fft256(3)

}

WmanIf2OfdmFecCodeType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"FEC code type and modulation type"

REFERENCE

"Table 356 and Table 362 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

bpskCc1Over2(0),

qpskRsCcCc1Over2(1),

qpskRsCcCc3Over4(2),

sixteenQamRsCcCc1Over2(3),

sixteenQamRsCcCc3Over4(4),

sixtyFourQamRsCcCc2Over3(5),

sixtyFourQamRsCcCc3Over4(6),

qpskBtc1Over2(7),

qpskBtc3Over4(8),

sixteenQamBtc3Over4(9),

sixteenQamBtc4Over5(10),

sixtyFourQamBtc2Over3(11),

sixtyFourQamBtc5Over6(12),

qpskCtc1Over2(13),

qpskCtc2Over3(14),

qpskCtc3Over4(15),

sixteenQamCtc1Over2(16),

sixteenQamCtc3Over4(17),

sixtyFourQamCtc2Over3(18),

sixtyFourQamCtc3Over4(19)

}

WmanIf2HarqAckDelay ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"HARQ ACK delay for UL and DL bursts

1 = one frame offset

2 = two frames offset

3 = three frames offset"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

oneframeoffset(1),

twoframesoffset(2),

threeframesoffset(3)

}

WmanIf2AasBeamSel ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Boolean to indicate whether unsolicited AAS Beam Select

messages (see 6.3.2.3.41 in IEEE 802.16e-2005) should be

sent by the MS.

0: MS should not send AAS Beam Select Messages

1: MS may send AAS Beam Select Messages"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

notAllowed(0),

allowed(1)

}

WmanIf2OfdmaFftSize ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"FFT size for OFDMA PHY

0b000 = 2048

0b001 = 1024

0b010 = 512

0b100 = 128"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

fft2048(0),

fft1024(1),

fft512(2),

reserved(3),

fft128(4)

}

WmanIfPermutationType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Permutation type for broadcast region in HARQ zone"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

pusc(1),

fusc(2),

optionalFusc(3),

amc(4)

}

WmanIf2HoSupportType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The types of handover supported."

REFERENCE

"Table 358 in IEEE Std 802.16e-2005"

SYNTAX BITS

{

handover(0),

mdHandover(1),

fbssHandover(2)

}

WmanIf2OfdmaUcdFecCode ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"UCD FEC code type and modulation type"

REFERENCE

"Table 357 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

qpskCc1Over2(0),

qpskCc3Over4(1),

sixteenQamCc1Over2(2),

sixteenQamCc3Over4(3),

sixtyFourQamCc1Over2(4),

sixtyFourQamCc2Over3(5),

sixtyFourQamCc3Over4(6),

qpskBtc1Over2(7),

qpskBtc3Over4(8),

sixteenQamBtc3Over5(9),

sixteenQamBtc4Over5(10),

sixtyFourQamBtc5Over8(11),

sixtyFourQamBtc4Over5(12),

qpskCtc1Over2(13),

reserved(14),

qpskCtc3Over4(15),

sixteenQamCtc1Over2(16),

sixteenQamCtc3Over4(17),

sixtyFourQamCtc1Over2(18),

sixtyFourQamCtc2Over3(19),

sixtyFourQamCtc3Over4(20),

sixtyFourQamCtc5Over6(21),

qpskZtCc1Over2(22),

qpskZtCc3Over4(23),

sixteenQamZtCc1Over2(24),

sixteenQamZtCc3Over4(25),

sixtyFourQamZtCc1Over2(26),

sixtyFourQamZtCc2Over3(27),

sixtyFourQamZtCc3Over4(28),

qpskLdpc1over2(29),

qpskLdpc2over3A(30),

qpskLdpc3over4A(31),

sixteenQamLdpc1over2(32),

sixteenQamLdpc2over3A(33),

sixteenQamLdpc3over4A(34),

sixtyFourQamLdpc1over2(35),

sixtyFourQamLdpc2over3A(36),

sixtyFourQamLdpc3over4A(37),

qpskLdpc2over3B(38),

qpskLdpc3over4B(39),

sixteenQamLdpc2over3B(40),

sixteenQamLdpc3over4B(41),

sixtyFourQamLdpc2over3B(42),

sixtyFourQamLdpc3over4B(43),

qpskCcOptIntv1over2(44),

qpskCcOptIntv3over4(45),

sixteenQamCcOptIntv1over2(46),

sixteenQamCcOptIntv3over4(47),

sixtyFourQamCcOptIntv2over3(48),

sixtyFourQamCcOptIntv3over4(49),

qpskLdpc5over6(50),

sixteenQamLdpc5over6(51),

sixtyFourQamLdpc5over6(52)

}

WmanIf2OfdmaDcdFecCode ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"DCD FEC code type and modulation type"

REFERENCE

"Table 363 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

qpskCc1Over2(0),

qpskCc3Over4(1),

sixteenQamCc1Over2(2),

sixteenQamCc3Over4(3),

sixtyFourQamCc1Over2(4),

sixtyFourQamCc2Over3(5),

sixtyFourQamCc3Over4(6),

qpskBtc1Over2(7),

qpskBtc3Over4Or2Over3(8),

sixteenQamBtc3Over5(9),

sixteenQamBtc4Over5(10),

sixtyFourQamBtc2Over3Or5Over8(11),

sixtyFourQamBtc5Over6Or4Over5(12),

qpskCtc1Over2(13),

reserved14(14),

qpskCtc3Over4(15),

sixteenQamCtc1Over2(16),

sixteenQamCtc3Over4(17),

sixtyFourQamCtc1Over2(18),

sixtyFourQamCtc2Over3(19),

sixtyFourQamCtc3Over4(20),

sixtyFourQamCtc5Over6(21),

qpskZtCc1Over2(22),

qpskZtCc3Over4(23),

sixteenQamZtCc1Over2(24),

sixteenQamZtCc3Over4(25),

sixtyFourQamZtCc1Over2(26),

sixtyFourQamZtCc2Over3(27),

sixtyFourQamZtCc3Over4(28),

qpskLdpc1over2(29),

qpskLdpc2over3A(30),

qpskLdpc3over4A(31),

sixteenQamLdpc1over2(32),

sixteenQamLdpc2over3A(33),

sixteenQamLdpc3over4A(34),

sixtyFourQamLdpc1over2(35),

sixtyFourQamLdpc2over3A(36),

sixtyFourQamLdpc3over4A(37),

qpskLdpc2over3B(38),

qpskLdpc3over4B(39),

sixteenQamLdpc2over3B(40),

sixteenQamLdpc3over4B(41),

sixtyFourQamLdpc2over3B(42),

sixtyFourQamLdpc3over4B(43),

qpskCcOptIntv1over2(44),

qpskCcOptIntv3over4(45),

sixteenQamCcOptIntv1over2(46),

sixteenQamCcOptIntv3over4(47),

sixtyFourQamCcOptIntv2over3(48),

sixtyFourQamCcOptIntv3over4(49)

}

WmanIf2SdmaPilotCap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This field indicates SDMA pilot pattern support for AMC

zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

noSupport(0),

sdmaPilotAandB(1),

allSdmaPilotPatterns(2)

}

WmanIf2GlobalSrvClass ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Global Service Class Name contains 8 information fields

that map to predefined QoS attributes as shown in

subclause 6.3.14.4.1.

bit#0:     Uplink/Downlink indicator

0 - uplink

1 - downlink

bit#1-6:   Maximum sustained traffic rate in bps that is

defined in Table 124b

bit#7:     0 - no traffic indication

1 - traffic indication

bit#8-13:  Maximum traffic burst defines the maximum burst

size that must be accommodated for the service.

bit#14-19: Minimum reserved traffic rate parameter

specifies the minimum rate, in bits per second,

reserved for this service flow.

bit#20-25: Maximum latency specifies the maximum interval

between the reception of a packet at CS of BS

or SS and the arrival of the packet to the peer

device.

bit#26:    SDU indicator specifies whether the SDUs on the

service flow are fixed-length or variable-length.

0 - variable length

1 - fixed length

bit#27:    Paging indicator of an MS preference for the

reception of paging advisory messages during

idle mode. When set, it indicates that the BS

may present paging advisory messages or other

indicative messages to the MS when data SDUs

bound for the MS are present while the MS is in

Idle Mode.

0 - no paging generation

1 - paging generation"

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

SYNTAX BITS

{

ulDlIndicator(0),

maxSustainedRate0(1),

maxSustainedRate1(2),

maxSustainedRate2(3),

maxSustainedRate3(4),

maxSustainedRate4(5),

maxSustainedRate5(6),

trafficIndication(7),

maxTrafficBurst0(8),

maxTrafficBurst1(9),

maxTrafficBurst2(10),

maxTrafficBurst3(11),

maxTrafficBurst4(12),

maxTrafficBurst5(13),

minReservedRate0(14),

minReservedRate1(15),

minReservedRate2(16),

minReservedRate3(17),

minReservedRate4(18),

minReservedRate5(19),

maxLatency0(20),

maxLatency1(21),

maxLatency2(22),

maxLatency3(23),

maxLatency4(24),

maxLatency5(25),

sduIndicator(26),

pagingGeneration(27),

reserved0(28),

reserved1(29),

reserved2(30),

reserved3(31)

}

WmanIf2LinkDirection ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The direction of a connection."

SYNTAX INTEGER

{

downstream(1),

upstream(2)

}

WmanIf2DataEncryptAlgId ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Data encryption algorithm identifiers."

REFERENCE

"Table 375 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

none(0),

des56BitCbcMode(1),

aes128BitCcmMode(2),

aes128BitCbcMode(3),

aes128BitCtrMode(128)

}

WmanIf2DataAuthAlgId ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Data authentication algorithm identifiers."

REFERENCE

"Table 376 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

noDataAuthentication(0),

aes128BitCcmMode(1)

}

WmanIf2TekEncryptAlgId ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"TEK encryption algorithm identifiers."

REFERENCE

"Table 377 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

tripleDes128BitKey(1),

rsa1024BitKey(2),

aes128BitKeyEcbMode(3),

aes128BitKeyWrap(4)

}

WmanIf2PkmErrorCode ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This error code provides further information about an

Authorization Reject, Key Reject, Authorization Invalid,

or TEK Invalid.

0 - no failure

1 - unauthorized SS

2 - unauthorized SAID

3 - unsolicited

4 - invalid key sequence

5 - key request authentication failure

The following are error code for permanent authorization

failure that indicates any reattempts at authorization

would continue to result in Authorization Rejects.

6 - the BS does not have the CA certificate belonging

to the issuer of an SS certificate

7 - SS certificate has an invalid signature

8 - ASN.1 parsing failure during verification of SS

certificate

9 - SS certificate is on the 'hot list'

10 - inconsistencies between certificate data and data

in accompanying PKM attributes

11 - SS and BS have incompatible security capabilities"

REFERENCE

"Subclause 11.9.10 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

noFailure(0),

unauthorizedSs(1),

unauthorizedSaid(2),

unsolicited(3),

invalidKeySequence(4),

keyReqAuthFailure(5),

umknownManufactur(6),

invalidSignature(7),

asn1ParsingFailure(8),

ssCaOnHotList(9),

dataInconsistency(10),

ssBsIncompatibleSc(11)

}

WmanIf2CertificateStat ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The reason why a SS's certificate is deemed valid

or invalid:

0 - return unknown if the SS is running PKM mode

1 - means the certificate is valid because it chains

to a valid certificate

2 - means the certificate is valid because it has been

provisioned to be trusted

3 - means the certificate is invalid because it has been

provisioned to be untrusted.

4 - means the certificate is invalid because it chains

to an untrusted certificate.

5 - refer to errors in parsing, validity periods, etc,

of SS certificate

6 - refer to errors in parsing, validity periods, etc,

of CA certificate"

REFERENCE

"Subclause 7.2.5.1 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

unknown(0),

validSsChained(1),

validSsTrusted(2),

invalidSsUntrusted(3),

invalidCAUntrusted(4),

invalidSsOther(5),

invalidCAOther(6)

}

WmanIf2SaType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The type of Security Association (SA)."

REFERENCE

"Table 379 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

primarySa(0),

staticSa(1),

dynamicSa(2)

}

WmanIf2SaServiceType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This attribute indicates service types of the corresponding

SA type. This attribute shall be defined only when the SA

type is Static SA or Dynamic SA. The GTEK shall be used to

encrypt connection for group multicast service. The MTK

shall be used to encrypt connection for MBS service."

REFERENCE

"Subclause 11.9.35 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

unicastService(0),

groupMulticastService(1),

mbsService(2)

}

WmanIf2SfSchedulingType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The scheduling service provided by a SC for an upstream

service flow. If the parameter is omitted from an upstream

QOS Parameter Set, this object takes the value of

bestEffort (2). This parameter must be reported as

undefined (1) for downstream QOS Parameter Sets."

SYNTAX INTEGER

{

undefined(1),

bestEffort(2),

nonRealTimePollingService(3),

realTimePollingService(4),

extRealTimePollingService(5),

unsolicitedGrantService(6)

}

WmanIf2PhsRuleVerify ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression. If PHSV is

enabled, the sender shall compare the bytes in the packet

header with the bytes in the PHSF that are to be

suppressed as indicated by the PHSM."

REFERENCE

"Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

phsVerifyEnable(0),

phsVerifyDisable(1)

}

WmanIf2ClassifierBitMap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16e-2005"

SYNTAX BITS

{

priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12),

actionRule(13)

}

WmanIf2SfState ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Defines the state of a service flow."

SYNTAX INTEGER

{

authorized(1),

admitted(2),

active(3)

}

WmanIf2ServClassName ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Defines the type of service class name."

SYNTAX OCTET STRING (SIZE (2..128))

WmanIf2CsSpecification ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Defines the types of convergence sublayer."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

SYNTAX INTEGER

{

reserved(0),

packetIpV4(1),

packetIpV6(2),

packet802dot3Ethernet(3),

packet802dot1QVlan(4),

packetIpV4Over802dot3(5),

packetIpV6Over802dot3(6),

packetIpV4Over802dot1Q(7),

packetIpV6Over802dot1Q(8),

atm(9),

packet802dot3EthernetRohcHc(10),

packet802dot3EthernetEcrtpHc(11),

packetIp2RohcHc(12),

packetIp2EcrtpHc(13)

}

WmanIf2TekState ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"TEK State."

REFERENCE

"Subclause 7.2.5.1 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

start(1),

opWait(2),

opReauthWait(3),

operational(4),

rekeyWait(5),

rekeyReauthWait(6)

}

WmanIf2OfdmaFecCodeType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"FEC code type and modulation type"

REFERENCE

"Table 356 and Table 362 in IEEE Std 802.16-2004"

SYNTAX INTEGER

{

qpskCc1Over2(0),

qpskCc3Over4(1),

sixteenQamCc1Over2(2),

sixteenQamCc3Over4(3),

sixtyFourQamCc2Over3(4),

sixtyFourQamCc3Over4(5),

qpskBtc1Over2(6),

qpskBtc2Over3(7),

sixteenQamBtc3Over5(8),

sixteenQamBtc4Over5(9),

sixtyFourQamBtc5Over8(10),

sixtyFourQamBtc4Over5(11),

qpskCtc1Over2(12),

qpskCtc2Over3(13),

qpskCtc3Over4(14),

sixteenQamCtc1Over2(15),

sixteenQamCtc3Over4(16),

sixtyFourQamCtc2Over3(17),

sixtyFourQamCtc3Over4(18),

sixtyFourQamCtc5Over6(19),

qpskZtCc1Over2(20),

qpskZtCc3Over4(21),

sixteenQamZtCc1Over2(22),

sixteenQamZtCc3Over4(23),

sixtyFourQamZtCc2Over3(24),

sixtyFourQamZtCc3Over4(25)

}

WmanIf2Ipv6FlowLabel ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field. As the flow label field has a

length of 20 bits, the first 4 bits of the most significant

byte shall be set to 0x0 and disregarded."

SYNTAX OCTET STRING (SIZE (3))

WmanIf2ActionRule ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule

Bit #0: 0 = none.

1 = Discard packet"

REFERENCE

"Subclause 11.13.19.3.4.17 in IEEE Std 802.16e-2005"

SYNTAX BITS { discardPacket(0) }

WmanIf2BsSsCapType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"xxxReq: Indicates that SS reports its capabilities in

RNG-REQ, SBC-REQ, or REG-REQ messages.

xxxRsp: Indicates that BS confirms SS capabilities in

RNG-RSP, SBC-RSP, or REG-RSP messages."

SYNTAX INTEGER

{

xxxReq(1),

xxxRsp(2)

}

WmanIf2BsCapType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"inheritedBsCap: Indicates BS' basic HW and SW capabilities.

configedBsCap: Indicates BS' capabilities that has been

configured."

SYNTAX INTEGER

{

inheritedBsCap(1),

configedBsCap(2)

}

WmanIf2CapMaxNumBurstTx ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS,

including all bursts without CID or with CIDs matching the MS's CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

SYNTAX Integer32 (1..16)

WmanIf2CapMaxNumProvSf ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

SYNTAX Integer32 (0..255)

WmanIf2CapMinNumFrmsPwrCtrl ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Defines the minimum number of frames that SS takes to switch

from the open loop power control scheme to the closed loop

power control scheme or vice versa."

REFERENCE

"Subclause 11.8.3.6.7 in IEEE Std 802.16-2008"

SYNTAX Integer32 (0..255)

WmanIf2CapOfdmaNoHarqChan ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"Specifies the number of H-ARQchannels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

REFERENCE

"Subclause 11.8.3.7.2 (demodulator) and 11.8.3.7.3 (modulator) in IEEE 802.16-2008."

SYNTAX Integer32 (0..15)

WmanIf2SsCapOptionsGeneral ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This type combines all the general SS capabilities (excluuding Phy and mobility specific)

with the binary encodded fields.

It reserves the space equivalent to the size of corresponding TLV

and within this space it maps directly to the encoding specified in TLVs including

all defined reserved bits."

REFERENCE

"IEEE Std 802.16-2008 subclauses:

bit 0:       11.7.8.1 (ARQ support)

1-7:     reserved

bit 8-9:     11.7.4 (IP version)

10-15:   reserved

bit 16-29:   11.7.7.1 (CS support)

30-47:   reserved

bit 48-49:   11.7.7.3 (PHS support)

50-55:   reserved

bit 56:      reserved

57:      11.8.1 (BW allocation - duplex support)

58-63:   reserved

bit 64-65:   11.8.2 (PDU construction and transmission)

66-71:   reserved

bit 72:      11.7.8.7 (Packing)

73-79:   reserved

bit 80:      11.7.8.8 (Extended rtPS)

81-87:   reserved

bit 88-91:   11.7.11 (SMC IP address allocation method)

92-95:   reserved

bit 96-100:  11.7.23 (ARQ ACK type)

101-103: reserved

bit 104-120: 11.7.25 (Mac header support)

121-127: reserved

bit 128-129: 11.8.4.1 (Pkm version)

130-135: reserved

bit 136-141: 11.8.4.2 (Authorization policy support)

142-143: reserved

bit 144:     11.8.4.3 (Message Authentication Code mode - HMAC)

145:     reserved

146-149: 11.8.4.3 (Message Authentication Code mode - HMAC follow up)

150-151: reserved

bit 152-154: 11.8.6 (Extended subheader support)

155-159: reserved"

SYNTAX BITS

{

arqSupport(0),

reserved1(1),

reserved2(2),

reserved3(3),

reserved4(4),

reserved5(5),

reserved6(6),

reserved7(7),

ipv4Support(8),

ipv6Support(9),

reserved10(10),

reserved11(11),

reserved12(12),

reserved13(13),

reserved14(14),

reserved15(15),

csSupportAtm(16),

csSupportIpv4(17),

csSupportIpv6(18),

csSupport802Dot3(19),

csSupport802Dot1Q(20),

csSupportIpv4Over802Dot3(21),

csSupportIpv6Over802Dot3(22),

csSupportIpv4Over802Dot1Q(23),

csSupportIpv6Over802Dot1Q(24),

csSupport802Dot3RohcHc(25),

csSupport802Dot3EcrtpHc(26),

csSupportIpv4Over6RohcHc(27),

csSupportIpv4Over6EcrtpHc(28),

csSupportGpcs(29),

reserved30(30),

reserved31(31),

reserved32(32),

reserved33(33),

reserved34(34),

reserved35(35),

reserved36(36),

reserved37(37),

reserved38(38),

reserved39(39),

reserved40(40),

reserved41(41),

reserved42(42),

reserved43(43),

reserved44(44),

reserved45(45),

reserved46(46),

reserved47(47),

phsSupportAtm(48),

phsSupportPacket(49),

reserved50(50),

reserved51(51),

reserved52(52),

reserved53(53),

reserved54(54),

reserved55(55),

reserved56(56),

bwAllocSupportFullDuplex(57),

reserved58(58),

reserved59(59),

reserved60(60),

reserved61(61),

reserved62(62),

reserved63(63),

pduConstructionpiggybackedRequests(64),

pduConstructionFsn3Bits(65),

reserved66(66),

reserved67(67),

reserved68(68),

reserved69(69),

reserved70(70),

reserved71(71),

packingSupported(72),

reserved73(73),

reserved74(74),

reserved75(75),

reserved76(76),

reserved77(77),

reserved78(78),

reserved79(79),

extendedRtpsSupported(80),

reserved81(81),

reserved82(82),

reserved83(83),

reserved84(84),

reserved85(85),

reserved86(86),

reserved87(87),

smcIpAddressAllocDhcp(88),

smcIpAddressAllocMobileIpv4(89),

smcIpAddressAllocMobileDhcpv6(90),

smcIpAddressAllocMobileIpv6Autoconfig(91),

reserved92(92),

reserved93(93),

reserved94(94),

reserved95(95),

arqSelectiveAck(96),

arqCumulativeAck(97),

arqCumWithSelAckEntry(98),

arqCumWithBlockSeqAck(99),

arqSeqBlockAck(100),

reserved101(101),

reserved102(102),

reserved103(103),

headerSupportBwReqUlTxPowerReport(104),

headerSupportBwReqCinrReport(105),

headerSupportCqichAllocationReq(106),

headerSupportPhyChannelReport(107),

headerSupportBwReqUlSleepCntl(108),

headerSupportSnReport(109),

headerSupportFeedbackReport(110),

headerSupportSduSn(111),

headerSupportSduSnPeriod0(112),

headerSupportSduSnPeriod1(113),

headerSupportSduSnPeriod2(114),

headerSupportDlSleepControl(115),

headerSupportFeedbackRequest(116),

headerSupportMimcModeFeedback(117),

headerSupportUlTxPowerReport(118),

headerSupportMiniFeedback(119),

headerSupportSnRequest(120),

reserved121(121),

reserved122(122),

reserved123(123),

reserved124(124),

reserved125(125),

reserved126(126),

reserved127(127),

pkmVersionSupport1(128),

pkmVersionSupport2(129),

reserved130(130),

reserved131(131),

reserved132(132),

reserved133(133),

reserved134(134),

reserved135(135),

authPolicySupportRsaInitialEntry(136),

authPolicySupportEapInitialEntry(137),

authPolicySupportAuthEapInitialEntry(138),

authPolicySupportRsaReentry(139),

authPolicySupportEapReentry(140),

authPolicySupportAuthEapReentry(141),

reserved142(142),

reserved143(143),

macModeHmac(144),

reserved145(145),

macModeHmac64(146),

macModeHmac80(147),

macModeHmac96(148),

macModeCmac(149),

reserved150(150),

reserved151(151),

extSubheader(152),

headerSupportShortPduSn(153),

headerSupportLongPduSn(154),

reserved155(155),

reserved156(156),

reserved157(157),

reserved158(158),

reserved159(159)

}

WmanIf2SsCapOptionsOfdm ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This type combines all OFDM specific SS capabilities

with the binary encodded fields.

It reserves the space equivalent to the size of corresponding TLV

and within this space it maps directly to the encoding specified in TLVs including

all defined reserved bits."

REFERENCE

"IEEE Std 802.16-2008 subclauses:

bits 0-1:     11.8.3.6.1 (OFDM FFT sizes)

2-7:     reserved

bits 8-13:    11.8.3.6.2 (OFDM SS demodulator)

14-15:   reserved

bits 16-21:   11.8.3.6.3 (OFDM SS modulator)

22-23:   reserved

bits 24:      11.8.3.6.5 (OFDM TC sublayer)

25-31:   reserved

bits 32-33:   11.8.3.6.6 (OFDM private map)

34-39:   reserved

bits 40-41:   11.8.3.6.7 (OFDM SS uplink power control)

42-47:   reserved"

SYNTAX BITS

{

ofdmFftSize256(0),

ofdmFftSize2048(1),

reserved2(2),

reserved3(3),

reserved4(4),

reserved5(5),

reserved6(6),

reserved7(7),

ofdmSsDemodulatorQam64(8),

ofdmSsDemodulatorBtc(9),

ofdmSsDemodulatorCtc(10),

ofdmSsDemodulatorStc(11),

ofdmSsDemodulatorAas(12),

ofdmSsDemodulatorSuchan(13),

reserved14(14),

reserved15(15),

ofdmSsModulatorQam64(16),

ofdmSsModulatorBtc(17),

ofdmSsModulatorCtc(18),

ofdmSsModulatorSubchan(19),

ofdmSsModulatorFocusedCtBwReq(20),

ofdmSsModulatorUlCyclicDelay(21),

reserved22(22),

reserved23(23),

ofdmTcSublayerSupport(24),

reserved25(25),

reserved26(26),

reserved27(27),

reserved28(28),

reserved29(29),

reserved30(30),

reserved31(31),

ofdmRegularMap(32),

ofdmCompressedMap(33),

reserved34(34),

reserved35(35),

reserved36(36),

reserved37(37),

reserved38(38),

reserved39(39),

ofdmUlOpenLoopPwrCtrl(40),

ofdmUlAasPreamblePwrCtrl(41),

reserved42(42),

reserved43(43),

reserved44(44),

reserved45(45),

reserved46(46),

reserved47(47),

reserved48(48)

}

WmanIf2SsCapOptionsOfdma ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"This type combines all OFDMA specific SS capabilities

with the binary encodded fields.

It reserves the space equivalent to the size of corresponding TLV

and within this space it maps directly to the encoding specified in TLVs including

all defined reserved bits."

REFERENCE

"IEEE Std 802.16-2008 subclauses:

bits 0:          reserved

1-4:        11.8.3.7.1 (OFDMA FFT sizes)

5-7:        reserved

bits 8-14,16-18: 11.8.3.7.2 (OFDMA SS demodulator)

15,19-23:   reserved

bits 24-31:      11.8.3.7.3 (OFDMA SS modulator)

bits 32-39:      11.8.3.7.4 (OFDMA permutation)

bits 40-61:      11.8.3.7.5 (OFDMA SS demodulator for MIMO)

62-63:      reserved

bits 64-71:      11.8.3.7.7 (OFDMA private map)

bits 72-76:      11.8.3.7.8 (OFDMA AAS)

77-79:      reserved

bits 80-87:      11.8.3.7.9 (OFDMA CINR measurements)

bits 88-89:      11.8.3.7.11 (OFDMA power control)

90-95:      reserved

bits 96-100:     11.8.3.7.12 (OFDMA map)

101-103:    reserved

bits 104-106,108-111: 11.8.3.7.13 (OFDM uplink control)

107:        reserved

bits 112-123:    11.8.3.7.14 (OFDMA CSIT capability)

124-127:    reserved

bits 128-135:    11.8.3.7.15 (OFDMA max bursts in HARQ)

bits 136-144:    11.8.3.7.16 (OFDMA modulator for MIMO)

145-151:    reserved

bits 152-153:    11.8.3.7.18 (OFDMA multicast bursts)

154-159:    reserved

bits 160-172:    11.8.3.7.19.1 (OFDMA HARQ incremental buffer)

173-175:    reserved

bits 176-190:    11.8.3.7.19.1 (OFDMA HARQ chase)

191:        reserved

bits 192-198:    11.8.3.7.20 (OFDMA parameter set)

199:        reserved"

SYNTAX BITS

{

reserved0(0),

ofdmaFftSize2048(1),

ofdmaFftSize128(2),

ofdmaFftSize512(3),

ofdmaFftSize1024(4),

reserved5(5),

reserved6(6),

reserved7(7),

ofdmaDemodulatorQam64(8),

ofdmaDemodulatorBtc(9),

ofdmaDemodulatorCtc(10),

ofdmaDemodulatorStc(11),

ofdmaDemodulatorCcWithInterleaver(12),

ofdmaDemodulatorHarqChase(13),

ofdmaDemodulatorHarqCtclr(14),

reserved15(15),

ofdmaDemodulatorHarqCclr(16),

ofdmaDemodulatorLdpc(17),

ofdmaDemodulatorDedicatedPilots(18),

reserved19(19),

reserved20(20),

reserved21(21),

reserved22(22),

reserved23(23),

ofdmaModulatorQam64(24),

ofdmaModulatorBtc(25),

ofdmaModulatorCtc(26),

ofdmaModulatorStc(27),

ofdmaModulatorHarqChase(28),

ofdmaModulatorHarqCtcIr(29),

ofdmaModulatorHarqCcIr(30),

ofdmaModulatorHarqLdpc(31),

odfmaPermutationOptionalPusc(32),

odfmaPermutationOptionalFusc(33),

odfmaPermutationAmc1x6(34),

odfmaPermutationAmc2x3(35),

odfmaPermutationAmc3x2(36),

odfmaPermutationAmcWithHarqMap(37),

odfmaPermutationTusc1(38),

odfmaPermutationTusc2(39),

ofdmaDemodMimo2AntStcMatrixA(40),

ofdmaDemodMimo2AntStcMatrixBVCoding(41),

ofdmaDemodMimo2AntStcMatrixBHCoding(42),

ofdmaDemodMimo4AntStcMatrixA(43),

ofdmaDemodMimo4AntStcMatrixBVCoding(44),

ofdmaDemodMimo4AntStcMatrixBHCoding(45),

ofdmaDemodMimo4AntStcMatrixCVCoding(46),

ofdmaDemodMimo4AntStcMatrixCHCoding(47),

ofdmaDemodMimo3AntStcMatrixA(48),

ofdmaDemodMimo3AntStcMatrixB(49),

ofdmaDemodMimo3AntStcMatrixCVCoding(50),

ofdmaDemodMimo3AntStcMatrixCHCoding(51),

ofdmaDemodMimoCalcPrecodeWight(52),

ofdmaDemodMimoAdaptiveRateCtrl(53),

ofdmaDemodMimoCalcChanMatrix(54),

ofdmaDemodMimoAntGroup(55),

ofdmaDemodMimoAntSelect(56),

ofdmaDemodMimoCodebookPrecode(57),

ofdmaDemodMimoLongTermPrecode(58),

ofdmaDemodMimoMidamble(59),

ofdmaDemodAllocGranDlPuscStc(60),

ofdmaDemodConcurentAllocDlPuscStc(61),

reserved62(62),

reserved63(63),

ofdmaPrivateMapHarqMap(64),

ofdmaPrivateMap(65),

ofdmaPrivateMapReduced(66),

ofdmaPrivateMapChainEnable(67),

ofdmaPrivateMapDlFrameOffset(68),

ofdmaPrivateMapUlFrameOffset(69),

ofdmaPrivateMapChainConcurency0(70),

ofdmaPrivateMapChainConcurency1(71),

ofdmaAasZone(72),

ofdmaAasDiverityMapScan(73),

ofdmaAasFeedbackRsp(74),

ofdmaAasDlPreamble(75),

ofdmaAasUlPreamble(76),

reserved77(77),

reserved78(78),

reserved79(79),

ofdmaCinrPhysicalPreamble(80),

ofdmaCinrPhysicalPilotSubc(81),

ofdmaCinrPhysicalDataSubc(82),

ofdmaCinrEffectivePreamble(83),

ofdmaCinrEffectivePilotSubc(84),

ofdmaCinrEffectiveDataSubc(85),

ofdmaCinr2CqiChannel(86),

ofdmaFreqSelectivityReport(87),

ofdmaPwrCtrlOpenLoop(88),

ofdmaPwrCtrlAasPreamble(89),

reserved90(90),

reserved91(91),

reserved92(92),

reserved93(93),

reserved94(94),

reserved95(95),

ofdmaMapHarq(96),

ofdmaMapExtendedHarqIe(97),

ofdmaMapSubMapForFirstZone(98),

ofdmaMapSubMapForOtherZones(99),

ofdmaMapDlRegionDefinition(100),

reserved101(101),

reserved102(102),

reserved103(103),

ofdmaUlCtrlt3BitMimoFastFeedback(104),

ofdmaUlCtrltEnhancedFastFeedback(105),

ofdmaUlCtrltUlAck(106),

reserved107(107),

ofdmaUlCtrltUepFastFeedback(108),

ofdmaUlCtrltFastDlMeasurementFeedback(109),

ofdmaUlCtrltPriSecFastFeedback(110),

ofdmaDiucCqiFastFeedback(111),

ofdmaCsitTypeA(112),

ofdmaCsitTypeB(113),

ofdmaPowerAssignment(114),

ofdmaSoundingRspTime0(115),

ofdmaSoundingRspTime1(116),

ofdmaSoundingRspTime2(117),

ofdmaMaxSimuSoundInst0(118),

ofdmaMaxSimuSoundInst1(119),

ofdmaMaxSimuSoundInst2(120),

ofdmaMaxSimuSoundInst3(121),

ofdmaNoP9Or18ForCsitTypeA(122),

ofdmaCsitNotSupported(123),

reserved124(124),

reserved125(125),

reserved126(126),

reserved127(127),

ofdmaMaxHarqBurstsUl0(128),

ofdmaMaxHarqBurstsUl1(129),

ofdmaMaxHarqBurstsUl2(130),

ofdmaMaxHarqBurstsUlNonHarqIncluded(131),

ofdmaMaxHarqBurstsDl0(132),

ofdmaMaxHarqBurstsDl1(133),

ofdmaMaxHarqBurstsDl2(134),

ofdmaMaxHarqBurstsDl3(135),

ofdmaMimoMod2AntStcMatrixA(136),

ofdmaMimoMod2AntStcMatrixBVCoding(137),

ofdmaMimoMod2AntStcMatrixBHCoding(138),

ofdmaMimoModBeamforming(139),

ofdmaMimoModAdaptiveRateControl(140),

ofdmaMimoModSingleAnt(141),

ofdmaMimoModCollaborativeSm1Ant(142),

ofdmaMimoModCollaborativeSm2Ants(143),

ofdmaMimoModDisableUlSubchRotation(144),

reserved145(145),

reserved146(146),

reserved147(147),

reserved148(148),

reserved149(149),

reserved150(150),

reserved151(151),

ofdmaMcastBurstsDlMultiFecTypes(152),

ofdmaMcastBurstsUlMultiFecTypes(153),

reserved154(154),

reserved155(155),

reserved156(156),

reserved157(157),

reserved158(158),

reserved159(159),

ofdmaHarqIncrBufDlNep0(160),

ofdmaHarqIncrBufDlNep1(161),

ofdmaHarqIncrBufDlNep2(162),

ofdmaHarqIncrBufDlNep3(163),

ofdmaHarqIncrBufDlAggFlag(164),

reserved165(165),

reserved166(166),

reserved167(167),

ofdmaHarqIncrBufUlNep0(168),

ofdmaHarqIncrBufUlNep1(169),

ofdmaHarqIncrBufUlNep2(170),

ofdmaHarqIncrBufUlNep3(171),

ofdmaHarqIncrBufUlAggFlag(172),

reserved173(173),

reserved174(174),

reserved175(175),

ofdmaHarqChaseBufDlComb0(176),

ofdmaHarqChaseBufDlComb1(177),

ofdmaHarqChaseBufDlComb2(178),

ofdmaHarqChaseBufDlComb3(179),

ofdmaHarqChaseBufDlComb4(180),

ofdmaHarqChaseBufDlComb5(181),

ofdmaHarqChaseBufDlAggFlag(182),

reserved183(183),

ofdmaHarqChaseBufUlComb0(184),

ofdmaHarqChaseBufUlComb1(185),

ofdmaHarqChaseBufUlComb2(186),

ofdmaHarqChaseBufUlComb3(187),

ofdmaHarqChaseBufUlComb4(188),

ofdmaHarqChaseBufUlComb5(189),

ofdmaHarqChaseBufUlAggFlag(190),

reserved191(191),

odfmaParamSetPhyA(192),

odfmaParamSetPhyB(193),

odfmaParamSetHarq0(194),

odfmaParamSetHarq1(195),

odfmaParamSetHarq2(196),

odfmaParamSetMacA(197),

odfmaParamSetMacB(198),

reserved199(199)

}

--

-- Node definitions

--

std OBJECT IDENTIFIER ::= { iso 0 }

iso8802 OBJECT IDENTIFIER ::= { std 8802 }

wman OBJECT IDENTIFIER ::= { iso8802 16 }

wmanIf2MibObjects OBJECT IDENTIFIER ::= { wmanIf2BsMibD6Cap 1 }

wmanIf2BsCm OBJECT IDENTIFIER ::= { wmanIf2MibObjects 2 }

wmanIf2BsRegisteredSsTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2BsRegisteredSsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the basic capability information

of SSs that have been negotiated and agreed between

BS and SS via REG-REQ and REG-RSP messages. An entry

in this table indicates the SS has entered and registered

into the BS."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsCm 1 }

wmanIf2BsRegisteredSsEntry OBJECT-TYPE

SYNTAX WmanIf2BsRegisteredSsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. The primary index is the ifIndex

with an ifType of ieee80216WMAN, indicating the BS sector

with which the SS is associated. wmanIf2BsSsMacAddress

identifies the SS being registered."

INDEX { ifIndex, wmanIf2BsSsMacAddress }

::= { wmanIf2BsRegisteredSsTable 1 }

WmanIf2BsRegisteredSsEntry ::=

SEQUENCE {

wmanIf2BsSsMacVersion

WmanIf2MacVersion,

wmanIf2BsSsMaxTxPower64Qam

WmanIf2MaxTxPowerType,

wmanIf2BsSsMaxTxPower16Qam

WmanIf2MaxTxPowerType,

wmanIf2BsSsMaxTxPowerQpsk

WmanIf2MaxTxPowerType,

wmanIf2BsSsMaxTxPowerBpsk

WmanIf2MaxTxPowerType,

wmanIf2BsSsAasBroadcastPermission

INTEGER,

wmanIf2BsSsVendorIdEncoding

OCTET STRING,

wmanIf2BsSs2ndMgmtArqBlockSize

INTEGER,

wmanIf2BsSs2ndMgmtArqRxPurgeTimeout

INTEGER,

wmanIf2BsSs2ndMgmtArqDeliverInOrder

TruthValue,

wmanIf2BsSs2ndMgmtArqSyncLossTimeout

INTEGER,

wmanIf2BsSs2ndMgmtArqBlockLifetime

INTEGER,

wmanIf2BsSs2ndMgmtArqUpLinkRxDelay

INTEGER,

wmanIf2BsSs2ndMgmtArqDnLinkRxDelay

INTEGER,

wmanIf2BsSs2ndMgmtArqUpLinkTxDelay

INTEGER,

wmanIf2BsSs2ndMgmtArqDnLinkTxDelay

INTEGER,

wmanIf2BsSs2ndMgmtArqWindowSize

INTEGER,

wmanIf2BsSs2ndMgmtArqEnable

TruthValue,

wmanIf2BsSsIpManagementMode

INTEGER,

wmanIf2BsSsManagementSupport

INTEGER,

wmanIf2BsSsSecondaryCid

WmanIf2CidType,

wmanIf2BsSsPrimaryCid

WmanIf2CidType,

wmanIf2BsSsBasicCid

WmanIf2CidType,

wmanIf2BsSsMacAddress

MacAddress

}

wmanIf2BsSsMacVersion OBJECT-TYPE

SYNTAX WmanIf2MacVersion

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This parameter specifies the version of 802.16 to which the

message originator conforms."

REFERENCE

"Subclause 11.1.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 24 }

wmanIf2BsSsMaxTxPower64Qam OBJECT-TYPE

SYNTAX WmanIf2MaxTxPowerType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum available power for 64-QAM constellations.

The maximum power parameters are reported in dBm and

quantized in 0.5 dBm steps ranging from -64 dBm (encoded

0x00) to 63.5 dBm (encoded 0xFF). Values outside this

range shall be assigned the closest extreme. SSs that do

not support QAM64 shall report the value of 0x00. This

parameter is only applicable to systems supporting the SCa,

OFDM or OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 23 }

wmanIf2BsSsMaxTxPower16Qam OBJECT-TYPE

SYNTAX WmanIf2MaxTxPowerType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum available power for 16-QAM constellations.

The maximum power parameters are reported in dBm and

quantized in 0.5 dBm steps ranging from -64 dBm (encoded

0x00) to 63.5 dBm (encoded 0xFF). Values outside this

range shall be assigned the closest extreme. This parameter

is only applicable to systems supporting the SCa, OFDM or

OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 22 }

wmanIf2BsSsMaxTxPowerQpsk OBJECT-TYPE

SYNTAX WmanIf2MaxTxPowerType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum available power for QPSK. The maximum power

parameters are reported in dBm and quantized in 0.5 dBm

steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

(encoded 0xFF). Values outside this range shall be assigned

to closest extreme. This parameter is only applicable to

systems supporting the SCa, OFDM or OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 21 }

wmanIf2BsSsMaxTxPowerBpsk OBJECT-TYPE

SYNTAX WmanIf2MaxTxPowerType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum available power for BPSK. The maximum power

parameters are reported in dBm and quantized in 0.5 dBm

steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

(encoded 0xFF). Values outside this range shall be assigned

the closest extreme. This parameter is only applicable to

systems supporting the SCa, OFDM or OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 20 }

wmanIf2BsSsAasBroadcastPermission OBJECT-TYPE

SYNTAX INTEGER

{

contBasedBwReqPermitted(0),

contBasedBwReqNotPermitted(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This parameter specifies if SS can issue contention-based

bandwidth request or not."

REFERENCE

"Subclause 11.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 19 }

wmanIf2BsSsVendorIdEncoding OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (3))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value field contains the vendor identification

specified by the 3 byte vendor-specific organizationally

unique identifier of the SS or BS MAC address.A vendor ID

used in a REG-REQ shall be the Vendor ID of the SS sending

the request. A vendor ID used in a REG-RSP shall be the

Vendor ID of the BS sending the response."

REFERENCE

"Subclause 11.1.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 18 }

wmanIf2BsSs2ndMgmtArqBlockSize OBJECT-TYPE

SYNTAX INTEGER (1..2040)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This parameter specifies the size of a ARQ block. This

parameter shall be established by negotiation during the

connection setup. The requester includes its desired

setting in the REQ message. The receiver of the REQ

message shall take the smaller of the value it prefers and

value in the REQ message. The minimum value is included in

the RSP message."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 17 }

wmanIf2BsSs2ndMgmtArqRxPurgeTimeout OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "10 us"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.7 in IEEE Std 802.16-2004"

DEFVAL { 0 }

::= { wmanIf2BsRegisteredSsEntry 16 }

wmanIf2BsSs2ndMgmtArqDeliverInOrder OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Indicates whether or not data is to be delivered

by the receiving MAC to its client application

in the order in which data was handed off to the

originating MAC."

REFERENCE

"Subclause 11.13.18.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 15 }

wmanIf2BsSs2ndMgmtArqSyncLossTimeout OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "10 us"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum interval before declaring a loss

of synchronization of the sender and receiver

state machines. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.5 in IEEE Std 802.16-2004"

DEFVAL { 0 }

::= { wmanIf2BsRegisteredSsEntry 14 }

wmanIf2BsSs2ndMgmtArqBlockLifetime OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "10 us"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum time interval an ARQ fragment will be

managed by the transmitter ARQ machine, once

initial transmission of the fragment has occured.

If transmission or retransmission of the fragment

is not acknowledged by the receiver before the

time limit is reached, the fragment is discarded.

A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.4 in IEEE Std 802.16-2004"

DEFVAL { 0 }

::= { wmanIf2BsRegisteredSsEntry 13 }

wmanIf2BsSs2ndMgmtArqUpLinkRxDelay OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The object defines the ARQ receiver delay for

uplink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 12 }

wmanIf2BsSs2ndMgmtArqDnLinkRxDelay OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The object defines the ARQ receiver delay for

downlink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 11 }

wmanIf2BsSs2ndMgmtArqUpLinkTxDelay OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The object defines the ARQ transmitter delay for

uplink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 10 }

wmanIf2BsSs2ndMgmtArqDnLinkTxDelay OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The object defines the ARQ transmitter delay for

downlink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 9 }

wmanIf2BsSs2ndMgmtArqWindowSize OBJECT-TYPE

SYNTAX INTEGER (1..1024)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Indicates the maximum number of unacknowledged

fragments at any time for 2nd management connection."

REFERENCE

"Subclause 11.13.18.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 8 }

wmanIf2BsSs2ndMgmtArqEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"True(1) ARQ enabling is requested for the 2nd

management channel."

REFERENCE

"Subclause 11.13.18.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 7 }

wmanIf2BsSsIpManagementMode OBJECT-TYPE

SYNTAX INTEGER

{

unmanaged(0),

ipManaged(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The IP management mode parameter dictates whether

the provider intends to manage the SS on an ongoing

basis via IP-based mechanisms."

REFERENCE

"Subclause 11.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 6 }

wmanIf2BsSsManagementSupport OBJECT-TYPE

SYNTAX INTEGER

{

unmanagedSs(0),

managedSs(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates whether or not the SS is managed."

REFERENCE

"Subclause 11.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 5 }

wmanIf2BsSsSecondaryCid OBJECT-TYPE

SYNTAX WmanIf2CidType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this object indicates the secondary

management CID present in the REG-RSP message. The value

should be null if the 2nd management connection is not

available."

REFERENCE

"Subclause 6.4.2.3.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 4 }

wmanIf2BsSsPrimaryCid OBJECT-TYPE

SYNTAX WmanIf2CidType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this object indicates the primary CID of the

SS received from the RNG-RSP message."

REFERENCE

"Subclause 6.3.2.3.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 3 }

wmanIf2BsSsBasicCid OBJECT-TYPE

SYNTAX WmanIf2CidType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this object indicates the SS's basic CID

that was sent in the RNG-RSP message."

REFERENCE

"Subclause 6.3.2.3.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 2 }

wmanIf2BsSsMacAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The MAC address of SS is received from the RNG-REQ

message. When SS registers, this MAC address is entered

into the table, and used as the identifier to the SS."

REFERENCE

"Subclause 6.3.2.3.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 1 }

wmanIf2BsSsReqCapabilitiesTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2BsSsReqCapabilitiesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the basic capability information of SSs

that have been reported by SSs to BS using RNG-REQ, SBC-REQ

and REG-REQ messages. Entries in this table should be

created when an SS registers with a BS."

::= { wmanIf2BsCm 3 }

wmanIf2BsSsReqCapabilitiesEntry OBJECT-TYPE

SYNTAX WmanIf2BsSsReqCapabilitiesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. This table augments the table

wmanIf2BsRegisteredSsTable."

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsReqCapabilitiesTable 1 }

WmanIf2BsSsReqCapabilitiesEntry ::=

SEQUENCE {

wmanIf2BsSsReqCapUplinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsSsReqCapDsxFlowControl

WmanIf2MaxDsxFlowType,

wmanIf2BsSsReqCapMcaFlowControl

WmanIf2MaxMcaFlowType,

wmanIf2BsSsReqCapMcpGroupCidSupport

WmanIf2MaxMcpGroupCid,

wmanIf2BsSsReqCapPkmFlowControl

WmanIf2MaxPkmFlowType,

wmanIf2BsSsReqCapMaxNumOfSupportedSA

WmanIf2MaxNumOfSaType,

wmanIf2BsSsReqCapMaxNumOfClassifier

WmanIf2MaxClassifiers,

wmanIf2BsSsReqCapTtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsSsReqCapRtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsSsReqCapDownlinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsSsReqCapMaxNumBurstToMs

WmanIf2CapMaxNumBurstTx,

wmanIf2BsSsReqCapMaxMacLevelDlFrame

WmanIf2MaxMacLevel,

wmanIf2BsSsReqCapMaxMacLevelUlFrame

WmanIf2MaxMacLevel,

wmanIf2BsSsReqCapPnWindowSize

INTEGER,

wmanIf2BsSsOfdmReqCapLoopPwrControlSw

Unsigned32,

wmanIf2BsSsOfdmaReqCapSdmaPilot

WmanIf2SdmaPilotCap,

wmanIf2BsSsOfdmaReqCapNoUlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsSsOfdmaReqCapNoDlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsSsReqCapOptionsGeneral

WmanIf2SsCapOptionsGeneral,

wmanIf2BsSsReqCapOptionsOfdm

WmanIf2SsCapOptionsOfdm,

wmanIf2BsSsReqCapOptionsOfdma

WmanIf2SsCapOptionsOfdma

}

wmanIf2BsSsReqCapUplinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shows the number of Uplink transport CIDs the SS

can support."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 1 }

wmanIf2BsSsReqCapDsxFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxDsxFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent

DSA, DSC, or DSD transactions that SS is capable of having

outstanding."

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 3 }

wmanIf2BsSsReqCapMcaFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxMcaFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent MCA

transactions that SS is capable of having outstanding."

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 4 }

wmanIf2BsSsReqCapMcpGroupCidSupport OBJECT-TYPE

SYNTAX WmanIf2MaxMcpGroupCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the maximum number of

simultaneous Multicast Polling Groups the SS is

capable of belonging to."

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 5 }

wmanIf2BsSsReqCapPkmFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxPkmFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent PKM

transactions that SS is capable of having outstanding."

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 6 }

wmanIf2BsSsReqCapMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX WmanIf2MaxNumOfSaType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the maximum number of supported

security associations of the SS."

DEFVAL { 1 }

::= { wmanIf2BsSsReqCapabilitiesEntry 7 }

wmanIf2BsSsReqCapMaxNumOfClassifier OBJECT-TYPE

SYNTAX WmanIf2MaxClassifiers

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the maximum number of admitted

Classifiers that the SS can support."

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 10 }

wmanIf2BsSsReqCapTtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the SS's transition speed SSTTG

for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsReqCapabilitiesEntry 14 }

wmanIf2BsSsReqCapRtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the SS's transition speed SSRTG

for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsReqCapabilitiesEntry 15 }

wmanIf2BsSsReqCapDownlinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shows the number of Downlink transport CIDs the

SS can support."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 16 }

wmanIf2BsSsReqCapMaxNumBurstToMs OBJECT-TYPE

SYNTAX WmanIf2CapMaxNumBurstTx

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS's CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 19 }

wmanIf2BsSsReqCapMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame."

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 23 }

wmanIf2BsSsReqCapMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame."

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 24 }

wmanIf2BsSsReqCapPnWindowSize OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 28 }

wmanIf2BsSsOfdmReqCapLoopPwrControlSw OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsSsReqCapabilitiesEntry 30 }

wmanIf2BsSsOfdmaReqCapSdmaPilot OBJECT-TYPE

SYNTAX WmanIf2SdmaPilotCap

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsSsReqCapabilitiesEntry 31 }

wmanIf2BsSsOfdmaReqCapNoUlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsSsReqCapabilitiesEntry 32 }

wmanIf2BsSsOfdmaReqCapNoDlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of downlink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsSsReqCapabilitiesEntry 33 }

wmanIf2BsSsReqCapOptionsGeneral OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsGeneral

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded general capability options as reported by SS."

::= { wmanIf2BsSsReqCapabilitiesEntry 34 }

wmanIf2BsSsReqCapOptionsOfdm OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdm

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDM specific capability options as reported by SS."

::= { wmanIf2BsSsReqCapabilitiesEntry 35 }

wmanIf2BsSsReqCapOptionsOfdma OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdma

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDMA specific capability options as reported by SS."

::= { wmanIf2BsSsReqCapabilitiesEntry 36 }

wmanIf2BsSsRspCapabilitiesTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2BsSsRspCapabilitiesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the basic capability information of SSs

that have been negotiated and agreed between BS and SS via

RNG-REQ/RSP, SBC-REQ/RSP and REG-REQ/RSP messages.

This table augments the wmanIf2BsRegisteredSsTable."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsCm 4 }

wmanIf2BsSsRspCapabilitiesEntry OBJECT-TYPE

SYNTAX WmanIf2BsSsRspCapabilitiesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. This table augments the

wmanIf2BsRegisteredSsTable. "

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsRspCapabilitiesTable 1 }

WmanIf2BsSsRspCapabilitiesEntry ::=

SEQUENCE {

wmanIf2BsSsRspCapUplinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsSsRspCapDsxFlowControl

WmanIf2MaxDsxFlowType,

wmanIf2BsSsRspCapMcaFlowControl

WmanIf2MaxMcaFlowType,

wmanIf2BsSsRspCapMcpGroupCidSupport

WmanIf2MaxMcpGroupCid,

wmanIf2BsSsRspCapPkmFlowControl

WmanIf2MaxPkmFlowType,

wmanIf2BsSsRspCapMaxNumOfSupportedSA

WmanIf2MaxNumOfSaType,

wmanIf2BsSsRspCapMaxNumOfClassifier

WmanIf2MaxClassifiers,

wmanIf2BsSsRspCapTtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsSsRspCapRtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsSsRspCapDownlinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsSsRspCapMaxNumBurstToMs

WmanIf2CapMaxNumBurstTx,

wmanIf2BsSsRspCapMaxMacLevelDlFrame

WmanIf2MaxMacLevel,

wmanIf2BsSsRspCapMaxMacLevelUlFrame

WmanIf2MaxMacLevel,

wmanIf2BsSsRspCapNumOfProvisionedSf

WmanIf2CapMaxNumProvSf,

wmanIf2BsSsRspCapPnWindowSize

INTEGER,

wmanIf2BsSsOfdmRspCapLoopPwrControlSw

Unsigned32,

wmanIf2BsSsOfdmaRspCapSdmaPilot

WmanIf2SdmaPilotCap,

wmanIf2BsSsOfdmaRspCapNoUlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsSsOfdmaRspCapNoDlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsSsRspCapOptionsGeneral

WmanIf2SsCapOptionsGeneral,

wmanIf2BsSsRspCapOptionsOfdm

WmanIf2SsCapOptionsOfdm,

wmanIf2BsSsRspCapOptionsOfdma

WmanIf2SsCapOptionsOfdma

}

wmanIf2BsSsRspCapUplinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated number of Uplink transport CIDs the SS can

support."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 1 }

wmanIf2BsSsRspCapDsxFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxDsxFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated maximum number of concurrent DSA, DSC, or DSD

transactions that may be outstanding."

::= { wmanIf2BsSsRspCapabilitiesEntry 3 }

wmanIf2BsSsRspCapMcaFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxMcaFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated maximum number of concurrent

MCA transactions that may be outstanding."

DEFVAL { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 4 }

wmanIf2BsSsRspCapMcpGroupCidSupport OBJECT-TYPE

SYNTAX WmanIf2MaxMcpGroupCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated maximum number of simultaneous Multicast

Polling Groups the SS is capable of belonging to."

DEFVAL { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 5 }

wmanIf2BsSsRspCapPkmFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxPkmFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated maximum number of concurrent PKM

transactions that may be outstanding."

DEFVAL { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 6 }

wmanIf2BsSsRspCapMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX WmanIf2MaxNumOfSaType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated maximum number of supported security

association of the SS."

DEFVAL { 1 }

::= { wmanIf2BsSsRspCapabilitiesEntry 7 }

wmanIf2BsSsRspCapMaxNumOfClassifier OBJECT-TYPE

SYNTAX WmanIf2MaxClassifiers

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Negotiated maximum number of admitted Classifiers

that the SS is allowed to have."

DEFVAL { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 10 }

wmanIf2BsSsRspCapTtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the negotiated transition speed

SSTTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsRspCapabilitiesEntry 14 }

wmanIf2BsSsRspCapRtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the negotiated transition speed

SSRTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsRspCapabilitiesEntry 15 }

wmanIf2BsSsRspCapDownlinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shows the number of Downlink transport CIDs the

SS can support."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 17 }

wmanIf2BsSsRspCapMaxNumBurstToMs OBJECT-TYPE

SYNTAX WmanIf2CapMaxNumBurstTx

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 19 }

wmanIf2BsSsRspCapMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 23 }

wmanIf2BsSsRspCapMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 24 }

wmanIf2BsSsRspCapNumOfProvisionedSf OBJECT-TYPE

SYNTAX WmanIf2CapMaxNumProvSf

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 25 }

wmanIf2BsSsRspCapPnWindowSize OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 29 }

wmanIf2BsSsOfdmRspCapLoopPwrControlSw OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsSsRspCapabilitiesEntry 31 }

wmanIf2BsSsOfdmaRspCapSdmaPilot OBJECT-TYPE

SYNTAX WmanIf2SdmaPilotCap

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsSsRspCapabilitiesEntry 32 }

wmanIf2BsSsOfdmaRspCapNoUlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsSsRspCapabilitiesEntry 33 }

wmanIf2BsSsOfdmaRspCapNoDlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of downlink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsSsRspCapabilitiesEntry 34 }

wmanIf2BsSsRspCapOptionsGeneral OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsGeneral

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded general capability options as granted by BS for SS."

::= { wmanIf2BsSsRspCapabilitiesEntry 35 }

wmanIf2BsSsRspCapOptionsOfdm OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdm

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDM specific capability options as granted by BS for SS."

::= { wmanIf2BsSsRspCapabilitiesEntry 36 }

wmanIf2BsSsRspCapOptionsOfdma OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdma

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDMA specific capability options as granted by BS for SS."

::= { wmanIf2BsSsRspCapabilitiesEntry 37 }

wmanIf2BsBasicCapabilitiesTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2BsBasicCapabilitiesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the basic capabilities of the BS as

implemented in BS hardware and software. These capabilities

along with the configuration for them

(wmanIf2BsCapabilitiesConfigTable) are used for negotiation

of basic capabilities with SS using RNG-RSP, SBC-RSP and

REG-RSP messages. The negotiated capabilities are obtained

by interSubclause of SS raw reported capabilities, BS raw

capabilities and BS configured capabilities. The objects in

the table have read-only access. The table is maintained

by BS."

::= { wmanIf2BsCm 5 }

wmanIf2BsBasicCapabilitiesEntry OBJECT-TYPE

SYNTAX WmanIf2BsBasicCapabilitiesEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX { ifIndex }

::= { wmanIf2BsBasicCapabilitiesTable 1 }

WmanIf2BsBasicCapabilitiesEntry ::=

SEQUENCE {

wmanIf2BsCapUplinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsCapDsxFlowControl

WmanIf2MaxDsxFlowType,

wmanIf2BsCapMcaFlowControl

WmanIf2MaxMcaFlowType,

wmanIf2BsCapMcpGroupCidSupport

WmanIf2MaxMcpGroupCid,

wmanIf2BsCapPkmFlowControl

WmanIf2MaxPkmFlowType,

wmanIf2BsCapMaxNumOfSupportedSA

WmanIf2MaxNumOfSaType,

wmanIf2BsCapMaxNumOfClassifier

WmanIf2MaxClassifiers,

wmanIf2BsCapTtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsCapRtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsCapDownlinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsCapMaxNumBurstToMs

WmanIf2CapMaxNumBurstTx,

wmanIf2BsCapMaxMacLevelDlFrame

WmanIf2MaxMacLevel,

wmanIf2BsCapMaxMacLevelUlFrame

WmanIf2MaxMacLevel,

wmanIf2BsCapNumOfProvisionedSf

WmanIf2CapMaxNumProvSf,

wmanIf2BsCapPnWindowSize

INTEGER,

wmanIf2BsSsOfdmCapLoopPwrControlSw

Unsigned32,

wmanIf2BsSsOfdmaCapSdmaPilot

WmanIf2SdmaPilotCap,

wmanIf2BsOfdmaCapNoUlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsOfdmaCapNoDlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsCapOptionsGeneral

WmanIf2SsCapOptionsGeneral,

wmanIf2BsCapOptionsOfdm

WmanIf2SsCapOptionsOfdm,

wmanIf2BsCapOptionsOfdma

WmanIf2SsCapOptionsOfdma

}

wmanIf2BsCapUplinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shows the number of Uplink transport CIDs the

BS can support per SS."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 1 }

wmanIf2BsCapDsxFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxDsxFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent

DSA, DSC, or DSD transactions that BS allows each SS to

have outstanding."

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 3 }

wmanIf2BsCapMcaFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxMcaFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent

MCA transactions that BS allows each SS to have."

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 4 }

wmanIf2BsCapMcpGroupCidSupport OBJECT-TYPE

SYNTAX WmanIf2MaxMcpGroupCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the maximum number of simultaneous

Multicast Polling Groups the BS allows each SS to belong

to."

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 5 }

wmanIf2BsCapPkmFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxPkmFlowType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent

PKM transactions that BS allows each SS to have."

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 6 }

wmanIf2BsCapMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX WmanIf2MaxNumOfSaType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies maximum number of supported security

associations per SS that the BS allows."

DEFVAL { 1 }

::= { wmanIf2BsBasicCapabilitiesEntry 7 }

wmanIf2BsCapMaxNumOfClassifier OBJECT-TYPE

SYNTAX WmanIf2MaxClassifiers

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the maximum number of admitted

Classifiers per SS that the BS allows."

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 10 }

wmanIf2BsCapTtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the transition speed SSTTG for TDD

and H-FDD SSs allowed by the BS. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsBasicCapabilitiesEntry 14 }

wmanIf2BsCapRtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the transition speed SSRTG for TDD

and H-FDD SSs allowed by the BS. The usage is defined

by WmanIf2SsTransitionGap."

::= { wmanIf2BsBasicCapabilitiesEntry 15 }

wmanIf2BsCapDownlinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shows the number of Downlink transport CIDs the

SS can support."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 16 }

wmanIf2BsCapMaxNumBurstToMs OBJECT-TYPE

SYNTAX WmanIf2CapMaxNumBurstTx

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 19 }

wmanIf2BsCapMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 23 }

wmanIf2BsCapMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 24 }

wmanIf2BsCapNumOfProvisionedSf OBJECT-TYPE

SYNTAX WmanIf2CapMaxNumProvSf

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 25 }

wmanIf2BsCapPnWindowSize OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 29 }

wmanIf2BsSsOfdmCapLoopPwrControlSw OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsBasicCapabilitiesEntry 31 }

wmanIf2BsSsOfdmaCapSdmaPilot OBJECT-TYPE

SYNTAX WmanIf2SdmaPilotCap

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsBasicCapabilitiesEntry 32 }

wmanIf2BsOfdmaCapNoUlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsBasicCapabilitiesEntry 33 }

wmanIf2BsOfdmaCapNoDlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of downlink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsBasicCapabilitiesEntry 34 }

wmanIf2BsCapOptionsGeneral OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsGeneral

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded general BS capability options."

::= { wmanIf2BsBasicCapabilitiesEntry 35 }

wmanIf2BsCapOptionsOfdm OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdm

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDM specific BS capability options."

::= { wmanIf2BsBasicCapabilitiesEntry 36 }

wmanIf2BsCapOptionsOfdma OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdma

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDMA specific BS capability options."

::= { wmanIf2BsBasicCapabilitiesEntry 37 }

wmanIf2BsCapabilitiesConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2BsCapabilitiesConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains the configuration for basic

capabilities of BS. The table is intended to be used to

restrict the Capabilities implemented by BS, for example in

order to comply with local regulatory requirements. The BS

should use the configuration along with the implemented

Capabilities (wmanIf2BsBasicCapabilitiesTable) for

negotiation of basic capabilities with SS using RNG-RSP,

SBC-RSP and REG-RSP messages. The negotiated capabilities

are obtained by interSubclause of SS reported capabilities,

BS raw capabilities and BS configured capabilities. The

objects in the table have read-write access. The rows are

created by BS as a copy of wmanIf2BsBasicCapabilitiesTable

and can be modified by NMS."

::= { wmanIf2BsCm 6 }

wmanIf2BsCapabilitiesConfigEntry OBJECT-TYPE

SYNTAX WmanIf2BsCapabilitiesConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX { ifIndex }

::= { wmanIf2BsCapabilitiesConfigTable 1 }

WmanIf2BsCapabilitiesConfigEntry ::=

SEQUENCE {

wmanIf2BsCapCfgUplinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsCapCfgDsxFlowControl

WmanIf2MaxDsxFlowType,

wmanIf2BsCapCfgMcaFlowControl

WmanIf2MaxMcaFlowType,

wmanIf2BsCapCfgMcpGroupCidSupport

WmanIf2MaxMcpGroupCid,

wmanIf2BsCapCfgPkmFlowControl

WmanIf2MaxPkmFlowType,

wmanIf2BsCapCfgMaxNumOfSupportedSA

WmanIf2MaxNumOfSaType,

wmanIf2BsCapCfgMaxNumOfClassifier

WmanIf2MaxClassifiers,

wmanIf2BsCapCfgTtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsCapCfgRtgTransitionGap

WmanIf2SsTransitionGap,

wmanIf2BsCapCfgDownlinkCidSupport

WmanIf2NumOfCid,

wmanIf2BsCapCfgMaxNumBurstToMs

INTEGER,

wmanIf2BsCapCfgMaxMacLevelDlFrame

WmanIf2MaxMacLevel,

wmanIf2BsCapCfgMaxMacLevelUlFrame

WmanIf2MaxMacLevel,

wmanIf2BsCapCfgNumOfProvisionedSf

Unsigned32,

wmanIf2BsCapCfgPnWindowSize

INTEGER,

wmanIf2BsSsOfdmCapCfgLoopPwrControlSw

Unsigned32,

wmanIf2BsSsOfdmaCapCfgSdmaPilot

WmanIf2SdmaPilotCap,

wmanIf2BsOfdmaCapCfgNoUlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsOfdmaCapCfgNoDlHarqChannel

WmanIf2CapOfdmaNoHarqChan,

wmanIf2BsCapCfgOptionsGeneral

WmanIf2SsCapOptionsGeneral,

wmanIf2BsCapCfgOptionsOfdm

WmanIf2SsCapOptionsOfdm,

wmanIf2BsCapCfgOptionsOfdma

WmanIf2SsCapOptionsOfdma

}

wmanIf2BsCapCfgUplinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shows the configured number of Uplink transport

CIDs the BS can support per SS."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 1 }

wmanIf2BsCapCfgDsxFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxDsxFlowType

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the configured maximum number of

concurrent DSA, DSC, or DSD transactions that BS allows

each SS to have outstanding."

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 3 }

wmanIf2BsCapCfgMcaFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxMcaFlowType

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent

MCA transactions that BS is configured to allow each SS to

have."

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 4 }

wmanIf2BsCapCfgMcpGroupCidSupport OBJECT-TYPE

SYNTAX WmanIf2MaxMcpGroupCid

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object indicates the maximum number of simultaneous

Multicast Polling Groups the BS is configured to allow

each SS to belong to."

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 5 }

wmanIf2BsCapCfgPkmFlowControl OBJECT-TYPE

SYNTAX WmanIf2MaxPkmFlowType

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the maximum number of concurrent

PKM transactions that BS is configured to allow each SS

to have."

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 6 }

wmanIf2BsCapCfgMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX WmanIf2MaxNumOfSaType

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This field specifies configured maximum number of supported

security association per SS."

DEFVAL { 1 }

::= { wmanIf2BsCapabilitiesConfigEntry 7 }

wmanIf2BsCapCfgMaxNumOfClassifier OBJECT-TYPE

SYNTAX WmanIf2MaxClassifiers

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object indicates the configured maximum number of

admitted Classifiers per SS that the BS can support."

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 10 }

wmanIf2BsCapCfgTtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This field indicates the configured transition speed

SSTTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsCapabilitiesConfigEntry 14 }

wmanIf2BsCapCfgRtgTransitionGap OBJECT-TYPE

SYNTAX WmanIf2SsTransitionGap

UNITS "microsecond"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This field indicates the configured transition speed

SSRTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsCapabilitiesConfigEntry 15 }

wmanIf2BsCapCfgDownlinkCidSupport OBJECT-TYPE

SYNTAX WmanIf2NumOfCid

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shows the number of Downlink transport CIDs

the SS can support."

REFERENCE

"Subclause 11.7.6.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 16 }

wmanIf2BsCapCfgMaxNumBurstToMs OBJECT-TYPE

SYNTAX INTEGER (1..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 19 }

wmanIf2BsCapCfgMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 23 }

wmanIf2BsCapCfgMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX WmanIf2MaxMacLevel

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 24 }

wmanIf2BsCapCfgNumOfProvisionedSf OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 25 }

wmanIf2BsCapCfgPnWindowSize OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 29 }

wmanIf2BsSsOfdmCapCfgLoopPwrControlSw OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsCapabilitiesConfigEntry 31 }

wmanIf2BsSsOfdmaCapCfgSdmaPilot OBJECT-TYPE

SYNTAX WmanIf2SdmaPilotCap

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsCapabilitiesConfigEntry 32 }

wmanIf2BsOfdmaCapCfgNoUlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsCapabilitiesConfigEntry 33 }

wmanIf2BsOfdmaCapCfgNoDlHarqChannel OBJECT-TYPE

SYNTAX WmanIf2CapOfdmaNoHarqChan

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This field specifies the number of downlink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsCapabilitiesConfigEntry 34 }

wmanIf2BsCapCfgOptionsGeneral OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsGeneral

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded general capability options cofigured on BS.

These object typically restricts availability of BS supported features."

::= { wmanIf2BsCapabilitiesConfigEntry 35 }

wmanIf2BsCapCfgOptionsOfdm OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdm

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDM specific capability options cofigured on BS.

These object typically restricts availability of BS supported OFDM specific features."

::= { wmanIf2BsCapabilitiesConfigEntry 36 }

wmanIf2BsCapCfgOptionsOfdma OBJECT-TYPE

SYNTAX WmanIf2SsCapOptionsOfdma

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Combined BITS encoded OFDMA specific capability options cofigured on BS.

These object typically restricts availability of BS supported OFDMA specific features."

::= { wmanIf2BsCapabilitiesConfigEntry 37 }