| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAP Overhead Reduction through Efficient Signaling of MCS and Allocation Size |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Shantidev Mohanty, Roshni Srinivasan, Apostolos Papathanassiou, Hassan Yaghoobi Intel Corporation | shantidev.mohanty@intel.com, roshni.m.srinivasan@intel.com, apostolos.papathanassiou@intel.com, hassan.yaghoobi@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Working Group 802.16 Letter Ballot #26a |  |
| Abstract | Proposal to reduce MAP overhead by eliminating the repetitive occurrence of MCS and duration information in consecutive MAP IEs. |  |
| Purpose | To discuss and adopt the proposed specification changes into IEEE 802.16 Rev 2/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MAP Overhead Reduction through Efficient Signaling of MCS and Allocation Size

Shantidev Mohanty, Roshni Srinivasan, Apostolos Papathanassiou, and Hassan Yaghoobi

Intel Corporation

# Introduction

Multiple HARQ subbursts may be allocated in an HARQ region. Information about these allocations is specified in the MAP using Sub-Burst_IEs. When Chase combining is used, the allocated MCS is indicated using the ‘DIUC’ and ‘Repetition Coding Indication’ fields of Sub-Burst IEs. The size of subbursts is indicated using the ‘Duration’ field of the Sub-Burst IEs.

In an IEEE 802.16Rev2/D2 OFDMA frame, consecutive HARQ subbursts may have the same MCS and/or size. For example, HARQ subbursts K, K+1, and K+2 in a DL burst may have the same size L. Currently, the HARQ Sub-Bursts IEs corresponding to each of these subbursts specify L in their ‘Duration’ fields. It can be observed that repetition of the value of the ‘Duration’ field in consecutive sub-burst IEs increases MAP overhead. Similarly when consecutive HARQ subbursts have same the MCS, repetition of the MCS in every Sub-burst IE also increases MAP overhead. This contribution proposes methods to reduce MAP overhead by eliminating the signaling of redundant MCS and duration information.

In the DL Chase HARQ Sub-Burst IE and DL IR HARQ for CC IE, the ‘Sub-Burst DIUC Indicator’ flag is used in IEEE 802.16Rev2/D2 to eliminate the repetitive specification of MCS in two or more consecutive subbursts. This is referred to as MCS optimization in the rest of this document. 802.16Rev2/D2 does not specify any such mechanism to eliminate the repetitive specification of size of two or more consecutive subbursts. The mechanism of eliminating the repetitive specification of size is referred to as duration optimization in the rest of this document.

The repetitive specification of size in the Sub-Burst IEs occurs more frequently in the case of VoIP, when many subbursts in a HARQ region have the same size. It must be noted that MCS optimization can also be applied to data traffic. Additionally, by limiting subburst allocations to a predetermined set of sizes, the BS scheduler can also take advantage of duration optimization for data traffic. The following example illustrates the benefit of MCS and duration optimization for VoIP.

In the example scenario, 115 bidirectional VoIP users using AMR 12.2 Kbps are assumed to be active in an IEEE 802.16Rev2/D2 based system. The speech source model assumes a voice activity factor of 50 %. MPDU size, including the GMH, CRC and VoIP payload with compressed protocol headers for RTP, UDP and IPv4 is 44bytes in the active state and 18 bytes in the inactive state. Asynchronous HARQ with Chase combining is used and 50 ms delay budget that accounts for queuing and HARQ retransmission delay is applied.  Monte Carlo system level simulations are used to evaluate user experience with mixed mobility (60% ITU Ped-B 3km/hr, 30% ITU Veh-A 30km/hr, and 10% ITU Veh-A 120km/hr) and the PUSC permutation for a 10MHz TDD system with a 26:21 symbol DL:UL frame ratio. Frequency reuse of 1 in a 19-cell/3 sector deployment with a 2x2 antenna configuration on the DL with adaptive (STC/SM) MIMO switching and a1x2 antenna configuration on the UL with CSM is assumed.  MAP overhead is dynamically modeled in every frame using compressed MAP and sub-MAPs. For the bidirectional VoIP traffic model considered, UL MAP overhead is also modeled in the DL.

Simulation results for the evaluation of MAP overhead with MCS optimization only, duration optimization only, and both MCS and duration optimization are shown in Figure 1, Figure 2 and Figure 3 respectively. In each figure, dynamic MAP overhead with compressed MAPs and sub-MAPs is evaluated with and without optimization. It must be observed that the gains from MCS optimization and duration optimization are independent of each other. Consequently, Figure 3 shows the additive gain obtained by using both MCS and duration optimization. The results clearly show that the combination of MCS and duration optimization offers the highest benefit with about 20% reduction in MAP overhead.

Figure 1: MAP Overhead Reduction with MCS Optimization

Figure 2: MAP Overhead Reduction with Duration Optimization

Figure 3: MAP Overhead Reduction with MCS and Duration Optimization

# Proposed Solution

For duration optimization, it is proposed that the size of the subbursts is specified only in the first subburst IE when the size of N consecutive HARQ subbursts is same. For the remaining (N-1) subbursts, a 'Duration Indication’ flag is used to indicate that the subburst size is the same as the previous subbursts. Some Sub-Burst IEs in IEEE 802.16Rev2/D2 have a reserved bit. It is proposed that this reserved bit be used as a 'Duration Indication’ flag. It must be noted that for Chase combining, IEEE 802.16Rev2/D2 provides similar mechanism to eliminate the repetitive specification of the MCS by using the 'Sub-Burst DIUC Indicator' flag.

Similarly for HARQ modes in IEEE 802.16Rev2/D2 for which MCS optimization is not applied, it is proposed that when the MCS of N consecutive HARQ subbursts is same, the MCS of the subbursts is specified only for the first subburst. For the remaining (N-1) subbursts, the 'Sub-Burst DIUC Indication' flag is used to indicate that the MCS is the same as the previous subbursts.

In order to minimize changes to the IEEE 802.16Rev2/D2, it is proposed that MCS and duration optimization are applied to the Sub-Burst IEs with unused reserved bit(s). This optimization can also be applied to all scenarios of HARQ operation with new Sub-Burst IE formats.

Table 1 indicates whether or not MCS and Duration optimizations are present in IEEE 802.16Rev2/D2 and in the proposed solution for different HARQ modes for DL. Similarly, Table 2 indicates whether or not MCS and Duration optimizations are present in IEEE 802.16Rev2/D2 and in the proposed solution for different HARQ modes for UL. It may be noted that due to the lack of reserved bit(s), the MCS and duration optimizations can not be applied in certain scenarios. However, these optimizations could be easily incorporated for all HARQ modes in DL and UL by using the new Sub-Burst IE format.

Table 1: MCS and Duration optimization in IEEE 802.16Rev2/D2 and proposed solution for DL.

| HARQ Mode | HARQ Type | MCS Optimization |  | Duration Optimization |  |
| --- | --- | --- | --- | --- | --- |
|  |  | IEEE 802.16Rev2/D2 | Proposed | IEEE 802.16Rev2/D2 | Proposed |
| 0b0000 | DL Chase HARQ | Yes | Yes (no change to 16Rev2/D2) | No | Yes |
| 0b0001 | DL Incremental redundancy HARQ for CTC | No | Yes | No | Yes |
| 0b0010 | DL Incremental redundancy for HARQ for Convolutional Code | Yes | Yes (no change to 16Rev2/D2) | No | Yes |
| 0b0011 | DL MIMO Chase HARQ | No | No | No | No |
| 0b0100 | DL MIMO IR HARQ | No | No | No | No |
| 0b0101 | DL MIMO IR HARQ for Convolutional Code | No | No | No | No |
| 0b0110 | DL MIMO STC HARQ | No | Yes | No | Yes |
| 0b0111-0b1111 | Reserved | NA | NA | NA | NA |

Table 2 MCS and Duration optimization in IEEE 802.16Rev2/D2 and proposed solution for UL.

| HARQ Mode | HARQ Type | MCS Optimization |  | Duration Optimization |  |
| --- | --- | --- | --- | --- | --- |
|  |  | IEEE 802.16Rev2/D2 | Proposed | IEEE 802.16Rev2/D2 | Proposed |
| 0b000 | UL Chase HARQ | No | No | No | Yes |
| 0b001 | UL Incremental redundancy HARQ for CTC | No | Yes | No | Yes |
| 0b010 | UL Incremental redundancy for HARQ for Convolutional Code | No | Yes | No | Yes |
| 0b011 | UL MIMO Chase HARQ | No | No | No | No |
| 0b100 | UL MIMO IR HARQ | No | No | No | No |
| 0b101 | UL MIMO IR HARQ for Convolutional Code | No | No | No | No |
| 0b110* | UL MIMO STC HARQ | No | No | No | No |
| 0b111 | Reserved | NA | NA | NA | NA |

* It is not clear from Table 420 of IEEE 802.16Rev2/D2 whether a reserved bit is available or not. If the reserved bit is available it can be used for Duration optimization.

# Recommended Text Changes

8.4.5.3.21 HARQ DL MAP IE

DL Chase HARQ

Table 347-DL HARQ Chase Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL_HARQ_Chase_Sub-Burst_IE () { | - | - |
| N sub burst | 4 | “Number of sub-bursts” in the 2D rectangular region in this field value plus 1 |
| N ACK channel | 4 | Number of HARQ ACK enabled subbursts in the 2D region |
| For (j=0; j< Number of sub-bursts; j++){ | - | - |
| RCID_IE() | variable | - |
| Duration | 10 | Duration in slots |
| If (j==0) { | - | - |
| Duration | 10 | Duration in slots |
| } | - | - |
| Else if (Duration Indicator of next Sub-Burst for (j-1)== 1) { | - | - |
| Duration | 10 | Duration in slots |
| } | - | - |
| Sub-Burst DIUC Indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, the this subburst will use the same DIUC as the previous If j is 0 then this indicator shall be 1. |
| Reserved Duration Indicator of next Sub-Burst | 1 | Shall be set to zero. If Duration Indicator of next Sub-Burst is 1, it indicates that Duration of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same Duration as this subburst. If j is (Number of sub-bursts-1) then this indicator shall be 0. |
| If (Sub-Burst DIUC Indicator == 1) { | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| Reserved | 2 | Shall be set to zero. |
| } | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.25). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. |
| Dedicated DL Control Indicator | 2 | LSB#0 indicates inclusion of CQI control LSB#1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of Dedicated DL Control Indicator ==1){ | - | - |
| Duration (d) | 4 | A CQI feedback in transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d-1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration != 0b0000){ | - | - |
| Allocation Index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame I specified, the MS should start reporting in eight frames. |
| } | - | - |
| } | - | - |
| If (LSB #1 of Dedicated DL Control Indicator ==1){ | - | - |
| Dedicated DL Control IE () | variable | - |
| } | - | - |
| } | - | - |
| } | - | - |

# Incremental redundancy HARQ for CTC

Table 348-DL HARQ IR CTC Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL_HARQ_IR_CTC_Sub-Burst_IE () { | - | - |
| N sub burst | 4 | “Number of sub-bursts” in the 2D region in this field value plus 1 |
| N ACK channel | 4 | Number of HARQ ACK enabled subbursts in the 2D region |
| For (j=0; j< Number of sub-bursts; j++){ | - | - |
| RCID_IE() | variable | - |
| NEP | -4- | - |
| NSCH | -4- | - |
| If (j==0) { | - | - |
| NEP | 4 | - |
| } | - | - |
| Else if (NEP Indicator of next Sub-Burst for (j-1)== 1) { | - | - |
| NEP | 4 | - |
| } | - | - |
| If (j==0) { | - | - |
| NSCH | 4 | - |
| } | - | - |
| Else if (NSCH Indicator of next Sub-Burst for (j-1)== 1) { | - | - |
| NSCH | 4 | - |
| } | - | - |
| SPID | 2 | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| Reserved NEP Indicator of next Sub-Burst NSCH Indicator of next Sub-Burst | 2 1 1 | Shall be set to zero. If NEP Indicator of next Sub-Burst is 1, it indicates that NEP of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same NEP as this subburst. If j is (Number of sub-bursts-1) then this indicator shall be 0. If NSCH Indicator of next Sub-Burst is 1, it indicates that NSCH of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same NSCH as this subburst. If j is (Number of sub-bursts-1) then this indicator shall be 0. |
| Dedicated DL Control Indicator | 2 | LSB#0 indicates inclusion of CQI control LSB#1 indicates inclusion of Dedicated DL Control IE. |
| If (LSB #0 of Dedicated DL Control Indicator ==1){ | - | - |
| Duration (d) | 4 | A CQI feedback in transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d-1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration != 0b0000){ | - | - |
| Allocation Index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame I specified, the MS should start reporting in eight frames. |
| } | - | - |
| } | - | - |
| If (LSB #1 of Dedicated DL Control Indicator ==1){ | - | - |
| Dedicated DL Control IE () | variable | - |
| } | - | - |
| } | - | - |
| } | - | - |

# DL HARQ IR CC

Table 349-DL HARQ IR CC Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL_HARQ_IR_CC_Sub-Burst_IE () { | - | - |
| N sub burst | 4 | “Number of sub-bursts” in the 2D region in this field value plus 1 |
| N ACK channel | 4 | Number of HARQ ACK enabled subbursts in the 2D region |
| For (j=0; j< Number of sub-bursts; j++){ | - | - |
| RCID_IE() | variable | - |
| Duration | 10 | - |
| If (j==0) { | - | - |
| Duration | 10 | - |
| } | - | - |
| Else if (Duration Indicator of next Sub-Burst for (j-1)== 1) { | - | - |
| Duration | 10 | - |
| } | - | - |
| Sub-Burst DIUC Indicator | 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, the this subburst will use the same DIUC as the previous If j is 0 then this indicator shall be 1. |
| Reserved Duration Indicator of next Sub-Burst | 1 | If Duration Indicator of next Sub-Burst is 1, it indicates that Duration of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same Duration as this subburst. If j is (Number of sub-bursts-1) then this indicator shall be 0. |
| If (Sub-Burst DIUC Indicator == 1) { | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| Reserved | 2 | - |
| } | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| SPID | 2 | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| Dedicated DL Control Indicator | 2 | LSB#0 indicates inclusion of CQI control LSB#1 indicates inclusion of Dedicated DL Control IE. |
| Reserved | 2 | Shall be set to zero. |
| If (LSB #0 of Dedicated DL Control Indicator ==1){ | - | - |
| Duration (d) | 4 | A CQI feedback in transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS for 2(d-1) frames. If d is 0b0000, deallocates all CQI feedback when the current ACID is completed successfully. If d is 0b1111, the MS should report until the BS command for the MS to stop. |
| If (Duration != 0b0000){ | - | - |
| Allocation Index | 6 | Index to the channel in a frame the CQI report should be transmitted by the SS. |
| Period (p) | 3 | A CQI feedback is transmitted on the CQI channels indexed by the (CQI Channel Index) by the SS in every 2p frames. |
| Frame offset | 3 | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame I specified, the MS should start reporting in eight frames. |
| } | - | - |
| } | - | - |
| If (LSB #1 of Dedicated DL Control Indicator ==1){ | - | - |
| Dedicated DL Control IE () | variable | - |
| } | - | - |
| } | - | - |
| } | - | - |

# MIMO DL STC HARQ

Table 353-MIMO DL STC HARQ Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MIMO_DL_STC_HARQ_Sub-Burst_IE () { | - | - |
| N sub burst | 4 | “Number of sub-bursts” in the 2D region in this field value plus 1 |
| N ACK channel | 4 | Number of HARQ ACK enabled subbursts in the 2D region |
| For (j=0; j< Number of sub-bursts; j++){ | - | - |
| Tx count | 2 | 0b00: initial transmission 0b01: odd retransmission 0b10: even retransmission 0b11: Reserved |
| Duration | 10 | - |
| If (j==0) { | - | - |
| Duration | 10 | - |
| } | - | - |
| Else if (Duration Indicator of next Sub-Burst for (j-1)== 1) { | - | - |
| Duration | 10 | - |
| } | - | - |
| Sub-burst offset indication | 1 | Indicates the inclusion of subburst offset. |
| Reserved Sub-Burst DIUC Indicator Sub-Burst Duration Indicator of next Sub-Burst | 3 1 1 1 | If Sub-Burst DIUC Indicator is 1, it indicates that DIUC is explicitly assigned for this subburst. Otherwise, this subburst will use the same DIUC as the previous subburst. If j is 0 then this indicator shall be 1. If Sub-Burst Duration Indicator of next Sub-Burst is 1, it indicates that Duration of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same Duration as this subburst. If j is (Number of sub-bursts-1) then this indicator shall be 0. |
| If (Sub-burst offset indication == 1) { | - | - |
| Sub-burst offset | 8 | Offset in slots with respect to the previous subburst defined in this region. If this is the first subburst within the data region, this offset is with respect to slot 0 of the data region. |
| } | - | - |
| RCID IE () | variable | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the SS in the ACKCH Region (see 8.4.5.4.24). In this case, no ACK channel is allocated for the sub-burst in the ACKCH Region. For the burst, BS shall not perform HARQ retransmission and MS shall ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| If (Tx count == 00) { | - | - |
| Dedicated MIMO DL Control Indicator | 1 | - |
| If (Dedicated MIMO DL Control Indicator == 1){ | - | - |
| Dedicated MIMO DL Control IE () | variable | - |
| } | - | - |
| DIUC | -4- | - |
| Repetition Coding Indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| If (Sub-Burst DIUC Indicator == 1) { | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } | - | - |
| If (ACK Disable == 0) { | - | - |
| ACID | 4 | - |
| } | - | - |
| } | - | - |
| } | - | - |

8.4.5.4.24 HARQ UL-MAP IE

UL HARQ Chase

Table 414-UL HARQ Chase Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HARQ Chase UL Sub-Burst IE () { | - | - |
| RCID IE () | variable | - |
| Dedicated UL Control Indicator | 4 | - |
| If (Dedicated UL Control Indicator==1) | - | - |
| Dedicated UL Control IE () | variable | - |
| } | - | - |
| DIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| Duration | 10 | - |
| If (j==0) { | - | - |
| Duration | 10 | - |
| } | - | - |
| Else if (Duration Indicator of next Sub-Burst for previous subburst== 1) { | - | - |
| Duration | 10 | - |
| } | - | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the BS in the HARQ BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the subburst in the HARQ ACK BITMAP. For the burst, MS shall not perform HARQ retransmission and ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| Reserved Duration Indicator of next Sub-Burst | 1 | If Duration Indicator of next Sub-Burst is 1, it indicates that Duration of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same Duration as this subburst. |
| } | - | - |

UL HARQ IR CTC

Table 415-UL HARQ IR CTC Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HARQ_IR_CTC_UL_Sub-Burst_IE () { | - | - |
| RCID_IE() | variable | - |
| Dedicated UL Control Indicator | 1 | - |
| If (Dedicated UL Control Indicator==1) |  | - |
| Dedicated UL Control IE () | variable | - |
| } | - | - |
| NEP | -4- | - |
| NSCH | -4- | - |
| If first subbursts in HARQ region for the HARQ mode) { | - | - |
| NEP | 4 | - |
| } | - | - |
| Else if (NEP Indicator of next Sub-Burst for previous subburst== 1) { | - | - |
| NEP | 4 | - |
| } | - | - |
| If (first subbursts in HARQ region for the HARQ mode) { | - | - |
| NSCH | 4 | - |
| } | - | - |
| Else if (NSCH Indicator of next Sub-Burst for previous subburst== 1) { | - | - |
| NSCH | 4 | - |
| } | - | - |
| SPID | 2 | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the BS in the HARQ BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the subburst in the HARQ ACK BITMAP. For the burst, MS shall not perform HARQ retransmission and ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| Reserved NEP Indicator of next Sub-Burst NSCH Indicator of next Sub-Burst | 3 1 1 1 | If NEP Indicator of next Sub-Burst is 1, it indicates that NEP of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same NEP as this subburst. If this is the last subburst of a particular HARQ mode then this indication shall be zero. If NSCH Indicator of next Sub-Burst is 1, it indicates that NSCH of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same NSCH as this subburst. If this is the last subburst of a particular HARQ mode then this indication shall be zero. |
| } | - | - |

UL HARQ IR CC

Table 416-UL HARQ IR CC Subburst IE Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HARQ_IR_CC_UL_Sub-Burst_IE () { | - | - |
| RCID_IE() | variable | - |
| Dedicated UL Control Indicator | 1 | - |
| If (Dedicated UL Control Indicator==1) | - | - |
| Dedicated UL Control IE () | variable | - |
| } | - | - |
| UIUC | -4- | - |
| Repetition Coding Information | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| If (first subbursts in HARQ region for the HARQ mode) { | - | - |
| UIUC | 4 | - |
| Repetition Coding Information | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } | - | - |
| Else if (Sub-Burst DIUC Indicator of next Sub-Burst for previous subburst== 1) { | - | - |
| UIUC | 4 | - |
| Repetition Coding Indication | 2 | 0b00: No repetition coding 0b01: Repetition coding of 2 used 0b10: Repetition coding of 4 used 0b11: Repetition coding of 6 used |
| } | - | - |
| Duration | 10 | - |
| If (first subbursts in HARQ region for the HARQ mode) { | - | - |
| Duration | 10 | - |
| } | - | - |
| Else if (Sub-Burst Duration Indicator of next Sub-Burst for previous subburst == 1) { | - | - |
| Duration | 10 | - |
| } | - | - |
| SPID | 2 | - |
| ACID | 4 | - |
| AI_SN | 1 | - |
| ACK disable | 1 | When ACK Disable ==1, the allocated subburst does not require an ACK to be transmitted by the BS in the HARQ BITMAP (see 8.4.5.3.22). In this case, no bit position is allocated for the subburst in the HARQ ACK BITMAP. For the burst, MS shall not perform HARQ retransmission and ignore ACID, AI_SN, and SPID, which shall be set to 0 by the BS if they exist. The CRC shall be appended at the end of each sub-burst regardless of the ACK disable bit. |
| Reserved Sub-Burst UIUC Indicator of next Sub-Burst Sub-Burst Duration Indicator of next Sub-Burst | 3 1 1 1 | If Sub-Burst UIUC Indicator of next Sub-Burst is 1, it indicates that UIUC and Repetition Coding Indication of the next subburst explicitly assigned. Otherwise, the next subburst will use the same UIUC and Repetition Coding Indication as this subburst. If this is the last subburst of a particular HARQ mode then this indication shall be zero. If Sub-Burst Duration Indicator of next Sub-Burst is 1, it indicates that Duration of the next subburst is explicitly assigned. Otherwise, the next subburst will use the same Duration as this subburst. If this is the last subburst of a particular HARQ mode then this indication shall be zero. |
| } | - | - |