| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Indication of UL Interference and Noise Level |  |
| Date Submitted | January 14, 2008 |  |
| Source(s) | JueJun Liu and Zhigang Rong Huawei Technologies Co., Ltd. | E-mail: juejunliu@huawei.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #26a |  |
| Abstract | This contribution describes a way to provide an indication of UL interference and noise level for multiple regions of the same permutation type. |  |
| Purpose | When the BS assigns multiple regions of the same permutation type, the BS can provide different indications of UL interference and noise level for open loop power control. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Indication of UL Interference and Noise Level

JueJun Liu and Zhigang Rong

Huawei Technologies Co., Ltd.

# Explanation

For open-loop power control, an indication of the UL interference and noise level is broadcast to the mobile stations using the UL Interference and Noise Level IE. The NI values indicate the estimated average power level (dBm) per subcarrier in a particular region. In current 16e, the BS may assign multiple regions of the same permutation type. For FDD mode or TDD mode when longer frames are used, the BS tends to assign multiple regions with the same permutation type. For example, when the BS assigns two PUSC regions in one UL subframe, the PUSC region NI indicates the average power level per subcarrier in both PUSC regions. The BS cannot separately indicate the exact noise and interference level for each PUSC region. The same problem exists when the BS assigns multiple AMC, AAS, CQI/ACK/PeriodicRanging, Optional PUSC, Sounding, or MIMO regions. This will affect the open loop power settings of MSs in different PUSC regions.

We propose BS should separately indicate the exact NI for each region, which have same permutation type. For example, When the BS assigns multiple PUSC regions, the PUSC region NI in the UL Noise and Interference Level IE indicates the average power level per subcarrier for the first PUSC region. The NI of first PUSC region is used as base value. The exact interference and noise level of other PUSC region is then defined as follows.    NIexact  =   NIbase + NI_Offset. NI_Offset can be transmitted using the reserved bits in the UL zone IE. Other permutations are similar to PUSC. AAS UL zone IE need also to be modified.

# Proposed Text Changes

8.4.5.4.6 AAS UL IE format

Page 761 Line 60 add NI_Offset description

NI_Offset

The NI_Offset shall be set to the offset value of NI in the defined AAS UL zone. This is a relative value to the NI field of the UL Noise and Interference Level IE with same permutation type. The exact NI is equal to NI + NI_Offset.

Modify table 370 as following

Table 370—OFDMA AAS UL IE format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| AAS_UL_IE { |  |  |
| Extended UIUC | 4 | AAS = 0x02 |
| Length | 4 | Length = 0x03 |
| Permutation | 2 | 0b00: PUSC permutation 0b01: Optional PUSC permutation 0b10: Adjacent subcarrier permutation 0b11: Reserved |
| UL_PermBase | 7 | － |
| OFDMA symbol offset | 8 | － |
| AAS zone length | 8 | Number of OFDMA symbols in AAS zone |
| Uplink_preamble_config | 2 | 0b00: 0 symbols 0b01: 1 symbols 0b10: 2 symbols 0b11: 3 symbols |
| NI_Offset | 4 | 0000：0 dB；0001：+1 dB；0010：+2 dB；0011：+3 dB；0100：+4 dB；0101：+5 dB；0110：+6 dB；0111：+7 dB；1000：－1 dB；1001：－2 dB；1010：－3 dB；1011：－4 dB；1100：－5 dB；1101：－6 dB；1110：－7 dB；1111：－8 dB； |
| Reserved | 1 | Shall be set to zero |

8.4.5.4.7 UL Zone Switch IE format

Page 762 Line 58 add NI_Offset description

NI_Offset

The NI_Offset shall be set to the offset value of NI in the UL zone. This is a relative value to the NI field of the UL Noise and Interference Level IE with same permutation type. The exact NI is equal to NI + NI_Offset.

Modify table 371 as following

Table 371—OFDMA UL Zone IE format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| UL_Zone_IE { |  |  |
| Extended UIUC | 4 | UL_Zone = 0x04 |
| Length | 4 | Length = 0x03 |
| OFDMA symbol offset | 7 | － |
| Permutation | 2 | 0b00: PUSC permutation 0b01: Optional PUSC permutation 0b10: Adjacent subcarrier permutation 0b11: Reserved |
| UL_PermBase | 7 | － |
| AMC type | 2 | Indicates the AMC type in case permutation type = 0b10, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved |
| Use All SC indicator | 1 | 0: Do not use all subchannels 1: Use all subchannels |
| Disable subchannel rotation | 1 | Only applies to PUSC permutation (see section 8.4.6.2.6) 0 = subchannel rotation enabled 1 = subchannel rotation disabled |
| NI_Offset | 4 | 0000：0 dB；0001：+1 dB；0010：+2 dB；0011：+3 dB；0100：+4 dB；0101：+5 dB；0110：+6 dB；0111：+7 dB；1000：－1 dB；1001：－2 dB；1010：－3 dB；1011：－4 dB；1100：－5 dB；1101：－6 dB；1110：－7 dB；1111：－8 dB； |

Backward compatibility

Legacy mobile station will use the existing NI level and will ignore the reserved bits in the UL Zone IE and the AAS UL IE. New mobile stations will interpret the reserved bits in the UL Zone IE and the AAS UL IE as described above, thereby resulting in improved performance with respect to legacy mobile stations.