| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | LBS_ADV TLVs |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Jaewon Lim, Kiseon Ryu LG Electronics Lei Wang, Michael Wengler, Yair Bourlas, Wee Peng Goh, Erik Colban NextWave Broadband* Kamran Etemad Intel | Voice: [Telephone Number (optional)]] E-mail: Jaewon.lim@lge.com ksryu@lge.com lwang@nextwave.com; ybourlas@nextwave.com; wgoh@nextwave.com; mwengler@nextwave.com ecolban@nextwave.com kamran.etemad@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Working Group 802.16 Letter Ballot #26a as announced in IEEE 802.16-07/058 |  |
| Abstract | This document proposes additional fields in LBS_ADV TLVs. |  |
| Purpose | To be discussed and adopted by 802.16 Rev2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

LBS_ADV TLVs

Jaewon Lim, Kiseon Ryu

LG Electronics

Lei Wang, Michael Wengler, Yair Bourlas, Wee Peng Goh, Erik Colban

NextWave Broadband Inc.

Kamran Etemad

Intel

# Problem statement

The LBS_ADV message can be optimized by adopting relative position indication and referencing the MOB_NBR-ADV message

# Suggested Changes in Rev2/D1

In Rev2/D2, modify the text on page 258 line 11 by the following:

6.3.2.3.64 Location Based Services (LBS-ADV) message

A BS may use the LBS-ADV message to broadcast the LBS information. The message may be broadcast periodically without solicitation. This message is sent from the BS to all MSs on a broadcast CID.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| LBS-ADV_Message_Format( ) { |  |  |
| Management message type = 69 |  |  |
| Number_of BS | 8 | Number of BS that are identified using the BSID |
| For (j = 0 ; j < Number_of_BS ; j++) { |  |  |
| Length | 8 | Length of message information within the iteration of Number_of_BS_Index in bytes. |
| BSID | 24 | The least significant 24 bits of the Base Station ID parameter in the DL-MAP message of the Serving BS or Neighbor BS. |
| TLV encoded information | variable | TLV specific |
| } |  |  |
| Number_of_BS_Index | 8 | Number of BS included in this message that are identified using an index to their position in the MOB_NBR-ADV message |
| If (Number_of_BS_Index != 0 ) { |  |  |
| Configuration change count for MOB_NBR-ADV | 8 |  |
| } | — | — |
| For (j = 0 ; j < Number_of_BS_Index ; j++) { |  |  |
| Length | 8 | Length of message information within the iteration of Number_of_BS_Index in bytes. |
| Neighbor_BS_Index | 8 | Index that corresponds to the position of the BS in the MOB_NBR-ADV message |
| TLV encoded information | variable |  |
| } |  |  |
| } |  |  |

The LBS-ADV shall include the following TLV:

Device Base Station Coordinates (see 11.20 11.21.1)

The BS's coordinates.

In Rev2/D2, replace the contents on pages 1192 line 1 through 1193 line 39 by the following, where the new text is marked by blue and underlined; the deleted text is red with strikethrough:

11.20 Device Coordinates

11.21 LBS-ADV message encodings

### 11.21.1 Base Station Coordinates

This compound TLV is used for encoding information used to determine the position of a device BS.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Base Station Coordinates | 45 | variable | Compound | LBS-ADV |

Either the Absolute Position TLV or the Relative Position TLV shall be present in the Base Station Coordinates TLV. The first occurrence of this compound TLV in a message shall contain the Absolute Position TLV, and the position given by this TLV serves as a reference position of all instances of the Relative Position TLV in the same message.

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Absolute Position | 45.1 | 15 | Bits # 0-5: longitude resolution 1-34 - number of valid bits in fixed-point value of longitude value 35 - LBS not supported Others - reserved Bits # 6-14: longitude integer Bits # 15-39: longitude fraction Bits # 40-45: latitude resolution 1-34 - number of valid bits in fixed-point value of latitude value 35 - LBS not supported Others - reserved Bits # 46-54: latitude integer Bits # 55-79: latitude fraction Bits # 80-83: altitude type 1 - meters 2 - floors Others - reserved Bits # 84-89: altitude resolution 1-30 - number of valid bits in fixed-point value of altitude value 31 - LBS not supported Others - reserved Bits # 90-111: altitude integer Bits # 112-119: altitude fraction |
| Relative Position | 45.2 | 6 | Bit #0-15: Relative longitude Bits #0: direction 0 – west(-) 1 – east(+) Bits #1-15 distance difference value (m) Bit #16-31: Relative latitude Bits #16: direction 0 – south(-) 1 – north(+) Bits #17-31 distance difference value (m) Bit #32-47: Relative altitude Bits #32: height 0 – lower (-) 1 – higher (+) Bits #33-47 distance difference value (m) |

The fields indicate the MS / BS location in latitude, longitude, and altitude fields within the Absolute Position TLV indicate the MS/BS location  in latitude, longitude, and altitude that are based on the LCI (Location Configuration Information) format as defined in RFC3825. Latitude and longitude are represented in 34 bits fixed-point 2s-complement number, consisting of 9 bits of integer and 25 bits of fraction. Altitude is represented in 30 bits fixed-point 2s-complement number with 22 bits of integer and 8 bits of fraction. Latitude and longitude should be normalized to within +/- 90 degrees and +/- 180 degrees, respectively. Each field also includes resolution bits that define the number of valid bits in the fixed-point value. Here are the definition of 2s-complement number.

- Positive numbers

  - LatitideLatitude - North

  - Longitude - East

  - Altitude - above ground

- Negative numbers

  - LatitideLatitude– South

  - Longitude - West

  - Altitude - below ground

The Relative Position TLV bits are used to indicate the positions of additional BSs as offsets in meters from the Latitude, Longitude and Altitude values of the first Absolute Position TLV in the message.  Negative values denote positions South, West and below the reference position. The scaling of these numbers allows positions of BS within 32 km from the first BS to be described.