| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Authenticator ID Support |  |
| Date Submitted | 2007-11-14 |  |
| Source(s) | Shahab Sayeedi Joe Schumacher Motorola* | Voice: +1 (847) 632-3877 E-mail: Shahab.sayeedi@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Letter Ballot 26a (802.16 Rev2/D2) |  |
| Abstract | MS is allowed to optionally include network authenticator ID in the RNG-REQ message to enable faster idle mode exit, handover, and location update procedures in the network. This allows network to authenticate MS faster without having to pull the information from the SBS. Network provides MS with authenticator ID in RNG-RSP and DREG-CMD messages similar to PC_ID. |  |
| Purpose | Approve the proposed changes for the Revision 2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Authenticator ID Support

Shahab Sayeedi, Joe Schumacher

Motorola

# Proposed Changes:

6.3.2.3.5 RNG-REQ (ranging request) message

An RNG-REQ shall be transmitted by the SS at initialization and periodically to determine network delay

and to request power and/or DL burst profile change. The format of the RNG-REQ message is shown in

Table 43. The RNG-REQ message may be sent in initial ranging and data grant intervals.

[Change  the following text as shown]

The following TLV parameter shall be included in the RNG-REQ message when the MS is attempting to

perform re-entry, handover, or Location Update:

Ranging Purpose Indication

Presence of item in message indicates MS action as follows:

If Bit #0 is set to 1, in combination with serving BS ID BSID indicates the MS is currently attempting

to HO; or in combination with Paging Controller ID the MS is attempting Network Re-entry

from Idle Mode to the BS. The authenticator may be included in both cases when the Ranging Purpose Indication is included.

If Bit #1 is set to 1, indicates MS action of Idle Mode Location Update Process.

[Add the following text to Section 6.3.2.3.5 as shown]

The following parameter should be included in the RNG-REQ message by the MS if available when it is attempts to perform re-entry, association, or handover:

Authenticator ID (see x.x.x) The Authenticator field provides the network with the location of the authenticator address.

HMAC/CMAC Tuple (see 11.1.2) The HMAC/CMAC Tuple shall be the last attribute in the message.

6.3.2.3.6 RNG-RSP (ranging response) message

[Add the following text to Section 6.3.2.3.6 as shown]

Next Periodic Ranging Indicates the Frame Offset for the next periodic ranging opportunity. This value shall be set to zero to indicate that there has been DL traffic addressed to the MS. The following parameter, necessary to expedite security authentication, shall be included in the RNG-RSP message when the BS notifies the MS through the HO Process Optimization TLV that the PKM-REQ/RSP sequence may be omitted for the current HO reentry attempt, or when the BS wishes to acknowledge a valid HMAC/CMAC Tuple in the acknowledged RNG-REQ management message:

Authenticator ID (see x.x.x) The Authenticator field provides the network with the location of the authenticator address.

HMAC/CMAC Tuple (see 11.1.2) The HMAC/CMAC Tuple shall be the last attribute in the message. The following TLV may be present in RNG-RSP (see 7.8.1, 11.6.1)

6.3.2.3.26 DREG-CMD (de/reregister command) message The DREG-CMD message shall be transmitted by the BS on an SS’s Basic CID to force the SS to change its access state. The BS may transmit the DREG-CMD message unsolicited or in response to a DREG-REQ message. Upon receiving a DREG-CMD message, the SS shall take the action indicated by the action code.

[Add the following text to the end of Section 6.3.2.3.26 as shown]

When the DREG-CMD message is sent with action code 0x05, the following TLVs may be included:

MAC Hash Skip Threshold Maximum number of successive MOB_PAG-ADV messages that may be sent from a BS with-out individual notification for an MS, including MAC Address hash of an MS for which the action code is 00 (No Action Required). If a BS receives the DREG-REQ message containing MAC Hash Skip Threshold TLV, the BS may include MAC Hash Skip Threshold TLV in the DREG-CMD message. If the value is set to 0xFF, a BS shall omit MAC Address hash of the MS with No Action Required for every MOB_PAG-ADV message. If the value is set to zero, BS shall include the MS MAC Address hash in every MOB_PAG-ADV message. The DREG-CMD message may include the following parameters encoded as TLV tuples:

REQ-duration Waiting value for the DREG-REQ message re-transmission (measured in frames) if this is included with action code 0x06 in DREG-CMD. If serving BS includes REQ-duration in a DREG-CMD message including an Action Code = 0x05, the MS may initiate an Idle Mode request through a DREG-REQ with Action Code = 0x01, request for MS De-Registration from serving BS and initiation of MS Idle Mode, at REQ-duration expiration.

The following parameter should be included in the RNG-REQ message by the MS if available when it is attempts to perform re-entry, association, or handover:

[Add the following text to Section 6.3.22.2.7 as shown]

6.3.22.2.7 Network entry/reentry Unless otherwise indicated in this subclause, MS mobile network entry/reentry is processed according to 6.3.9.

An MS and a target BS shall conduct ranging per 6.3.9.5 except when dedicated ranging opportunity is available, in which case, the procedure described in 6.3.23.2.4 shall be employed. For identification of the MS, RNG-REQ message may include MS MAC Address or HO_ID (if assigned in MOB_BSHO-REQ or MOB_BSHO-RSP). The target BS shall assign to the MS Basic CID and Primary CID in the RNG-RSP management message. The MS shall signal the target BS of a current HO attempt by including a serving BSID TLV and Ranging Purpose Indication TLV with Bit #0 set to 1 in the RNG-REQ management message. The MS may also include the authenticator ID field if previously received from the network. The MS shall not include a Ranging Purpose Indication TLV in the RNG-REQ management message unless actually in the process of conducting an HO, location update, or network reentry from idle mode attempt. If an MS RNG-REQ includes a serving BSID and Ranging Purpose Indication TLV with Bit #0 set to 1, and the target BS had not previously received MS information over the backbone network, then the target BS may make an MS information request of the serving BS over the backbone network and the serving BS may respond. Regardless of having received MS information from the serving BS, the target BS may request MS information from another network entity via the backbone network.

<:::>

[Change the following text in Section 6.3.24.8.2.1 as shown]

6.3.24.8.2.1 Secure location update process  If the MS shares a valid security context with the target BS so that the MS may include a valid HMAC/ CMAC Tuple in the RNG-REQ, then the MS shall conduct initial ranging with the target BS by sending a RNG-REQ including Ranging Purpose Indication TLV with Bit #1 set to 1, Location Update Request and Paging Controller ID TLVs (11.1.9.2) Authenticator ID, and HMAC/CMAC Tuple. If the target BS evaluates the HMAC/ CMAC Tuple as valid and can supply a corresponding authenticating HMAC/CMAC Tuple, then the target BS shall reply with a RNG-RSP including the Location Update Response TLV and HMAC/CMAC Tuple completing the location update process. If the paging group has changed, then target BS shall include Paging Group ID TLV in the RNG-RSP. If the target BS responds with a successful Location Update Response = 0x00 (Success of Location Update), the target BS shall notify the paging controller via the backbone network of the MS new location information, the MS shall assume the Paging Group ID of the target BS, and the paging controller may send a message over the backbone network to inform the BS at which the MS entered idle mode that the MS has transitioned to a different Paging Group. If the target BS evaluates the HMAC/CMAC Tuple as invalid, cannot supply a corresponding authenticating HMAC/CMAC Tuple, or otherwise elects to direct the MS to use unsecure location update, then the target BS shall instruct the MS to continue network reentry using the unsecure location update process by inclusion of Location Update Response TLV in RNG-RSP with a value of 0x01 (Failure of Location Update).

[Change the following text in Section 6.3.24.9 as shown]

6.3.24.9 Network reentry from idle mode

For the network reentry from idle mode method, the MS shall initiate network reentry with the target BS by sending a RNG-REQ including Ranging Purpose Indication TLV with Bit #0 set to 1, Paging Controller ID TLVs (11.1.9.2), and Authenticator ID field. If the MS shares a valid security context with the target BS so that the MS may include a valid HMAC/ CMAC Tuple in the RNG-REQ, then the MS shall conduct initial ranging with the target BS by sending a RNG-REQ including HMAC/CMAC Tuple.

If MS RNG-REQ includes an Ranging Purpose Indication TLV with Bit #0 set to 1, Paging Controller ID TLVs and Authenticator ID field, and target BS had not previously received MS information over the backbone network, then target BS may make an MS information request of paging controller over the backbone network and paging controller may respond.

Regardless of having received MS information from paging controller, target BS may request MS information from another network management entity via the backbone network. Network reentry proceeds per 6.3.9.5 except as may be shortened by target BS possession of MS information obtained from paging controller or other network entity over the backbone network.

For the target BS to notify an MS seeking network reentry from idle mode of reentry process management messages that may be omitted during the current reentry attempt due to the availability of MS service and operational context information obtained over the backbone network, the target BS shall place an HO Process Optimization TLV in the RNG-RSP indicating which reentry management messages may be omitted.

The target BS shall not direct the omission of any reentry process management messages that would compromise the security or integrity of normal operation of the communications as established through an unabridged Initial Entry. If the target BS evaluates a HMAC/CMAC Tuple included in the RNG-REQ as valid and can supply a corresponding authenticating HMAC/CMAC Tuple, then the target BS may reply with a RNG-RSP including the valid HMAC/CMAC Tuple. The target BS shall not indicate through the HO Process Optimization TLV that the PKM-REQ/RSP management messages may be omitted in the current reentry attempt without inclusion of a valid HMAC/CMAC Tuple. If an MS detects an invalid HMAC/CMAC Tuple included as part of a RNG-RSP during network reentry from idle mode, the MS shall continue with network reentry but shall process full PKM security rekeying regardless of HO Process Optimization TLV settings.

Regardless of the HO Process Optimization TLV settings, the target BS may elect to use MS service and operational information obtained over the backbone network to build and send unsolicited SBC-RSP and/or REG-RSP management messages to update MS operational information, or to include REG-RSP-specific (11.7) or SBC-RSP-specific (11.8) message items as TLV items in the RNG-RSP.

The target BS may ignore only the first corresponding REQ management message received if it sends an unsolicited SBC-RSP or unsolicited REG-RSP message. MS is not required to send the complimentary REQ management message if it receives an unsolicited SBC-RSP or unsolicited REG-RSP management message prior to MS attempt to send the corresponding REQ management message. Target BS reentry unsolicited response management messages may be grouped into the same DL frame transmission and may be grouped into the same DL frame transmission with the RNG-RSP. However, unsolicited SBC-RSP and unsolicited REG-RSP may not be grouped together into the same DL frame transmission when the PKM-REQ/RSP management message process is required. For a security keying process that has not been determined to be omitted in the HO Process Optimization TLV settings, if MS RNG-REQ includes Ranging Purpose Indication TLV with Bit #0 set to 1 and Paging Controller ID TLVs, and target BS has keying material for the MS, the MS and target BS shall use the RNGRSP including the SA Challenge Tuple TLV to initiate the 3-way handshake reauthorization process as defined in 7.8.1. If MS RNG-REQ includes Ranging Purpose Indication TLV with Bit #0 set to 1 and Paging Controller ID TLVs, and target BS has received a message over the backbone network containing MS information, the target BS may use MS service and operational information obtained over the backbone network to build and send a REG-RSP message or a RNG-RSP message with REG-RSP encodings TLV that includes service flow remapping information in SFID, New_CID, and Connection_Info TLVs. During network reentry, the target BS may notify the MS, through the Bit #7 MS DL data pending element of the HO Process Optimization TLV item in RNG-RSP, of post- network reentry MS DL data pending. Upon MS successful reentry at target BS, now new serving BS, and new serving BS completing reception of any network reentry pending MS DL data retained and forwarded, MS may reestablish IP connectivity and the new serving BS may send a message over the backbone network to request the old serving BS or other network entity to stop forwarding pre-HO pending MS DL data. Network entry/reentry process completes with establishment of normal operations. The target BS shall notify the paging controller via the backbone of MS successful network reentry, and the paging controller may send a message over the backbone network to inform the BS at which the MS entered idle mode that the MS has resumed normal operations at the new serving BS.

11.5 RNG-REQ management message encodings

The encodings in Table 622 are specific to the RNG-REQ message (6.3.2.3.5).

[Add the following row to the end of Table 622 RNG-REQ message as shown]

| Name | Type (1 byte) | Length | Value (variable-length) | Phy Scope |
| --- | --- | --- | --- | --- |
| Authenticator ID | 14 | Variable | This field contains the Authenticator ID used by the network to locate the authenticator function in the network. This field is sent by the MS to the BS when performing handover, network re-entry from idle mode, and location updates. | -- |

11.1.9.x Authenticator ID

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| ? | . | This field contains the Authenticator ID used by the network to locate the authenticator function in the network. | RNG-REQ RNG-RSP DREG-CMD |