| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS UL power consumption reduction |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Itay Lusky Yigal Bitran Altair-Semiconductor | itay@altair-semi.com yigal@altair-semi.com |
|  | Darcy Poulin SiGe | dp@sige.com |
|  | Peter Stewart Aryan Saed PMC Sierra | Peter_Stewart@pmc-sierra.com Aryan_Saed@pmc-sierra.com |
|  | Jeff Zhuang Motorola | Jeff.Zhuang@motorola.com |
|  | Clif Barber CJ Barber Consulting Services | cbarber@cjbarberconsulting.com |
| Re: | IEEE 802.16-07/049: IEEE 802.16 Working Group Working Group Letter Ballot #26 |  |
| Abstract | Current 802.16 specifications exhaust MS battery during UL transmission. A substantial MS UL power consumption saving is introduced. |  |
| Purpose | Adopt the proposed solution and incorporate it in the P802.16Rev2 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MS UL Power Consumption Reduction

Itay Lusky, Yigal Bitran- Altair-Semiconductor;

Darcy Poulin- SiGe;

Peter Stewart, Aryan Saed- PMC Sierra;

Jeff Zhuang- Motorola;

Clif Barber- CJ Barber Consulting Services

# Problem Statement

  - Background

UL power consumption is a major roadblock for widespread adoption of 802.16e in handsets and handheld devices. LTE is expected to challenge 802.16e primarily on UL power consumption. Feasibility of devices with small batteries (<1000mAh) requires significant improvement in the average power consumption of mobile WiMAX

A key component is the power amplifier (PA) which typically consumes 1W or more during transmission (at 23dBm output)

We propose a significant reduction of the average battery consumption.

  - Improving average Tx power consumption

The standard defines “horizontal” (snake-like) allocations in the uplink to maximize allowable transmit power per subchannel for MSs far from the BS. For nearer MSs, the power control mechanism reduces the total transmitted power as needed

As is shown below, for MSs that are not in the cell edge, it is significantly better to keep the total transmit power and reduce the transmission time.

We consider two possible data allocations for UL transmissions: “horizontal” allocations as is defined in current standard and “vertical” as is described in Figure 1 below.

Example: Assume a 10-slot allocation. It can be transmitted the following way:

- Horizontal allocation: 2 subchannels x 15 symbols

- Vertical allocation: 10 subchannels x 3 symbols

The vertical allocation will transmit only 1/5 of the time of the horizontal allocation but with output power 5 times higher (i.e. 7 dB higher).

The MS power consumption however does not scale linearly with transmit power:

- Baseband and RF transceiver's power consumption remain constant

- The PA’s efficiency significantly degrades when reducing output power

- For example total current at 21dBm is 400mA while it is 280mA at 14dBm

Figure 1: “Horizontal” vs. “Vertical” UL transmission definition

Since in “vertical” allocation the UL transmit time is reduced by 5x there is a significant power to be saved. Following the above example:

  - In horizontal allocation MS consumes 280mA for 100% of the UL zone

  - In vertical allocation MS consumes: 400mA for 20% of the UL zone or 80mA on average

 As a result power consumption in “vertical” allocation compared to “horizontal” allocation is 3.5x lower !

  - Suggested UL transmit scheme and backward compatibility

It is suggested to allow BS to schedule “vertical” allocations for power sensitive MS that are not transmitting at maximal power (i.e. at zero headroom) following the next rules:

- Vertical allocations are defined and protected by UIUC 13 (Safety Region referred herein as “protected region”).

- Legacy 802.16e MSs will simply skip those regions since they are allocated using UIUC 13.

- UL power-save allocation IE compliant MSs may use this zone for vertical UL transmissions using a new extended-2 IE. To use the new Extended-2 IE, UL-MAP contains:

- Regular IE’s (for Legacy 802.16e MS allocations)

- One or more UIUC 13 IE’s (safety zones, to create protected regions)

- One or more Extended-2 IE (wrapping all allocations to UIUC=13 safety zones)

The combination of possible UL transmissions is depicted in Figure 2:

Figure 2: UL subframe containing “horizontal” and “vertical” allocations

- In any case there is no change in transmitted spectral density or compliance with power control loop rules.

- It is up to the BS to decide which UL power-save allocation IE compliant MSs are allocated into protected regions based on reported power headroom and other implementation specific BS parameters.

- BS may know if registered MS is power sensitive or not according to proposed new TLV transmitted during SBC-REQ capability exchange phase. This will allow BS to apply the power saving schemes only to MSs that will benefit from it.

# Proposed text changes to the 802.16Rev2 draft

## In page 760 change the following:

Table 368—Extended-2 UIUC code assignment for UIUC = 11

| Extended-2 Type (hexadecimal) |  |
| --- | --- |
| 00 | CQICH Enhanced Allocation IE |
| 01 | HO Anchor Active UL-MAP IE |
| 02 | HO Active Anchor UL-MAP IE |
| 03 | Anchor BS Switch IE |
| 04 | UL Sounding Command IE |
| 05 | Reserved |
| 06 | MIMO UL Enhanced IE |
| 07 | HARQ UL MAP IE |
| 08 | HARQ ACKCH Region Allocation IE |
| 09 | MIMO UL Basic IE |
| 0A | Mini-subchannel allocation IE |
| 0B | UL power-save allocation IE |
| 0B 0C…0D | Reserved |
| 0E | AAS SDMA UL IE |
| 0F | Feedback Polling IE |

## At the beginning of page 828 add the following section, figure and table:

8.4.5.4.29 UL Power Save IE

MS may save UL power consumption by transmitting over larger part of active subcarriers over shorter amount of time. To achieve that BS may set special protection regions defined by UIUC=13 with bit “PAPR Reduction/Safety Zone” set to 1 (safety zone). Bursts are allocated in those protection regions using UL power save allocation IE defined below. In case multiple UIUC=13 safety zones are defined within UL subframe they are identified using Protection region ID variable defined below.

An example of how UIUC=13 safety zone is used within UL subframe to allow power-efficient UL transmission is described in Figure 244

Figure 244: Battery optimized uplink allocation using UIUC=13 (safety zone)

Table 426: UL power save allocation IE format

| Syntax | Size | Note |
| --- | --- | --- |
| UL_Power_Save_Allocation_IE() { |  |  |
| Extended-2 UIUC | 4 | 0x0B |
| Length | 8 | Length in bytes of data field |
| Protection region ID | 4 | Points to one of the protection regions defined in UL-MAP as UIUC=13 safety zone. The ID is at range of 0…15 according to order of UIUC=13 safety zone regions definition in UL-MAP. |
| Reserved | 4 |  |
| while (map data remains){ |  |  |
| UL-MAP_IE() | Variable |  |
| } |  |  |
| } |  |  |

## UL-MAP_IE() defined within the UL_Power_Save_Allocation_IE follows same rules for UL allocations and may include any UL IEs (for example: UL HARQ IE).

## In page 1119 add the following section:

11.8.3.7.21 OFDMA Power Sensitive MS

This field indicates that MS is a power sensitive device that can possibly benefit from using UL Power Save IE defined in 8.4.5.4.29

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Power Sensitive MS | 205 | 1 | 0: MS is not power sensitive device 1: MS is power sensitive device | SBC-REQ, SBC-RSP |