| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CDD Announcement |  |
| Date Submitted | 2008-3-10 |  |
| Source(s) | Frank Zhou, Bert Hochwald, Louay Jalloul, Arogyaswami Paulraj, Beceem Ji-Yun Seol, Samsung Amir Francos, Alvarion Philippe Sehier, Dan Gal, Alcatel-Lucent Sylvain Labonte, Sequans Peretz Shkalim, Runcom Hang Jin, Herbert Ruck, Cisco Systems Weili Ren, Yuefeng Zhou, Nader Zein, Tomohiro Kikuma, NEC | fzhou@beceem.com jiyun.seol@samsung.com Amir.Francos@alvarion.com Philippe.Sehier@alcatel-lucent.fr sylvain@sequans.com peretzs@runcom.co.il hang_jin@navini.com weili.ren@nectech.co.uk |
| Re: | 802.16 Working Group Letter Ballot #26b |  |
| Abstract | We propose a BS to announce its CDD parameters if it uses CDD. |  |
| Purpose | For inclusion in IEEE802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# CDD Announcement

Frank Zhou, Bert Hochwald, Louay Jalloul, Arogyaswami Paulraj, Beceem

Ji-Yun Seol, Samsung

Amir Francos, Alvarion

Philippe Sehier, Dan Gal, Alcatel-Lucent

Sylvain Labonte, Sequans

Peretz Shkalim, Runcom

Hang Jin, Herbert Ruck, Cisco Systems

Weili Ren, Yuefeng Zhou, Nader Zein, Tomohiro Kikuma, NEC

- Motivations

CDD has been proposed and implemented in a number of BS vendors to provide additional frequency diversity and maximize usage of Tx PAs for preamble, MAP, and SIMO zones.  If the MS knows the BS has CDD, it can use this info to improve the modem performance in a number of ways:

  - Enable optimal UL rank 1 Beam Forming for CPE which can significantly enhance UL performance, a bottleneck for WiMax

    - Without the CDD announcement, the CPE can only BF to antenna 0 (the one that carries the preamble); with the CDD announcement, the CPE can perform SVD type BF toward all BS antennas.  Note this rank 1 UL BF is transparent to the BS

  - Symbol timing estimation

    - The signal transmitted from the antenna with delay D has the same symbol timing as the normal signal, but in frequency domain processing (channel estimation, correlation, etc) it would appear to have a delay of D. In order to correctly set the symbol timing without causing ISI and artificial phase roll, the SS needs to know the value of D

      - Without the CDD announcement, 1% CDD delay, for example, could result in about 1 us timing error which will cause 1% ISI and limit the SNR at max 20 dB.  This will cause problem for 64 QAM

      - 1 us timing error can also cause phase roll resulting in interpolation error, particularly for AMC and FUSC when pilot density is low.

  - The MS can learn the number of Tx antenna of a BS from the CDD descriptors (defined in the following page)

    - Number of BS Tx antennas is an important piece of information for the MS.  For example it can be used for setting AGC backoff in BF zones, expecting maximum BF power gain of 10log10(Nant)

  - Help the MS to learn the full MIMO channel estimation, particularly when midamble is not present, e.g., in the 2x2 open loop case.

    - Preamble pilot tones are 3.5 dB hotter than MIMO pilot tones

    - Preamble pilot tones always enjoy frequency reuse 3 therefore lower inference

    - The number of preamble pilot tones is larger than the number of MIMO pilot tones, e.g., for 1024 FFT size, there are 284 preamble pilot tones vs. 120 MIMO pilot tones for PUSC

    - An example is shown in the appendix of C80216maint-08_070

  - Point to point performance will be affected by CDD.  Link adaptation algorithm can be improved if CDD info is known to the MS

  - BAMC selection process can also be improved with the MS knowing that the BS is using CDD

So our proposal is that if a BS uses CDD (in TDD or FDD) which artificially changes the natural channel, the mobiles want to know about it.

- Proposed Text Changes in Rev2/D3

Add the following CDD Descriptor at the end of Table 543 in Section 11.4.1 DCD Channel Encodings at Page 1071

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| CDD Descriptor for Preamble, MAP, and SIMO zones | 155 | variable | Each byte represents one CDD parameter pair: 5 bits for delay in samples (1 to 32) from physical antenna #0, 3 bits for power attenuation (0 to 7 dB). If the Length of this CDD Descriptor is n bytes, then the number of Tx antenna is n+1. | OFDMA |
| CDD Descriptor for MIMO zones | 156 | variable | Each byte represents one CDD parameter pair: 5 bits for delay in samples (1 to 32) from physical antenna #0, 3 bits for power attenuation (0 to 7 dB). If the Length of this CDD Descriptor is n bytes, then the number of Tx antenna is 2(n+1). | OFDMA |

- Conclusion

Here we propose CDD announcement for Base Stations.  If a BS uses CDD which artificially alters the channel a MS sees, the MS would like to know.  The MS can choose to use or not use the CDD Descriptor info in DCD.  If the MS choose not to use the extra info, CDD is transparent to the MS and the channel just appears more frequency selective.

The proposal is strictly backward compatible and provides a number of usages to the MS.   The added overhead is negligible, e.g., one byte for 2 Tx antennas, three bytes for 4 Tx antennas in the 100+ byte DCD message which is transmitted in second time scale.