| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reporting tables in WMAN-IF2F-BS-MIB |  |
| Date Submitted | 2008-01-24 |  |
| Source(s) | Krzysztof Dudzinski Airspan Joey Chou Intel | E-mail: kdudzins@airspan.com E-mail: joey.chou@intel.com |
| Re: | P802.16Rev2/D2 (LB26a) |  |
| Abstract | Document provides specific remedy for comment #1389. |  |
| Purpose | Approve the contribution and apply the changes to subclauses 13.1.5 and 13.2.5 of the standard document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Reporting tables in WMAN-IF2F-BS-MIB

Krzysztof Dudzinski, Airspan

Joey Chou, Intel

The MIB module was created as a part of restructuring of WMAN-IF2-MIB. Some reporting tables for classifiers and PHS rule were lost in this process. Also in the new structure the tables are at the same level in the tree and they need some better naming to make them apart. This contribution proposes the following changes to address these issues. All the changes are for the subclause 13.2.5 (ASN.1 definition) unless specified otherwise.

1. Change the name of the table in subclauses 13.1.5.1 (description) and 13.2.5 (ASN.1 definition)

from wmanIf2fBsProvisionedSfTable

to wmanIf2fBsProvServiceFlowTable

2. Change the name of the table in subclauses 13.1.5.2 (description)  and 13.2.5 (ASN.1 definition)

from wmanIf2fBsServiceClassTable

to wmanIf2fBsProvServiceClassTable

3. Change the name of the table in subclauses 13.1.5.4 (description)  and 13.2.5 (ASN.1 definition)

from wmanIf2fBsClassifierRuleTable

to wmanIf2fBsProvClassifierRuleTable

3.1 Change the prefix of the name of all objects of the table wmanIf2fBsClassifierRuleTable (wmanIf2fBsProvClassifierRuleTable)

from wmanIf2fBsClassifierRule

to wmanIf2fBsProvClsfRule

3.2 Change the name of the last index of the table wmanIf2fBsClassifierRuleTable (wmanIf2fBsProvClassifierRuleTable)

from wmanIf2fBsClassifierRuleIndex

to wmanIf2fBsProvClsfRuleIndex

3.3 Correct table wmanIf2fBsClassifierRuleTable (wmanIf2fBsProvClassifierRuleTable) description accordingly

4. Change the name of the table in subclauses 13.1.5.5 (description)  and 13.2.5 (ASN.1 definition)

from wmanIf2fBsPhsRuleTable

to wmanIf2fBsProvPhsRuleTable

4.1 Change the prefix of the name of all objects in the table wmanIf2fBsPhsRuleEntry (wmanIf2fBsProvPhsRuleEntry)

from wmanIf2fBsPhsRule

to wmanIf2fBsProvPhsRule

4.2 Change the access rights of all the read-only objects in the table wmanIf2fBsPhsRuleEntry (wmanIf2fBsProvPhsRuleEntry)

from read-only

to read-create

4.3 Add row status object to the table  wmanIf2fBsPhsRuleEntry (wmanIf2fBsProvPhsRuleEntry), defined as follows:

wmanIf2fBsProvPhsRuleRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Row status."

::= { wmanIf2fBsProvPhsRuleEntry 6 }

4.4 Correct table wmanIf2fBsPhsRuleEntry (wmanIf2fBsProvPhsRuleEntry) description accordingly

5. Add two new reporting tables to the module wmanIf2fBsMib: wmanIf2fBsClassifierRuleTable and wmanIf2BsPhsRuleTable, defined as follows:

wmanIf2fBsClassifierRuleTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2fBsClassifierRuleEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains packet classifier rules associated

with created service flows."

::= { wmanIf2fBsMib 6 }

wmanIf2fBsClassifierRuleEntry OBJECT-TYPE

SYNTAX WmanIf2fBsClassifierRuleEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each packet classifier

rule."

INDEX { ifIndex, wmanIf2fBsSfMacAddress, wmanIf2fBsSfId, wmanIf2fBsClassifierRuleIndex }

::= { wmanIf2fBsClassifierRuleTable 1 }

WmanIf2fBsClassifierRuleEntry ::=

SEQUENCE {

wmanIf2fBsClassifierRuleIndex

Unsigned32,

wmanIf2fBsClassifierRulePriority

INTEGER,

wmanIf2fBsClassifierRuleIpTosLow

INTEGER,

wmanIf2fBsClassifierRuleIpTosHigh

INTEGER,

wmanIf2fBsClassifierRuleIpTosMask

INTEGER,

wmanIf2fBsClassifierRuleIpProtocol

Integer32,

wmanIf2fBsClassifierRuleIpSourceAddr

InetAddress,

wmanIf2fBsClassifierRuleIpSourceMask

InetAddress,

wmanIf2fBsClassifierRuleIpDestAddr

InetAddress,

wmanIf2fBsClassifierRuleIpDestMask

InetAddress,

wmanIf2fBsClassifierRuleSourcePortStart

Integer32,

wmanIf2fBsClassifierRuleSourcePortEnd

Integer32,

wmanIf2fBsClassifierRuleDestPortStart

Integer32,

wmanIf2fBsClassifierRuleDestPortEnd

Integer32,

wmanIf2fBsClassifierRuleDestMacAddr

MacAddress,

wmanIf2fBsClassifierRuleDestMacMask

MacAddress,

wmanIf2fBsClassifierRuleSourceMacAddr

MacAddress,

wmanIf2fBsClassifierRuleSourceMacMask

MacAddress,

wmanIf2fBsClassifierRuleEnetProtocolTyp

INTEGER,

wmanIf2fBsClassifierRuleEnetProtocol

Integer32,

wmanIf2fBsClassifierRuleUserPriLow

Integer32,

wmanIf2fBsClassifierRuleUserPriHigh

Integer32,

wmanIf2fBsClassifierRuleVlanId

Integer32,

wmanIf2fBsClassifierRulePkts

Counter64,

wmanIf2fBsClassifierRuleIpv6FlowLabel

WmanIf2Ipv6FlowLabel,

wmanIf2fBsClassifierRuleActionRule

WmanIf2ActionRule,

wmanIf2fBsClassifierRuleBitMap

WmanIf2ClassifierBitMap,

wmanIf2fBsClassifierRuleAssociatedPhsi

INTEGER

}

wmanIf2fBsClassifierRuleIndex OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An index is assigned to each classifier in the classifiers

table"

REFERENCE

"Subclause 11.13.19.3.4.14 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 1 }

wmanIf2fBsClassifierRulePriority OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value specifies the order of evaluation of the

classifiers. The higher the value the higher the

priority. The value of 0 is used as default in

provisioned service flows classifiers. The default

value of 64 is used for dynamic service flow classifiers.

If the referenced parameter is not present in a classifier

, this object reports the default value as defined above"

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL { 0 }

::= { wmanIf2fBsClassifierRuleEntry 2 }

wmanIf2fBsClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 3 }

wmanIf2fBsClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values.

If the referenced parameter is not present in a classifier

, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 4 }

wmanIf2fBsClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The mask value is bitwise ANDed with TOS byte in an IP

packet and this value is used for the range checking of

TosLow and TosHigh. If the referenced parameter is not

present in a classifier, this object reports the value

of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 5 }

wmanIf2fBsClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX Integer32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 6 }

wmanIf2fBsClassifierRuleIpSourceAddr OBJECT-TYPE

SYNTAX InetAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2CmnClassifierRuleIpSourceMask value

equals the wmanIf2CmnClassifierRuleIpSourceAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 7 }

wmanIf2fBsClassifierRuleIpSourceMask OBJECT-TYPE

SYNTAX InetAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2CmnClassifierRuleIpSourceMask value equals the

wmanIf2CmnClassifierRuleIpSourceAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 8 }

wmanIf2fBsClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX InetAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2CmnClassifierRuleIpDestMask value equals the

wmanIf2CmnClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 9 }

wmanIf2fBsClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX InetAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2CmnClassifierRuleIpDestMask value equals the

wmanIf2CmnClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 10 }

wmanIf2fBsClassifierRuleSourcePortStart OBJECT-TYPE

SYNTAX Integer32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared

. This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 11 }

wmanIf2fBsClassifierRuleSourcePortEnd OBJECT-TYPE

SYNTAX Integer32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 12 }

wmanIf2fBsClassifierRuleDestPortStart OBJECT-TYPE

SYNTAX Integer32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 13 }

wmanIf2fBsClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX Integer32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of

65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 14 }

wmanIf2fBsClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2CmnClassifierRuleDestMacMask equals the value of

wmanIf2CmnClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 15 }

wmanIf2fBsClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2CmnClassifierRuleDestMacMask equals the value of

wmanIf2CmnClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 16 }

wmanIf2fBsClassifierRuleSourceMacAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"An Ethernet packet matches this entry when its source

MAC address  bitwise ANDed with

wmanIf2CmnClassifierRuleSourceMacMask equals the value

of wmanIf2CmnClassifierRuleSourceMacAddr. If the

referenced parameter is not present in a classifier,

this object reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 17 }

wmanIf2fBsClassifierRuleSourceMacMask OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2CmnClassifierRuleSourceMacMask equals the value of

wmanIf2CmnClassifierRuleSourceMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 18 }

wmanIf2fBsClassifierRuleEnetProtocolTyp OBJECT-TYPE

SYNTAX INTEGER

{

none(0),

ethertype(1),

dsap(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 19 }

wmanIf2fBsClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX Integer32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If wmanIf2CmnClassifierRuleEnetProtocolTyp is none(0),

this object is ignored when considering whether a packet

matches the current rule.

If wmanIf2CmnClassifierRuleEnetProtocolTyp is ethertype(1)

,this object gives the 16-bit value of the EtherType that

the packet must match in order to match the rule.

If wmanIf2CmnClassifierRuleEnetProtocolTyp is dsap(2), the

lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 20 }

wmanIf2fBsClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX Integer32 (0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2CmnClassifierRulePriLow and

wmanIf2CmnClassifierRulePriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 21 }

wmanIf2fBsClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX Integer32 (0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2CmnClassifierRulePriLow

and wmanIf2CmnClassifierRulePriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 22 }

wmanIf2fBsClassifierRuleVlanId OBJECT-TYPE

SYNTAX Integer32 (0..4095)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 23 }

wmanIf2fBsClassifierRulePkts OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object counts the number of packets that have

been classified using this entry."

::= { wmanIf2fBsClassifierRuleEntry 25 }

wmanIf2fBsClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX WmanIf2Ipv6FlowLabel

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2fBsClassifierRuleEntry 26 }

wmanIf2fBsClassifierRuleActionRule OBJECT-TYPE

SYNTAX WmanIf2ActionRule

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule. If this classification action rule

exists, its action shall be applied on the packets that

match this classifier rule."

::= { wmanIf2fBsClassifierRuleEntry 27 }

wmanIf2fBsClassifierRuleBitMap OBJECT-TYPE

SYNTAX WmanIf2ClassifierBitMap

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2fBsClassifierRuleEntry 28 }

wmanIf2fBsClassifierRuleAssociatedPhsi OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Associated PHSI has a value between 1 and 255, which

shall mirror the PHSI value of a PHS rule. Packets matching

the Packet Classification Rule containing the Associated

PHSI parameter shall undergo PHS according to the

corresponding PHS rule. A value '0' indicates that no PHS

rule is associated with this classifier rule."

REFERENCE

"Subclause 11.13.19.3.4.13 in IEEE Std 802.16-2004"

::= { wmanIf2fBsClassifierRuleEntry 29 }

wmanIf2BsPhsRuleTable OBJECT-TYPE

SYNTAX SEQUENCE OF WmanIf2BsPhsRuleEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2fBsMib 7 }

wmanIf2BsPhsRuleEntry OBJECT-TYPE

SYNTAX WmanIf2BsPhsRuleEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX { ifIndex, wmanIf2fBsSfMacAddress, wmanIf2fBsSfCid, wmanIf2fBsPhsRulePhsIndex }

::= { wmanIf2BsPhsRuleTable 1 }

WmanIf2BsPhsRuleEntry ::=

SEQUENCE {

wmanIf2fBsPhsRulePhsIndex

INTEGER,

wmanIf2fBsPhsRulePhsField

OCTET STRING,

wmanIf2fBsPhsRulePhsMask

OCTET STRING,

wmanIf2fBsPhsRulePhsSize

Integer32,

wmanIf2fBsPhsRulePhsVerify

WmanIf2PhsRuleVerify

}

wmanIf2fBsPhsRulePhsIndex OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsPhsRuleEntry 1 }

wmanIf2fBsPhsRulePhsField OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (0..65535))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsPhsRuleEntry 2 }

wmanIf2fBsPhsRulePhsMask OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (0..65535))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2CmnPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsPhsRuleEntry 3 }

wmanIf2fBsPhsRulePhsSize OBJECT-TYPE

SYNTAX Integer32 (0..255)

UNITS "byte"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL { 0 }

::= { wmanIf2BsPhsRuleEntry 4 }

wmanIf2fBsPhsRulePhsVerify OBJECT-TYPE

SYNTAX WmanIf2PhsRuleVerify

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL { phsVerifyEnable }

::= { wmanIf2BsPhsRuleEntry 5 }

5.1 Add subclause 13.1.5.6 as follows:

13.1.5.6 wmanIf2fBsClassifierRuleTable

wmanIf2fBsClassifierRuleTable contains the properties of classification rules of created service flows.

5.2 Add subclause 13.1.5.7 as follows:

13.1.5.7 wmanIf2BsPhsRuleTable

wmanIf2BsPhsRuleTable contains the properties of PHS rules of created service flows.