| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for efficient H-FDD support in OFDMA physical layer |  |
| Date Submitted | January 23, 2008 |  |
| Source(s) | Roberto Albanese, Andrea Bacioccola and Zexian Li Nokia Aik Chindapol and Giovanni Maggi NSN | E-mail: roberto.albanese@nokia.com andrea.bacioccola@nokia.com zexian.li@nokia.com aik.chindapol@nsn.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16Rev2/D2, Letter Ballot 26a. Technical Comments |  |
| Abstract | This document proposes a mechanism for efficient support of H-FDD MSs, specifically related to OFDMA physical layer |  |
| Purpose | Accept the proposed specification changes on IEEE P802.16Rev2/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for efficient H-FDD support in OFDMA physical layer

Roberto Albanese, Andrea Bacioccola and Zexian Li

Nokia

Aik Chindapol and Giovanni Maggi

NSN

# Introduction

In an H-FDD system, a half duplex MS cannot transmit and receive at the same time.. Due to inherent handling of resources in IEEE 802.16 OFDMA frame (UL transmissions are based on a time-first allocation), it is convenient for the BS to separate the DL and UL allocations of H-FDD MSs into zones within the same frame. For this reason, we define a strategy for efficient support of H-FDD MS, where MSs are divided into groups and allowed to use specific zones in each frame.

This contribution proposes a set of minimal changes to the IEEE 802.16Rev2/D2 [1] OFDMA PHY specifications to support efficient H-FDD operations in OFDMA physical layer..

An H-FDD MS shall listen to the MAP messages and to the zones to which it belongs. Regarding UL sub-frame, an H-FDD shall transmit only into those zones to which it belongs. The BS is responsible for defining both UL and DL zone to guarantee that the requirements on SSTTG and SSRTG are satisfied for all the H-FDD MSs.

# Proposed text changes

In order to support the aforementioned proposal, the following changes are needed to the standard.

Change 0: Intro text for Grouping

[Add the following paragraph after the second paragraph in section 8.4.4.2]

During any DL transmission, an H-FDD MS is entitled to listen to the MAP message and to those zones allocated to its group. The H-FDD MS should not listen to DL or transmit in UL zones which are not associated to its half duplex group, except during network entry.

[Change Table 327 page 375 as indicated as follows]

Table 327—OFDMA STC DL Zone IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| STC_DL_Zone_IE() { |  |  |
| Extended DIUC | 4 | STC/DL_Zone_SWITCH |
| Length | 4 | Length = 0x04 |
| OFDMA symbol offset | 8 | Denotes the start of the zone (counting from the frame preamble and starting from 0) |
| Permutation | 2 | 0b00: PUSC permutation 0b01: FUSC permutation 0b10: Optional FUSC permutation 0b11: Adjacent subcarrier permutation |
| Use All SC indicator | 1 | 0: Do not use all subchannels 1: Use all subchannels |
| STC | 2 | 0b00: No STC 0b01: STC using 2/3 antennas 0b10: STC using 4 antennas 0b11: FHDC using 2 antennas |
| Matrix Indicator | 2 | STC matrix (see 8.4.8.1.4) if (STC == 0b01 or STC == 0b10) { 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Reserved } else if (STC == 0b11) { 0b00 = Matrix A 0b01 = Matrix B 0b10–11 = Reserved } |
| DL_PermBase | 5 |  |
| PRBS_ID | 2 | Values: 0..2. Refer to 8.4.9.4.1 |
| AMC type | 2 | Indicates the AMC type in case permutation type = 0b11, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved Note that only 2x3 band AMC subchannel type (AMC Type = 0b01) is supported by MS |
| Midamble presence | 2 | 0: Not present 1: MIMO midamble present at the first symbol in STC Zone |
| Midamble boosting | 2 | 0: No boost 1: Boosting (3 dB) |
| Dedicated Pilots |  | 0: STC using 2 antennas 1: STC using 3 antennas Selects 2/3 antennas when STC = 0b01 |
| Dedicated Pilots |  | 0: Pilot symbols are broadcast 1: Pilot symbols are dedicated. An MS should use only pilots specific to its burst for channel estimation |
| H-FDD Group Indication | 1 | 1: The zone is intended for H-FDD Group 1 0: The zone is intended for H-FDD Group 2 |
| Reserved | 4 3 | Shall be set to zero |
| } | - | - |

[Change Table 371 page 763 as indicated as follows]

Table 371—OFDMA UL Zone IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL_Zone_IE() { |  |  |
| Extended UIUC | 4 | UL_Zone_SWITCH |
| Length | 4 | Length = 0x04 |
| OFDMA symbol offset | 7 | - |
| Permutation | 2 | 0b00: PUSC permutation 0b01: Optional PUSC permutation 0b10: Adjacent subcarrier permutation 0b11: Reserved |
| UL_PermBase | 7 | - |
| AMC Type | 2 | Indicates the AMC type in case permutation type = 0b10, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved |
| Use All SC indicator | 1 | 0: Do not use all subchannels 1: Use all subchannels |
| Disable subchannel rotation | 1 | Only applies to PUSC permutation (see section 8.4.6.2.6) 0 = subchannel rotation enabled 1 = subchannel rotation disabled |
| H-FDD Group Indication | 1 | 1: The zone is intended for H-FDD Group 1 0: The zone is intended for H-FDD Group 2 |
| Reserved | 4 3 | Shall be set to zero |

# References

- IEEE Std 802.16Rev2-D2 (Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007), “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems,” Dec 13, 2007