| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover Optimization: Reduction in HO interruption latency for IEEE 802.16Rev2 |  |
| Date Submitted | 2008-01-16 |  |
| Source(s) | Kiran Thakare, Sten Sjoberg Ericsson AB SE-164 80 Stockholm, Sweden | Voice: +46 8 58532591 E-mail: kiran.thakare@ericsson.com |
| Re: | IEEE 802.16 Rev2 |  |
| Abstract | Within the framework, IEEE standard 802.16 Rev2 we propose a handover optimization method for 802.16 Rev2 |  |
| Purpose | To discuss and include the proposal in 802.16 Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reduction in HO interruption latency for IEEE 802.16 Rev2

Kiran Thakare, Sten Sjoberg

Ericsson AB Sweden

# Introduction

There are ongoing efforts in 802.16 Rev2 maintenance group to improve the overall handover performance.  This contribution outlines a technique to improve the handover interruption latency.  A considerable, around 15-25%, latency improvement is achieved using this technique.

We propose that the proposed method below be included in IEEE 802.16e Rev 2.

# HO Background in existing IEEE 802.16e & subsequent revisions

Current Existing Handover (HO) techniques, in Mobile WiMax technology involves couple of steps in handover procedure i.e. neighbor BS scanning, handover decision step and handover execution step.  Figure 1 shows the existing, optimized signaling flow for the MS initiated hard handover, intra FA, non-collocated, fully optimized network reentry procedure that uses RNG_REQ/RES signaling message [1].  As shown in the Figure 1, the HO involves three nodes MS, Serving BS and Target BS. The figure below shows the handover procedures and processing time estimated in frames.

HO interruption time is a time delay from when MS transmits the MOB_HO_IND message to its serving BS and then does not transmit or receive the traffic data anymore to when the MS starts to exchange the traffic data with its target BS as shown in Figure 1.

T-BS/s

MS

S-BS

(1) CQI Code for MOB_MSHO_REQ

(2) Allocation for BR

(3) BR for MOB-MSHO_REQ

(4) Allocation for MOB_MSHO_REQ

(5) MOB_MSHO_REQ

(6) HO_REQ

(7) HO_RES

(8) MOB BSHO RSP/Allocation for MOB-HO-IND

(11) Allocation of RNG_REQ

(9) MOB_HO_IND

(10) HO_Cnf

(12) RNG_REQ

(13) RNG_RSP

(14) Allocation for CQICH

(15) CQI code by CQICH

Data Transfer

2 frame

1 frame

1 frame

1 frame

4 frame

1 frame

1 frame

1 frame

2 frame

2 frame

1 frame

1 frame

1  frame

2 frame

3 frame

1 frame

2 frame

Preparation

time

Interruption

time

Figure 1 MS initiated hard handover, intra FA, non-collocated Handover Scenario

In the scenario of figure 1, the handover interruption latency of the Reference WirelessMAN OFDMA system is shown in terms of frame numbers [1].   Interruption time is 9 frames (45ms) in the above example.  Please note that 4 frames elapse during the attempt to read the ‘allocation for RNG-REQ’ message.   We aim to reduce the this time.  This is due to synchronization with T-BS, then waiting time for DL/UL MAP and then decoding of DL/UL MAP and acquiring the fast ranging IE included in MAP.

# Proposed method for HO interruption time Reduction

Figure 2 shows the MSC for the proposed handover method.  This method minimizes the handover interruption time up to 2 to 5frames (around 25% improvement) as shown in figure 2.

This Proposal shifts one of the signaling flow from the handover step 2 (interruption time) to the step 1 (preparation time).  Moreover, this message, “Allocation for RNG_REQ” is piggybacked over backbone between BS-S and BS-T.  This message is further piggybacked by the BS-S to the MS in the message MOB_BSHO_RSP/Allocation for MOB_HO_IND as an IE.  This results in considerable reduction of HO interruption time as well as processing time at MS.  This in tern results in a lesser buffer requirement at MS during HO interruption phase.

The message from BS-T to MS: “Allocation for RNG_REQ” is shifted from interruption step (fig1) to HO preparation step as shown in figure 2.  This means that the BS-T allocates the exact UL slot (code, frame number etc, for MS in advance.  The UL slot IE carries information such as code, sub-channel, frame number etc to the MS.  BS-T allocates k =2 consecutive frame numbers for MS in which it can make a dedicated request.  Value of k could be 1 to 2 frames.  The UL Slot IE is, then piggybacked by BS-Ts to BS-S in HO_RSP message.  BS-S then piggybacks this IE in MOB_HO_RES message as shown in figure 2.  This way, with exact allocation the MS avoids having to search & decode and process the allocation IE in UL MAP, sent by target BS, as shown in Figure 1.

MS having received the UL slot IE, constructs the ranging request message based on this IE, selects the BS for handover, makes sure it has synchronization with selected target BS, by checking the last DL/UL MAP counter that it has acquired earlier during scanning phase and sends the MOB_HO_IND message to the serving BS.  MS then immediately sends the dedicated ranging request to target BS in a given frame number using the dedicated radio resources.

The procedure is explained as shown in MSC-2 and MSC-3.

- The Target BSs allocates the ‘UL slot IE’ to the MS with all the required information (code, frame number etc).  This is complete UL slot IE information that a MS needs in order to make a dedicated ranging request.

- The target BSs piggybacks the “UL Slot IE” IE with the HO_Res messages that is intended for the serving BS over the backbone interface.

- The Serving BS, upon reception of this message from all the relevant target BSs, piggybacks these IEs to MS with the MS_BSHO_RSP message, over the air interface. MS_BSHO_RSP message may contain more than one UL Slot IEs allocated by the number of Target BSs.

- The MS, upon reception of ‘UL slot’ IEs, selects the target BS and makes sure it has synchronizes with BS-T, by checking the lost DL/UL MAP counter value acquired earlier during scanning phase and constructs the message.  MS then terminates the connection with BS-S in MOB_HO_IND message and immediately sends the RNG_REQ message in the given frame number (UL Slot IE) to selected BS-T in the UL slot acquired earlier.

- MS may send MOB_HO_IND message to S-BS and RNG_REQ message to T-BS in any order as follows.  MS may send the RNG_REQ message to T-BS just prior to sending the MOB_HO_IND.

T-BS/s

MS

S-BS

(1) CQI Code for MOB-MSHO-REQ

(2) Allocation for BR

(3) BR for MOB-MSHO_REQ

(4) Allocation for MOB_MSHO_REQ

(5) MOB-MSHO-REQ

(6) HO-REQ

(7) HO-RES

(8) MOB BSHO RSP/Allocation for MOB-HO-IND

(9) MOB-HO-IND

(10) HO-Cnf

(11) RNG-REQ

(12) RNG-RSP

(13) Allocation for CQICH

(14) CQI code by CQICH

Data Transfer

2 frame

1 frame

1 frame

1 frame

1 frame

1 frame

1 frame

2 frame

2 frame

1 frame

1 frame

1  frame

2 frame

1 frame

2 frame

Preparation

time

Interruption time

Allocate exact UL slot with code, frame number etc

Piggyback UL slot IE with code, frame number etc for RNG-REQ

Piggyback UL slot IE with code, frame number etc for RNG-REQ for one or more BS-T

Extract UL slot info from UL Slot IE, Select BS-T

Check the last DL/UL MAP counters. Use the dedicated UL Slot info for making RNG-REQ

Figure 2: MSC-1, showing the ‘UL slot allocation’ of T-BS in HO preparation phase.

T-BS/s

MS

S-BS

(1) CQI Code for MOB-MSHO-REQ

(2) Allocation for BR

(3) BR for MOB-MSHO_REQ

(4) Allocation for MOB_MSHO_REQ

(5) MOB-MSHO-REQ

(6) HO-REQ

(7) HO-RES

(8) MOB BSHO RSP/Allocation for MOB-HO-IND

(9) MOB-HO-IND

(10) HO-Cnf

(11) RNG-REQ

(12) RNG-RSP

(13) Allocation for CQICH

(14) CQI code by CQICH

Data Transfer

2 frame

1 frame

1 frame

1 frame

1 frame

1 frame

1 frame

2 frame

2 frame

1 frame

1 frame

2 frame

1 frame

2 frame

Preparation

time

Interruption time

Allocate exact UL slot with code, frame number etc

Piggyback UL slot IE with code, frame number etc for RNG-REQ

Piggyback UL slot IE with code, frame number etc for RNG-REQ for one or more BS-T

Extract UL slot info from UL Slot IE, Select BS-Ts

Check the last DL/UL MAP counters. Use the dedicated UL Slot info for making RNG-REQ

Figure 3: MSC-2, showing the change of order of MOB-HO-IND message to S-BS and RNG-REQ to T-BS

# 3.4 Proposal

We propose that the following proposed text be included in the IEEE802.16 Rev2 Draft 3.

Addition 1: Addition to the Section 6.3.22.2 HO Process (page 429)

Add following at the end of the ‘ranging subsection’ (page 430, line 16)

--MS may receive the dedicated UL ranging parameters of the target BSs in the form of UL Slot IE piggybacked by the serving BS in the MOB_BSHO-RSP message or MOB_BSHO-REQ message.  Upon decoding the non-contentious uplink radio resources sent in UL slot IE, MS shall check that it has the synchronization with the target BS by checking the Lost DL MAP interval (600ms).  If this interval has not elapsed then MS without having to decode the DL MAP, DCD message of target BS shall construct the RNG-REQ message for the target BS and send it, in a given frame number using the given non-contentious uplink radio resources received in UL slot UE.  MS may send the RNG-REQ to target BS and MOB-HO-IND message to Serving BS in any order.  That is MS may initiate the RNG-REQ to target BS prior to sending the MOB-HO-IND message to serving BS.

Addition 2: Addition to the Section 6.3.22.2.2 HO Decision & Initiation (page 433)

Add following at the end of the ‘line 12’ (page 433, line 12)

--When MOB_BSHO-REQ is sent by a BS, the BS may indicate one or more possible target BSs. It may also indicate the dedicated UL resources (UL Slot IE) for ranging request for one or more possible target BSs.  SS check if this information is included in the message by checking the piggyback indicator.  If this indicator is enabled the MS decodes the UL slots IE.  Upon decoding the non-contentious uplink radio resources sent in UL slot IE, MS shall check that it has the synchronization with the target BS by checking the Lost DL MAP interval (600ms).  If this interval has not elapsed then MS without having to decode the DL MAP, DCD message of target BS, shall construct the RNG-REQ message for the target BS and send it, in a given frame number using the given non-contentious uplink radio resources.  MS may send the RNG-REQ to target BS and MOB-HO-IND message to Serving BS in any order.  That is MS may initiate the RNG-REQ to target BS prior to sending the MOB-HO-IND message to serving BS.

Addition 3: Section 6.3.2.3.49 MOB_BSHO-RSP message and Section 6.3.2.3.47 MOB_BSHO-REQ message format

A piggy back indicator bit is used to indicate whether each recommended neighbor BS includes the UL slot IE information or not.

If this bit is set, UL Slot IE field should be included as a parameter of the BS.

[Include following parameters in line 49 on page 227 for MOB_BSHO-RSP and line 23 on page 214 for MOB_BSHO-REQ ]

| Piggyback Indicator | 1 | To indicate whether piggyback information is included or not 0: Not included; 1 : Included |
| --- | --- | --- |

[Include following parameters in line 15 on page 228 for MOB_BSHO-RSP and line 46 on page 214  for MOB_BSHO-REQ]

| UL Slot IE () { |  |  |
| --- | --- | --- |
| Frame Number Index n+1 | 4 | Identifies the frame index in which UE shall make UL access. LSB of the relevant frame number. Two consecutive frames |
| Frame Number Index n+2 | 4 | Identifies the frame index in which UE shall make UL access. LSB of the relevant frame number. Two consecutive frames |
| UIUC | 4 | UIUC =12 Burst type to be used for UL transmission |
| Ranging Code | 8 | To be used by UE during UL access |
| Ranging subchannel | 7 | To be used by UE during UL access |
| UL Initial Transmit Timing | 8 | Start of UL subframe |
| Symbol offset | 2 | To be used by UE. Symbol offset from the start of UL sub frame |
| CID field in MAC header = Initial Ranging CID =0000 | 16 | CID field to be used the RNG-REQ message |
| HO ID } | 8 | HO ID allocated by target BS to identify the UE instead of MAC address. Used during initial ranging to the selected target BS |

# References

- ITU-R document R03-WP8F-C-1199MSW-E[1]

- WiMAX Forum Network Architecture – Stage 3 – Detailed Protocols and Procedures - Release 1.1.0

- NWG_R1.1.0-Stage-3-Annex-R6-R8-ASN-Anchored-Mobility-Scenarios

- IEEE C802.16maint-08/085

- IEEE802.16 Rev2 Draft 3