| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | C80216maint-08_086 (Handoffs between 5 and 10 MHz Channel Bandwidths¦) |  |
| Date Submitted | 17-March 2008 |  |
| Source(s) | Subir Varma John Humbert Chris Seagren Sprint 6220 Sprint Parkway Overland Park, KS USA Phil Barber – Huawei | Voice: 816-210-9611 Fax: +1(913) 794-0420 John.J.Humbert@sprint.com |
| Re: | Letter Ballot 26 |  |
| Abstract |  |  |
| Purpose | Clarifications needed to support 5 and 10 MHz. Handoffs |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chair@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Modifications to the IEEE 802.16 Rev2/D3 Specification to Accommodate 10 MHz to 5 MHz Handovers

Subir Varma, Phil Barber, John Humbert

Sprint, Huawei, Sprint

The following modifications need to be made to the IEEE802.15Rev2/D3 Specification in order to accommodate 10 MHz to 5 MHz (and vice-versa) handovers. All new text is marked in blue and underlined:

- Remove the definition of the PHY Profile ID field in Section 6.3.2.3.42, Table 142 since it is not correct. The correct definition is given on page 195, to which an additional line should be added: Change the text from line #1 to 10 in page 195 to following;

PHY Profile ID The PHY Profile ID contains a subset of the MOB_NBR-ADV message fields.  is the aggregate ID’s including the Co-located FA Indicator bit, the FA Configuration indicator bit, Time/Frequency Synchronization Indicator, BS EIRP Indicator, DCD/UCD Reference Indicator, FA Index Indicator, and the FA (Frequency Assignment) number. For systems using OFDM or OFDMA, the bit-by-bit definition of the PHY Profile is shown in Table 143. If the Co-located FA Indicator bit is set, the following field of the NBR-ADV element including Preamble Index, HO Process Optimization, DCD/UCD Configuration Change Count, and TLV Encoded Neighbor Information may be omitted.

Change the text in Table 143 in page 195 to following;

Table 143—Bit-by-bit definition of PHY Profile ID of the BS

| Item | Size (bit) | Notes |
| --- | --- | --- |
| Co-located FA Indicator | 1 | If this bit is set, the following fields of the MOB_NBR-ADV TLV may be omitted: Preamble Index, HO Process Optimization, DCD/UCD Configuration Change Count, and TLV Encoded Neighbor Information. |

- Add the definition of the following TLV field, in the MOB_NBR-ADV message. This definition should occur after the UCD_settings TLV definition (Section 6.3.2.3.42, page 197). Also change the definition of the Neighbor BS Trigger entry in Table 576 as per the definition given below.

Neighbor BS Trigger
The Neighbor BS trigger is a compound TLV value that indicates defines the complete set of triggers that the MS shall apply being applied to this neighbor BS for initiating actions 0x1 (Respond to trigger with MOB_SCN-REP) and 0x2 (Respond to trigger with MOB_MSHO-REQ) (see Tables 5776 and 578). The BS shall include the Neighbor BS trigger TLV set shall be included in the MOB_NBR-ADV message only if it is defines a set different from the trigger set for actions 0x1 and 0x2 that is defined in the serving BS’s DCD message or from the Neighbor BS trigger set for actions 0x1 and 0x2 that the MS shall apply to the preceding neighbor BS, whichever is referenced by the Trigger Reference Indicator in the PHY Profile ID.

- Change the Value field definition in the Type/Function/Action TLV in Table 577 (Section 11.18.1), to the following:

See Table 577578 for a description

- Clarify the preamble index field for the OFDMA PHY case, in Table 142, page 192, line 54.  Make the following changes in the notes box of the parameter “Preamble Index / Subchannel Index”

For the OFDMA PHY this parameter defines the PHY specific preamble. For the OFDM PHY, the 5 LSB contain the active DL subchannel index and the 3 MSB shall be Reserved and set to ‘0b000’. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measure-ment for handoff. A value of '0' indicates a reuse fac-tor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit #6 to bit #0 represent the preamble index of the neighbor BS.

- Add clarification about the use of DCD/UCD reference indicator in PHY profile ID. The following changes are proposed in Rev2/D3:

  - On page 197, make the following change in the text in line 1 to 13.

DCD_settings

The DCD_settings is a compound TLV that encapsulates TLVs from the neighbor BS’ DCD message that may be transmitted in the advertised BS downlink channel. This information is intended to enable fast synchronization of the MS with the advertised BS downlink. The DCD settings fields shall contain only neighbor’s DCD TLV values that are different from the serving BS corresponding values or from the DCD_settings of the previous neighbor BS, whichever is referenced by the DCD/UCD Reference Indicator in the PHY Profile ID. Neighbor BS TLVs that are already represented within the fixed fields of the MOB_NBR-ADV message (e.g., BS_EIRP, DCD configuration change count, neighbor BSID) shall be excluded. For values that are not included, the MS shall assume they are identical to the corresponding values of the serving BS. The duplicate TLV encoding parameters within a Neighbor BS shall not be included in DCD setting.

  - On page 197, make the following change in the text in line 19 to 30.

UCD_settings

The UCD_settings is a compound TLV that encapsulates TLVs from the neighbor BS’ UCD message that may be transmitted in the advertised BS downlink channel. This information is intended to enable fast synchronization of the MS with the advertised BS uplink. The UCD set-tings fields shall contain only neighbor’s UCD TLV values that are different from the serving BS’s corresponding values or from the UCD_settings of the previous neighbor BS, whichever is referenced by the DCD/UCD Reference Indicator in the PHY Profile ID. Neighbor BS UCD TLVs that are already represented within the fixed fields of the MOB_NBR-ADV message (e.g., UCD configuration change count) shall be excluded. For values that are not included, the MS shall assume they are identical to the serving BS’s corresponding values. The duplicate TLV encoding within a Neighbor BS shall not be included in UCD setting.

  - On page 1188, Table 576, make the following changes.

Table 576—MOB_NBR-ADV encodings

| Name | Type (1 byte) | Length (1 byte) | Notes |
| --- | --- | --- | --- |
| UCD_settings | 2 | variable | The UCD_settings is a compound TLV that encapsulates TLVs from the neighbor BS’ UCD message that may be transmitted in the advertised BS downlink channel. This information is intended to enable fast synchronization of the MS with the advertised BS uplink. The UCD settings fields shall contain only neighbor’s UCD TLV values that are different from the serving BS’s corresponding values or from the UCD_settings of the previous neighbor BS, whichever is referenced by the DCD/UCD Reference Indicator in the PHY Profile ID. Neighbor BS UCD TLVs that are already represented within the fixed fields of he MOB_NBR-ADV message (e.g., UCD configuration change count) shall be excluded. For values that are not included, the MS shall assume they are identical to the serving BS’s corresponding values. |
| Neighbor BS trigger | 4 | variable | The Neighbor BS trigger is a compound TLV value that indicates the trigger being applied to this neighbor BS (Table 545). The Neighbor BS trigger set is included in MOB_NBR-ADV message, only if it is different from trigger set that is defined in the serving BS’s DCD message or from the Neighbor BS trigger set for the preceding neighbor BS, whichever is referenced by the Trigger Reference Indicator in the PHY Profile ID. |