| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definitions for matrix A in 2x3 AMC structure |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Yuval Lomnitz Dov Andelman Intel Corp. | Voice: +972-3-9205773 E-mail: Yuval.lomnitz@intel.com dov.andelman@intel.com |
| Re: | Letter Ballot 26b |  |
| Abstract | Completing definitions of AMC for MIMO zone |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Definitions for matrix A in 2x3 AMC structure

Yuval Lomnitz, Dov Andelman - Intel Corp.

# Purpose

Contribution C802.16maint-08/007r3 clarifies and extends AMC+MIMO support for various slot structures. Conditioned on adoption of C802.16maint-08/007r3, the support for matrix A encoding requires further clarification since the matrix A encoding done over adjacent symbols needs to be done on symbols belonging to different slots.

Note: the contribution C802.16maint-08/007r2 covering different topics related to the AMC+MIMO zone was split into the following contributions in order to enable separate discussion/decision on each item, and this contribution is part of this pack:

- C802.16maint-08/007r3 (Definitions for AMC permutation slot structure in MIMO zone): covers definitions of AMC slot structures in the map IE-s and STC mapping (DL and UL), except matrix A support for 2x3 slot structure

- C802.16maint-08/087 (Definitions for matrix A in 2x3 AMC structure): adds the definition of matrix A support (both DL and UL)

- C802.16maint-08/088 – Modified pilot splitting pattern for AMC MIMO: both UL and DL.

- C80216maint-08/089 – Generalization of allocation granularity definitions: this contribution generalizes the allocation granularity definitions that were defined for PUSC+MIMO+dedicated pilots to AMC.

- C80216maint-08/090 - Correction to MIMO Pilot and Data Power

Text Changes

8.4.8.3.1.2.1 STC Mapping for optional AMC permutation

[Add the following text at the end of the second paratraph (starting with “For 2-antenna matrix A”)

The mapping of matrix A for the 3x2 slot is the same as the mapping for PUSC in Table 537. For the 2x3 slot, the data is first mapped frequency-first to each 2x3 slot, then matrix A encoding is performed over each pair of symbols. The symbol pairs for matrix A encoding numbered from the beginning of the STC zone are 2n, 2n+1 (n³0). Symbols 6n+2 and 6n+3 belong to different slots, starting at symbols 6n and 6n+3 of the STC zone respectively (n³0). The BS shall guarantee that the two slots belong to the same burst or sub-burst therefore matrix A bursts and sub-bursts shall be 6n symbols in time (n>0) allocated slots starting at a 6n symbol boundary. Figure YYY illustrates encoding of 192 QAM symbols into over 2 subchannels and 6 symbols for Matrix A with AMC 2x3 slot.

[add a new figure after figure 276]

Figure 276a—Example of 2TX matrix A mapping for AMC 2x3 for a burst of 2 subchannels by 6 symbols